| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0057 |
| --- | --- |

| Title: | CE6 subset 5.2.2 and 6.2.2: Intra coding improvements |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongbing Lin Lingzhi Liu Jianhua Zheng Haitao Yang Haoping Yu | Tel: Email: | +86 10 60612142 yblin@huawei.com |
| Source: | HiSilicon Technologies, Huawei Technologies |  |  |

_____________________________

# Abstract

This contribution reports test results of CE6 subset 5.2.2 and 6.2.2. In the proposed algorithms, the total remaining Intra mode number (i.e. exclude 2 MPMs) becomes a power of 2 for all PU sizes, and thus the remaining mode index can be binarized with FLC. Simulation results reveal that subset5.2.2 improves the coding efficiency by 0.1% in AI-HE and AI-LC, whereas subset6.2.2 leads to performance loss of 0.01% for AI-HE and AI-LC. Based on the test results, it is recommended to adoption of subset5.2.2 due to its simplified Intra mode coding, improved coding efficiency, and coherent design of Intra prediction directions.

# Introduction

This contribution reports test results on CE6 subset 5.2.2 and 6.2.2.

# Algorithm description

In the proposed algorithms, the two existing diagonal intra prediction modes (HOR+8 and VER+8) are merged into one. Meanwhile HOR+6 is added as a supplement mode for 4x4 PU. As a result, the total remaining Intra mode number (i.e. exclude 2 MPMs) becomes a power of 2 for all PU sizes, and thus the remaining mode index can be binarized with FLC.

Two variants described in CE6 are tested [1].

1) CE6 subset 5.2.2 proposes to replace the two existing diagonal intra prediction modes (HOR+8 and VER+8) with one bi-prediction mode which averages the two diagonal prediction values.

2) CE6 subset 6.2.2 proposes to remove Intra prediction mode HOR+8 for all PUs.

As for the proposed bi-prediction Intra mode in subset5.2.2, it is designed to average the main and side reference samples before performing angular Intra prediction. Only the following change to angular Intra prediction is made. It is noted the proposed bi-prediction mode utilizes DST for 4x4 block residual in both vertical and horizontal directions.

8.3.3.1.6 Specification of Intra_Angular prediction mode

…

The reference pixel array refMain[ x ], with x=-nS..2*nS is specified as follows.

–	If intraPredOrder is equal to 17,

refMain[ x ] = ( p[ -1+x, -1 ] + p[ -1, -1+x ] )>>1,  with x=0..2*nS	(8-1)

–	Otherwise, if intraPredOrder is less than 18,

refMain[ x ] = p[ -1+x, -1 ], with x=0..nS		(8-2)

- If intraPredAngle is less than 0,

refMain[ x ] = p[ -1, -1+( ( x*invAngle+128 )>>8 ) ], with x=( nS*intraPredAngle ) >>5..-1	(8-3)

- Otherwise,

refMain[ x ] = p[ -1+x, -1 ], with x=nS+1..2*nS		(8-4)

Otherwise,

…

# Test Settings and Conditions

The simulations of this document have used HM5.0 software. The common test conditions and reference configurations specified in [2] are followed. The following simulation platform and compiler are used.

| Simulation platform | Compiler |
| --- | --- |
| Intel Xeon X7560 @2.27GHz, 256G RAM Windows Server 2003, 64 bits | Microsoft VS2005 |

# Experimental results

The experimental results are summarized in Table 1 and Table 2. The CE6 subset5.2.2 improves the coding efficiency by 0.1% in AI-HE and AI-LC, whereas subset6.2.2 leads to minor coding efficiency loss by 0.01%. It is also observed the two subsets lead to 0.1-0.2% loss for class F sequences.

Table 1. Experimental results for subset 5.2.2 (with bi-prediction intra mode)

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.12% | 0.09% | 0.10% | -0.15% | -0.01% | -0.09% |
| Class B | -0.05% | 0.02% | 0.04% | -0.09% | -0.04% | -0.07% |
| Class C | -0.09% | -0.05% | -0.03% | -0.12% | -0.09% | -0.10% |
| Class D | -0.07% | -0.02% | 0.01% | -0.10% | -0.06% | -0.11% |
| Class E | -0.04% | 0.16% | 0.16% | -0.07% | 0.06% | -0.03% |
| Overall | -0.07% | 0.03% | 0.05% | -0.10% | -0.04% | -0.08% |
|  | -0.07% | 0.03% | 0.05% | -0.10% | -0.03% | -0.09% |
| Class F | 0.16% | 0.15% | 0.23% | 0.14% | 0.05% | 0.01% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

Table 2. Experimental results for subset 6.2.2 (removal of intra prediction mode HOR+8)

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.02% | 0.01% | 0.03% | -0.02% | 0.03% | 0.00% |
| Class B | 0.01% | 0.02% | 0.04% | 0.01% | 0.02% | 0.03% |
| Class C | 0.02% | 0.00% | 0.03% | 0.01% | 0.02% | -0.02% |
| Class D | 0.02% | 0.09% | 0.04% | 0.03% | 0.04% | -0.01% |
| Class E | 0.01% | 0.03% | 0.12% | 0.01% | 0.04% | 0.04% |
| Overall | 0.01% | 0.03% | 0.05% | 0.01% | 0.03% | 0.01% |
|  | 0.01% | 0.03% | 0.05% | 0.01% | 0.03% | 0.01% |
| Class F | 0.12% | 0.11% | 0.05% | 0.15% | 0.11% | 0.10% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusion and recommendation

The BD performance of subset 5.2.2 and 6.2.2 are reported. The two subsets can achieve simplified FL binarization of the remaining intra mode coding. Compared with subset6.2.2, subset5.2.2 becomes a more coherent design of intra prediction directions for all PU sizes. Based on the experimental results we recommend to adoption of subset5.2.2 due to its simplified Intra mode coding, improved coding efficiency, and coherent design of Intra prediction directions.

# Acknowledge

We would like to express our thanks to SONY, LG and I2R for the cross-check.

# References

- Ali Tabatabai, Edouard Francois, Keiichi Chono, “CE6: Intra Coding Improvements,” JCT-VC Document, JCTVC-G1206, Geneva, CH, Nov. 2011

- F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-G1200, Geneva, CH, Nov. 2011

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “WD4: Working Draft 5 of High-Efficiency Video Coding,” JCT-VC Document, JCTVC-G1103, Geneva, CH, Nov. 2011

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Proposed changes to WD5 text

8.3.1 Derivation process for luma intra prediction mode

Inputs to this process are:

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

–	a variable log2PUSize specifying the size of the current prediction unit,

–	variable arrays IntraPredMode (If available) that are previously (in decoding order) derived for adjacent coding units.

Output of this process is the variable IntraPredMode[ xB ][ yB ].

Table 8-1 specifies the value for the intra prediction mode and the associated names.

Table 8-1 – Specification of intra prediction mode and associated names

| Intra prediction mode | Associated names |
| --- | --- |
| 0 | Intra_Planar |
| 1 | Intra_Vertical |
| 2 | Intra_Horizontal |
| 3 | Intra_DC |
| Otherwise (4..33) | Intra_Angular |
| 34 | Intra_FromLuma (used only for chroma) |

Table 8-2 specifies the number of luma intra prediction modes intraPredModeNum depending on log2PUSize.

Table 8-2 – Specification of intraPredModeNum

| log2PUSize | intraPredModeNum |
| --- | --- |
| 2 | 18 |
| 3 | 34 |
| 4 | 34 |
| 5 | 34 |
| 6 | 34 |

Error: Reference source not found specifies the mapping table used for converting the number of intra prediction modes.

IntraPredMode[ xB ][ yB ] labelled 0, 1, 2, .., 34 represents directions of predictions as illustrated in .

4

18

10

19

5

20

11

21

1

22

12

23

6

24

25

13

7

0:

Intra_Planar

3:

Intra_DC

34:

Intra_FromLuma

26

14

27

8

28

15

29

2

30

16

31

9

32

17

33

7

Figure 8-1 – Intra prediction mode directions (informative)

…

8.3.3.1.6 Specification of Intra_Angular prediction mode

…

The reference pixel array refMain[ x ], with x=-nS..2*nS is specified as follows.

–	If intraPredOrder is equal to 17,

refMain[ x ] = ( p[ -1+x, -1 ] + p[ -1, -1+x ] )>>1,  with x=2..2*nS	(8-5)

–	Otherwise, If intraPredOrder is less than 18,

refMain[ x ] = p[ -1+x, -1 ], with x=0..nS		(8-6)

- If intraPredAngle is less than 0,

refMain[ x ] = p[ -1, -1+( ( x*invAngle+128 )>>8 ) ], with x=( nS*intraPredAngle ) >>5..-1	(8-7)

- Otherwise,

refMain[ x ] = p[ -1+x, -1 ], with x=nS+1..2*nS		(8-8)

Otherwise,

refMain[ x ] = p[ -1, -1+x ], with x=0..nS		(8-9)

- If intraPredAngle is less than 0,

refMain[ x ] = p[ -1+( ( x*invAngle+128 )>>8 ), -1 ], with x=( nS*intraPredAngle ) >>5..-1	(8-10)

- Otherwise,

refMain[ x ] = p[ -1, -1+x ], with x=nS+1..2*nS		(8-11)

The values of the prediction samples predSamples[ x, y ], with x, y = 0..nS-1 are derived by the following procedures.

…

9.2.2.8  Binarization process for rem_intra_luma_pred_mode

Input to this process is a request for the syntax element rem_intra_luma_pred_mode and cNumBins.

Output of this process is the binarization of the syntax element.

The binarization for rem_intra_luma_pred_mode is given by Table 9-3.

Table 9-3 – Binarization for rem_intra_luma_pred_mode

| Value of rem_intra_luma_pred_mode | Bin string |
| --- | --- |
| less than 32 | FL, cMax = cNumBins |