| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0058 |
| --- | --- |

| Title: | Cross-verification of CE6 subset 6.2.1: On intra prediction mode coding (JCTVC-H0075) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yongbing Lin Lingzhi Liu | Tel: Email: | +86 10 60612142 yblin@huawei.com |
| Source: | HiSilicon Technologies |  |  |

_____________________________

# Abstract

This contribution reports cross-check results of I2R’s proposal on intra prediction mode coding (JCTVC-H0075). The remaining intra mode coding is modified by using a binary+FLC binarization, allowing for 19 prediction modes with 4x4 PUs and 35 prediction modes with 8x8 and larger PUs. The rest results show all bit-rate and PSNR values match exactly with those reported by proponent, and runtime ratios are very similar.

# Introduction

This document reports cross-check results of I2R’s proposal on the remaining intra mode coding. As described in CE6 subtest6.2.1, the remaining intra mode coding is modified by using a binary+FLC binarization, allowing for 19 prediction modes with 4x4 PUs and 35 prediction modes with 8x8 and larger PUs.

We got the source code and independently compiled the code with VS2005, and ran in environment of Windows server 2003 R2 64bits. The common test conditions and software reference configurations [1] are followed in our simulation.

# Experimental results

There are two variants for the CE6 subtest6.2.1. Variant 1 adds a context for coding the binary flag indicating whether the first remaining mode is used, and variant 2 codes that flag in bypass mode.

The experimental results are summarized in Table 1 and Table 2, respectively. It is observed variant 1 improves coding efficiency by 0.2% in AI-HE and AI-LC, and variant 2 achieves performance gain by 0.04% and 0.00% in AI-HE and AI-LC. It is confirmed that all bit-rate and PSNR values match exactly with those reported by proponent, and encoding/decoding run time has similar character.

Table 1. Experimental results for variant 1 (using context for the flag indicating the first remain mode)

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.29% | -0.08% | -0.03% | -0.32% | -0.11% | -0.05% |
| Class B | -0.20% | -0.11% | -0.07% | -0.22% | -0.08% | -0.02% |
| Class C | -0.19% | -0.15% | -0.13% | -0.19% | -0.11% | -0.11% |
| Class D | -0.16% | -0.03% | -0.12% | -0.15% | -0.11% | -0.12% |
| Class E | -0.29% | -0.08% | -0.03% | -0.32% | -0.08% | -0.01% |
| Overall | -0.22% | -0.09% | -0.08% | -0.23% | -0.10% | -0.06% |
|  | -0.21% | -0.09% | -0.07% | -0.22% | -0.10% | -0.07% |
| Class F | -0.18% | -0.11% | -0.18% | -0.15% | -0.16% | -0.20% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 2. Experimental results for variant 2 (bypass mode for the flag indicating the first remain mode)

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.16% | 0.01% | 0.08% | -0.15% | -0.05% | 0.06% |
| Class B | -0.10% | -0.08% | -0.02% | -0.08% | -0.07% | 0.02% |
| Class C | 0.05% | -0.07% | -0.01% | 0.13% | -0.03% | -0.03% |
| Class D | 0.05% | 0.07% | 0.01% | 0.13% | -0.06% | -0.03% |
| Class E | -0.10% | 0.11% | 0.16% | -0.11% | 0.06% | 0.14% |
| Overall | -0.04% | 0.00% | 0.03% | 0.00% | -0.03% | 0.02% |
|  | -0.04% | 0.00% | 0.04% | 0.00% | -0.03% | 0.02% |
| Class F | 0.06% | -0.22% | 0.01% | 0.14% | -0.03% | -0.07% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Recommendation

The test results reported in JCTVC-H0075 has been verified. Due to the improved performance, it is recommended to review the variant 1.

# References

- F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-G1200, Geneva, CH, Nov. 2011