| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0059 |
| --- | --- |

| Title: | Cross-verification of JVC's proposal on Intra mode reduction (JCTVC-H0081) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yongbing Lin | Tel: Email: | +86 10 60612142 yblin@huawei.com |
| Source: | HiSilicon Technologies |  |  |

_____________________________

# Abstract

This contribution reports cross-check results of JVC’s proposal on Intra mode reduction (JCTVC-H0081).

For 8x8 and larger PUs, total number of intra modes is reduced from 35 to 34, so intra mode coding can be simplified. The rest results show all bit-rate and PSNR values match exactly with those reported by proponent, and runtime ratios are very similar.

# Introduction

This document reports cross-check results of JVC’s proposal on Intra mode reduction. Simplification of intra mode coding is proposed by removing one intra mode (i.e. HOR+7) from 35 to 34 for 8x8 and larger PUs. As a result, the remaining intra mode index can be binarized with FLC.

We got the source code and independently compiled the code with VS2005, and ran in environment of Windows server 2003 R2 64bits. The common test conditions and software reference configurations [1] are followed in our simulation.

# Experimental results

The experimental results are summarized in Table 1. It is observed the proposal has minor impact on BD performance. All bit-rate and PSNR values match exactly with those reported by proponent, and encoding/decoding run time has similar character.

Table 1. Experimental results for Intra mode reduction (removal of HOR+7)

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.07% | 0.08% | 0.14% | 0.08% | 0.08% | 0.10% |
| Class B | 0.02% | 0.01% | 0.02% | 0.02% | 0.02% | 0.03% |
| Class C | 0.03% | -0.01% | 0.02% | 0.02% | 0.02% | 0.00% |
| Class D | 0.01% | 0.07% | 0.03% | 0.02% | 0.03% | 0.01% |
| Class E | 0.01% | 0.02% | -0.01% | 0.01% | 0.04% | 0.05% |
| Overall | 0.02% | 0.03% | 0.03% | 0.03% | 0.03% | 0.03% |
|  | 0.02% | 0.03% | 0.04% | 0.03% | 0.03% | 0.02% |
| Class F | 0.02% | 0.08% | 0.00% | 0.02% | 0.00% | 0.02% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Recommendation

The test results reported in JCTVC-H0081 has been verified. Due to the minimum change to WD and minor performance impact, it is recommended to review the proposal.

# References

- F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-G1200, Geneva, CH, Nov. 2011