| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, USA, 1-10 February, 2012 | Document: JCTVC-H0064 |
| --- | --- |

| Title: | CE5.c: Mode-dependent significance scans for transform skip |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Mrak, Andrea Gabriellini BBC R&D, Centre House, 56 Wood Lane W12 7SB London, UK |  | marta.mrak@bbc.co.uk andrea.gabriellini@bbc.co.uk |
| Source: | British Broadcasting Corporation |  |  |

_____________________________

# Abstract

This contribution describes test CE5.c and reports its results. Core Experiment 5 is described in [JCTVC-G1205]. Test CE5.c evaluates the performance of mode-dependent significance scans applied to transform skip modes (TSM) 1 and 2. Results are compared to HM 5.0 and HM 5.0 with TSM. Overall this test shows that mode-dependent significance scans do not affect the complexity of TSM nor improve its compression performance, as both BD-rate and running time results are almost identical to HM 5.0 with TSM.

# Introduction

This contribution describes test CE5.c on mode-dependent significance scans for TSM and reports its result. The test is part of the work of Core Experiment 5, described in [JCTVC-G1205].

# Test description

TSM was proposed in [JCTVC-G575] and was the subject of the work of AdHoc Group 19 ([JCTVC-G019]). This technique allows to skip one or both uni-dimensional transforms for inter-predicted blocks. Four transform skip modes are defined, TSM0 to TSM3. TSM0 is the standard bi-directional Discrete Cosine Transform (DCT) used in HEVC. TSM1 skips the vertical DCT transform, thus only the horizontal DCT is performed. TSM2 skips the horizontal DCT, thus only the vertical DCT is performed. Finally, TSM3 skips both vertical and horizontal DCT, thus the following encoding stage, quantisation, is applied to the inter prediction residual values directly.

If we consider the spatial distribution of the transformed coefficients for TSM1 and TSM2 we can observe that the DC values generated by the uni-dimensional DCT are grouped on the first column and row, respectively. Therefore it may prove more effective to apply a vertical and horizontal scan, respectively, when coding the significance map for blocks processed with TSM1 and TSM2.

Vertical and horizontal significance scans are only applied to blocks of size 4x4 and 8x8, and only when the transform skip mode is 1 or 2. For all other blocks sizes or transform skip modes the standard diagonal scan is used instead.

# Experimental results

The proposed technique is implemented on top of the software developed for CE5.a, which is built on top of HM 5.0. The simulation is applied to the mandatory test points defined in [JCTVC-G1200]. Since the technique is only applied to inter predicted blocks, no intra-only test points were considered.

The simulation was run on a cluster of 38 machines running a 64-bit Linux operating system. The performance of the 38 machines is homogenous but each processor can increase its clock when not all its four cores are being used, thus some small variability in the running times results is expected.

Two sets of results are presented here: a comparison of the proposed technique versus HM 5.0, to show the overall performance reached by TSM, and a comparison with HM 5.0 with TSM, showing the contribution of mode-dependent significance scans to the performance of TSM.

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -0.1% | -0.3% | 0.0% | -0.2% | -0.2% | 0.0% | -0.2% | -0.1% |
| Class B | -0.3% | 0.1% | 0.1% | -0.7% | -0.1% | 0.1% | -0.2% | 0.0% | 0.1% |
| Class C | -0.5% | -0.4% | -0.1% | -1.0% | -0.3% | 0.0% |  |  |  |
| Class D | -0.8% | -0.3% | -0.1% | -1.5% | 0.2% | 0.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.4% | -0.2% | -0.1% | -0.9% | -0.1% | 0.1% | -0.1% | -0.1% | 0.0% |
|  | -0.4% | -0.2% | -0.1% | -0.9% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% |
| Class F | -2.0% | -1.1% | -1.0% | -2.8% | -1.6% | -1.7% |  |  |  |
| Enc Time[%] | 130% |  |  | 119% |  |  | 129% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  | 101% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | 0.6% | 0.6% | -1.0% | -0.4% | 0.3% |
| Class C | -0.7% | -0.2% | 0.1% | -1.5% | -0.1% | 0.6% |
| Class D | -1.2% | 0.5% | 0.8% | -2.2% | 1.4% | 1.3% |
| Class E | -0.2% | 0.6% | 0.3% | -1.2% | 1.3% | 1.3% |
| Overall | -0.6% | 0.4% | 0.5% | -1.5% | 0.4% | 0.8% |
|  | -0.6% | 0.3% | 0.4% | -1.4% | 0.3% | 0.7% |
| Class F | -4.5% | -2.6% | -2.9% | -6.3% | -4.4% | -4.2% |
| Enc Time[%] | 125% |  |  | 115% |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  |

Table 1 Comparison of HM 5.0 TSM with mode-dependent significance scans and HM 5.0

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.1% | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | -0.2% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |  |  |  |
| Class D | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% |
| Class F | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.2% | 0.0% | 0.3% | 0.3% |
| Class C | 0.1% | -0.1% | 0.3% | 0.0% | -0.1% | 0.2% |
| Class D | 0.0% | 0.1% | 0.4% | -0.1% | -0.2% | -0.6% |
| Class E | 0.0% | -0.1% | 0.0% | 0.1% | 0.0% | 0.6% |
| Overall | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.1% | 0.3% | 0.0% | 0.0% | 0.1% |
| Class F | -0.1% | 0.5% | 0.3% | -0.1% | -0.1% | 0.0% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 103% |  |  | 100% |  |  |

Table 2 Comparison of HM 5.0 with TSM and significance scans and HM 5.0 with TSM

Table 2 shows how the proposed technique does not affect the running times of either the decoder or the encoder. The only tiny difference in the running times for low delay high efficiency is within the margin of variability displayed by the cluster used for the test. Table 2 also shows that the proposed technique does not affect the compression performance of TSM in a significant way.

Table 1 shows the overall performance of TSM, with its marked improvement for encoding of class F sequences where it achieves up to 6.3% smaller average BD-rates.

# Patent right declaration(s)

British Broadcasting Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[JCTVC-G1205] M. Mrak et al.,“Description of Core Experiment 5: Transform Skipping”, JCTVC-G1205, 7th JCTVC Meeting, Geneva, CH, November 2011

[JCTVC-G1200] F. Bossen, “Common test conditions and software reference configurations”, JCTVC-G1200, 7th JCTVC Meeting, Geneva, CH, November 2011

[JCTVC-G019] M. Mrak, J. Sole, I.-K. Kim, J. Xu, H. Yu, “JCT-VC AHG Report: Transform Skipping (AHG19)”, JCTVC-G019, 7th JCT-VC Meeting, Geneva, CH, November 2011

[JCTVC-G575] M. Mrak, A. Gabriellini, D. Flynn, “Transform skip mode”, JCTVC-G575, 7th JCT-VC Meeting, Geneva, CH, November 2011