| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0065 |
| --- | --- |

| Title: | Mixed Chroma Sampling-rate coding: combining the merits of 4:4:4 and 4:2:0 and increasing the value of past 4:2:0 investment |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tao Lin Shuhui Wang | Tel: Email: | 86-21-5103-0961 lintao@tongji.edu.cn shw@tongji.edu.cn |
| Source: | Tongji University, Shanghai, China |  |  |

_____________________________

# Abstract

For compound image compression, this contribution proposes a Mixed Chroma Sampling-rate (MCS) coding technique that combines and mixes one chroma sampling rate (such as full-chroma YUV444) coding with another chroma sampling rate (such as chroma-subsampled YUV420) coding at the coding unit (CU) boundary, the coding chain component (such as prediction, transform, quantization, entropy coding etc.) boundary, or other properly selected boundary. Typically, the full-chroma coder is discontinuous-tone content oriented to code mostly computer-generated discontinuous-tone content such as text, chart, graphics, and icon in a compound image like screen content, while the chroma-subsampled coder is continuous-tone content oriented to code mostly camera-captured continuous-tone content such as photograph, movie/TV clips, and natural picture video sequences in a compound image.

The first intention of MCS coding is to combine the merits of both full-chroma coding and chroma-subsampled coding. The second intention of MCS coding is to increase the value of chroma-subsampled CODEC software, hardware, and human assets and investment accumulated in the past 25 years and to reduce the additional efforts and investment required to raise the coding quality to the next level, especially for applications like screen content coding in cloud-mobile computing environment.

# Introduction

The onset of cloud computing, cloud rendering, mobile computing, and cloud-mobile computing has brought many new topics in video coding. In particular, low complexity and high quality screen content coding is indispensable in many rapidly growing cloud and mobile related applications such as virtualized screen, thin client-server platform, remote desktop, remote display and wireless or wired external, second, or mirror display for iPad/iPhone like mobile device at office or home.

In compound image such as screen content, the image content can be classified into two types: computer-generated discontinuous tone content such as text, chart, graphics, and icon, and camera-captured continuous tone content such as photograph, movie/TV clips, and natural picture video sequences. The discontinuous tone content features very sharp edges and thin lines, even one-pixel thin lines while continuous tone content features relatively smooth edges and thick lines. It is well known that for continuous tone content, chroma subsampling is almost visually lossless, hence all existing video coding standards such as MPEG, JPEG, JPEG-2000, H.264/AVC, VC-1, and AVS are designed primarily to code chrom-subsampled contents. Actually, majority of CODEC products on the market today can handle only YUV 4:2:0 or 4:2:2 format video. On the other hand, chroma subsampling significantly degrades the visual quality of discontinuous tone content in a compound image.

For compound image compression, although the visual quality of chrom-subsampled continuous tone content oriented coders is not good, almost all applications today still uses them primarily due to two reasons: 1) the majority of video CODEC software, hardware, and human assets developed in the past 25 years are chrom-subsampled ones and completely switching to full-chroma coders (at least two coders, one for continuous tone content and the other for discontinuous tone content) not only significantly increase the unit cost but also requires tremendous new investment; 2) since quite some portions of a compound image are continuous tone content, coding the entire image using two full-chroma coders is a overkill and too expensive.

Hence, the optimal solution to achieve best 3-way balance between bit-rate, coding quality and coding complexity for compound image compression is to combines one chroma sampling rate (such as full-chroma YUV444) coding with another chroma sampling rate (such as chroma-subsampled YUV420) coding and mix them in some properly selected boundaries in an adaptive way or in a pre-defined way.

This contribution proposes a Mixed Chroma Sampling-rate (MCS) coding technique that combines one chroma sampling rate coding with another chroma sampling rate coding at certain boundaries during the coding process. One approach is to mix the two chroma sampling-rates at macroblock (or CU) boundary in an adaptive way. Another approach is to mix the two chroma sampling-rates at the coding chain component tool (such as prediction, transform, quantization, entropy coding etc.) boundary. The two approaches are complementary and can be used together.

As well known, both full-chroma coding and chroma-subsampled coding have their own advantages. MCS coding can combine the merits of both. Furthermore, there were tremendous investment in the past 25 years for developing chroma-subsampled CODEC software, hardware, and human assets. MCS coding can fully utilize the assets and increase the value of past investment.

# MCS Coding System

For compound images consisting of both continuous-tone content and discontinuous-tone content, an MCS coding system can be decomposed into four coding tool (component) chains as shown in Figure 1. The Discontinuous-tone Content Oriented (DCO) chroma coding chain #1 usually takes full-chroma format while the Continuous-tone Content Oriented (CCO) chroma coding chain #2 usually takes chroma-subsampled format.

(a) Encoder

Input CU

Discontinuous-tone content oriented luma encoding tool chain #1

R-D Based Selector

Encoding tool #N1

Encoding tool #1

Continuous-tone content oriented luma encoding tool chain #2

Encoding tool #N2

Encoding tool #1

luma

Discontinuous-tone content oriented chroma encoding tool chain #1

R-D Based Selector

Encoding tool #N3

Encoding tool #1

Continuous-tone content oriented chroma encoding tool chain #2

Encoding tool #N4

Encoding tool #1

chroma

MCS Bitstream

(b) Decoder

Constru-cted CU

Discontinuous-tone content oriented luma decoding tool chain #1

CU Coding Type Parser

Decoding tool #N1

Decoding tool #1

Continuous-tone content oriented luma decoding tool chain #2

Decoding tool #N2

Decoding tool #1

luma

Discontinuous-tone content oriented chroma decoding tool chain #1

CU Coding Type Parser

Decoding tool #N3

Decoding tool #1

Continuous-tone content oriented chroma decoding tool chain #2

Decoding tool #N4

Decoding tool #1

chroma

MCS Bitstream

- MCS coding system.

In a particular implementation of the MCS coding system, some coding tools may be shared by different tool chains and these common tools and even the entire tool chains may be merged. For instance, an MCS coding system is shown in Figure 2. This MCS coding system is upgraded from existing chroma-subsampled hybrid coding system with the only additions being a chroma-subsampler/upsampler and a String Search & Matching/Construction tool like that used in gzip or lzma, which is very effective for discontinuous-tone content coding as reported in [1]-[3].

CCO luma and chroma-subsampled hybrid encoding tool chain

Pred

Tran

Entropy Encode

Scan

Qu

(a) Encoder

Input CU

DCO luma and full-chroma dictionary-entropy encoding tool chain

R-D Based Selector

Entropy encoding

String search & matching

MCS Bitstream

(b) Decoder

Constru-cted CU

DCO luma and full-chroma dictionary-entropy decoding tool chain

CU Coding Type Parser

Entropy decoding

CCO luma and chroma-subsampled hybrid decoding tool chain

MCS Bitstream

UV

shared

The existing hevc/avc/mpeg1/2/4 4:2:0 hybrid coders are used as is

= the only new additions

String construction

Pred

Inv.Tran

Entropy Decode

Inv.Scan

DeQu

UV

The existing hevc/h264/mpeg1/2/4 4:2:0 hybrid coders are used as is

= the only new additions

shared

- An MCS coding system upgraded from an existing hevc/avc/mpeg1/2/4 4:2:0 hybrid coder.

Another MCS coding system upgraded from an existing chroma-subsampled hybrid coding system is shown in Figure 3. The only additions are a chroma-subsampler/upsampler and a Direct Predictor tool, which can be a 4:4:4 version of that reported in [4].

CCO luma and chroma-subsampled hybrid encoding tool chain

Pred

Tran

Entropy Encode

Scan

Qu

(a) Encoder

Input CU

DCO luma and full-chroma dictionary-entropy encoding tool chain

R-D Based Selector

Entropy encoding

4:4:4 Direct Predictor

MCS Bitstream

(b) Decoder

Constru-cted CU

DCO luma and full-chroma dictionary-entropy decoding tool chain

CU Coding Type Parser

Entropy decoding

CCO luma and chroma-subsampled hybrid decoding tool chain

MCS Bitstream

UV

shared

The existing hevc/avc/mpeg1/2/4 4:2:0 hybrid coders are used as is

= the only new additions

4:4:4 Direct Predictor

Pred

Inv.Tran

Entropy Decode

Inv.Scan

DeQu

UV

The existing hevc/h264/mpeg1/2/4 4:2:0 hybrid coders are used as is

= the only new additions

shared

- Another MCS coding system upgraded from an existing hevc/avc/mpeg1/2/4 4:2:0 hybrid coder.

The existing 4:2:0 hybrid coders themselves used in the above MCS coding system can also be upgraded towards MCS direction. For instance, the 4:2:0 prediction can be upgraded to 4:4:4 prediction as shown in Figure 4 (inter only) and Figure 5 (both inter and intra), without any change to the existing 4:2:0 hybrid coding syntax and semantics including prediction modes and motion vector formats. Furthermore, both 4:2:0 prediction and transform can be upgraded to 4:4:4 prediction and transform as shown in Figure 6, without any change to the existing 4:2:0 hybrid coding syntax and semantics including prediction modes and motion vector formats. Note that in transform coefficient domain, subsampling becomes truncating three quarters of coefficients and upsampling becomes padding three quarters of coefficients. All other 4:2:0 hybrid coding tools are not changed and can be used as is.

String construction

= the only new additions

= the only new additions

UV -Pred

UV

Pred

CCO luma and chroma-subsampled hybrid encoding tool chain

Pred-UV

Tran

Entropy Encode

Scan

Qu

(a) Encoder

Input CU

DCO luma and full-chroma dictionary-entropy encoding tool chain

R-D Based Selector

Entropy encoding

String search & matching

MCS Bitstream

(b) Decoder

Constru-cted CU

DCO luma and full-chroma dictionary-entropy decoding tool chain

CU Coding Type Parser

Entropy decoding

CCO luma and chroma-subsampled hybrid decoding tool chain

MCS Bitstream

shared

Inv.Tran

Entropy Decode

Inv.Scan

DeQu

The existing hevc/h264/mpeg1/2/4 4:2:0 coding tools are used As Is

shared

Inter

UV

Intra

Pred

The existing hevc/avc/mpeg1/2/4 4:2:0 coding tools are used As Is

- Upgrading inter 4:2:0 prediction to 4:4:4 pred w/o any change to 4:2:0 syntax and semantics.

CCO luma and chroma-subsampled hybrid encoding tool chain

Pred

Tran

Entropy Encode

Scan

Qu

(a) Encoder

Input CU

DCO luma and full-chroma dictionary-entropy encoding tool chain

R-D Based Selector

Entropy encoding

String search & matching

MCS Bitstream

(b) Decoder

Constru-cted CU

DCO luma and full-chroma dictionary-entropy decoding tool chain

CU Coding Type Parser

Entropy decoding

CCO luma and chroma-subsampled hybrid decoding tool chain

MCS Bitstream

UV

shared

The existing hevc/avc/mpeg1/2/4 4:2:0 coding tools are used as is

= the only new additions

String construction

Pred

Inv.Tran

Entropy Decode

Inv.Scan

DeQu

UV

The existing hevc/h264/mpeg1/2/4 4:2:0 coding tools are used as is

= the only new additions

shared

- Upgrading 4:2:0 prediction to 4:4:4 prediction w/o any change to 4:2:0 syntax and semantics.

3/4 coef

CCO luma and chroma-subsampled hybrid encoding tool chain

Pred

Tran

Entropy Encode

Scan

Qu

(a) Encoder

Input CU

DCO luma and full-chroma dictionary-entropy encoding tool chain

R-D Based Selector

Entropy encoding

String search & matching

MCS Bitstream

(b) Decoder

Constru-cted CU

DCO luma and full-chroma dictionary-entropy decoding tool chain

CU Coding Type Parser

Entropy decoding

CCO luma and chroma-subsampled hybrid decoding tool chain

MCS Bitstream

3/4 coef

shared

The existing hevc/avc/mpeg1/2/4 4:2:0 coding tools are used as is

= the only new additions

String construction

Pred

Inv.Tran

Entropy Decode

Inv.Scan

DeQu

The existing hevc/h264/mpeg1/2/4 4:2:0 coding tools are used as is

= the only new additions

shared

- Upgrading 4:2:0 prediction/trans. to 4:4:4 ones w/o any change to 4:2:0 syntax and semantics.

Since all 4:2:0 hybrid coding tools and even a complete 4:2:0 hybrid coder can be used AS IS in an MCS coding system, the value of 4:2:0 CODEC investment in the past 25 years is appreciated. Since the additional full-chroma coding tools in an MSC coding system for discontinuous-tone content are usually simple ones, the development cost and unit cost of a coding system for full-chroma content can be minimized.

Not all discontinuous-tone contents are created equal. One particular DCO coder may be very effective to some contents, but may be not very effective to other contents. Therefore, a combo of different DCO coders may be used together to achieve better coding efficiency. Fortunately, DCO coders are usually much simpler than Hybrid Coder. Therefore, using multiple DCO coders does not increase too much coding complexity. A combo of four different DCO coders is shown in Figure 7.

DCO luma and full-chroma coding tool chain combo

R-D Based Selector

Entropy encoding

String search & matching

RLE

Coder 1

Coder 2

Entropy encoding

String search & matching

Hextile

(2D RLE)

Entropy encoding

String search & matching

Coder 3

Entropy encoding

String search & matching

RLE

A 4-mode intra predictor

Coder 4

- A combo of four different DCO coders.

# Experimental Results

See JCTVC-H0065-v1.ppt.

# Conclusions

Full-chroma such as YUV4:4:4 coding is indispensable for many emerging applications especially in cloud-mobile computing areas. Completely switching to a full-chroma coding system is unnecessary and too expensive. MCS coding can achieve a good 3-way balance between bit-rate, coding quality and coding complexity for compression of compound image consisting of both continuous-tone content and discontinuous-tone content.

# References

- S. Wang and T. Lin, “A unified LZ and hybrid coding for compound image partial-lossless compression,” 2nd International Conference on Image and Signal Processing, Oct. 2009, pp. 99-103.

- S. Wang and T. Lin, “United coding for compound image compression,” 3rd International Conference on Image and Signal Processing, Oct. 2010, pp. 566-570

- C. Lan, X. Peng, J. Xu and F. Wu, “Intra and inter coding tools for screen contents,” JCT-VC Doc JCTVC-E145, Mar. 2011.

- W. Gao, M. Jiang, Y. He, J. Song, and H. Yu, “A Lossless Coding Solution for HEVC,” JCT-VC Doc JCTVC-G664, Nov. 2011.

# Patent rights declaration(s)

Tongji University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).