| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0067 |
| --- | --- |

| Title: | Non-CE8.a.2 SAO with LCU-based syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Fuldseth, Cisco Systems G. Bjøntegaard, Cisco Systems | Tel: Email: Tel: Email: | +47 92414082 arild.fuldseth@cisco.com gbjonteg@cisco.com |
| Source: | Cisco Systems |  |  |

_____________________________

# Abstract

The contribution proposes an SAO technique with support for sub-frame encoder delay. One single set of SAO parameters are transmitted for each LCU using single-pass estimation. This supports low encoder-side delay by allowing for estimation and signaling of the SAO coefficients on the LCU level without aggregating pixel data of the whole frame. The proposed SAO scheme uses only edge offset with offset parameter always equal to 1. BDR losses between 0.0% and 1.1% are observed..

# Introduction

SAO operations as specified in HM 5.0 are mainly performed on a frame basis which would typically result in additional latency caused by the coding process.  This makes SAO less attractive for low delay real time applications like video conferencing where encoding delay less than one frame period is desired.  This contribution describes an SAO version with LCU-based syntax that is useful for low latency applications requiring encoding delays significantly less than one frame period.

# Description

the proposed method uses a simplified edge offset method and no band offset. In the following, the proposed method is described in detail.

Decision on making an offset (correction) of a pixel X is based on two neighboring pixels, X1 and X2.  Four structures are considered as indicated in the figure:

Structure:    1                                  2                                3                              4

X

X

X1

x2

X1

X2

X

X1

X2

X

X1

X2

For each structure, the decision process on whether to modify the pixel X can be described as follows:

if (abs(X1+X2-X0-X0) > 1)
{
  if (X1 >= X && X2 >= X)
  {
    X = X + 1
  }
  else if (X1 <= X & X2 <= X)
  {
    X = X – 1
  }
}

Enabling/disabling SAO on the LCU level

One flag is signaled for each LCU to enable/disable SAO.

Signaling of structure
Two bits are used to signal one of the four structures illustrated above.

Enabling/disabling chroma SAO on the LCU level

If SAO is enabled, one flag is signaled for each LCU to enable/disable SAO for chroma. All other SAO parameters are shared between luma and choma.

SAO parsing process

The SAO parsing process for each LCU can be summarized by the following pseudo-code.

SAObit           // SAO on/off

if (SAObit){

SAOchromaBit   // SAO on/off for chroma

EOtype         // Indicate type 1-4

}

# Simulation results

Table 1 shows BD-rate results for the proposed SAO method (preliminary results)

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A (8bit) | 0,2 % | 0,3 % | 0,3 % |
| Class B | 0,0 % | 1,1 % | 1,7 % |
| Class C | -0,3 % | 1,0 % | 1,6 % |
| Class D | 0,0 % | 0,1 % | 0,3 % |
| Class E | 0,0 % | 1,3 % | 1,4 % |
| Overall | -0,1 % | 0,8 % | 1,2 % |
|  | -0,1 % | 0,9 % | 1,2 % |
| Class F | 0,9 % | 2,0 % | 4,0 % |
| Enc Time[%] | 99,4 % |  |  |
| Dec Time[%] | 101,9 % |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A (8bit) | 0,4 % | 0,5 % | 0,5 % |
| Class B | 0,5 % | 2,0 % | 2,0 % |
| Class C | 0,0 % | 1,5 % | 2,3 % |
| Class D | 0,3 % | 0,8 % | 0,4 % |
| Class E |  |  |  |
| Overall | 0,3 % | 1,3 % | 1,5 % |
|  | 0,3 % | 1,4 % | 1,6 % |
| Class F | 1,2 % | 3,0 % | 4,9 % |
| Enc Time[%] | 99,8 % |  |  |
| Dec Time[%] | 99,6 % |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0,7 % | 3,1 % | 3,3 % |
| Class C | 0,2 % | 2,2 % | 3,1 % |
| Class D | 0,5 % | 1,2 % | 1,1 % |
| Class E | 2,0 % | 3,8 % | 4,2 % |
| Overall | 0,8 % | 2,5 % | 2,9 % |
|  | 0,8 % | 2,6 % | 3,0 % |
| Class F | 1,9 % | 4,6 % | 7,5 % |
| Enc Time[%] | 100,0 % |  |  |
| Dec Time[%] | 99,6 % |  |  |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0,6 % | 2,9 % | 3,1 % |
| Class C | 0,2 % | 1,9 % | 2,7 % |
| Class D | 0,3 % | 0,3 % | 0,6 % |
| Class E | 1,8 % | 3,1 % | 3,7 % |
| Overall | 0,6 % | 2,0 % | 2,5 % |
|  | 0,6 % | 2,1 % | 2,5 % |
| Class F | 2,0 % | 4,6 % | 6,9 % |
| Enc Time[%] | 99,9 % |  |  |
| Dec Time[%] | 101,1 % |  |  |

# Conclusions

Results have been presented for a low delay SAO method with LCU-based. The method is proposed adopted into the HM.

# References

# Patent rights declaration(s)

Cisco Systems may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).