| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0067 |
| --- | --- |

| Title: | Non-CE8.a.2: SAO with LCU-based syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Fuldseth, Cisco Systems G. Bjøntegaard, Cisco Systems | Tel: Email: Tel: Email: | +47 92414082 arild.fuldseth@cisco.com gbjonteg@cisco.com |
| Source: | Cisco Systems |  |  |

_____________________________

# Abstract

The contribution proposes an SAO technique with LCU-based syntax. The proposed syntax allows for signaling new SAO parameters for every LCU. The SAO syntax elements are located at the LCU level enabling immediate insertion of SAO information into the bit-stream once encoding of an LCU is completed. It is asserted that the proposed SAO syntax is friendlier to single-pass low delay encoder implementations compared to HM5.0-SAO. The proposed SAO method uses edge offset only, and without signaling any offset value. For each LCU, SAO can be enabled/disabled independently for luma and chroma. Using HM5.0 as anchor and with high efficiency settings (i.e. including HM5.0-SAO), BDR results between -0.1% and +1.1% are observed.

# Introduction

SAO as implemented in HM5.0 is frame oriented. This implies that all SAO syntax elements are shared by one frame (or a region of a frame) and are located in the APS. Furthermore the HM5.0 encoder estimates all SAO parameters for one frame as a separate pass once deblocking is complete. This means that there is an encoding delay of one frame associated with the performance of HM5.0-SAO. It is not known how HM5.0-SAO would perform with tighter constraint on the encoding delay, e.g. by using SAO parameters estimated from the previous frame. This makes HM5.0-SAO less attractive for low delay real time applications like video conferencing where encoding delay less than one frame period is desired.

The proposed SAO method is friendlier towards single-pass encoders with sub-frame encoding delay. By doing both SAO parameter estimation and SAO parameter signaling at the LCU level, SAO bits can be transmitted instantly and there is no additional encoding delay associated with the SAO process.

# Description

The proposed method uses a simplified edge offset method and no band offset. In the following, the proposed method is described in detail.

2.1 Edge offset

The decision on whether or not to make a correction of a pixel X is based on two neighboring pixels, X1 and X2.  Four structures are considered as indicated in the figure 1.

1                              2                            3                             4

x

x2222

x1111

x1

x

x2

x2

x

x1

x1

x

x2

Figure 1. Four structures for edge offset.

For each structure, the decision process on whether to modify the pixel X can be described by the following pseudo-code:

if (abs(x1+x2-x-x) > 1)
{
  if (x1 >= x && x2 >= x)
  {
    x = x + 1
  }
  else if (x1 <= x && x2 <= x)
  {
    x = x – 1
  }
}

2.2 SAO parsing process

The SAO parsing process for each LCU can be summarized by the following pseudo-code.

sao_enabled_flag            // SAO on/off

if (sao_enabled_flag){

sao_chroma_enabled_flag   // SAO on/off for chroma

sao_type_idx              // Indicate EO type 1-4
}

2.3 Determining SAO parameters

SAO parameters are determined using rate-distortion optimization for each LCU separately. When simulating single-pass low delay encoding mode, the 3 bottom rows of each LCU are not used for the estimation process, as these have not been deblocked yet.

2.4 Enabling/disabling SAO on the LCU level

One flag is signaled for each LCU to enable/disable SAO.

2.5 Signaling of structure
Two bits are used to signal one of the four structures illustrated above.

2.6 Enabling/disabling chroma SAO on the LCU level

If SAO is enabled for an LCU, one flag is signaled to enable/disable SAO for chroma. the EO type is shared between luma and choma.

2.7 Other considerations

Slice- and tile boundary processing

For slice- and tile boundary processing is proposed to use the same mechanisms as HM5.0-SAO.

Chroma filtering

The proposed method uses the same edge offset structure (i.e. 1-4) for luma and chroma samples within an LCU (provided both luma and chroma SAO is signaled). Alternatively, one could signal separate SAO structures for luma and chroma.

Recursive or non-recursive SAO processing at left and top LCU border

When doing LCU-by-LCU processing one can chose to use already SAO-processed samples of the LCU to the left and above when defining x1 and x2 for the leftmost pixel column and the top pixel row of the current LCU. This introduces a slight degree of recursive processing at the LCU border. Alternatively one can define the SAO process to be strictly non-recursive (i.e. always use pixel values that have not been SAO processed to define x1 and x2).

# Simulation results

BD-rate results for the proposed method are shown below. A strict sub-frame encoding delay constraint is imposed on the SAO filter estimation process, implying that the bottom three pixel of each LCU are not used to determine the SAO parameters.

In Table 1 BD-rate results for the proposed SAO method using one slice per frame and recursive processing at the LCU borders are shown. Common coding conditions [1] are used.

In Table 2 BD-rate results for the proposed SAO method using one slice per frame and non-recursive processing at the LCU borders are shown. Common coding conditions [1] are used.

In Table 3 BD-rate results for the proposed SAO method using multiple slices per frame and recursive processing at the LCU borders are shown. Except for multiple slices per frame, common coding conditions [1] are used. As described in [2], the numbers of LCUs per slice are 64, 104, 28, 60, and 96 for 1920x1080, 832x480, 416x240, 1280x720, and 1024x768 resolutions, respectively. The anchor is HM5.0 with multi-slice patches of revisions #1703, #1704, #1705, and #1706, multiple slices per frame and with ALF disabled [2]. Although this definition of anchor is not useful for direct comparison with the proposal, it is useful as a common reference between various CE8a proposals.

Table 1. BD-rate results for the proposed SAO method, single-slice configuration recursive processing at the LCU borders. Anchor: HM5.0 with SAO and ALF enabled

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A (8bit) | 0,2 % | 0,2 % | 0,3 % |
| Class B | -0,1 % | 1,2 % | 1,9 % |
| Class C | -0,3 % | 1,1 % | 1,7 % |
| Class D | 0,0 % | 0,1 % | 0,3 % |
| Class E | 0,0 % | 1,6 % | 1,3 % |
| Overall | -0,1 % | 0,9 % | 1,2 % |
|  | -0,1 % | 0,9 % | 1,3 % |
| Class F | 1,1 % | 2,5 % | 4,0 % |
| Enc Time[%] | 100 % |  |  |
| Dec Time[%] | 102 % |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A (8bit) | 0,4 % | 0,4 % | 0,3 % |
| Class B | 0,4 % | 2,2 % | 2,6 % |
| Class C | 0,0 % | 1,5 % | 2,3 % |
| Class D | 0,4 % | 1,0 % | 0,7 % |
| Class E |  |  |  |
| Overall | 0,3 % | 1,4 % | 1,7 % |
|  | 0,3 % | 1,4 % | 1,7 % |
| Class F | 2,1 % | 3,4 % | 6,3 % |
| Enc Time[%] | 100 % |  |  |
| Dec Time[%] | 100 % |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 1,0 % | 4,8 % | 6,1 % |
| Class C | 0,2 % | 3,6 % | 6,2 % |
| Class D | 0,3 % | 2,4 % | 3,5 % |
| Class E | 1,7 % | 4,8 % | 5,2 % |
| Overall | 0,8 % | 3,9 % | 5,3 % |
|  | 0,8 % | 3,9 % | 5,4 % |
| Class F | 2,9 % | 6,5 % | 11,1 % |
| Enc Time[%] | 100 % |  |  |
| Dec Time[%] | 100 % |  |  |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0,9 % | 4,7 % | 5,4 % |
| Class C | 0,3 % | 3,4 % | 5,4 % |
| Class D | 0,4 % | 2,7 % | 3,0 % |
| Class E | 1,5 % | 4,0 % | 4,7 % |
| Overall | 0,7 % | 3,7 % | 4,7 % |
|  | 0,7 % | 3,7 % | 4,7 % |
| Class F | 3,1 % | 7,0 % | 10,9 % |
| Enc Time[%] | 100 % |  |  |
| Dec Time[%] | 100 % |  |  |
|  | Random Access HE10 |  |  |
|  | Y | U | V |
| Class A (8bit) | 1,1 % | 6,8 % | 7,7 % |
| Class B | 1,0 % | 2,4 % | 2,9 % |
| Class C |  |  |  |
| Class D |  |  |  |
| Class E |  |  |  |
| Overall | 1,1 % | 4,4 % | 5,1 % |
|  | 1,1 % | 4,3 % | 5,1 % |
| Class F |  |  |  |
| Enc Time[%] | 100 % |  |  |
| Dec Time[%] | 99 % |  |  |

Table 2. BD-rate results for the proposed SAO method, single-slice configuration non-recursive processing at the LCU borders. Anchor: HM5.0 with SAO and ALF enabled.

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A (8bit) | 0,2 % | 0,2 % | 0,3 % |
| Class B | -0,1 % | 1,2 % | 1,9 % |
| Class C | -0,3 % | 1,1 % | 1,7 % |
| Class D | 0,0 % | 0,1 % | 0,3 % |
| Class E | 0,0 % | 1,6 % | 1,3 % |
| Overall | -0,1 % | 0,9 % | 1,2 % |
|  | -0,1 % | 0,9 % | 1,3 % |
| Class F | 1,1 % | 2,5 % | 4,0 % |
| Enc Time[%] | 105 % |  |  |
| Dec Time[%] | 120 % |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A (8bit) | 0,4 % | 0,3 % | 0,4 % |
| Class B | 0,4 % | 2,2 % | 2,6 % |
| Class C | 0,0 % | 1,6 % | 2,2 % |
| Class D | 0,4 % | 0,8 % | 0,8 % |
| Class E |  |  |  |
| Overall | 0,3 % | 1,4 % | 1,7 % |
|  | 0,3 % | 1,4 % | 1,8 % |
| Class F | 2,1 % | 3,5 % | 6,3 % |
| Enc Time[%] | 101 % |  |  |
| Dec Time[%] | 110 % |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 1,0 % | 4,8 % | 6,0 % |
| Class C | 0,3 % | 3,6 % | 6,3 % |
| Class D | 0,3 % | 2,3 % | 3,3 % |
| Class E | 1,7 % | 4,1 % | 5,6 % |
| Overall | 0,8 % | 3,7 % | 5,3 % |
|  | 0,8 % | 3,8 % | 5,3 % |
| Class F | 2,9 % | 7,0 % | 10,8 % |
| Enc Time[%] | 101 % |  |  |
| Dec Time[%] | 109 % |  |  |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0,9 % | 4,5 % | 5,2 % |
| Class C | 0,3 % | 3,5 % | 5,3 % |
| Class D | 0,4 % | 2,5 % | 2,5 % |
| Class E | 1,5 % | 3,9 % | 4,4 % |
| Overall | 0,7 % | 3,6 % | 4,4 % |
|  | 0,7 % | 3,7 % | 4,5 % |
| Class F | 3,1 % | 6,7 % | 10,4 % |
| Enc Time[%] | 100 % |  |  |
| Dec Time[%] | 107 % |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A (8bit) | 1,1 % | 6,5 % | 8,0 % |
| Class B | 1,1 % | 2,5 % | 3,0 % |
| Class C |  |  |  |
| Class D |  |  |  |
| Class E |  |  |  |
| Overall | 1,1 % | 4,2 % | 5,2 % |
|  | 1,1 % | 4,2 % | 5,2 % |
| Class F |  |  |  |
| Enc Time[%] | 100 % |  |  |
| Dec Time[%] | 103 % |  |  |

Table 3. BD-rate results for the proposed SAO method, multi-slice configuration and recursive processing at the LCU borders. Anchor: HM5.0 with multiple slices per frame, SAO enabled and ALF disabled.

|  | Low delay B HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -1,8 % | 4,2 % | 6,2 % |
| Class C | -2,3 % | 3,3 % | 5,3 % |
| Class D | -1,9 % | 1,9 % | 1,5 % |
| Class E | -1,1 % | 4,5 % | 8,1 % |
| Overall | -1,9 % | 3,4 % | 5,2 % |
|  | -1,8 % | 3,4 % | 5,2 % |
| Class F | 0,7 % | 6,6 % | 10,7 % |
| Enc Time[%] | 106 % |  |  |
| Dec Time[%] | 117 % |  |  |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -4,9 % | 3,1 % | 4,7 % |
| Class C | -2,2 % | 3,0 % | 4,9 % |
| Class D | -0,7 % | 1,9 % | 1,7 % |
| Class E | -4,6 % | 2,7 % | 6,6 % |
| Overall | -3,1 % | 2,7 % | 4,3 % |
|  | -3,1 % | 2,7 % | 4,4 % |
| Class F | 1,2 % | 6,0 % | 10,4 % |
| Enc Time[%] | 110 % |  |  |
| Dec Time[%] | 122 % |  |  |

# Conclusions

Results have been presented for an SAO method with LCU-based syntax. The method is particularly useful for single-pass encoder implementations with sub-frame encoding delay. When comparing with the HM5.0 anchor, small BD-rate losses are observed. The method is proposed adopted into the HM/WD.

# References

- F. Bossen, "Common HM test conditions and software reference configurations," JCTVC-G1200, Geneva, November, 2011.

- T. Yamakage, I. S. Chong, M. Narroschke, Y.-W. Huang, "Description of Core Experiment 8 (CE8): Non-deblocking loop filtering," JCTVC-G1208, Geneva, November, 2011.

# Patent rights declaration(s)

Cisco Systems may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix A

This appendix contains suggested WD text for the proposed method. The text below is proposed to replace the relevant sections of the existing WD.

Syntax

| sao_parameters( ) { | Descriptor |
| --- | --- |
| sao_enabled_flag | ae(v) |
| if( alf_enabled_flag ) { |  |
| sao_chroma_enabled_flag | ae(v) |
| sao_type_idx | ae(v) |
| } |  |
| } |  |

Semantics

sao_enabled_flag equal to 1 specifies that the sample adaptive offset process is applied to the luma samples of the current LCU. sao_enabled_flag equal to 0 specifies that that the sample adaptive offset process is not enabled for the luma samples of the current LCU.

sao_chroma_enabled_flag equal to 1 specifies that the sample adaptive offset process is applied to the chroma samples of the current LCU. sao_chroma_enabled_flag equal to 0 specifies that that the sample adaptive offset process is not enabled for the chroma samples of the current LCU. When not present, sao_chroma_enabled_flag shall be inferred to be equal to 0.

sao_type_idx specifies which of he SAO types shall be used in the adaptive loop filter process. sao_type_id shall b in the range of 0 to 3, inclusive.

Decoding process

Modification  process  for sample adaptive offset

Inputs to this process are:

–	a variable cIdx specifying chroma component index,

–	a sample location ( x0, y0 ) specifying the top-left sample of the current largest coding unit,

–	Output of this process is a modified picture buffer for the chroma component cIdx.

Let recSaoPicture represent the processed sample array of the current picture of chroma component cIdx.

Next, the following ordered steps apply:

- Variable nS is set equal to ( 1 << Log2MaxCUSize ).

- Arrays hPos[2] and vPos[2] are specified in Table 8-16

- Define the variables X, X1, and X2 as

X = recPicture[ xC + i , yC + j )

X1 = recPicture[ xC + i + hPos[ 0 ], yC + j + vPos[ 0 ] )

X2 = recPicture[ xC + i + hPos[ 1 ], yC + j + vPos[ 1 ] )

with i = 0..nS-1 and j = 0..nS-1

- Define the variable Offset as

  if ((abs(x1+x2-x-x) > 1) && (x1 >= x && x2 >= x))
      Offset  =  1
  else if ((abs(x1+x2-x-x) > 1) && (x1 <= x && x2 <= x))
      Offset  =  -1
else
      Offset  =  0

- The reconstructed picture buffer is modified as

  recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ] + Offset

  with i = 0..nS-1 and j = 0..nS-1

Table 8-16 – Specification of hPos[2] and vPos[2] according to the type of sample adaptive offset process

| sao_type_idx | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| hPos[0] | -1 | 0 | -1 | 1 |
| hPos[1] | 1 | 0 | 1 | -1 |
| vPos[0] | 0 | -1 | -1 | 1 |
| vPos[1] | 0 | 1 | 1 | -1 |