| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0072 |
| --- | --- |

| Title: | NAL Unit loss software |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Stephan Wenger Vidyo, Inc 433 Hackensack Ave., Hackensack N.J. 07601, USA | Tel: Email: | +1-415-713-5473 stewe@stewe.org |
| Source: | Vidyo, Inc. |  |  |

_____________________________

# Abstract

Attached is a nal unit loss simulator (loss.c) that can be used to apply error pattern files to a bitstream according to Annex B bitstream format.  Further included is a converter program (convert.c) that converts the error pattern files as generated by the G.1050 software to a more compact interim format—the same format used previously in JCT, as well as converted error pattern files.  Both software and patterns could benefit from testing.

# NAL Unit Loss Simulator

The NAL unit simulator can be found in the file loss.c and loss.exe.  The source file was written using MS Visual C++ 2010, but the file should compile without major modification under other operating systems or compilers as well.  No special compiler options were used.

Usage

Loss <infile> <outfile> <errfile> <options> where

<infile> is the input NAL unit bitstream (Annex B, with start codes) as generated by the HM software

<outfile> is a NAL unit bitstream file that may miss certain NAL units

<errfile> is a textfile with error patters.  Within the file, whitespace is ignored.  A ‘0’ character indicates a lost NAL unit.  Any other character indicates a transmitted NAL unit.

<options> are one or more options.  Of any given type, when more than one option is provided, only the last instance is used.  An option starts with a ‘-‘, followed by a single character, followed by an integer.  No whitespace is allowed in options.  The following options are supported:

-r<offset> start reading the error pattern file at offset <offset> (can be used ot implement pseudo-random entry into the error pattern file from a script)

-d<debuglevel> 0 is quiet mode.  1 provides based statistic info after termination.  Higher values produce an increasing number of debug messages

-a<apply-level> allows to exclude certain NAL unit types from the application of the error patterns, as shown in the table below.

| A | excluded NAL unit types | Remark |
| --- | --- | --- |
| 0 | all | All NAL units get through; option mostly good for debugging |
| 1 | 7, 8 | SPS and PPS get through |
| 2 | 5, 7, 8 | SPS, PPS, and IDR slices |
| 3 | 4, 5, 7, 8 | SPS, PPS, IDR and CRA slices |
| 4 | 1, 4, 5, 7, 8 | SPS, PPS, and all coded slices |
| 5 | none | All NAL units subject to errors |

The source file has a compile time switch adapting the NAL unit header format to the format used for HM 5.0 and  HM 5.1 (minus the APS).  The executable is compiled using the HM5.0 format.  The first byte of the NAL unit header in HM5.0 is the same as in H.264, and the author used his old testing environment and JM bitstreams to test the simulator.  This format is believed to be without obvious bugs.

The HM5.1 format is essentially untested, and may require some additional work.

# Conversion Software

The conversion software is provided as source file convert.c and executable convert.exe.  Convert.c should compile in most environments without modifications.  It is used as follows:

Convert <G.1050-input-textfile >old-format-output-textfile

Error messages and statistics are provided on stderr.

# Converted Error Patterns

Also included are the JCTVC error patterns in converted form.  The filenames are 1.txt, 3.txt. 5.txt, and 10.txt, and the number in the filename indicates the average loss rate in the file.

# Patent statement

Vidyo, Inc. does not have any current or pending patent rights relating to the technology described in this contribution.