| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0073 |
| --- | --- |

| Title: | 4:4:4 screen content coding using Macroblock-Adaptive Mixed Chroma-Sampling-Rate |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shuhui Wang Tao Lin | Tel: Email: | 86-21-5103-0961 vlsi@tongji.edu.cn lintao@tongji.edu.cn |
| Source: | Tongji University, Shanghai, China |  |  |

_____________________________

# Abstract

This contribution presents a Macroblock(or CU)-Adaptive Mixed Chroma-sampling-rate (MAMC) technique for 444 screen content coding. MAMC coding consists of two coders, one is traditional continuous-tone content oriented (CCO) chroma-subsampled (420 or 422) coder such as the existing HEVC/H.264/AVC coder with 420 or 422 chroma format. The other is discontinuous-tone content oriented (DCO) 444 full-chroma coder such as a dictionary-entropy coder. One of the two coders is selected to code a macroblock(or CU) based on the rate-distortion performance.

Another aspect of the contribution is to propose a modification to the existing 420 or 422 chroma format HEVC/H.264/AVC coder to be used in MAMC coding. Since the full-chroma pixels are available in MAMC coding, the prediction can be done in full-chroma format without any change to the existing syntax and semantics including prediction modes of the 420 or 422 chroma format HEVC/H.264/AVC standard specs except adding a “MAMC flag” bit in the SPS.

The main point of MAMC is to enable 444 chroma format content coding in HEVC and H.264/AVC, which could expand their market adoption especially in cloud-mobile computing and wireless display areas, with minimum addition, modification, and complexity increment to the current 420/422 chroma format HEVC/H.264/AVC coder design.

Coding experimental results are that MAMC coding consisting of a gzip-based dictionary-entropy coder and an H.264/AVC high profile coder can get much higher PSNR than the H.264/AVC high profile coding. Using full-chroma prediction H.264/AVC coder instead of traditional H.264/AVC high profile coder in MAMC coding can get further PSNR improvement in two pictures which have a lot of discontinuous-tone content.

# Introduction

Screen content features two different types of content: 1) discontinuous tone content such as text, chart, graphics, and icon, which are rendered by computers; 2) continuous tone content such as photograph, movie/TV clips, and natural picture video sequences, which are mostly captured from cameras. The discontinuous tone content features very sharp edges and thin lines, even one-pixel thin lines while continuous tone content features relatively smooth edges and thick lines. It is well known that for continuous tone content, chroma subsampling is almost visually lossless. On the other hand, chroma subsampling significantly degrades the visual quality of discontinuous tone content in a screen.

For screen content coding, using full-chroma coders all the way is an overkill and too expensive. Hence, the optimal solution to achieve 3-way balance between bit-rate, coding quality and coding complexity for SCC is to add a full-chroma coder to a traditional chrom-subsampled coder such as HEVC/H.264/AVC 420/422 format coder.

This contribution proposes a macroblock(or CU)-adaptive mixed chroma-sampling-rate (MAMC) approach for SCC. An original input macroblock of an image is chroma-subsampled. Then, a discontinuous tone content oriented (DCO) coder such as dictionary-entropy coder is used to code the original full-chroma macroblock and a continuous tone content oriented (CCO) coder such as HEVC/H.264/AVC 420/422 format coder is used to code the chroma-subsampled version of the macroblock. Finally, the coder that has the optimal rate-distortion performance is selected as the optimal coder to code the macroblock and the corresponding coded bitstream data is put into the output bitstream.

Furthermore, the contribution proposes a modification to the existing 420 or 422 chroma format HEVC/H.264/AVC coder used in MAMC coding. Since the full-chroma pixels are available in MAMC coding, the prediction or both prediction and transform can be done in full-chroma format without any change to the existing syntax and semantics including prediction modes of the 420 or 422 chroma format HEVC/H.264/AVC standard specs except adding a “MAMC flag” bit in the SPS.

# MAMC Coding Architecture

A general architecture and major components of MAMC coding are shown in Figure 1.

For ref., pred. or dict.

(a) Encoder

Original Input Macroblock O

Chroma Subsampler 2

(b) Decoder

Coder 1

Constructed Pixel Buffer 1

Constructed Pixel Buffer 2

R-D Performance Based Selector

MAMC Bitstream

Chroma Subsampler 1

Chroma Upsampler

Coder 2

MAMC Bitstream

Constructed MacroblockÕ

Chroma Upsampler 2

Decoder 1

Constructed Pixel Buffer 1

Constructed Pixel Buffer 2

Chroma Subsampler

Chroma Upsampler 1

Decoder 2

2-to-1 Multiplexer

MB Chroma Type Parser

1-to-2 Demultiplexer

S

O

P1

b1

P2

b2

For ref., pred. or dict.

For ref., pred. or dict.

For ref., pred. or dict.

T

O

MB_chroma_type_bit

P1

b1

P2

b2

- MAMC coding architecture.

In the encoder, the original full-chroma input macroblock O is fed to Coder 1 that is discontinuous tone content oriented. The same O is also sent to a chroma subsampler that subsamples the chroma of O but keeps the luma of O unchanged to generate chrom-subsampled macroblock S. S is then fed to Coder 2 that is continuous tone content oriented. Coder 1 codes O to generate the coded bitsream b1 and the constructed macroblock P1. Coder 2 codes S to generate the coded bitsream b2 and the constructed macroblock P2. P1 and P2 are stored in Constructed Pixel Buffer (CPB) 1 and CPB 2 to be used by Coder 1 and Coder 2, respectively, as reference, prediction or dictionary to code the other pixels and macroblocks. P2 is also chroma upsampled to get T for rate-distortion (R-D) evaluation. The data O, P1, b1, T, and b2 are sent to R-D Performance Based Selector that calculates the R-D performance of the two coders and selects the one with the best R-D performance as the final coder for the macroblock. The corresponding coded bitstream b1 or b2 is selected and put into the output MAMC bitstream.

To seamlessly unify the two coders and achieve maximum coding efficiency for both coders, each CPB should have as many relevant pixels as possible. Therefore, in the encoding process, if Coder 1 is selected as the final coder, then P1 in CPB 1 is chrom-subsampled and then put into CPB 2 to replace P2, otherwise, P2 in CPB 2 is chroma upsampled and then put into CPB 1 to replace P1. In this way, the two CPBs are always synchronized. No matter which coder is selected as the final coder, all constructed pixels from the final coder are available not only to the final coder itself but also to the other coder to maximize the coding gain of both coders.

In the decoder, the input MAMC bitstream is first parsed by Macroblock Chroma Type Parser to get the MB_chroma_type_bit that indicates if the current macroblock to be decoded is chrom-subsampled or not. If the coded macroblock is not chrom-subsampled, then it is fed to Decoder 1 which decodes and constructs the full-chroma macroblock P1, otherwise, it is fed to Decoder 2 which decodes and constructs the chrom-subsampled macroblock P2. P1 or P2 is stored in CPB 1 or CPB 2 to be used by Decoder 1 or Decoder 2, respectively, for reference, prediction or dictionary. Furthermore, P1 or P2 is subsampled or upsampled and stored in CPB 2 or CPB 1 to be used by Decoder 2 or Decoder 1, respectively. If the coded macroblock is not chrom-subsampled, then P1 is the output Constructed Macroblock Õ, otherwise, P2 is chroma upsampled to become the output Constructed Macroblock Õ. It should be mentioned that if the chroma upsampling filter in Chroma Upsampler 2 for Õ involves the pixels of neighboring macroblocks that is constructed by Decoder 1, then the pixels from CPB 1 should also be supplied to Chroma Upsampler 2 for the filtering operations, as indicated by the dash line in Figure 1(b).

It should be noted that Chroma Subsampler 1 and Chroma Subsampler 2 in Figure 1(a) play two different roles. The role of Chroma Subsampler 1 is to provide constructed pixels to Coder 2 for reference, prediction or dictionary, so the chroma subsampling filter in Chroma Subsampler 1 can involve only the left and above macroblocks that have already been constructed. Therefore, conventional symmetric filters with more than 2 taps cannot be used in Chroma Subsampler 1. The role of Chroma Subsampler 2 is to subsample the chroma of the original input image, so any subsampling filter can be used. Similarly, Chroma Upsampler 1 and Chroma Upsampler 2 in Figure 1(b) play two different roles. The role of Chroma Upsampler 1 is to provide constructed pixels to Decoder 1 for reference, prediction or dictionary, so the chroma subsampling filter in Chroma Upsampler 1 can involve only the left and above macroblocks that have already been constructed. Therefore, conventional symmetric filters with more than 2 taps cannot be used in Chroma Upsampler 1. The role of Chroma Upsampler 2 is to upsample the chroma of the final output image, so the upsampling operation can be delayed until all neighboring macroblocks are available, thus any upsampling filter can be used. Similarly, conventional symmetric filters with more than 2 taps cannot be used in Chroma Upsampler of Figure 1 (a) and Chroma Subsampler of Figure 1 (b).

# A Partial-lossless MAMC Coding System

For screen content coding, there are quite a few tools proposed in the past few JCTVC meetings [1]-[3]. It was reported [4] that “It is remarked that among the results shown LZ performs almost best at least for intra”, where the “results” are from [5] and LZ means a type of dictionary-entropy coding. Therefore, in the MAMC coding system proposed in this contribution, we use a lossless dictionary-entropy coder such as gzip or lzma as the base full-chroma coder (Coder 1 in Figure 1) and a lossy hybrid video coder such as high profile H.264 as the base chrom-subsampled coder (Coder 2 in Figure 1). Therefore, CPB 1 in Figure 1 becomes the dictionary for string searching and matching, and CPB 2 in Figure 1 becomes the neighboring pixels for intra prediction and the reference pictures for inter prediction.

  - Dictionary in MAMC Coding

In a typical dictionary-entropy coder, the maximum dictionary size ranges from 32KB (in gzip) to 4GB (in LZMA), which is far more than the data size of one macroblock. Obviously, if only the current macroblock data of 256x3=768 bytes or the current LCU data of 4096x3=12K are used in the dictionary, the compression performance will suffer seriously. Therefore, in the proposed MAMC coding system, pixels data in previously constructed macroblocks are also used in the dictionary. The constructed pixels from the macroblocks (or CUs) previously coded by the lossless dictionary-entropy coder are all used in the dictionary. The constructed pixels from the macroblocks previously coded by the lossy hybrid coder can also optionally be used in the dictionary.

  - Pixel Format and Scan Order in Macroblock and Dictionary

In hybrid coding, pixels are organized in planar format as luma blocks and chroma blocks, and the scan order is zig-zag. It turns out that this pixel format and scan order are not suitable for lossless dictionary-entropy coder in MAMC coding. In fact, if the input macroblock to lossless coders use packed pixel format (such as R0, G0, B0, R1, G1, B1, …… or Y0, U0, V0, Y1, U1, V1, ……where Rn, Gn, Bn or Yn, Un, Vn are the three components of a pixel) and column-wise scanning as shown in Figure 2, the compression performance can be significantly improved. Therefore, the pixel format and scan order of an input macroblock to the lossless coder is different from that to the lossy coder. For the lossless coder, both the input macroblock and the dictionary use packed pixel format and column-wise scan order.

left

neighboring

macroblock

current

macroblock

- Column-wise scanning of packed pixel in macroblocks and dictionary.

  - Enhancement to Hybrid Coding

Enhancement to traditional hybrid coding is necessary to take full advantage of hybrid coding. In particular, when doing intra- or inter-prediction in hybrid coding, the full-chroma pixels of a neighboring macroblock or a reference picture can be used as the prediction pixels to significantly improve the hybrid coding performance without any change to the basic chrom-subsampled 420/422 hybrid coding syntax and semantics including the prediction modes and motion vector formats by keeping the prediction residuals still coded in the chrom-subsampled format. This is essentially to change the traditional chroma-subsampled (420 or 422) hybrid coder used in MAMC coding as shown in Figure 3 into a Mixed Chroma Sampling-rate hybrid coder. There are two ways to make the change. One way is to do full-chroma prediction and then chroma-subsample the prediction residuals before transform as shown in Figure 4 (a). The other way is to do both full-chroma prediction and full-chroma transform but only keep one quarter of the transform coefficients, i.e., truncate the other three quarters of coefficients as shown in Figure 4 (b).

T

MAMC Bitstream

Traditional

hybrid coder

YUV444 Input Macroblock O

Chroma Subsampler 2

YUV444 Constructed Pixel Buffer 1

YUV420 Constructed Pixel Buffer 2

R-D Performance Based Selector

Chroma Subsampler 1

Chroma Upsampler

Pred

S

P2

b2

Trans

Entropy Coding

IQ

IT

Scan

Quan

- Traditional Chroma-subsampled (420 or 422) Hybrid Coder used in MAMC coding.

MAMC Bitstream

MCS hybrid coder

YUV444 Constructed Pixel Buffer 1

R-D Performance Based Selector

Chroma Upsampler

Pred

T

Trans

Quan

IQ

IT

YUV444 Input Macroblock O

Chroma Subsampler

YUV420

YUV444

Entropy Coding

Scan

(a) Only prediction is in YUV444 format

b2

MCS hybrid coder

¾ UV Coeff Padding

Pred

Trans

Quan

IQ

IT

¾ UV Coeff Truncation

YUV420

YUV444

b2

Entropy Coding

Scan

(b) Both prediction and transform are in YUV444 format

YUV444 Input Macroblock O

YUV444 Constructed Pixel Buffer 1

MAMC Bitstream

R-D Performance Based Selector

T

- Mixed chroma sampling-rate hybrid coder used in MAMC coding.

# Experimental Results

In the experiments, we use six screen capture pictures as shown in Figure 5 (a) – (f).

(a) Integrated-circuit design debug waveform.

(b) IC design debug waveform plus nature picture.

(c) Web page.

(d) PowerPoint, Word, and Excel file windows.

(e) Document editing.

(f) Various windows mix.

- Six screen capture pictures.

We also use the four YUV4:4:4 test sequences for screen content coding presented in [6].

The experimental results are presented JCTVC-H0073-v1.ppt.

# Conclusions

In this document, we have proposed MAMC coding and mixed chroma sampling-rate hybrid coding for full-chroma screen content coding. The coding performance for full-chroma (YUV444) screen contents is significantly improved.

# References

- Oscar C. Au, Jizheng Xu, and Haoping Yu, “JCT-VC AHG report: Screen Content Coding (SCC),” JCT-VC Doc JCTVC-G013, Nov. 2011.

- Wen Gao, Keiichi Chono, Jizheng Xu, and Minhua Zhou, “JCT-VC AHG report: Lossless Coding,” JCT-VC Doc JCTVC-G022, Nov. 2011.

- Oscar C. Au, Jizheng Xu, and Haoping Yu, “JCT-VC AHG report: Screen Content Coding (SCC),” JCT-VC Doc JCTVC-F013, July 2011.

- Gary Sullivan, and Jens-Rainer Ohm, “Meeting report of the fifth meeting of the Joint Collaborative Team on Video Coding (JCT-VC), Geneva, CH, 16-23 March 2011,” JCT-VC Doc JCTVC-E_Notes_d8, Mar. 2011.

- C. Lan, X. Peng, J. Xu and F. Wu, “Intra and inter coding tools for screen contents,” JCT-VC Doc JCTVC-E145, Mar. 2011.

- J. Cao, K. Zhou, S. Wang, T. Lin, “YUV4:4:4 test sequences for screen content coding,” JCT-VC Doc JCTVC-H0294, Feb. 2012

# Patent rights declaration(s)

Tongji University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).