| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0076 |
| --- | --- |

| Title: | CE1: Cross-check of Subtest B2 – Test 3 (H0266) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Chuohao Yeo1 Yih Han Tan1 Zhengguo Li1 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 Liwei Guo2 Marta Karczewicz2 5775 Morehouse Drive San Diego, CA 92121 USA 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 | Tel: Email: | +65 6408 2000 chyeo@i2r.a-star.edu.sg liweig@qualcomm.com |
| Source: | 1 Institute for Infocomm Research 2 Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents a summary of a cross-check performed by the Institute for Infocomm Research and Qualcomm on HHI’s proposal for 8-bit value CABAC context initialization. The cross-check performance results reportedly match those provided by HHI. The implemented algorithm is also reportedly in line with HHI’s proposal.

# Introduction

In JCTVC-H0266, 2 modifications of the 8-bit value CABAC context initialization in HM5 are proposed. In the first modification (Test 1), “linear”, the initialization used is such that the initial CABAC state chosen has a corresponding probability that is linear in the slice QP. In the second modification (Test 3), “disabled”, the initialization used is such that the initial CABAC state chosen is linear in QP.

In the source code of the implementation provided by HHI, the only relevant modification to the initialization routine is that the entries in the 2 look-up tables used by initialization are modified. In HM5, the following table is used to determine the slope used for computing an intermediary value:

const Int ContextModel::m_slopes[16] = { -239, -143, -85, -51, -31, -19, -11, 0, 11, 19, 31, 51, 85, 143, 239, 399 };

In addition, the following 2 tables are used to determine the final CABAC state by using a piece-wise linear approximation:

const Int ContextModel::m_segOffset[8]            = {   6,  7,  5,  7, 10, 14, 16,  1 };

const Int ContextModel::m_accumulatedSegOffset[8] = {  -3,  3, 10, 15, 22, 32, 46, 62 };

In Test 1, the above 2 tables are modified such that the initial CABAC state chosen has a corresponding probability that is linear in the slice QP:

const Int ContextModel::m_segOffset[8]            = {   4,  3,  3,  4,  6,  8, 13, 21 };

const Int ContextModel::m_accumulatedSegOffset[8] = {  -2,  2,  5,  8, 12, 18, 26, 39 };

This has been checked numerically.

In Test 3, the 2 tables are modified such that the initial CABAC state chosen is linear in QP:

const Int ContextModel::m_segOffset[8]            = {   8,  8,  8,  8,  8,  8,  8,  8 };

const Int ContextModel::m_accumulatedSegOffset[8] = {  -4,  4, 12, 20, 28, 36, 44, 52 };

This is in fact equivalent to replacing lines 62-67 of ContextModel.cpp:

Int val = max( 0, min( 3839, (n<<8) + m * (qp-26) + 128 ) );

val -= 1920;

Int valMps = val >= 0;

val = ( val ^ (val>>11) ) + 128;

Int segmentIdx = val >> 8;

Int pStateIdx = m_accumulatedSegOffset[segmentIdx] + ( (val&255) * m_segOffset[segmentIdx] >> 8);

with:

Int val = max( 0, min( 3839, (n<<8) + m * (qp-26) + 128 ) );

val -= 1920;

Int valMps = val >= 0;

Int pStateIdx = ( val ^ (val>>11) ) >> 5;

In this cross-check, Qualcom has performed simulations for Test 1 and I2R has performed simulations for Test 3.

Code Review:

At some places, bit operations are used to replace logical and arithmetic operations. For example,

val = ( val ^ (val>>11) ) + 128;        (1)

Note that this line of code is also used by the initialization in HM5.

We believe the purpose of this code is to flip the value of val when val < 0, which can be alternatively implemented as follows:

if (val<0 )

{ val = -val - 1;}

val = val + 128;             (2)

While implementations (1) and (2) are equivalent for -2048<val<2048 (which is always true due to the clipping at the first step), the readability and intent of implementation (1) may not be as clear as (2).

# Simulation results

The test conditions as specified in CE1 Subtest B [1] are followed. Tests were conducted on configurations as specified in the common test conditions [2]. Low-Delay P configurations were also tested (and included in the attached Excel spreadsheets).

The results for Test 1 for QP=(22,27,32,37) are shown below:

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | -0.1% | 0.1% | 0.0% | 0.1% | -0.1% |  |  |  |
| Enc Time[%] | 101% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 94% |  |  | 94% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | -0.3% | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 0.3% |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | -0.1% | -0.1% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |  |  |  |
| Class D | 0.0% | 0.1% | 0.3% | 0.0% | 0.6% | -0.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.2% | 0.0% | 0.0% | -0.1% | 0.1% |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class F | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 99% |  |  | 100% |  |  | 95% |  |  |
| Dec Time[%] | 97% |  |  | 98% |  |  | 96% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.2% | 0.4% | 0.0% | 0.0% | 0.1% |  |  |  |
| Class C | 0.1% | 0.1% | 0.3% | 0.0% | 0.5% | 0.3% |  |  |  |
| Class D | 0.0% | 0.3% | 0.1% | 0.0% | 1.3% | 0.6% |  |  |  |
| Class E | 0.1% | 0.3% | -0.1% | 0.0% | -0.6% | -0.3% |  |  |  |
| Overall | 0.0% | 0.2% | 0.2% | 0.0% | 0.4% | 0.2% |  |  |  |
|  | 0.0% | 0.2% | 0.2% | 0.0% | 0.3% | 0.3% |  |  |  |
| Class F | -0.1% | -0.2% | -0.4% | 0.3% | -0.5% | 0.0% |  |  |  |
| Enc Time[%] | 98% |  |  | 102% |  |  |  |  |  |
| Dec Time[%] | 96% |  |  | 94% |  |  |  |  |  |

The results for Test 1 for QP=(17,22,27,32) are shown below:

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 101% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 95% |  |  | 95% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | -0.1% |
| Class C | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.2% |  |  |  |
| Class D | -0.1% | 0.1% | 0.2% | 0.0% | 0.3% | 0.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.1% | 0.1% | 0.0% | 0.2% | 0.1% | 0.0% | -0.1% | 0.0% |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.2% | 0.1% | 0.0% | -0.1% | 0.0% |
| Class F | -0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 98% |  |  | 100% |  |  | 96% |  |  |
| Dec Time[%] | 96% |  |  | 96% |  |  | 96% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.2% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class C | 0.0% | 0.1% | 0.3% | 0.0% | 0.4% | 0.3% |  |  |  |
| Class D | 0.0% | 0.4% | 0.2% | 0.0% | 1.0% | 0.6% |  |  |  |
| Class E | 0.1% | 0.0% | 1.0% | 0.0% | 0.1% | 0.3% |  |  |  |
| Overall | 0.0% | 0.2% | 0.4% | 0.0% | 0.4% | 0.3% |  |  |  |
|  | 0.0% | 0.2% | 0.3% | 0.0% | 0.4% | 0.3% |  |  |  |
| Class F | -0.1% | -0.1% | -0.2% | 0.3% | 0.2% | 0.2% |  |  |  |
| Enc Time[%] | 100% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 96% |  |  | 95% |  |  |  |  |  |

The results for Test 1 for QP=(27,32,37,42) are shown below:

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.1% | -0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | -0.1% | 0.1% | 0.0% | 0.1% | -0.1% |  |  |  |
| Enc Time[%] | 101% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 96% |  |  | 94% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -0.1% | -0.3% | 0.0% | 0.3% | 0.0% | 0.0% | 0.3% | 0.8% |
| Class B | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | -0.1% |
| Class C | 0.0% | -0.3% | 0.0% | 0.0% | 0.1% | -0.1% |  |  |  |
| Class D | 0.0% | 0.0% | 0.1% | -0.1% | 0.6% | -0.4% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.2% | -0.1% | 0.0% | 0.0% | 0.3% |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.2% | -0.1% | 0.0% | 0.0% | 0.2% |
| Class F | 0.0% | 0.1% | -0.1% | 0.0% | -0.2% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 95% |  |  |
| Dec Time[%] | 97% |  |  | 98% |  |  | 97% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.1% | 0.2% | 0.6% | 0.0% | -0.1% | 0.6% |  |  |  |
| Class C | 0.1% | 0.2% | 0.1% | 0.0% | 0.5% | 0.1% |  |  |  |
| Class D | 0.0% | -0.6% | -0.4% | 0.0% | 1.0% | 0.4% |  |  |  |
| Class E | 0.0% | 1.0% | -2.1% | 0.1% | -2.7% | -0.7% |  |  |  |
| Overall | 0.0% | 0.1% | -0.3% | 0.0% | -0.2% | 0.2% |  |  |  |
|  | 0.0% | 0.2% | -0.3% | 0.0% | -0.1% | 0.1% |  |  |  |
| Class F | -0.1% | -0.3% | -0.9% | 0.1% | -1.4% | -0.9% |  |  |  |
| Enc Time[%] | 99% |  |  | 102% |  |  |  |  |  |
| Dec Time[%] | 97% |  |  | 95% |  |  |  |  |  |

The results for Test 3 for QP=(22,27,32,37) are shown below:

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.2% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | -0.3% | 0.0% | 0.2% | -0.1% | 0.0% | -0.1% | 0.4% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | -0.1% | -0.1% | 0.1% | 0.0% | 0.2% | -0.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.2% |
| Class F | -0.1% | -0.2% | 0.1% | 0.0% | -0.1% | -0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  | 98% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.2% | 0.0% | 0.1% | 0.6% |  |  |  |
| Class C | 0.0% | 0.2% | 0.1% | 0.0% | 0.3% | 0.0% |  |  |  |
| Class D | 0.0% | -0.5% | -0.3% | 0.0% | 0.9% | 0.5% |  |  |  |
| Class E | 0.1% | 0.1% | -0.1% | -0.1% | 0.4% | -0.4% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.4% | 0.2% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.3% | 0.2% |  |  |  |
| Class F | -0.2% | -0.3% | -0.3% | -0.1% | -0.7% | -0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |  |  |  |

The results for Test 3 for QP=(17,22,27,32) are shown below:

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% | 0.3% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class D | -0.1% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | -0.1% | -0.1% | 0.1% | 0.0% | -0.1% | -0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 99% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.2% | 0.4% |  |  |  |
| Class C | 0.0% | 0.1% | 0.2% | 0.0% | 0.3% | 0.2% |  |  |  |
| Class D | 0.0% | -0.2% | -0.2% | 0.0% | 0.6% | 0.3% |  |  |  |
| Class E | 0.1% | -0.1% | 0.6% | -0.1% | 0.4% | 0.1% |  |  |  |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.4% | 0.3% |  |  |  |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.4% | 0.3% |  |  |  |
| Class F | -0.1% | -0.1% | -0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

The results for Test 3 for QP=(27,32,37,42) are shown below:

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | -0.1% |  |  |  |
| Class E | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | 0.1% | 0.2% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -0.1% | -0.2% | 0.0% | 0.1% | -0.1% | 0.0% | 0.2% | 0.4% |
| Class B | -0.1% | 0.0% | 0.2% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| Class D | 0.0% | -0.3% | -0.3% | -0.1% | -0.1% | -0.3% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | 0.1% |
|  | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class F | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 99% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 98% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.4% | 0.0% | -0.3% | 0.3% |  |  |  |
| Class C | 0.0% | 0.1% | -0.1% | 0.0% | 0.2% | -0.4% |  |  |  |
| Class D | 0.1% | -1.1% | -0.8% | 0.0% | 0.8% | 0.8% |  |  |  |
| Class E | 0.1% | -0.1% | -1.0% | -0.1% | -0.8% | -0.3% |  |  |  |
| Overall | 0.0% | -0.2% | -0.3% | 0.0% | 0.0% | 0.1% |  |  |  |
|  | 0.0% | -0.4% | -0.3% | 0.0% | 0.1% | 0.2% |  |  |  |
| Class F | -0.2% | -0.8% | -0.7% | -0.2% | -1.9% | -1.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

These results match those provided by the proponents.

# Conclusions

We have cross-checked CE1 Subtest B2 Test 1 and Test 3, studied the algorithm, and verified the coding simulation results. We also note that the current initialization method requires the use of 3 look-up tables.

# References

[1] R. Joshi et al., “Description of Core Experiment 1: Entropy Coding,” JCTVC-G1201, Geneva, Switzerland, Nov. 2011.

[2] Frank Bossen, “Common test conditions and software reference configurations,” JCTVC-G1200, Geneva, Switzerland, Nov. 2011.

Institute for Infocomm Research may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).