| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0077 |
| --- | --- |

| Title: | Non-CE9: Merge candidates pruning without reference index |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shigeru Fukushima Kazumi Arakage JVC KENWOOD Corporation 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-6597 fukushima.shigeru@ jvckenwood.com someya.kazumi@ jvckenwood.com |
| Source: | JVC KENWOOD Corporation |  |  |

_____________________________

# Abstract

In the pruning process of the HM5.0, redundant candidates among the spatial and temporal candidates in the merge list are eliminated by comparing each motion vector, reference indices and prediction mode. In this proposal, reference indices and prediction mode are not compared, and only motion vectors are used for a decision whether the candidate is redundant or not. The simulation results report that the proposed technique provides less than 0.1% loss for all configurations.

# Introduction

Merge is one of the techniques to reduce motion information. In the HM5.0[1], the pruning process are invoked for elimination of redundant candidates from the merge list. In the pruning process, motion vectors and reference indices according to prediction mode of all merge candidates are compared each other. Therefore, many comparison operations are needed to decide whether the candidate is redundant and it makes increase of complexity of the pruning process.

# Algorithm Description

## Current algorithm (HM5.0)

The merge candidate pruning process of the HM5.0 is illustrated in Figure 1 where CurrInterDir, RefIdxLX and CurrMv are prediction mode, reference index and motion vector of the candidate comparing in the pruning processing respectively, OthrRefIdxLX and OthrMv are reference index and motion vector of the other candidate for comparison respectively.

Figure 1 pruning process in the HM5.0

## Proposed algorithm

The proposed algorithm aims to reduce complexity of elimination of the redundant candidates with the pruning process of merge candidates in the HM5.0. For this purpose, it is proposed that reference indices and prediction mode are not compared, and only motion vectors are compared for a decision whether the candidate is redundant or not. Even if the prediction mode of current candidate is not Pred_Bi, motion vector of L0 and L1 are compared as Pred_Bi in B-Slice. Since the motion vector of the list which is not used for reference is set to (0,0), zero motion vector is compared. In P-Slice, only motion vector of L0 is compared.

Figure 2 illustrates the flowchart of the pruning process for merge candidates in the proposed algorithm.

By this modification, following effects can be obtained.

- Reduce comparison between the candidates in the pruning process.

- Share logic with AMVP pruning process.

- Simplify software.

- Simplify WD.

Figure 2 pruning process in this proposal

## Implementation

### Software

This proposed technique is implemented into HM5.0 software.

### Syntax structure

The syntax of this proposed technique is not revised from HM5.0.

# Experiments

## Test conditions

The common test conditions [2] are used.

## Experimental environment

The experimental environment used is presented in Table1.

Table 1 Experimental environment

| CPU | Intel Core i7-2600K 3.40GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Notes: The decoding time is measured with the writing of yuv file.

## Simulation results

Table2 and Table3 present the simulation results for intra only, random access and low delay B, respectively.

Table 2 Simulation results for random access

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.04% | 0.10% | -0.04% | 0.04% | 0.07% | 0.02% | 0.06% | 0.03% | 0.08% |
| Class B | 0.05% | 0.06% | -0.01% | 0.03% | 0.02% | 0.02% | 0.05% | 0.07% | 0.08% |
| Class C | 0.07% | 0.06% | 0.04% | 0.08% | 0.14% | 0.15% |  |  |  |
| Class D | 0.02% | -0.10% | -0.05% | 0.02% | 0.00% | 0.05% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.05% | 0.02% | -0.01% | 0.04% | 0.05% | 0.06% | 0.06% | 0.05% | 0.08% |
|  | 0.05% | 0.02% | -0.01% | 0.04% | 0.05% | 0.06% | 0.06% | 0.06% | 0.09% |
| Class F | 0.04% | 0.04% | 0.05% | 0.10% | 0.13% | 0.12% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  | 100% |  |  |

Table 3 Simulation results for low delay B

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.06% | 0.13% | 0.36% | 0.06% | -0.14% | 0.36% |
| Class C | 0.07% | -0.05% | 0.08% | -0.02% | 0.09% | 0.10% |
| Class D | -0.01% | -0.19% | 0.09% | -0.03% | 0.54% | -0.10% |
| Class E | 0.17% | -0.02% | -0.10% | 0.10% | -0.25% | 0.03% |
| Overall | 0.06% | -0.02% | 0.14% | 0.03% | 0.07% | 0.12% |
|  | 0.06% | -0.01% | 0.10% | 0.03% | 0.09% | 0.12% |
| Class F | -0.02% | 0.20% | -0.19% | 0.02% | 0.20% | -0.12% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusions

This contribution reports that the proposed technique can reduce complexity of the merge pruning process without coding loss. Therefore, we recommend that the proposed technique be adopted to the CD and HM.

# References

[1] “WD5: Working Draft 5of High-Efficiency Video Coding”,

JCTVC-G, Geneva, November 2011

[2] “Common test conditions and software reference configurations”,

JCTVC-G1200, Geneva, November 2011

# Patent rights declaration(s)

JVC KENWOOD Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD changes

##### 8.4.2.1.1  Derivation process for luma motion vectors for merge mode

This process is only invoked when PredMode is equal to MODE_SKIP or PredMode is equal to MODE_ INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

–	the luma motion vectors mvL0 and mvL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags predFlagL0 and predFlagL1.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps:

- The derivation process for merging candidates from neighboring prediction unit partitions in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH and the partition index PartIdx as inputs and the output is assigned to the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N and the motion vectors mvL0N and mvL1N with N being replaced by A0, A1, B0, B1 or B2.

- The reference index for temporal merging candidate is derived as follows..

- If the prediction unit covering luma location ( xP − 1, yP + nPSH − 1 ) is available and PredMode is not MODE_INTRA, refIdxLX is set equal to refIdxLX[ xP − 1, yP + nPSH − 1 ].

- Otherwise, refIdxLX is set equal to 0.

- The derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), refIdxLX as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector mvLXCol. The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

availableFlagCol = availableFlagL0Col || availableFlagL1Col	(8-1)

predFlagLXCol = availableFlagLXCol	(8-2)

- The merging candidate list, mergeCandList, is constructed as follows.

  - A1, if availableFlagA1 is equal to 1

  - B1, if availableFlagB1 is equal to 1

  - B0, if availableFlagB0 is equal to 1

  - A0, if availableFlagA0 is equal to 1

  - B2, if availableFlagB2 is equal to 1

  - Col, if availableFlagCol is equal to 1

- When merging candidates have the same motion vectors and the same reference indices, the merging candidates are removed from the list except the merging candidate which has the smallest order in the mergeCandList.