| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0078 |
| --- | --- |

| Title: | Non-CE9: Simplification of AMVP spatial candidate derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shigeru Fukushima Hiroya Nakamura Hideki Takehara JVC KENWOOD Corporation 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-6597 fukushima.shigeru@ jvckenwood.com nakamura.hiroya@ jvckenwood.com takehara.hideki@ jvckenwood.com |
| Source: | JVC KENWOOD Corporation |  |  |

_____________________________

# Abstract

In the 7th JCT-VC meeting, simplified AMVP candidate derivation process is adopted. However, there is still dependency between the group A (Left) and the group B (Above) in the HM5.0.

In this proposal, AMVP scaling candidate derivation of the group B is removed. The scaling candidate is never derived from the group B.

The simulation results report that the proposed technique provides less than 0.1% BD-rate loss for all configurations.

# Algorithm Description

## Current algorithm (HM5.0)

In the 7th JCT-VC meeting, simplified AMVP candidate derivation process is adopted [1]. However, there is still dependency between the group A (Left) and the group B (Above) in the HM5.0 [2]. AMVP candidate derivation for the group B needs to check whether the group A has at least one inter coded PU or not.

Figure 1 illustrates AMVP scaling candidate derivation in the HM5.0.

Figure 1 AMVP scaling candidate derivation in the HM5.0

## Proposed algorithm

In this proposal, AMVP scaling candidate derivation of the group B is removed. The scaling candidate is never derived from the group B. Figure 2 illustrates AMVP scaling candidate derivation in this proposal.

By this change, following effects can be obtained.

- Complete parallelization of AMVP derivation process for the group A and the group B

- Smoothing of the computational complexity for the group A and the group B (The group A consists of 2 neighbors with scaling calculation. The group B consists of 3 neighbors without scaling calculation.)

- WD simplified (changes are sentence removal only)

- Software simplified (changes are code removal only)

Figure 2 AMVP scaling candidate derivation in this proposal

## Implementation

### Software

This proposed technique is implemented into HM5.0 software.

### Syntax structure

The syntax of this proposed technique is not revised from HM5.0.

# Experiments

## Test conditions

The common test conditions [3] are used.

## Experimental environment

The experimental environment used is presented in Table1.

Table 1 Experimental environment

| CPU | Intel Core i7-2600K 3.40GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Notes: The decoding time is measured with the writing of yuv file.

## Simulation results

Table2, Table3, and Table4 present the simulation results for intra only, random access and low delay B, respectively.

Table 2 Simulation results for random access

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.05% | 0.13% | 0.03% | 0.05% | 0.13% | 0.13% | 0.07% | 0.05% | 0.13% |
| Class B | 0.04% | 0.06% | 0.03% | 0.07% | 0.10% | 0.01% | 0.05% | 0.02% | 0.07% |
| Class C | 0.06% | 0.13% | 0.09% | 0.09% | 0.13% | 0.17% |  |  |  |
| Class D | 0.05% | -0.01% | -0.07% | 0.07% | 0.12% | 0.04% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.05% | 0.07% | 0.02% | 0.07% | 0.11% | 0.08% | 0.06% | 0.03% | 0.10% |
|  | 0.05% | 0.07% | 0.02% | 0.07% | 0.11% | 0.07% | 0.06% | 0.02% | 0.11% |
| Class F | -0.01% | 0.02% | 0.02% | 0.06% | 0.04% | 0.07% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  | 100% |  |  |

Table 3 Simulation results for low delay B

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.01% | 0.03% | 0.08% | 0.01% | -0.17% | 0.26% |
| Class C | 0.05% | -0.10% | 0.09% | 0.01% | 0.21% | 0.05% |
| Class D | 0.08% | -0.25% | -0.03% | -0.04% | -0.16% | 0.11% |
| Class E | 0.03% | -0.61% | -0.07% | 0.07% | -0.48% | 0.21% |
| Overall | 0.04% | -0.19% | 0.03% | 0.01% | -0.13% | 0.16% |
|  | 0.04% | -0.23% | 0.00% | 0.01% | -0.07% | 0.16% |
| Class F | -0.04% | 0.11% | -0.17% | -0.06% | -0.04% | -0.53% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusions

This contribution reports the proposed technique can reduce complexity of AMVP derivation process without coding loss.

Therefore, we recommend that the proposed technique be adopted to the CD and HM.

# References

[1] “Non-CE9/Non-CE13: Simplification on AMVP/Merge”,

JCTVC-G542, Geneva, November 2011

[2] “WD5: Working Draft 5of High-Efficiency Video Coding”,

JCTVC-G1103, Geneva, November 2011

[3] “Common test conditions and software reference configurations”,

JCTVC-G1200, Geneva, November 2011

# Patent rights declaration(s)

JVC KENWOOD Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD changes

##### 8.4.2.1.7  Derivation process for motion vector predictor candidates

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Outputs of this process are (with N being replaced by A, or B)

- the motion vectors mvLXN of the neighbouring prediction units,

- the availability flags availableFlagLXN of the neighbouring prediction units.

Figure 8-2 – Spatial motion vector neighbours

The variable isScaledFlagLX with X being 0 or 1 is set equal to 0.

The motion vector mvLXA and the availability flag availableFlagLXA are derived in the following ordered steps:

- Let a set of two sample locations be (xAk, yAk), with k = 0, 1, specifies sample locations with xAk = xP – 1, yA0 = yP + nPSH and yA1 = yA0 - MinPuSize. The set of sample locations ( xAk, yAk ) represent the sample locations immediately to the left side of the left partition boundary and it’s extended line.

- Let the availability flag availableFlagLXA be initially set equal to 0 and the both components of mvLXA are set equal to 0.

- If one of the following conditions is true, the variable isScaledFlagLX is set equal to 1.

- the prediction unit covering luma location ( xA0, yA0 ) is available and PredMode is not MODE_INTRA.

- the prediction unit covering luma location ( xA1, yA1 ) is available and PredMode is not MODE_INTRA.

- For ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 - MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

- If the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1 and the reference index refIdxLX[ xAk ][ yAk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXA is set equal to 1 and the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ] and ListA is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1 and PicOrderCnt( RefPicListLY( refIdxLY[ xAk ][ yAk ] ) ) is equal to PicOrderCnt( RefPicListLX( refIdxLX ) ), availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ] ,  ListA is set equal to LY and mvLXA is set equal to mvLXA.

- When availableFlagLXA is equal to 0, for ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 - MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

- If the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ], ListA is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ], ListA is set equal to LY.

- When availableFlagLXA is equal to 1, mvLXA is derived as specified below.

tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td	(8-1)

DistScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )	(8-2)

mvLXA = Sign( DistScaleFactor * mvLXA ) * 			
		( (Abs( DistScaleFactor * mvLXA ) + 127 ) >> 8 )	(8-3)

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCntVal – PicOrderCnt( RefPicListListA( refIdxA ) ) )	(8-4)

tb = Clip3( -128, 127, PicOrderCntVal – PicOrderCnt( RefPicListLX( refIdxLX ) ) )	(8-5)

The motion vector mvLXB and the availability flag availableFlagLXB are derived in the following ordered steps:

- Let a set of three sample location (xBk, yBk), with k = 0,1,2, specifies sample locations with xB0 = xP + nPSW, xB1 = xB0 MinPuSize , xB2 = xP - MinPuSize and yBk = yP – 1. The set of sample locations ( xBk, yBk ) represent the sample locations immediately to the upper side of the above partition boundary and it’s extended line. [Ed.: (WJ) MinPuSize should be defined somewhere]

- If yP-1 is smaller than YLCU, the following applies.

xB0 = (xB0>>Log2MinCUSize)<<Log2MinCUSize) + ((xB0>>Log2MinCUSize)&1)*(MinPuSize*2-1)	(8-6)
xB1 = (xB1>>Log2MinCUSize)<<Log2MinCUSize) + ((xB1>>Log2MinCUSize)&1)*(MinPuSize*2-1)	(8-7)
xB2 = (xB2>>Log2MinCUSize)<<Log2MinCUSize) + ((xB2>>Log2MinCUSize)&1)*(MinPuSize*2-1)	(8-8)

where MinPuSize = 1 << ( Log2MinCUSize – 1)

- Let the availability flag availableFlagLXB be initially set equal to 0 and the both components of mvLXB are set equal to 0.

- For ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP +nPSW, xB1 = xB0 - MinPuSize , and xB2 =  xP - MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

- If the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, and the reference index refIdxLX[ xBk ][ yBk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXB is set equal to 1 and the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ] and ListB is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1 and PicOrderCnt( RefPicListLY( refIdxLY[ xBk ][ yBk ] ) ) is equal to PicOrderCnt( RefPicListLX( refIdxLX ) ), availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ],  refIdxB is set equal to refIdxLY[ xBk ][ yBk ],  and ListB is set equal to LY.

- When isScaledFlagLX is equal to 0, availableFlagLXB is set equal to 0 and for ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP +nPSW, xB1 = xB0 - MinPuSize , and xB2 =  xP - MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

- If the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ], ListB is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ], refIdxB is set equal to refIdxLY[ xBk ][ yBk ], ListB is set equal to LY.

- When availableFlagLXB is equal to 1 and PicOrderCnt( RefPicListListB( refIdxB ) ) is not equal to PicOrderCnt( RefPicListLX( refIdxLX ) ), mvLXB is derived as specified below.

tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td	(8-9)

DistScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )	(8-10)

mvLXB = Sign( DistScaleFactor * mvLXA ) * 			
		( (Abs( DistScaleFactor * mvLXA ) + 127 ) >> 8 )	(8-11)

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCntVal – PicOrderCnt( RefPicListListB( refIdxB ) ) )	(8-12)

tb = Clip3( -128, 127, PicOrderCntVal – PicOrderCnt( RefPicListLX( refIdxLX ) ) )	(8-13)