| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0079 |
| --- | --- |

| Title: | Non-CE6.c: Unified neighboring positions for intra mode coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shigeru Fukushima Hiroya Nakamura JVC KENWOOD Corporation 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-6597 fukushima.shigeru@ jvckenwood.com nakamura.hiroya@ jvckenwood.com |
| Source: | JVC KENWOOD Corporation |  |  |

_____________________________

# Abstract

In this proposal, intra neighboring positions are changed to be matched with inter neighboring positions.

The simulation results report that the proposed technique provides 0.04% BD-rate gain for intra only settings.

# Algorithm Description

## Current algorithm (HM5.0)

In the 6th JCT-VC meeting, unification of merge/AMVP spatial neighboring positions [1] is adopted. However neighboring positions for intra mode coding are still not unified in the HM5.0 [2]. Figure 1 illustrates the intra neighboring positions in the HM5.0.

Figure 1 Neighboring positions in the HM5.0

## Proposed algorithm

Intra neighboring positions are changed to be matched with inter neighboring positions. Figure 2 illustrates the intra neighboring positions in this proposal.

By this change, following effects can be obtained.

- In encoder, memory access to obtain neighboring information can be reduced.

- In decoder, neighboring information can be obtained without depending on intra/inter mode.

Figure 2 Intra neighboring positions in this proposal

## Implementation

### Software

This proposed technique is implemented into HM5.0 software.

### Syntax structure

The syntax of this proposed technique is not revised from HM5.0.

# Experiments

## Test conditions

The common test conditions [2] are used.

## Experimental environment

The experimental environment used is presented in Table1.

Table 1 Experimental environment

| CPU | Intel Core i7 870 2.93GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Notes: The decoding time is measured with the writing of yuv file.

## Simulation results

Table2 presents the simulation results for intra only.

Table 2 Simulation results for intra only

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.05% | -0.08% | -0.03% | -0.03% | -0.04% | -0.04% |
| Class B | -0.04% | -0.05% | -0.04% | -0.03% | 0.00% | 0.00% |
| Class C | -0.04% | -0.07% | -0.04% | -0.03% | -0.02% | -0.02% |
| Class D | -0.01% | 0.05% | -0.01% | 0.00% | 0.00% | -0.02% |
| Class E | -0.06% | -0.09% | -0.06% | -0.06% | -0.03% | -0.08% |
| Overall | -0.04% | -0.04% | -0.03% | -0.03% | -0.02% | -0.03% |
|  | -0.04% | -0.04% | -0.03% | -0.03% | -0.02% | -0.03% |
| Class F | -0.09% | -0.13% | -0.07% | -0.05% | 0.15% | -0.14% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

# Conclusions

This contribution reports the proposed technique can reduce complexity of both encoding and decoding. Furthermore, the proposed technique can bring coding gain.

Therefore, we recommend that the proposed technique be adopted to the CD and HM.

# References

[1] “WD5: Working Draft 5of High-Efficiency Video Coding”,

JCTVC-G, Geneva, November 2011

[2] “Common test conditions and software reference configurations”,

JCTVC-G1200, Geneva, November 2011

# Patent rights declaration(s)

JVC KENWOOD Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD changes

### 8.3.1  Derivation process for luma intra prediction mode

Inputs to this process are:

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

–	a variable log2PUSize specifying the size of the current prediction unit,

–	variable arrays IntraPredMode (If available) that are previously (in decoding order) derived for adjacent coding units.

Output of this process is the variable IntraPredMode[ xB ][ yB ].

Table 8-3 specifies the value for the intra prediction mode and the associated names.

Table 8-3 – Specification of intra prediction mode and associated names

| Intra prediction mode | Associated names |
| --- | --- |
| 0 | Intra_Planar |
| 1 | Intra_Vertical |
| 2 | Intra_Horizontal |
| 3 | Intra_DC |
| Otherwise (4..34) | Intra_Angular |
| 35 | Intra_FromLuma (used only for chroma) |

Table 8-4 specifies the number of luma intra prediction modes intraPredModeNum depending on log2PUSize.

Table 8-4 – Specification of intraPredModeNum

| log2PUSize | intraPredModeNum |
| --- | --- |
| 2 | 18 |
| 3 | 35 |
| 4 | 35 |
| 5 | 35 |
| 6 | 35 |

Error: Reference source not found specifies the mapping table used for converting the number of intra prediction modes.

IntraPredMode[ xB ][ yB ] labelled 0, 1, 2, .., 34 represents directions of predictions as illustrated in Figure 8-3.

Figure 8-3 – Intra prediction mode directions (informative)

IntraPredMode[ xB ][ yB ] is derived as the following ordered steps. [Ed. (WJ): proponent suggests to move this part to the syntax since the other syntax elements utilize IntraPredMode. But it seems too complex to move all the following process to the syntax table. Maybe it’s better to move this part to the semantics section or simply avoid the use of IntraPredMode to parse the syntax item]

- The derivation process for neighbouring treeblocks specified in subclause XXX with ( xB,  yB ) given as input and the output is assigned to tbAddrA and tbAddrB specifying the treeblock addresses of treeblocks covering ( xBA,  yBA ) and ( xBB, yBB ) respectively where ( xBA,  yBA ) is set equal to ( xB-1,  yB ) and ( xBB,  yBB ) is set equal to ( xB,  yB-1 ).

( xBA,  yBA ) is set equal to ( xB-1,  yB+(1<<log2PUSize)-1 ) and ( xBB,  yBB ) is set equal to ( xB+(1<<log2PUSize)-1,  yB-1 ).