| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0081 |
| --- | --- |

| Title: | Non-CE6.c: Fixed length binarization of remaining intra prediction mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Toru Kumakura Shigeru Fukushima Motoharu Ueda JVC KENWOOD Corporation 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-5798 kumakura.toru@ jvckenwood.com fukushima.shigeru@ jvckenwood.com ueda.motoharu@jvckenwood.com |
| Source: | JVC KENWOOD Corporation |  |  |

_____________________________

# Abstract

In the HM5.0, parsing process for intra luma prediction mode has branch process by variable length binarization if prev_intra_luma_pred_mode is equal to 0.

In this proposal, intra luma prediction mode 34 is removed to use fixed length binalization for remaining intra luma prediction mode.

The simulation results report that the proposed technique provides no BD-rate loss for all intra high efficiency setting and all intra low complexity setting.

# Algorithm Description

## Current algorithm (HM5.0)

Figure 1 illustrates the definition of intra luma prediction mode in the HM 5.0. For 4x4 blocks, mode 0 to 17 are defined. As the number of most probable mode is always two, the total number of rem_intra_luma_pred_mode is equal to 16 = 24 like illustrated in Table 1. Thus, rem_intra_luma_pred_mode can be always binarized to 4 bins fixed For 8x8 / 16x16 / 32x32 / 64x64 blocks, mode 0 to 34 are defined and the total number of rem_intra_luma_pred_mode is equal to 33 = 25 + 1 like illustrated in Table 1. Thus, rem_intra_luma_pred_mode is coded to 5 bins or 6 bins variable. If the value of first 5 bins is 31("11111"), additional read process is needed like illustrated in Figure 2.

Figure 1 Intra luma prediction modes in HM 5.0

Table 1 The number of intra luma prediction modes in HM 5.0

| Part size | Num of modes(defined mode) | Num of rem_intra_luma_pred_mode |
| --- | --- | --- |
| 4x4 | 18(0 - 17) | 16 = 24 |
| 8x8 | 35(0 - 34) | 33 = 25 + 1 |
| 16x16 | 35(0 - 34) | 33 = 25 + 1 |
| 32x32 | 35(0 - 34) | 33 = 25 + 1 |
| 64x64 | 35(0 - 34) | 33 = 25 + 1 |

Figure 2 parsing process for rem_intra_luma_pred_mode if prev_intra_luma_pred_flag is equal to 1 (2Nx2N blocks)

## Proposed algorithm

Intra luma prediction mode 34 is removed from HM 5.0. The definition of intra luma prediction modes are illustrated in Figure 3. This proposal do not need to change the definition for 4x4 blocks. For 8x8 / 16x16 / 32x32 / 64x64 blocks, intra luma prediction mode 0 to 33 and the total number of intra luma prediction modes are reduced to 34. The number of rem_intra_luma_pred_mode is 32 = 25 like illustrated in Table 2 and rem_intra_luma_pred_mode can be always binarized to 5 bins fixed. The value of first 5 bins comparison and additional one more bin read process can be removed.

Figure 3 Intra luma prediction modes in proposal

Table 2 The number of intra luma prediction modes in proposal

| Part size | Num of modes(defined mode) | Num of rem_intra_luma_pred_mode |
| --- | --- | --- |
| 4x4 | 18(0 - 17) | 16 = 24 |
| 8x8 | 34(0 - 33) | 32 = 25 |
| 16x16 | 34(0 - 33) | 32 = 25 |
| 32x32 | 34(0 - 33) | 32 = 25 |
| 64x64 | 34(0 - 33) | 32 = 25 |

As removed mode is last element of intra luma prediction mode, the modification of the tables for mode dependent intra smoothing, mode dependent coefficient scanning and mode dependent transform are minimum(delete only).

## Implementation

### Software

This proposed technique is implemented into HM5.0 software.

### Syntax structure

The syntax of this proposed technique is not revised from HM5.0.

# Experiments

## Test conditions

The common test conditions [2] are used.

## Experimental environment

The experimental environment used is presented in Table 3.

Table 3 Experimental environment

| CPU | Intel Core i7 870 2.93GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Notes: The decoding time is measured with the writing of yuv file.

## Simulation results

Table 4 presents the simulation results for intra only.

Table 4 Simulation results for intra only

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

# Conclusions

This contribution reports the proposed technique can reduce complexity of intra mode parsing process.

Therefore, we recommend that the proposed technique be adopted to the CD and HM.

# References

[1] “WD5: Working Draft 5of High-Efficiency Video Coding”,

JCTVC-G, Geneva, November 2011

[2] “Common test conditions and software reference configurations”,

JCTVC-G1200, Geneva, November 2011

# Patent rights declaration(s)

JVC KENWOOD Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD changes

### 8.3.1  Derivation process for luma intra prediction mode

Inputs to this process are:

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

–	a variable log2PUSize specifying the size of the current prediction unit,

–	variable arrays IntraPredMode (If available) that are previously (in decoding order) derived for adjacent coding units.

Output of this process is the variable IntraPredMode[ xB ][ yB ].

Table 8-5 specifies the value for the intra prediction mode and the associated names.

Table 8-5 – Specification of intra prediction mode and associated names

| Intra prediction mode | Associated names |
| --- | --- |
| 0 | Intra_Planar |
| 1 | Intra_Vertical |
| 2 | Intra_Horizontal |
| 3 | Intra_DC |
| Otherwise (4..34) | Intra_Angular |
| 35 | Intra_FromLuma (used only for chroma) |

Table 8-6 specifies the number of luma intra prediction modes intraPredModeNum depending on log2PUSize.

Table 8-6 – Specification of intraPredModeNum

| log2PUSize | intraPredModeNum |
| --- | --- |
| 2 | 18 |
| 3 | 3534 |
| 4 | 3534 |
| 5 | 3534 |
| 6 | 3534 |

#### 9.2.2.8  Binarization process for rem_intra_luma_pred_mode

Input to this process is a request for the syntax element rem_intra_luma_pred_mode and cNumBins.

Output of this process is the binarization of the syntax element.

The binarization for rem_intra_luma_pred_mode is given by Table 9-7.

Table 9-7 – Binarization for rem_intra_luma_pred_mode

| Value of rem_intra_luma_pred_mode | Bin string |
| --- | --- |
| less than 32All | FL, cMax = cNumBins |
| 32 | 111110 |
| 33 | 111111 |