| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0085 M22957 |
| --- | --- |

| Title: | Non-CE6.c: Cross-verification of unified neighboring positions for intra mode coding (JCTVC-H0079 from JVC) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA | Tel: Email: : | +1-214-480-3816 zhou@ti.com |
| Source: | Texas Instruments Inc; |  |  |

_____________________________

# Abstract

This contribution reports cross-check results JVC’s proposal JCTVC-H0079 entitled “Unified neighboring positions for intra mode coding”.  In the current HM5.0, left and upper neighboring PU positions are used for predictive intra mode code. In the proposed algorithm those two positions are replaced with down-left and upper-right neighboring PU positions.  No loss was observed in RA-HE and RA-LC when compared to the current HM design.  The BD-rate results match the ones reported by proponents. The source code has been carefully checked to verify its consistency with the proposal description.

.

# Introduction

This contribution reports cross-check results for JVC’s proposal JCTVC-H0079 entitled “Unified neighboring positions for intra mode coding”.  In the proposed algorithm the left and upper neighboring PU positions for predictive intra mode coding is replaced by down-left and upper-right neighboring PU positions, respectively.

# Test Settings and Conditions

The simulations of this document have used HM4.0 software, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz 64 bits Linux machines, the common test conditions and reference configurations specified in [1] are followed. Simulation is carried out for AI-HE and AI-LC only.

# Experimental results

The experimental results are summarized in Table 1.  The proposed algorithm is slightly better than the current design (up to 0.1% gain).  The results match the ones reported by proponents. Please be advised runtime here is not accurate. The proposed algorithm does not have an impact on complexity when compared to the current design.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | -0.1% | -0.1% | -0.1% | -0.1% | 0.1% | -0.1% |
| Enc Time[%] | 82% |  |  | 83% |  |  |
| Dec Time[%] | 82% |  |  | 78% |  |  |

Table 1. Experimental results for AI-HE and AI-LC configurations

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-G1200, Geneva, CH, Nov. 2011.

[2]	B. Bross, W.-J. Han,  J.-R. Ohm, G. J. Sullivan, T. Wiegand  “High Efficiency Video Coding (HEVC) text specification Working Draft 5” JCT-VC Document, JCTVC-G1103, Geneva, CH, Nov. 2011

[3] S. Fukushima,  H. Nakamura (JVC Kenwood), “Non-CE6.c: Unified neighboring positions for intra mode coding”, JCT-VC Document, JCTVC-H0079, San Jose, CA, USA, Feb.  2012