| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0086 M22958 |
| --- | --- |

| Title: | Non-CE9: Cross-verification of modification of derivation process for MVP candidate in AMVP/Merge (JCTVC-H0216 from Panasonic) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA | Tel: Email: : | +1-214-480-3816 zhou@ti.com |
| Source: | Texas Instruments Inc; |  |  |

_____________________________

# Abstract

This contribution reports cross-check results for Panasonic’s proposal on “Modification of derivation process for MVP candidate in AMVP/Merge (JCTVC-H0216)”.  In the current HM5.0 design, the precision of scaled MVP (temporal and spatial) for merge/skip and AMVP is not properly defined , which may lead to encoder and decoder mismatch. To resolve this issue, two solutions are proposed in JCTVC-G0216: 1) Not appending MVP candidate which exceeds the specified ranges, and 2) clipping MVP candidate into the specified ranges. Both solutions do not cause coding loss. The BD-rate results match the ones reported by proponents. The source code has been carefully checked to verify its consistency with the proposal description.

.

# Introduction

In HM5.0, the MVP scaling is defined as

Int Mvx = (iScale * getHor() + 127 + (iScale * getHor() < 0)) >> 8;

Int Mvy = (iScale * getVer() + 127 + (iScale * getVer() < 0)) >> 8;

Where iScale is 13-bit. For 8K x2K video, the motion vectors are 14-bit, so after scaling, the scaled MVP could have 19-bit. If the scaled MVP is used as temporally co-located or spatial neighboring MVP and scaled again, the dynamic range of the scaled MVP can further increase. Therefore, certain restrictions are needed.

# Test Settings and Conditions

The simulations of this document have used HM4.0 software, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz 64 bits Linux machines, the common test conditions and reference configurations specified in [1] are followed. Two settings are verified:

- if scaled MVP exceeds 16-bit range, it is removed from the merge/skip and AMVP list derivation process

- if scaled MVP exceeds 16-bit range, it is clipped to 16-bit range [-32768:32767]

# Experimental results

The experimental results are summarized in Table 1 and Table 2.  No loss has been observed for both settings. The results match the ones reported by proponents. Please be advised runtime may here is not accurate.

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 90% |  |  | 95% |  |  | 98% |  |  |
| Dec Time[%] | 84% |  |  | 92% |  |  | 97% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 88% |  |  | 89% |  |  |  |  |  |
| Dec Time[%] | 81% |  |  | 80% |  |  |  |  |  |

Table 1. Experimental results for setting 1 (“removal”)

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 97% |  |  | 99% |  |  | 96% |  |  |
| Dec Time[%] | 97% |  |  | 100% |  |  | 93% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 94% |  |  | 92% |  |  |  |  |  |
| Dec Time[%] | 91% |  |  | 86% |  |  |  |  |  |

Table 2. Experimental results for setting 2  (“clipping”)

# Comments

From the simulation results it does not seem to be the case that the scaled MVP exceeds 16-bit in the common test conditions.

However, the issue raised in JCTVC-G0216 does exist. For simplicity of both in hardware and software implementation, and of working draft text it  might be preferred to simply clip the scaled MVP to 16-bit range independent of levels. 16-bit is enough to support UHD video resolutions.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-G1200, Geneva, CH, Nov. 2011.

[2]	B. Bross, W.-J. Han,  J.-R. Ohm, G. J. Sullivan, T. Wiegand  “High Efficiency Video Coding (HEVC) text specification Working Draft 5” JCT-VC Document, JCTVC-G1103, Geneva, CH, Nov. 2011

[3]  T. Sugio, T. Nishi, “ Non-CE9: Modification of derivation process for MVP candidate in AMVP/Merge”, JCT-VC Document, JCTVC-H0216, San Jose, CA, USA, Feb.  2012