| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0087 M22959 |
| --- | --- |

| Title: | Non-CE9: Cross-verification of removing PU dependency in TMVP reference index derivation (JCTVC-H0092 from LGE and Qualcomm) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA | Tel: Email: : | +1-214-480-3816 zhou@ti.com |
| Source: | Texas Instruments Inc; |  |  |

_____________________________

# Abstract

This contribution reports cross-check results for LGE and Qualcomm’s proposal on “removing PU dependency in TMVP reference index derivation (JCTVC-H0092)”.  In the current HM5.0, the TMVP reference index derivation for merge/skip depends on the left neighboring PU. In the proposed algorithm two options are advocated, 1) fixing the TMVP reference index always to 0; and 2) all the PUs of a CU share a same TMVP reference index, which is derived based on the down-left neighboring PU of the CU. No loss was observed for both cases.  The BD-rate results match the ones reported by proponents. The source code has been carefully checked to verify its consistency with the proposal description.

.

# Test Settings and Conditions

The simulations of this document have used HM4.0 software, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz 64 bits Linux machines, the common test conditions and reference configurations specified in [1] are followed. Two settings are verified:

- Fix the refIdx for temporal TMVP to 0.

- For a CU of 2Nx2N, all the PUs of the CU share a same refIdx for TMVP. The refIdx is determined  by the availability of the down-left neighboring PU A1 of the CU.  If A1 is available, refIdx is set to that of A1. Otherwise, it is set to equal to 0.

# Experimental results

The experimental results are summarized in Table 1 and Table 2.  No loss has been observed for both settings. The results match the ones reported by proponents. Please be advised runtime may not be accurate.

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | 0.1% |
| Class B | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| Class D | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% |
| Class F | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 97% |  |  | 98% |  |  | 100% |  |  |
| Dec Time[%] | 93% |  |  | 94% |  |  | 95% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.6% | 0.8% | 0.0% | 0.3% | 0.5% |  |  |  |
| Class C | 0.1% | 0.1% | 0.2% | 0.0% | 0.3% | 0.2% |  |  |  |
| Class D | 0.2% | 0.5% | 1.1% | 0.1% | 1.0% | 0.8% |  |  |  |
| Class E | 0.2% | 0.6% | 0.4% | 0.1% | 0.4% | 0.7% |  |  |  |
| Overall | 0.1% | 0.4% | 0.7% | 0.1% | 0.5% | 0.6% |  |  |  |
|  | 0.1% | 0.4% | 0.6% | 0.1% | 0.5% | 0.6% |  |  |  |
| Class F | -0.2% | -0.1% | -0.5% | -0.1% | -0.7% | -0.4% |  |  |  |
| Enc Time[%] | 99% |  |  | 98% |  |  |  |  |  |
| Dec Time[%] | 98% |  |  | 93% |  |  |  |  |  |

Table 1. Experimental results for TMVP refIdx fixed to 0

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.1% | -0.1% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 90% |  |  | 91% |  |  | 94% |  |  |
| Dec Time[%] | 85% |  |  | 84% |  |  | 88% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | -0.2% | 0.1% | 0.0% | 0.2% | 0.0% |  |  |  |
| Class D | 0.0% | 0.1% | 0.2% | 0.0% | 0.3% | 0.0% |  |  |  |
| Class E | 0.1% | -0.2% | -0.3% | 0.0% | -0.3% | -0.1% |  |  |  |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% |  |  |  |
| Class F | -0.1% | -0.2% | -0.5% | -0.2% | -0.5% | -0.7% |  |  |  |
| Enc Time[%] | 88% |  |  | 90% |  |  |  |  |  |

Table 2. Experimental results for 2Nx2N CU based TMVP refIdx derivation

# Comments

The current HM5.0 allows CU-level parallel merge/skip MVP list derivation for part mode of PART_2NxN, PART_2NxnU and PART_2NxnD. The proposed algorithm further enables CU-level parallel merge/skip MVP list derivation for part mode of PART_Nx2N, PART_nLx2N and PART_nRx2N  by making TMVP refIdx derivation independent of PUs of a same CU. Although the proposed algorithm improves parallelism, it does not make the merge/skip MVP list derivation fully parallel for SCUs because in PART_NxN mode the merge/skip MVP list derivation of second, third and fourth PU are still inter-dependent.

Fixing TMVP reference index to 0 appears to be simpler and shall consume less memory bandwidth for motion compensation and motion estimation in multiple reference frame prediction cases. The outliers for fixing TMVP refIdx to 0 should also be considered.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-G1200, Geneva, CH, Nov. 2011.

[2]	B. Bross, W.-J. Han,  J.-R. Ohm, G. J. Sullivan, T. Wiegand  “High Efficiency Video Coding (HEVC) text specification Working Draft 5” JCT-VC Document, JCTVC-G1103, Geneva, CH, Nov. 2011

[3]  Y. Jeon, B. Jeon (LG), V. Seregin, X Wang, J. Chen, M. Karczewicz (Qualcomm), “Non-CE9: Removing PU dependency in TMVP reference index derivation”, JCT-VC Document, JCTVC-H0092, San Jose, CA, USA, Feb.  2012