| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, CA, USA, 1 - 10 February, 2012 | Document: JCTVC-H0089 M22961 |
| --- | --- |

| Title: | AHG7: Feasibility study results on virtual motion compensation memory bandwidth verifier (VMBV) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyungjoon Kim Minhua Zhou Texas Instruments Inc., USA | Tel: Email: | +1-214-480-3816 h-kim@ti.com, zhou@ti.com, |
| Source: | Texas Instruments Inc |  |  |

_____________________________

# Abstract

# This contribution reports feasibility study results on virtual motion compensation memory bandwidth verifier (VMBV) aimed to lower the worst case motion compensation memory bandwidth requirements, which is proposed in JCTVC-G095. To impose VMBV constraints on bitstreams, a motion compensation memory bandwidth control algorithm is implemented in the encoder motion estimation of HM5.0 to guarantee that resulting bitstreams meet the memory bandwidth budget. The control algorithm is realized by adjusting number of reference frames and CU depth and is operated on a LCU by LCU basis. The memory bandwidth measurement tool used is the one described in JCTVC-C007.  In the experiment, the memory bandwidth budget was set to 4 bytes per pixel, there were 13 sequences in common test conditions consumes more bandwidth than the budget. The algorithm was able to bring all the sequences to meet VMBV constraints. However, some of sequences had relatively high loss (up to 5.3%). Although the control algorithm is simple and not mature, the results reveal that it is feasible to impose VMBV compliance without causing significant burden on the encoder. It is recommended to conduct the further feasibility study of VMBV model in AHG.

# Introduction

Memory access bandwidth for motion compensation is always a bottleneck for decoder design as described in JCTVC-G095 [1]. In evil cases, an encoder can generate a motion vector field in which the reference blocks for neighboring PUs are totally independent without any overlapping, or each PU uses different reference frames, assumption of the reference block caching model [4] is broken leading to much higher memory access bandwidth requirements. In this contribution, we present an example algorithm that controls motion compensation memory bandwidth in encoder side in order to show the practicality of VMBV.

# Algorithm description

In our analysis, the amount of memory bandwidth consumed by motion compensation depends on various factors such as similarity among motion vectors, CU depths, PU types, number of reference frames, etc. Among the many factors, the proposed algorithm controls memory bandwidth by reducing number of reference frames and disabling motion vectors for small CU sizes.

When virtual target memory access bandwidth per frame is given based on profile & levels, the algorithm controls memory bandwidth on LCU basis to meet the target bandwidth. When a target bandwidth per LCU, T, is given, the overall flow of the algorithm as follows:

- Initialize a variable for memory bandwidth balance, B, to T at beginning of a frame: B = T

- If B is negative, reduce number of reference frames and disable small CU sizes for the current LCU

- Encode the current LCU and measure the consumed memory bandwidth U

- Update B with U: B = B – U + T

- Repeat step 2 to 4 until completion of the current frame

The overall procedure is illustrated in Figure 1. To minimize encoding efficiency degradation, the algorithm determines number of reference frames and use of small CU sizes based on absolute value of B, |B|, in step 3. In the current implementation, if B < 0 and T < |B| <= 3*T, then maximum number of reference frames is limited to two and 8x8 CUs are disabled, and if B < 0 and |B| > 3*T, only one reference frame are used and 8x8 CUs are disabled.

If B < 0, limit number of references and CU depths

B = T

Encode current LCU and measure U

B = B – U – T

Repeat for all LCUs

Figure 1. Overall procedure of the algorithm

# Experimental results

The proposed algorithm is implemented in HM5.0 [2] and tested under common conditions [3]. The algorithm uses a reference block caching model described in JCTVC-C228 [4] for memory bandwidth measurement. The details are as follows:

- Capacity (cache size): 128 KB

- Line size: 64 bytes

- Organization: 4-way set-associative

- Replacement strategy: FIFO

- DDR alignment: 64 bits

- DDR burst size 256 bits

For 8-bit 4:2:0 YUV (without internal bi-depth increase) and B-type frame, we allocate 4 bytes per pixel as target memory bandwidth. Thus, target memory bandwidth per LCU, T, can be calculated with following equation:

T = frame width * frame height * 1.5 * 4 / number of LCUs.

In the first test, we measured motion compensation memory bandwidth [5] without the proposed control algorithm, i.e., memory bandwidth consumption is not controlled in motion estimation. In this case, encoding performance is exactly same as that of original HM5.0. There are 9 and 4 video sequences consuming more memory bandwidth than target budget in RA_HE and LD_HE cases, respectively. Table 1 shows the sequences, maximum memory bandwidth consumption and number of frames that exceed the target. In the test, maximum memory bandwidth for BQSquare is 5.94 bytes per pixel. It is expected that memory bandwidth requirement in the worst case would be much higher than 5.94. Without considering DDR alignment and burst size, the rough number for the worst case memory bandwidth is around 10 bytes per pixel (4x8 PU, 8-tap luma filter and 4-tap chroma filter).

Table 1. Memory bandwidth without control algorithm

|  |  | Maximum consumed bandwidth (Bytes) per pixel / Target bandwidth (Bytes) per pixel | Number frames that exceed the target / Total number of frames |
| --- | --- | --- | --- |
| RA_HE | Traffic | 5.05 / 4 | 10 / 150 |
|  | PeopleOnStreet | 4.33 / 4 | 13 / 150 |
|  | BQTerrace | 4.94 / 4 | 33 / 600 |
|  | PartyScene | 4.87 / 4 | 62 / 500 |
|  | RaceHorses(Class C) | 4.32 / 4 | 1 / 300 |
|  | BQSquare | 5.94 / 4 | 93 / 600 |
|  | BlowingBubbles | 5.92 / 4 | 46 / 500 |
|  | RaceHorses(Class D) | 4.04 / 4 | 1 / 300 |
|  | ChinaSpeed | 4.37 / 4 | 6 / 500 |
| LD_HE | BQTerrace | 4.56 / 4 | 61 / 600 |
|  | PartyScene | 4.61 / 4 | 158 / 500 |
|  | BQSquare | 5.66 / 4 | 259 /600 |
|  | BlowingBubbles | 4.79 / 4 | 105 / 500 |

By enabling the proposed algorithm, all sequences consume memory bandwidth within the budget. However, the control algorithm lowers encoding efficiency. Table 2 shows maximum memory bandwidth consumption and BD-rate increase. The current control algorithm is very simple and easy to implement, which however leads to high BD-rate increases in some sequences.  We expect that encoding efficiency loss can be reduced when the control algorithm reaches a mature stage.

Table 2. Memory bandwidth with control algorithm

|  |  | Maximum consumed bandwidth (Bytes) per pixel / Target bandwidth (Bytes) per pixel | BD-rate increase (%) |
| --- | --- | --- | --- |
| RA_HE | Traffic | 3.60 / 4 | 0.3 |
|  | PeopleOnStreet | 3.63 / 4 | 0.2 |
|  | BQTerrace | 3.65 / 4 | 0.3 |
|  | PartyScene | 3.59 / 4 | 1.3 |
|  | RaceHorses(Class C) | 3.79 / 4 | 0.0 |
|  | BQSquare | 3.94 / 4 | 2.7 |
|  | BlowingBubbles | 3.91 / 4 | 1.4 |
|  | RaceHorses(Class D) | 3.83 / 4 | 0.1 |
|  | ChinaSpeed | 3.47 / 4 | 4.4 |
| LD_HE | BQTerrace | 3.47 / 4 | 0.1 |
|  | PartyScene | 3.52 / 4 | 1.3 |
|  | BQSquare | 3.73 / 4 | 5.3 |
|  | BlowingBubbles | 3.70 / 4 | 1.1 |

The BD-rate performance for all the test sequences is provided in the attached spreadsheet. Note that decoder includes the memory bandwidth measurement tool, thus the decode runtime is higher.

# Conclusion

# In this contribution, we investigated the practicality of VMBV by implementing a memory bandwidth control algorithm to impose the VMBV constraint on the encoder side. The test results show that the it is feasible to generate VMBV compliant  bitstreams  by simple control of  motion estimation from LCU to  LCU. It is recommended to conduct the further feasibility study of VMBV model in AHG.

# References

[1] M. Zhou, “Virtual motion compensation memory bandwidth verifier,” JCT-VC Document, JCTVC-G095, Geneva, Switzerland, November 2011.

[2]	B. Bross, W.-J. Han,  J.-R. Ohm, G. J. Sullivan, T. Wiegand  “WD4: Working Draft 5 of High-Efficiency Video Coding,” JCT-VC Document, JCTVC-G1103, Geneva, Switzerland, November 2011

[3]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-G1200, Geneva, Switzerland, November 2011.

[4] M. Budagavi, “A simple cache model for measuring motion compensation bandwidth”, JCTVC document JCTVC-C228, Guangzhou, CN, 7-15 October, 2010

[5] K. Chono, T. Chujoh, C. Lim “JVT-VC AHG report: Memory compression”, JCTVC document JCTVC-C007, Guangzhou, CN, 7-15 October, 2010

# Patent rights declaration(s)

Texas Instruments, Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation |ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).