| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0091 |
| --- | --- |

| Title: | Parallel Merge Candidate Derivation for Inter_NxN partition type |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongjoon Jeon (LG) *, Byeongmoon Jeon (LG) *, Vadim Seregin (Qualcomm) ** Xianglin Wang (Qualcomm) ** Jianle Chen (Qualcomm) ** Marta Karczewicz (Qualcomm) ** * #221 Yangjae-dong, Seoucho-gu, Seoul 137-130, Korea **5775 Morehouse Dr San Diego, CA 92121 USA | Tel: Email: | *+82-10-9043-9419 **1-858-651-5135 yongjoon.jeon@lge.com bm.jeon@lge.com vseregin@qualcomm.com xianglin@qualcomm.com cjianle@qualcomm.com martak@qualcomm.com |
| Source: | LG, Qualcomm Inc. |  |  |

_____________________________

# Abstract

In JCTVC-G681 which was adopted at the last meeting, it was proposed to exclude the MVPs located in the first PU in a CU from the merge list candidate for the second PU in the same CU, when the CU has 2NxN, Nx2N or AMP partition mode. With the adoption, merge list construction can potentially be processed in parallel for all PUs in a same CU. However, JCTVC-G681 did not cover NxN partition which may exist in HM when inter 4x4 prediction is enabled. This contribution simply extends the idea in that proposal to NxN partition. Simulation results revealed that with inter 4x4 mode enabled, extending the idea to NxN partition shows average 0.0% loss.

# Introduction

In JCTVC-G681 [1] which was adopted at the last meeting, it was proposed to exclude the MVPs located in the first PU in a CU from the merge list candidate for the second PU in the same CU, when the CU has 2NxN, Nx2N or AMP partition mode. Such a modification removed the inter-PU dependency between PUs in a same CU under those partition modes and made it possible to derive the merge/skip list for each PU in parallel. Fig. 1 shows an example of this process.

Fig.1 Example of MVP exclusion from merge candidate list

This contribution simply extends such a modification to the case of NxN partition. For example, when inter 4x4 prediction is enabled (by setting the marco DISABLE_4x4_INTER equal to 0), the HM can also have NxN partitions.

# Proposed method

Fig.2 shows the spatial MVP candidates location for merge list of each PU when a CU is coded with NxN partition type.

Fig. 2. Cases when a PU has dependency with other PUs in the same CU in the spatial merging candidate derivation process for NxN partition mode.

This contribution proposes to exclude the MVPs located in a previous PU in the same CU (marked by red circle in Fig.2) in the merge candidate list derivation under NxN partition. This is same to the scheme for 2NxN, Nx2N, and AMP partitions in the current HM.

In the WD, the following changes are made in the derivation process for spatial merging candidates. The changes are marked in yellow.

- 8.4.2.1.2 Derivation process for spatial merging candidates

…

For the derivation of availableFlagN, with N being A0, A1, B0, B1 or B2 and ( xN, yN ) being ( xP – 1,  yP + nPSH ), ( xP − 1,  yP + nPSH − 1 ), ( xP + nPSW,  yP – 1 ), ( xP + nPSW − 1,  yP – 1 ) or ( xP – 1,  yP – 1 ), the following applies.

–	If one of the following conditions is true, the availableFlagN is set equal to 0, both components mvLXN are set equal to 0, refIdxLXN and predFlagLX[ xN, yN ] of the prediction unit covering luma location ( xN, yN ) are assigned respectively to mvLXN, refIdxLXN and predFlagLXN.

  - N is equal to B2 and availableFlagA0 + availableFlagA1 + availableFlagB0 + availableFlagB1 is equal to 4.

  - The prediction unit covering luma location ( xN, yN ) is not available or PredMode is MODE_INTRA.

  - PartMode of the current prediction unit is PART_2NxN or PART_2NxnU or PART_2NxnD and PartIdx is equal to 1 and N is equal to B1

  - PartMode of the current prediction unit is PART_Nx2N or PART_nLx2N or PART_nRx2N and PartIdx is equal to 1 and N is equal to A1

  - PartMode of the current prediction unit is PART_NxN and PartIdx is equal to 1 and N is equal to A0 or A1

  - PartMode of the current prediction unit is PART_NxN and PartIdx is equal to 2 and N is equal to B0 or B1

  - PartMode of the current prediction unit is PART_NxN and PartIdx is equal to 3 and N is equal to A1 or B1 or B2

–	Otherwise, availableFlagN is set equal to 1 and the variables mvLX[ xN, yN ], refIdxLX[ xN, yN ] and predFlagLX[ xN, yN ] of the prediction unit covering luma location ( xN, yN ) are assigned respectively to mvLXN, refIdxLXN and predFlagLXN.

# Experimental results

In the simulation, partition NxN is enabled by setting DISABLE_4x4_INTER equal to 0 in HM5.0. The bug fix filed in ticket-265 is also applied in anchor software to solve the decoder crash problem when enabling partition_NxN mode in HM5.0. The following two software versions are tested with the common test condition and reference configuration [2] to check the performance.

- Anchor : HM5.0 + (setting DISABLE_4x4_INTER to 0) + Ticket-265

- Tested : Anchor + proposed method

Tab.1 shows the simulation results. Almost no coding performance change is found for all configurations in the common test condition when extending the existing parallel merge list derivation to NxN partition.

Tab.1 Simulation results for the proposed method

# Conclusion and recommendations

This contribution proposes to apply to NxN partition type the same merge list derivation scheme that is used for all other partition types in current HM. Simulation results revealed that the proposed change to  merge list derivation under NxN partition mode does not incur obvious loss to coding efficiency. It is recommended to adopt the proposed method into the next version of HM.

# References

[1] Y. Zheng, X. Wang, W. Chien, and M. Karczewicz “Non-CE9: Simplified Merge candidate derivation”, JCT-VC Document, JCTVC-G681,7th Meeting: Geneva, CH, 21-30 November, 2011.

[2]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-G1200, 7th Meeting: Geneva, CH, 21-30 November, 2011.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).