| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0092 |
| --- | --- |

| Title: | Non-CE9: Removing PU dependency in TMVP reference index derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongjoon Jeon (LG) * Byeongmoon Jeon (LG) * Vadim Seregin (Qualcomm) ** Xianglin Wang (Qualcomm) ** Jianle Chen (Qualcomm) ** Marta Karczewicz (Qualcomm) ** * #221 Yangjae-dong, Seoucho-gu, Seoul 137-130, Korea **5775 Morehouse Dr San Diego, CA 92121 USA | Email: | yongjoon.jeon@lge.com bm.jeon@lge.com vseregin@qualcomm.com xianglin@qualcomm.com cjianle@qualcomm.com martak@qualcomm.com |
| Source: | LG, Qualcomm Inc. |  |  |

_____________________________

# Abstract

In the current HM, reference index of TMVP is derived from the left neighbor PU of a current CU. When both the left neighbor PU and the current PU belong to a same CU, it incurs inter-PU dependency, which is undesirable for parallel merge motion estimation.

This contribution aims at breaking the inter-PU dependency in the TMVP reference index derivation process by using the reference index from a neighbor block outside of the current CU. The reported simulation results show that the proposed modifications do not incur any coding loss.

# Introduction

In the current HM, reference index of TMVP is derived from the left neighbor PU of a current PU. When both the left neighbor PU and the current PU belong to a same CU, it incurs inter-PU dependency. Fig. 1 shows some examples of such situation.

Fig.1 Examples of a neighboring PU of the same CU is referred in the TMVP reference index derivation

As a result, in these cases TMVP for the current PU cannot be derived until the encoding/decoding of the first PU has finished. This creates a problem for parallel derivation of the merge candidates list for all the PUs of a same CU.

# Proposed method

The inter-PU dependency can be removed by fixing the TMVP reference index value to 0. The idea was proposed last meeting, but was not adopted due to its relatively large coding loss on some video sequence under low-delay B configurations.

In this proposal, we propose to derive the TMVP reference index from a neighbor block outside of the current CU. More exactly, for all PUs in a CU regardless of its partition mode, TMVP reference index is always derived in a same way as it is done currently for 2Nx2N mode.

As shown in Fig. 2, for every PU inside the CU, its TMVP reference index is set equal to the reference index of the block at location A1. If a reference index of A1 is not available, the corresponding TMVP reference index is set to 0.

Fig.2 The position for the TMVP reference index used for all the PU partitions in method 2

# Simulation results

For reference, simulations were performed on HM5.0 by fixing the TMVP reference index value to 0. The results can be found in the associated data sheet. The average loss relative to the HM5.0 anchor of [1] is 0.0%, 0.0%, 0.1% and 0.1% for RA-HE, RA-LC, LB-HE, and LB-LC configurations respectively. However, the loss (0.5~0.7% ) for BQSquare sequence of LB configurations is still noticeable as shown in Fig. 3 and 4.

Fig.3 BD-rate Y(%) for LB-HE configuration of method 1

Fig.4 BD-rate Y(%) for LB-LC configuration of method 1

Simulations with the proposed scheme were also performed on HM5.0. Detailed results can be found in the associated datasheet. In average this method has no noticeable loss for all configurations.

Similar to above, Fig. 5 and 6 show the performance of the proposed scheme for each sequence under LB configurations. It can be seen that the proposed scheme provides rather consistent performance for different sequences.

Fig.5 BD-rate Y(%) for LB-HE configuration of method 2

Fig.6 BD-rate Y(%) for LB-LC configuration of method 2

# Conclusion

In this contribution, a method is proposed to remove the inter-PU dependency in the TMVP reference index derivation process to allow parallel processing for all partitions within the same CU. As shown in the simulation results, the proposed scheme does not incur obvious coding loss on any test sequence. It is also worth mentioning that according to the proposed scheme, TMVP reference index is derived only once and shared for all PUs. It has lower complexity compared with the current scheme in HM5.0. We recommend to adopt the proposed scheme into HM.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-G1200, 7th Meeting: Geneva, CH, 21-30 November, 2011.

# Patent rights declaration(s)

LG Electronics and Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD changes

The changes to the current WD are marked in yellow.

### 8.4.2 Decoding process for prediction units in inter prediction mode

Inputs to this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current prediction unit relative to the top left luma sample of the current coding unit,

–	a variable nCS specifying the size of the current coding unit,

–	a variable nPSW specifying the width of the current prediction unit,

–	a variable nPSH specifying the width of the current prediction unit,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are:

–	a (nCSL)x(nCSL) array predSamplesL of luma prediction samples, where nCSL is derived as specified below,

–	a (nCSC)x(nCSC) array predSamplesCb of chroma prediction samples for the component Cb, where nCSC is derived as specified below,

–	a (nCSC)x(nCSC) array predSamplesCr of chroma prediction samples for the component Cr, where nCSC is derived as specified below.

The variable nCSL is set equal to nCS and the variable nCSC is set equal to nCS >> 1. [Ed: (WJ) revisit for supporting other chroma formats]

The decoding process for prediction units in inter prediction mode consists of the following ordered steps:

- Derivation process for motion vector components and reference indices as specified in subclause .

Inputs to this process are

–	a luma location ( xC, yC ) of the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

–	a luma location ( xB, yB ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current coding unit,

–	a variable nCS specifying the size of the current coding unit,

–	variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

  - a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

–	luma motion vectors mvL0 and mvL1, and chroma motion vectors mvCL0 and mvCL1,

–	reference indices refIdxL0 and refIdxL1,

–	prediction list utilization flags predFlagL0 and predFlagL1.

…

#### 8.4.2.1 Derivation process for motion vector components and reference indices

Input to this process are

- a luma location ( xC, yC ) of the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

- a luma location ( xB, yB ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current coding unit,

- a variable nCS specifying the size of the current coding unit,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

- luma motion vectors mvL0 and mvL1 and chroma motion vectors mvCL0 and mvCL1,

- reference indices refIdxL0 and refIdxL1,

- prediction list utilization flags predFlagL0 and predFlagL1.

Let ( xP, yP ) specify the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture where xP = xC + xB and yP = yC + yB.

For the derivation of the variables mvL0 and mvL1, refIdxL0 and refIdxL1 as well as PredFlagL0 and PredFlagL1, the following applies.

- If PredMode is equal to MODE_SKIP, the derivation process for luma motion vectors for merge mode as specified in subclause  is invoked with the luma locations (xC, yC), ( xP, yP ), variables nCS, nPSW, nPSH and the partition index PartIdx as inputs and the output being the luma motion vectors mvL0, mvL1, the reference indices refIdxL0, refIdxL1, and the prediction list utilization flags predFlagL0 and predFlagL1.

- Otherwise, if PredMode is equal to MODE_INTER and merge_flag[ xP ][ yP ] is equal to 1,, the derivation process for luma motion vectors for merge mode as specified in subclause  is invoked with the luma locations (xC, yC), ( xP, yP ), variables nCS, nPSW and nPSH and the partition index PartIdx as inputs and the outputs being the luma motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, the prediction utilization flags predFlagL0 and predFlagL1.

…

##### 8.4.2.1.1 Derivation process for luma motion vectors for merge mode

This process is only invoked when PredMode is equal to MODE_SKIP or PredMode is equal to MODE_ INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xC, yC ) of the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- a variable nCS specifying the size of the current coding unit,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

–	the luma motion vectors mvL0 and mvL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags predFlagL0 and predFlagL1.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps:

- The derivation process for merging candidates from neighboring prediction unit partitions in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH and the partition index PartIdx as inputs and the output is assigned to the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N and the motion vectors mvL0N and mvL1N with N being replaced by A0, A1, B0, B1 or B2.

- The reference index for temporal merging candidate is derived as follows..

- If the prediction unit covering luma location ( xP − 1, yP + nPSH − 1 ) is available and PredMode is not MODE_INTRA, refIdxLX is set equal to refIdxLX[ xP − 1, yP + nPSH − 1 ].

- If the prediction unit covering luma location ( xC − 1, yC + nCS − 1 ) is available and PredMode is not MODE_INTRA, refIdxLX is set equal to refIdxLX[ xC − 1, yC + nCS − 1 ].

- Otherwise, refIdxLX is set equal to 0.

…