| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0094 |
| --- | --- |

| Title: | Non-CE8: Modification of directional classification for block-based adaptation on ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Masaaki Matsumura Seishi Takamura Atsushi Shimizu Hirohisa Jozawa 1-1, Hikari-no-oka, Yokosuka-shi, Kanagawa 239-0847, JAPAN | Tel: Email: | +81-46-859-3521 matsumura.masaaki@lab.ntt.co.jp +81-46-859-2371 takamura.seishi@lab.ntt.co.jp +81-46-859-2448 shimizu.atsushi@lab.ntt.co.jp +81-46-859-3382 jozawa.hirohisa@lab.ntt.co.jp |
| Source: | NTT (Nippon Telegraph and Telephone) Corporation |  |  |

_____________________________

# Abstract

This contribution presents a flexible directional classification framework for block-based adaptation (BA) on adaptive loop filter (ALF). In this contribution, the framework that allocates a directional classification pattern to each classified activity domain is proposed. Compared to HM5.0 anchor, the average BD-rate losses were 0.1, 0.1, 0.1, 0.1, 0.2, 0.2, 0.1 and 0.1% for HE-AI, HE-RA, HE-LB, HE-LP, HE-AI10, HE-RA10, HE-LB10 and HE-LP10 cases, respectively. The average encoding time reduction of 0.7 - 1.3% and the average decoding time reduction of 1.9 - 3.0% were achieved.

# Introduction

In the current ALF of HM5.0 [1], BA mode employs initial 15 filters consist of fixed five activity domains with fixed three directional classification patterns, and designs each filter coefficients for minimizing square difference between decoded picture and original picture. Although the merge process restricts the number of filters to necessary minimum number, how to classify the initial filters is one of the important factors that decide coding efficiency of ALF.

Proposed framework prepares two or more directional pattern classification sets (DirSets), and provides the method that each DirSet to be coded allocates to each activity domain (Figure 1). In this contribution, five DirSets described in Table 1 are prepared, and the activity is classified into six domains with a variable offset to be coded (Figure 2). The benefits of this framework are described below:

- The increase of directional classification pattern on the activity domain allocated lots of 4x4 blocks might design effective filter than conventional one.

- The implementation of DirNone that skips ALF process besides the CU-adaptive mode might reduce the computational complexity of ALF.

Figure 1: Proposed classification framework.

Figure 2: Proposed classification for activity.

| Table 1: Prepared directional pattern classification sets. |  |  |  |
| --- | --- | --- | --- |
| DirSet | Index | Number of filters | Classification patterns |
| DirNone | 0 | 0 | Luma samples are not filtered (such as JCTVC-G446 [2]). |
| DirSingle | 1 | 1 | Classification is unnecessary. |
| DirHV | 2 | 2 | Horizontal and vertical (described in JCTVC-G647 [3]). |
| DirHVO | 3 | 3 | Horizontal, vertical and other (HM default). |
| DirHVDLR | 4 | 4 | Horizontal, vertical, diagonal-left and diagonal-right. |

# Syntax and semantics

In this section, modified syntax and semantics are described. The modified syntax is specified in Table 2. The difference between conventional ALF and proposal set highlighted in yellow. The modified semantics used in the syntax are specified below:

alf_zero_offset specifies the offset for activity classification. If the alf_zero_offset is equal to 0, blocks classify into five domains. Otherwise, the blocks classify into six domains. The value of alf_zero_offset shall be in the range of 0 to 10, inclusive. The code for alf_zero_offset is specified in Table 3.

alf_dir_set[i] specifies the directional pattern classification set (DirSet). The index of Table 1 is assigned to alf_dir_set[i]. If the alf_dir_set[i] is equal to 0, the luma samples are not filtered. The range/code table for alf_dir_set is specified in Table 4.

Figure 2 illustrates the example of activity classification with alf_zero_offset (the variable activityIdx used in Figure 3 is specified at equation 3-11 in subclause 3.2). Each classified domain is allocated an alf_dir_set associated with DirSet (Table 1).

Figure 3: Activity classification with alf_zero_offset.

| Table 2: Adaptive loop filter parameter syntax. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| alf_param(){ |  |  |  |  | Descriptor |  |  |
| alf_region_adaptation_flag |  |  |  |  | u(1) ¦ ae(v) |  |  |
| if (alf_region_adaptation_flag == 0){ |  |  |  |  |  |  |  |
| alf_zero_offset |  |  |  |  | u(v) ¦ ae(v) |  |  |
| alf_dir_set[0] = 0 |  |  |  |  |  |  |  |
| if (alf_zero_offset != 0) |  |  |  |  |  |  |  |
| alf_dir_set[0] |  |  |  |  | u(1) ¦ ae(v) |  |  |
| for (i=1; i< 3; i++) |  |  |  |  |  |  |  |
| alf_dir_set[i] |  |  |  |  | f(2) ¦ ae(v) |  |  |
| for (i=3; i< 6; i++) |  |  |  |  |  |  |  |
| alf_dir_set[i] |  |  |  |  | f(2) ¦ ae(v) |  |  |
| } |  |  |  |  |  |  |  |
| alf_length_luma_minus_5_div2 |  |  |  |  | ue(v) |  |  |
| ... |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| Table 3: The code table for alf_zero_offset. |  |  | Table 4: The range/code table for alf_dir_set |  |  |  |  |
| Index | Code |  | Semantics | Index |  | Code |  |
| 1 | 0 |  | alf_dir_set[0] | 0 - 1 |  | 0, 1 |  |
| 2 | 10 |  | alf_dir_set[1..2] | 0 - 3 |  | 00, 01, 10, 11 |  |
| 3 | 110 |  | alf_dir_set[3..5] | 1 - 4 |  | 00, 01, 10, 11 |  |
| … | … |  |  |  |  |  |  |
| 10 | 111111110 |  |  |  |  |  |  |
| 0 | 111111111 |  |  |  |  |  |  |

# Decoder description

## Overview for subclauses of this section

In this section, the decoding process of proposed ALF is described. In this contribution, subclause 8.6.3.3 and 8.6.3.4 described in WD5 [1] are modified. On subclause 3.2 (as 8.6.3.3) and 3.3 (as 8.6.3.4), the differences between conventional ALF process and proposed one set highlighted in yellow.

## Derivation process for filter index array for luma samples (as 8.6.3.3)

Inputs of this process are:

- a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

- a variable log2CUSize specifying the size of the current coding unit.

Output of this process is the two-dimensional filter index array of (nS)x(nS), fIdx.

A variable nS is set equal to ( 1 << log2CUSize ).

The boundary padding process specified in subclause 8.6.3.1 is invoked with the luma location ( xC, yC ), the size of coding unit log2CUSize and the chroma component index cIdx set equal to 0, and the output is assigned to the luma sample array s’’. [Ed. (WJ): s’’ is now a picture-size array, but actually CU size + appropriate margin is enough]

The filter index array fIdx is specified in the follows:

- When alf_region_adaptation_flag is equal to 1, the following ordered steps apply.

  - The variables xIdx and yIdx are derived as

regionTab[16] = { 0, 1, 4, 5, 15, 2, 3, 6, 14, 11, 10, 7, 13, 12, 9, 8 }				(3-1)
offset = 1 << (Log2MaxCUSize – 1) 							(3-2)
xInterval = ( ( ( PicWidthInSamplesL + offset ) >> Log2MaxCUSize ) + 1 ) >> 2		(3-3)
yInterval = ( ( ( PicHeightInSamplesL + offset ) >> Log2MaxCUSize ) + 1 ) >> 2		(3-4)
xIdx = Min( 3, Floor( ( xC + x ) / ( xInterval << Log2MaxCUSize ) ) ) 			(3-5)
yIdx = Min( 3, Floor( ( yC + y ) / ( yInterval << Log2MaxCUSize ) ) ) 			(3-6)

  - The filter index fIdx[ x, y ] is derived as

fIdx[ x ][ y ] = regionTab[ ( yIdx << 2 ) + xIdx ]						(3-7)

- Otherwise (alf_region_flag is equal to 0), for each 4x4 block (in the current coding unit) with the luma location (xC4, yC4) specifying the top-left luma sample relative to the top left luma sample of the current picture, the following ordered steps apply.

  - The variables varH, varV and verActivity are derived as

varH = | ( s”[ xC4+1, yC4+1 ] << 1 ) – s”[ xC4+0, yC4+1 ] – s”[ xC4+2, yC4+1 ] |
      | ( s”[ xC4+2, yC4+1 ] << 1 ) – s”[ xC4+1, yC4+1 ] – s”[ xC4+3, yC4+1 ] |
      | ( s”[ xC4+1, yC4+2 ] << 1 ) – s”[ xC4+0, yC4+2 ] – s”[ xC4+2, yC4+2 ] |
      | ( s”[ xC4+2, yC4+2 ] << 1 ) – s”[ xC4+1, yC4+2 ] – s”[ xC4+3, yC4+2 ] |		(3-8)
varV = | ( s”[ xC4+1, yC4+1 ] << 1 ) – s”[ xC4+1, yC4+0 ] – s”[ xC4+1, yC4+2 ] |
      | ( s”[ xC4+2, yC4+1 ] << 1 ) – s”[ xC4+2, yC4+0 ] – s”[ xC4+2, yC4+2 ] |
      | ( s”[ xC4+1, yC4+2 ] << 1 ) – s”[ xC4+1, yC4+1 ] – s”[ xC4+1, yC4+3 ] |
      | ( s”[ xC4+2, yC4+2 ] << 1 ) – s”[ xC4+2, yC4+1 ] – s”[ xC4+2, yC4+3 ] |		(3-9)
varActivity = varH + varV 								(3-10)

  - The variables activityIdx and activityOffset are derived as

regionBuf[32] = { 1, 1, 2, 2, 3, 3, 3, 3, 3, 3, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5 }
regionTab[32] = { 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0 }
for(n = alf_zero_offset; n < 32; n++)
  regionTab[n] = regionBuf[n]
activityIdx = regionTab[ Min( ( varActivity ) >> ( 2 + BitDepthY ), 31 ) ]			(3-11)
activityOffset = 0
for(n = 0; n < activityIdx - 1; n++)
  activityOffset += alf_dir_set[n] 							(3-12)

  - The filter index fIdx[ x, y ] with x, y = xC4 – xP..xC4 – xP + 3 and yC4 – yP..yC4 – yP + 3 is derived as

    - If alf_dir_set[ activityIdx ] is equal to 0, fIdx[ x ][ y ] is assigned to -1.

    - Otherwise, if alf_dir_set[ activityIdx ] is equal to 1, fIdx[ x ][ y ] is assigned to activityOffset.

    - Otherwise, if alf_dir_set[ activityIdx ] is equal to 2, the filter index fIdx[ x, y ] is derived as

fIdx[ x ][ y ] = activityOffset + ( ( varH < varV ) ? 0 : 1 ) 				(3-13)

    - Otherwise, if alf_dir_set[ activityIdx ] is equal to 3, following steps apply:

      - If varH is greater than 2 * varV, fIdx[ x ][ y ] is assigned to activityOffset + 2.

      - Otherwise, if varV is greater than 2 * varH, fIdx[ x ][ y ] is assigned to activityOffset + 1.

      - Otherwise, fIdx[ x ][ y ] is assigned to activityOffset.

    - Otherwise (when alf_dir_set[ activityIdx ] is equal to 4), following steps apply:

      - If varH is greater than 4 * varV, fIdx[ x ][ y ] is assigned to activityOffset.

      - Otherwise, if varV is greater than 4 * varH, fIdx[ x ][ y ] is assigned to activityOffset + 1.

      - Otherwise, fIdx[ x ][ y ] is derived as

varDL = | ( s”[ xC4+1, yC4+1 ] << 1 ) – s”[ xC4+2, yC4+0 ] – s”[ xC4+0, yC4+2 ] |
        | ( s”[ xC4+2, yC4+1 ] << 1 ) – s”[ xC4+3, yC4+0 ] – s”[ xC4+1, yC4+2 ] |
        | ( s”[ xC4+1, yC4+2 ] << 1 ) – s”[ xC4+2, yC4+1 ] – s”[ xC4+2, yC4+1 ] |
        | ( s”[ xC4+2, yC4+2 ] << 1 ) – s”[ xC4+3, yC4+1 ] – s”[ xC4+3, yC4+1 ] |	(3-14)
varDR = | ( s”[ xC4+1, yC4+1 ] << 1 ) – s”[ xC4+0, yC4+0 ] – s”[ xC4+2, yC4+2 ] |
        | ( s”[ xC4+2, yC4+1 ] << 1 ) – s”[ xC4+1, yC4+0 ] – s”[ xC4+3, yC4+2 ] |
        | ( s”[ xC4+1, yC4+2 ] << 1 ) – s”[ xC4+0, yC4+1 ] – s”[ xC4+2, yC4+3 ] |
        | ( s”[ xC4+2, yC4+2 ] << 1 ) – s”[ xC4+1, yC4+1 ] – s”[ xC4+3, yC4+3 ] |	(3-15)
fIdx[ x ][ y ] = activityOffset + ((varDL > varDR) ? 2 : 3) 			(3-16)

## Filtering process for luma samples (as 8.6.3.4)

Inputs of this process are:

- a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

- a variable log2CUSize specifying the size of the current coding unit,

- a filter index array of (nS)x(nS), fIdx.

Output of this process is the filtered reconstruction of luma picture.

The boundary padding process specified in subclause 8.6.3.1 is invoked with the luma location ( xC, yC ), the size of coding unit log2CUSize and the chroma component index cIdx set equal to 0, and the output is assigned to the luma sample array s’’. [Ed. (WJ): s’’ is now a picture-size array, but actually CU size + appropriate margin is enough]

A variable nS is set equal to ( 1 << log2CUSize ) and a variable alfFilterShape is set equal to alf_filter_shape_flag.

A variable lcuHeight is set equal to ( 1 << Log2MaxCUSize ) and a variable vbLine is set equal to lcuHeight – 4.

Each sample of luma picture recFiltPictureL[ xC + x ][ yC + y ] with x, y = 0..(nS)-1, is derived as following ordered steps:

- A variable dist2VB is derived as follows.

dist2VB = ( ( yC + y ) % lcuHeight – vbLine )						(3-17)

- A variable dist2VB is modified as follows.

  - If dist2VB is less than –vbLine+2 and yC is larger than 2, dist2VB is set equal to dist2VB+lcuHeight,

  - Otherwise, if yC+lcuHeight is equal or larger than pic_height_in_luma_samples, dist2VB is set equal to 5.

- If “alfFilterShape is equal to 0 and dist2VB is equal to 0 or -1” or “fIdx[ x ][ y ] is equal to -1”, the following applies.

recFiltPictureL[ xC + x ][ yC + y ] = s”[ xC + x, yC + y ]					(3-18)

- Otherwise, if alfFilterShape is equal to 0 and dist2VB is equal to 1 or -2, the following applies.

recFiltPictureL[ xC + x ][ yC + y ]
=s \[  ital xC+x, ital yC+y \] + Sum cSub { size 8{i=0} }  cSup { size 8{N - 1} }  { left (s[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y][i]]>>1	(3-19)

- Otherwise, the following applies.

recFiltPictureL[ xC + x ][ yC + y ] 
=i=0N−1s''[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y][i]]				(3-20)

# Simulation results

In this section, objective evaluations are described. Since the full optimization of all the proposed syntax elements causes increase of encoding time, alf_zero_offset is set at suitable value by encoder, and the alf_dir_sets are selected by RDC (Rate-Distortion-Complexity) optimization from prepared three patterns (Figure 4).

Figure 4: Prepared three patterns.

Table 5 and 6 present coding efficiencies and computational complexities based on common condition [4] compared to HM5.0-r1686 anchors. The source code was compiled on 64-bit Windows 7 environment by using Microsoft Visual Studio 2010 Professional edition, and the experiments were performed on the cluster of PCs with 64-bit Windows 7, Intel Core i7 2600 (3.4GHz) and 16GB RAM.

| Table 5: Experimental results (mandatory) for proposal compared to the anchors. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra HE |  |  | Low delay B HE |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.3% | 0.0% | 0.0% |  |  |  |
| Class B | 0.2% | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% |
| Class C | 0.1% | 0.0% | 0.0% | 0.0% | -0.2% | 0.1% |
| Class D | 0.1% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Class E | 0.2% | 0.0% | 0.0% | 0.2% | 0.3% | -0.1% |
| Overall | 0.2% | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% |
|  | 0.2% | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% |
| Class F | 0.0% | 0.0% | 0.0% | -0.1% | -0.3% | -0.4% |
| Enc Time[%] | 100.3% |  |  | 100.0% |  |  |
| Dec Time[%] | 97.6% |  |  | 98.2% |  |  |
|  | Random Access HE |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.2% | 0.0% | 0.0% | 0.2% | -0.1% | 0.1% |
| Class B | 0.2% | -0.1% | -0.1% | 0.2% | 0.0% | 0.1% |
| Class C | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.1% | -0.2% | -0.1% |  |  |  |
| Class E |  |  |  |  |  |  |
| Overall | 0.1% | -0.1% | 0.0% | 0.2% | 0.0% | 0.1% |
|  | 0.1% | -0.1% | -0.1% | 0.2% | 0.0% | 0.1% |
| Class F | -0.1% | -0.1% | -0.1% |  |  |  |
| Enc Time[%] | 100.0% |  |  | 100.1% |  |  |
| Dec Time[%] | 98.0% |  |  | 98.4% |  |  |

| Table 6: Experimental results (all) for proposal compared to the anchors. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra HE |  |  | All Intra HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.2% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% |
| Class B | 0.2% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% |
| Class C | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Class D | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Class E | 0.2% | 0.0% | 0.0% | 0.4% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.1% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% |
|  | 0.1% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% |
| Enc Time[%] | 100.8% |  |  | 100.3% |  |  |
| Dec Time[%] | 98.0% |  |  | 97.8% |  |  |
|  | Random Access HE |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.2% | -0.1% | -0.1% | 0.2% | -0.1% | 0.1% |
| Class B | 0.2% | -0.1% | -0.1% | 0.2% | 0.0% | 0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% |
| Class D | 0.1% | -0.2% | -0.1% | 0.1% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F | -0.1% | -0.1% | -0.1% | 0.1% | 0.1% | 0.2% |
| Overall | 0.1% | -0.1% | -0.1% | 0.2% | 0.0% | 0.0% |
|  | 0.1% | -0.1% | -0.1% | 0.2% | 0.0% | 0.0% |
| Enc Time[%] | 100.1% |  |  | 100.1% |  |  |
| Dec Time[%] | 98.2% |  |  | 98.6% |  |  |
|  | Low delay B HE |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.0% | -0.1% | 0.2% | 0.0% | 0.0% |
| Class C | 0.0% | -0.2% | 0.1% | 0.1% | -0.2% | 0.1% |
| Class D | 0.0% | -0.1% | -0.1% | 0.1% | 0.0% | -0.2% |
| Class E | 0.2% | 0.3% | -0.1% | 0.1% | -0.8% | 0.2% |
| Class F | -0.1% | -0.3% | -0.4% | -0.1% | 0.1% | 0.2% |
| Overall | 0.1% | -0.1% | -0.1% | 0.1% | -0.1% | 0.1% |
|  | 0.1% | -0.1% | -0.2% | 0.1% | -0.1% | 0.0% |
| Enc Time[%] | 100.0% |  |  | 100.0% |  |  |
| Dec Time[%] | 98.3% |  |  | 98.7% |  |  |
|  | Low delay P HE |  |  | Low delay P HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | 0.1% | -0.2% | 0.2% | 0.0% | -0.1% |
| Class C | 0.2% | 0.1% | -0.2% | 0.1% | 0.0% | 0.2% |
| Class D | 0.1% | 0.1% | -0.3% | 0.2% | -0.4% | 0.1% |
| Class E | 0.1% | -0.2% | 0.2% | 0.2% | 0.3% | 0.1% |
| Class F | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 0.1% |
| Overall | 0.1% | 0.1% | -0.1% | 0.1% | 0.0% | 0.1% |
|  | 0.1% | 0.1% | -0.2% | 0.1% | 0.0% | 0.1% |
| Enc Time[%] | 100.1% |  |  | 100.1% |  |  |
| Dec Time[%] | 97.5% |  |  | 98.0% |  |  |

After the source code cleanup optimized for the proposal, the encoding time and decoding time were reduced as shown in Table 7, 8. No mismatch between the bit-stream and BD-rate before the source code cleanup and the bit-stream and BD-rate after the cleanup was confirmed.

| Table 7: Cleanup results (mandatory) for proposal compared to the anchors. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra HE |  |  | Low delay B HE |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.3% | 0.0% | 0.0% |  |  |  |
| Class B | 0.2% | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% |
| Class C | 0.1% | 0.0% | 0.0% | 0.0% | -0.2% | 0.1% |
| Class D | 0.1% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Class E | 0.2% | 0.0% | 0.0% | 0.2% | 0.3% | -0.1% |
| Overall | 0.2% | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% |
|  | 0.2% | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% |
| Class F | 0.0% | 0.0% | 0.0% | -0.1% | -0.3% | -0.4% |
| Enc Time[%] | 98.8% |  |  | 98.9% |  |  |
| Dec Time[%] | 97.3% |  |  | 97.6% |  |  |
|  | Random Access HE |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.2% | 0.0% | 0.0% | 0.2% | -0.1% | 0.1% |
| Class B | 0.2% | -0.1% | -0.1% | 0.2% | 0.0% | 0.1% |
| Class C | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.1% | -0.2% | -0.1% |  |  |  |
| Class E |  |  |  |  |  |  |
| Overall | 0.1% | -0.1% | 0.0% | 0.2% | 0.0% | 0.1% |
|  | 0.1% | -0.1% | -0.1% | 0.2% | 0.0% | 0.1% |
| Class F | -0.1% | -0.1% | -0.1% |  |  |  |
| Enc Time[%] | 99.1% |  |  | 99.4% |  |  |
| Dec Time[%] | 97.3% |  |  | 97.8% |  |  |

| Table 8: Cleanup results (all) for proposal compared to the anchors. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra HE |  |  | All Intra HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.2% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% |
| Class B | 0.2% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% |
| Class C | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Class D | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Class E | 0.2% | 0.0% | 0.0% | 0.4% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.1% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% |
|  | 0.1% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% |
| Enc Time[%] | 99.2% |  |  | 98.7% |  |  |
| Dec Time[%] | 97.7% |  |  | 97.5% |  |  |
|  | Random Access HE |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.2% | -0.1% | -0.1% | 0.2% | -0.1% | 0.1% |
| Class B | 0.2% | -0.1% | -0.1% | 0.2% | 0.0% | 0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% |
| Class D | 0.1% | -0.2% | -0.1% | 0.1% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F | -0.1% | -0.1% | -0.1% | 0.1% | 0.1% | 0.2% |
| Overall | 0.1% | -0.1% | -0.1% | 0.2% | 0.0% | 0.0% |
|  | 0.1% | -0.1% | -0.1% | 0.2% | 0.0% | 0.0% |
| Enc Time[%] | 99.2% |  |  | 99.3% |  |  |
| Dec Time[%] | 97.6% |  |  | 97.9% |  |  |
|  | Low delay B HE |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.0% | -0.1% | 0.2% | 0.0% | 0.0% |
| Class C | 0.0% | -0.2% | 0.1% | 0.1% | -0.2% | 0.1% |
| Class D | 0.0% | -0.1% | -0.1% | 0.1% | 0.0% | -0.2% |
| Class E | 0.2% | 0.3% | -0.1% | 0.1% | -0.8% | 0.2% |
| Class F | -0.1% | -0.3% | -0.4% | -0.1% | 0.1% | 0.2% |
| Overall | 0.1% | -0.1% | -0.1% | 0.1% | -0.1% | 0.1% |
|  | 0.1% | -0.1% | -0.2% | 0.1% | -0.1% | 0.0% |
| Enc Time[%] | 99.0% |  |  | 99.0% |  |  |
| Dec Time[%] | 97.7% |  |  | 98.1% |  |  |
|  | Low delay P HE |  |  | Low delay P HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | 0.1% | -0.2% | 0.2% | 0.0% | -0.1% |
| Class C | 0.2% | 0.1% | -0.2% | 0.1% | 0.0% | 0.2% |
| Class D | 0.1% | 0.1% | -0.3% | 0.2% | -0.4% | 0.1% |
| Class E | 0.1% | -0.2% | 0.2% | 0.2% | 0.3% | 0.1% |
| Class F | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 0.1% |
| Overall | 0.1% | 0.1% | -0.1% | 0.1% | 0.0% | 0.1% |
|  | 0.1% | 0.1% | -0.2% | 0.1% | 0.0% | 0.1% |
| Enc Time[%] | 99.2% |  |  | 99.2% |  |  |
| Dec Time[%] | 97.0% |  |  | 97.4% |  |  |

# Conclusion

In this contribution, the framework that allocates a directional classification pattern to each classified activity domain is proposed. Compared to HM5.0 anchor, the average BD-rate losses were 0.1, 0.1, 0.1, 0.1, 0.2, 0.2, 0.1 and 0.1% for HE-AI, HE-RA, HE-LB, HE-LP, HE-AI10, HE-RA10, HE-LB10 and HE-LP10 cases, respectively. The average encoding time reduction of 0.7 - 1.3% and the average decoding time reduction of 1.9 - 3.0% were achieved.

Thanks to KDDI’s cross-verification [5] for confirming the coding results.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “WD5: Working Draft 5 of High-Efficiency Video Coding,” JCTVC-G1103_d6.doc, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011.

- K. Miyazawa, K. Sugimoto, A. Minezawa, S. Sekiguchi, T. Murakami, “Reduction of the number of pixels used in Adaptive Loop Filter,” JCTVC-G446, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, Switzerland, 21-30 November, 2011.

- P. Lai, F. C. A. Fernandes, I.-K. Kim, “CE8 Subtest a, Tool 3: Block-based filter adaptation with 8 initial filter classes (HV8),” JCTVC-G647, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, Switzerland, 21-30 November, 2011.

- F. Bossen, “Common test conditions and software reference configurations,” JCTVC-G1200, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011.

- T. Yoshino, K. Kawamura, S. Naito, “Non-CE8: Cross-check of modified directional classification for block-based adaptation on ALF (JCTVC-H0094)” JCTVC-H0158, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012.

# Patent rights declaration(s)

NTT may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

As an additional experiment, Table 9 shows the comparison results between conventional method (as an anchor) with filter skipping by alf_zero_offset and proposal method (with filter skipping by alf_zero_offset and adaptive directional classification). The BD-rate improvements were 0.00 – 0.03 [%] with encoding time of 99.2 – 100.5 [%] and decoding time of 98.0 – 100.6 [%]. Especially, in 8-bit HE cases, proposed adaptive directional classification improves coding efficiency with reductions of encoding/decoding time.

| Table 9: The comparison results between conventional method (as an anchor) with filter skipping by alf_zero_offset and proposal method (with filter skipping and adaptive directional classification). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra HE |  |  | All Intra HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100.3% |  |  | 100.5% |  |  |
| Dec Time[%] | 99.5% |  |  | 100.4% |  |  |
|  | Random Access HE |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99.3% |  |  | 99.4% |  |  |
| Dec Time[%] | 98.9% |  |  | 100.6% |  |  |
|  | Low delay B HE |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | -0.2% | 0.0% | 0.1% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class D | 0.0% | -0.1% | 0.1% | 0.0% | -0.2% | -0.4% |
| Class E | 0.1% | 0.3% | 0.2% | 0.0% | -0.3% | 0.2% |
| Class F | 0.0% | 0.0% | -0.2% | -0.1% | -0.1% | 0.3% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 99.1% |  |  | 99.2% |  |  |
| Dec Time[%] | 98.7% |  |  | 100.6% |  |  |
|  | Low delay P HE |  |  | Low delay P HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.1% | -0.4% | -0.1% | 0.1% | -0.2% |
| Class C | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | -0.1% | -0.7% | 0.0% | -0.2% | 0.0% |
| Class E | -0.1% | -0.6% | 0.4% | 0.0% | 0.2% | 0.2% |
| Class F | 0.0% | 0.0% | 0.1% | -0.1% | -0.1% | -0.3% |
| Overall | 0.0% | -0.1% | -0.2% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99.1% |  |  | 99.2% |  |  |
| Dec Time[%] | 98.0% |  |  | 100.2% |  |  |