| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0095r2 |
| --- | --- |

| Title: | CE11.1 Simplified significant_coeff_flag context derivation at high frequency area |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba, JAPAN | Tel: Email: | +81 43 299 8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution presents two kinds of complexity reduction method of significant map coding of large (16x16, 16x4, 4x16, 32x32, 32x8, 8x32) transform at high frequency area. In the proposed method1 reduces the number of neighbors to 2 from 4-5, while method2 eliminate neighboring dependency using constant index. Both methods  also eliminate conditional branches in 4x4 sub-block at high frequency area to improve throughput. The ratio of the area where complexity reduction is applied is about 62.5 %. At the 2 neighbor method, it is reported that average BD-rate change is -0.01 % to 0.03 % on common condition and 0.00 % to 0.02 % on low qp condition (qp = 12, 17, 22, 27). At the 0 neighbor method, it is reported that average BD-rate change is -0.01 % to 0.03 % on common condition and -0.03 % to 0.01 % on low qp condition. Enc Time and Dec Time change is negligible.

The method 2.1 is added on January 27. The method 2.1 is the same as method2 except that it doesn’t add new contexts but shares cxtCnt = 0 context at high frequency area. it is reported that average BD-rate change is 0.00 % to 0.04 % on common condition and -0.02 % to 0.04 % on low qp condition.

# Introduction

In HM-5.0, the context of significant_coeff_flag is derived by the number of non-zero coefficients surrounding the target coefficient. The number of neighbor is 4 to 5. The derivation is conducted in 4x4 sub-block and the number of neighbor depends on the position in subblock to achieve parallelism. This dependency needs conditional branches which reduce throughput. And the large number of neighbor also reduce throughput if the context derivation and the bin decoding is not conducted parallel or if the context derivation is slower than the bin decoding in parallel. Therefore it is needed to eliminate the conditional branches in subblock and reduce the number of neighbors. The proposal addresses this issue at high frequency area based on JCTVC-G308 [1].

Figure 1: Elimination of condition in sub block

# Proposed method

In the proposed method, the coefficients of 16x16 and 32x32 transform are classified into 4 regions, R0 to R3 as Figure 3. In Table 1, the condition of the classification and its context derivation method is described.

In the proposal, in R3 region the number of neighbors decreases to 2. In R3, the condition in subblock is eliminated.  In method 1, the categories R2 and R3 share the common contexts so the number of contexts is not changed. In method 2, the number of contexts increases by two for luma and choroma.

In method 2.1, the number of contexts is not changed. At high frequency area, the fix context, or ctxCnt =  0 context, is used as JCTVC-G917 [2].

Figure 3: Definitions of the categories for context derivation of significant_coeff_flag

Table 1: Number of neighbours

| Region | Condition | HM5.0 | Method1 | Method2 |
| --- | --- | --- | --- | --- |
| R0 | xC + yC == 0 | 0 |  |  |
| R1 | xC + yC < (TU size >> 2) | 4-5 |  |  |
| R2 | (xC>>2) + (yC>>2) < 3*(TU size >> 4) | 4-5 |  |  |
| R3 | Otherwise | 4-5 | 2 | 0 |

where TU size is the width of transform block

Figure 3: Simplification of boundary conditional branches

# WD text

The changes of WD text are highlighted as below.

9.3.3.1.1.5 Derivation process of ctxIdxInc for the syntax element significant_coeff_flag

Inputs to this process are the color component index cIdx, the current coefficient scan position ( xC , yC ), the transform block width log2TrafoWidth and the transform block height log2TrafoHeight.

Output of this process is ctxIdxInc.

The variable sigCtx depends on the current position ( xC, yC ), the color component index cIdx, the transform block size and previsously decoded bins of the syntax element significant_coeff_flag. For the derivation of sigCtx, the following applies.

- If log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 2, sigCtx is derived using ctxIdxMap4x4[ ] specified in Error: Reference source not found as follows..

sigCtx  =  ctxIdxMap4x4[ ((cIdx > 0) ? 15 : 0) + (yC << 2) + xC ]	(9-1)

- Otherwise if log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 3, sigCtx is derived using ctxIdxMap8x8[ ] specified in Error: Reference source not found as follows.

sigCtx  =  ((xC + yC) = = 0) ? 10 : ctxIdxMap8x8[ ((yC >> 1 ) << 2) + (xC >> 1) ]	(9-2)

sigCtx  +=  ( cIdx > 0) ? 6: 9		(9-3)

- Otherwise if xC + yC is equal to 0, sigCtx is derived as follows.

sigCtx  =  ( cIdx > 0) ? 17: 20		(9-4)

- Otherwise if (xC>>2)  +  (yC>>2) is equal to or greater than 3 << (max(log2TrafoWidth, log2TrafoHeight) − 4), sigCtx is derived using previously decoded bins of the syntax element significant_coeff_flag as follows.

- The variable sigCtx is initialized as follows.

sigCtx  =  0		(9-5)

- When xC is less than ( 1 << log2TrafoWidth ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC ]	(9-6)

- When xC is less than ( 1 << log2TrafoWidth ) − 1 and yC is less than ( 1 << log2TrafoHeight ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC + 1 ]	(9-7)

- When xC is less than ( 1 << log2Width ) − 2, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 2 ][ yC ]	(9-8)

- When all of the following conditions are true,

  - yC is less than ( 1 << log2TrafoHeight ) − 1,

  - xC % 4 is not equal to 0 or yC % 4 is not equal to 0,

  - xC % 4 is not equal to 3 or yC % 4 is not equal to 2,

the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 1 ]	(9-9)

- When yC is less than ( 1 << log2TrafoHeight ) − 2 and sigCtx is less than 4, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 2 ]	(9-10)

- The variable sigCtx is modified as follows.

  - If cIdx is equal to 0 and xC + yC are greater than (1 << (max(log2TrafoWidth, log2TrafoHeight) − 2)) − 1, the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + 24		(9-11)

  - Otherwise, the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + ( (cIdx > 0) ? 18 : 21 )	(9-12)

The context index increment ctxIdxInc is derived using the color component index cIdx and sigCtx as follows.

- If cIdx is equal to 0, ctxIdxInc is derived as follows.

ctxIdxInc  =  sigCtx		(9-13)

- Otherwise (cIdx is greater than 0), ctxIdxInc is derived as follows.

ctxIdxInc  =  27  +  sigCtx		(9-14)

// for method1 (ref2)

- Otherwise ((xC>>2)  +  (yC>>2) is equal to or greater than 3 << (uiLog2BlkSize-4), sigCtx is derived using previously decoded bins of the syntax element significant_coeff_flag as follows.

- The variable ctxCnt is initialized as follows.

ctxCnt =  0		(9-x)

- When xC is less than ( 1 << log2TrafoSize ) − 1, the following applies.

ctxCnt = ctxCnt +  significant_coeff_flag[ xC + 1 ][ yC ]	(9-x)

- When yC is less than ( 1 << log2TrafoSize ) − 2, the following applies.

ctxCnt = ctxCnt +  significant_coeff_flag[ xC ][ yC + 2 ]	(9-x)

- cIdx is equal to 0, ctxIdxInc is derived as follows.

ctxIdxInc = 24 + ctxCnt

- Otherwise (cIdx is greater than 0), ctxIdxInc is derived as follows.

ctxIdxInc = 18 + ctxCnt

// for method2 (ref0)

- Otherwise ((xC>>2)  +  (yC>>2) is equal to or greater than 3 << (uiLog2BlkSize-4), sigCtx is derived as follows.

- cIdx is equal to 0, ctxIdxInc is derived as follows.

sigCtx = 27

- Otherwise (cIdx is greater than 0), ctxIdxInc is derived as follows.

sigCtx = 21

// for method2.1 (ref0)

- Otherwise ((xC>>2)  +  (yC>>2) is equal to or greater than 3 << (max(log2TrafoWidth, log2TrafoHeight) e-4), sigCtx is derived as follows.

- cIdx is equal to 0, ctxIdxInc is derived as follows.

sigCtx = 24

- Otherwise (cIdx is greater than 0), ctxIdxInc is derived as follows.

sigCtx = 18

# Experimental result

The proposed method is implemented on HM-5.0 and simulation is conducted based on common test condition. Table 1 summarizes the result of the experiment.

Table 1: Summary of performance

|  | Method1 (two neighbors) | Method2 (no neighbors) |
| --- | --- | --- |
| Normal QP | -0.01 % to 0.03 % | 0.00 % to 0.02 % |
| Low QP | -0.01 % to 0.03 % | -0.03 % to 0.01 % |

|  |  | Method2.1 (no neighbors) |
| --- | --- | --- |
| Normal QP |  | 0.00 % to 0.04 % |
| Low QP |  | -0.02 % to 0.04 % |

## Results of method1 (two neighbors)

Table 2: Performance of method1 (two neighbors, normal QP)

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class D | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.02% | 0.05% | 0.05% | 0.00% | 0.03% | 0.02% |  |  |  |
|  | 0.02% | 0.05% | 0.05% | 0.00% | 0.03% | 0.02% |  |  |  |
| Class F | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -0.1% | -0.3% | 0.0% | 0.1% | -0.1% | 0.0% | -0.1% | 0.1% |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class C | 0.0% | -0.1% | 0.1% | 0.1% | 0.2% | 0.1% |  |  |  |
| Class D | 0.0% | -0.1% | 0.1% | 0.0% | 0.3% | -0.2% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.01% | -0.04% | 0.03% | 0.03% | 0.18% | -0.03% | 0.03% | -0.05% | 0.04% |
|  | 0.01% | -0.04% | 0.04% | 0.03% | 0.18% | -0.05% | 0.03% | -0.04% | 0.01% |
| Class F | -0.1% | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.2% | 0.0% | 0.0% | -0.3% | 0.0% |  |  |  |
| Class C | 0.0% | -0.2% | 0.0% | 0.0% | 0.1% | -0.1% |  |  |  |
| Class D | 0.0% | 0.1% | -0.1% | 0.0% | 0.4% | 0.0% |  |  |  |
| Class E | 0.1% | 0.3% | -0.6% | -0.1% | -1.2% | -0.4% |  |  |  |
| Overall | 0.03% | 0.08% | -0.12% | -0.01% | -0.21% | -0.13% |  |  |  |
|  | 0.03% | 0.06% | -0.09% | -0.01% | -0.18% | -0.18% |  |  |  |
| Class F | -0.1% | 0.1% | -0.3% | -0.1% | -0.4% | -0.7% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

Table 3: Performance of method1 (two neighbors, low QP)

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U |  |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.01% | 0.03% | 0.05% | 0.01% | 0.02% | 0.02% |  |  |  |
|  | 0.01% | 0.03% | 0.05% | 0.01% | 0.02% | 0.02% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.02% | 0.02% | 0.03% | 0.01% | 0.07% | 0.01% | 0.00% | 0.00% | 0.01% |
|  | 0.02% | 0.02% | 0.02% | 0.01% | 0.07% | 0.01% | 0.00% | 0.04% | 0.06% |
| Class F | -0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |  |  |  |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | -0.1% |  |  |  |
| Class E | 0.0% | -0.3% | -0.1% | -0.1% | -0.3% | -0.5% |  |  |  |
| Overall | 0.01% | -0.06% | 0.04% | 0.00% | -0.03% | -0.13% |  |  |  |
|  | 0.01% | -0.04% | 0.05% | 0.00% | -0.03% | -0.11% |  |  |  |
| Class F | 0.1% | -0.1% | 0.0% | 0.2% | 0.2% | -0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

## Results of method2 (no neighbors)

Table 4: Performance of method2 (no neighbors, normal QP)

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |  |  |  |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | -0.1% | -0.1% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class D | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.02% | 0.06% | 0.06% | -0.01% | -0.01% | -0.02% |  |  |  |
|  | 0.02% | 0.06% | 0.06% | -0.01% | -0.01% | -0.02% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | -0.2% | 0.0% | 0.2% | 0.1% |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
| Class D | 0.0% | -0.2% | 0.1% | 0.0% | 0.4% | 0.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.01% | -0.03% | 0.09% | 0.02% | 0.20% | 0.03% | 0.03% | 0.10% | 0.01% |
|  | 0.01% | -0.03% | 0.09% | 0.02% | 0.20% | -0.02% | 0.04% | 0.14% | 0.06% |
| Class F | -0.1% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.4% | 0.0% | -0.3% | 0.6% |  |  |  |
| Class C | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |  |  |  |
| Class D | 0.0% | -0.3% | 0.2% | 0.0% | 0.6% | -0.4% |  |  |  |
| Class E | -0.1% | 0.5% | -0.1% | 0.1% | -1.3% | -0.7% |  |  |  |
| Overall | 0.01% | 0.05% | 0.16% | 0.02% | -0.16% | -0.03% |  |  |  |
|  | 0.01% | 0.04% | 0.17% | 0.02% | -0.18% | -0.04% |  |  |  |
| Class F | -0.1% | -0.2% | -0.2% | -0.1% | 0.0% | -0.5% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

Table 5: Performance of method2 (no neighbors, low QP)

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | -0.1% | -0.2% | -0.2% |  |  |  |
| Class B | 0.0% | 0.0% | 0.1% | -0.1% | -0.1% | -0.2% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class D | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class E | 0.0% | 0.0% | 0.1% | -0.1% | -0.1% | -0.1% |  |  |  |
| Overall | 0.01% | 0.04% | 0.06% | -0.03% | -0.05% | -0.05% |  |  |  |
|  | -0.01% | 0.05% | 0.06% | -0.03% | -0.05% | -0.05% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.2% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class D | 0.0% | 0.0% | -0.1% | 0.0% | 0.2% | 0.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.01% | 0.03% | 0.03% | 0.01% | 0.06% | 0.02% | 0.00% | 0.08% | 0.10% |
|  | 0.01% | 0.03% | 0.03% | 0.01% | 0.07% | 0.02% | -0.02% | 0.09% | 0.15% |
| Class F | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 99% |  |  | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.2% | 0.0% | 0.0% | 0.2% |  |  |  |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |  |  |  |
| Class D | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
| Class E | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
| Overall | 0.01% | 0.01% | 0.10% | 0.00% | 0.04% | 0.10% |  |  |  |
|  | 0.01% | 0.01% | 0.09% | 0.00% | 0.03% | 0.12% |  |  |  |
| Class F | 0.0% | -0.2% | 0.0% | 0.2% | 0.3% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

Table 6: Performance of method2.1 (no neighbors, normal QP)

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U |  |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.2% | 0.2% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.1% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class D | 0.1% | 0.2% | 0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
| Class E | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.6% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.2% | 0.2% | 0.1% | 0.0% | -0.1% |
| Class C | 0.0% | 0.1% | 0.1% | 0.1% | -0.1% | 0.1% |  |  |  |
| Class D | 0.0% | -0.2% | 0.1% | 0.0% | 0.3% | -0.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.2% |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.2% |
| Class F | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | -0.2% | 0.3% |  |  |  |
| Class C | 0.0% | -0.1% | 0.0% | 0.1% | 0.1% | 0.1% |  |  |  |
| Class D | 0.0% | -0.4% | 0.4% | 0.0% | 0.4% | 0.4% |  |  |  |
| Class E | 0.0% | 0.6% | -0.3% | 0.0% | -1.6% | -0.3% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | 0.2% |  |  |  |
|  | 0.0% | 0.1% | 0.0% | 0.0% | -0.2% | 0.1% |  |  |  |
| Class F | -0.2% | 0.1% | -0.3% | -0.1% | 0.0% | -0.6% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

Table 7: Performance of method2.1 (no neighbors, low QP)

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U |  |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% |  |  |  |
| Class B | 0.0% | 0.1% | 0.1% | -0.1% | -0.1% | -0.1% |  |  |  |
| Class C | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class D | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class E | 0.0% | 0.1% | 0.1% | 0.0% | -0.1% | -0.1% |  |  |  |
| Overall | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.2% |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class C | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% |
| Class F | -0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% |  |  |  |
| Class C | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class D | 0.0% | -0.1% | -0.1% | 0.0% | 0.1% | -0.1% |  |  |  |
| Class E | 0.0% | 0.0% | 0.1% | 0.0% | -0.6% | 0.4% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% |  |  |  |
| Class F | -0.1% | -0.1% | -0.1% | 0.1% | 0.0% | -0.2% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |  |  |  |

# Conclusion

This contribution presents two kinds of complexity reduction method of significant map coding of large (16x16, 16x4, 4x16, 32x32, 32x8, 8x32) transform at high frequency area. In the proposed method1 reduces the number of neighbors to 2 from 4-5, while method2 eliminate neighboring dependency using constant index. Both methods  also eliminate conditional branches in 4x4 sub-block at high frequency area to improve throughput. The ratio of the area where complexity reduction is applied is about 62.5 %. At the 2 neighbor method, it is reported that average BD-rate change is -0.01 % to 0.03 % on common condition and 0.00 % to 0.02 % on low qp condition (qp = 12, 17, 22, 27). At the 0 neighbor method, it is reported that average BD-rate change is -0.01 % to 0.03 % on common condition and -0.03 % to 0.01 % on low qp condition. Enc Time and Dec Time change is negligible.

The method 2.1 is added on January 27. The method 2.1 is the same as method2 except that it doesn’t add new contexts but shares cxtCnt = 0 context at high frequency area. The average coding loss is negligible (up to 0.04 %) on normal QP and low QP test. It is recommended to adopt method 2.1 to CD.

# Neighbors

[1] T. Ikai, “Complexity reduction of significant map coding,” JCTVC-G308, JCT-VC 7th Meeting, Geneva, November, 2011

[2] V. Sze, “On significance map coding for CABAC”, JCTVC-G917, JCT-VC 7th Meeting, Geneva, November, 2011

# IPR Statement

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).