| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0096_r2 |
| --- | --- |

| Title: | AHG7: Controllable memory bandwidth reduction with bi-pred to uni-pred conversion |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba, JAPAN | Tel: Email: | +81 43 299 8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution presents bi-prediction restriction to reduce motion compensation complexity.In the proposed method inter_4x4_enabled_flag is replaced with inter_prediction_restriction_flag to control both PU size restriction and bi-prediction PU size restriction. If inter_prediction_restriction_flag is enabled, bi-prediction of 4x8 and 8x4 PU or 8x8 PU is prohibited depending on MinCUSize. Specifically 4x8 and 8x4 bi-prediction is prohibited when MinCUSize is 8, while 8x8 bi-prediction is prohibited when MinCUSize is 16. If bi-prediction is disabled, the related syntax inter_pred_flag is omitted from the bitstream and uni-prediction to bi-prediction conversion is performed in merge motion parameter derivation process. It is reported that average BD-rate loss is 0.0 % to 0.3 % with 6 % to 10 % less encode time when 4x8 and 8x4 bi-prediction restriction case, while encoder only case’s average loss is 0.2 % to 0.8 %. It is reported that BD-rate loss is xx % to 3.7 % with 21 % to 31 % less encoding time when 8x8 bi-prediction restriction case, while encoder only case’s average loss is 1.2 % to 4.6 %.

# Introduction

In current HM, inter_4x4_enabled_flag is used for control PU size. This syntax is beneficial for motion compensation complexity deduction especially for the worst case. However, considering bi-prediction compensation, the prohibition is not enough.  Therefore we replace inter_4x4_enabled_flag with inter_prediction_restriction_flag to control both PU size restriction and bi-prediction PU size restriction. To minimize coding loss, which is large in encoder only restriction, syntax omitting and modification of the merge motion parameter derivation process is introduced.

# Proposed method

When inter_prediction_restriction_flag is true (inter_4x4_enable_flag is false), the restriction shown in Table 1 and Figure 1 is applied depending on log2_min_cu_size_minus3. MinCUSize (SCU size) is defined 1 << log2_min_cu_size_minus3 + 3.

Figure 1: Controllable memory bandwidth reduction with inter_prediction_restriction_flag

Table 1 Proposed restriction

| inter_prediction_restriction_flag |  | log2_min_cu_sizes_minus3 = 0 | log2_min_cu_sizes_minus3 = 1 |
| --- | --- | --- | --- |
| 0 | prohibited PU size | - | 4x4, 4x8, 8x4 |
|  | prohibited Bipred PU size | - | - |
| 1 | prohibited PU size | 4x4 | 4x4, 4x8, 8x4 |
|  | prohibited Bipred PU size | 8x4, 4x8, 4x4 (!= 2Nx2N) | 8x8 (=NxN) |

Bi-prediction is prohibited with the following changes.

- NoBipredFlag

NoBipredFlag is derived as follows.

NoBipredFlag = inter_prediction_restriction_flag is true && ((log2MinCUSize == 3 && log2CUSize == 3 && PartMode != PART_2Nx2N) || (log2MinCUSize > 3 && log2CUSize == log2MinCUSize && PartMode == PART_NxN))

- Inter_pred_flag syntax omitting

NoBipredFlag is true, bi-prediction is prohibited. In this case, inter_pred_flag is not present in the syntax.

- Modification of merge candidate derivation process

The inter prediction parameters of merge mode is inferred from one of neighboring blocks. If the inferred parameters uses bi-prediction, the parameter is converted to uni-prediction as the follows.

If both predFlagL0[ xN, yN ] and predFlagL1[ xN, yN ] is 1, predFlagL0[ xN, yN ] is set to  0 and refIdxL0 is set to -1

And combined bi-predictive merging and non-scaled bi-predictive merging process is skipped. And zero merge candidate is using L0.

# WD text

The changes of WD text are highlighted as below.

// Changes of syntax

7.3.7 Prediction unit syntax

| prediction_unit( x0, y0 , log2CUSize ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| … |  |
| } else { /* MODE_INTER */ |  |
| if( entropy_coding_mode_flag ¦¦ PartMode != PART_2Nx2N ) |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( slice_type = = B ) { |  |
| NoBipredFlag = inter_prediction_restriction_flag is true && ((log2MinCUSize == 3 && log2CUSize == 3 && PartMode != PART_2Nx2N) ¦¦ (log2MinCUSize > 3 && (log2CUSize == log2MinCUSize && PartMode == PART_NxN)) |  |
| if (! NoBipredFlag) |  |
| inter_pred_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

8.4.2.1.1 Derivation process for luma motion vectors for merge mode

…

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps:

…

- When slice_type is equal to B and NoBipredFlag is equal to 0, the following applies.

  - The derivation process for combined bi-predictive merging candidates specified in subclause Error: Reference source not found is invoked…

  - The derivation process for non-scaled bi-predictive merging candidates specified in subclause Error: Reference source not found is invoked….

8.4.2.1.2 Derivation process for spatial merging candidates

...

When noBipredFlag is equal to 1, predFlagL0[ xN, yN ] is equal to 1 and predFlagL1[ xN, yN ] is equal to 1, predFlagL0[ xN, yN ] is set to  0 and refIdxL0 is set to -1

8.4.2.1.3 Derivation process for temporal merging candidates

...

When noBipredFlag is equal to 1, predFlagL0[ xN, yN ] is equal to 1 and predFlagL1[ xN, yN ] is equal to 1, predFlagL0[ xN, yN ] is set to  0 and refIdxL0 is set to -1

8.4.2.1.5	Derivation process for zero motion vector merging candidates

…

If (slice_type is equal to P or noBipredFlag is equal to 1), the candidate zeroCandm with m equal to ( numMergeCand − numInputMergeCand ) is added at the end of mergeCandList ( mergeCandList[ numMergeCand ] = zeroCandm ) and the reference indices, the prediction list utilization flags and the motion vectors of zeroCandm are dervied as follows and numMergeCand is incremented by 1.

…

Otherwise (slice_type is equal to B and noBipredFlag is equal to 0), the candidate zeroCandm with m equal to ( numMergeCand − numInputMergeCand ) is added at the end of mergeCandList ( mergeCandList[ numMergeCand ] = zeroCandm ) and the reference indices, the prediction list utilization flags and the motion vectors of zeroCandm are dervied as follows and numMergeCand is incremented by 1.

# Experimental result

The proposed method is implemented on HM-5.0 and simulation is conducted based on the common test condition. Since the proposed method only impacts on inter frame coding with B-frame, Random Access High Efficiency (RAHE), Low Delay B High efficiency (LBHE), Random Access Low Complexity (RALC) and Low Delay B Low Complexity (LBLC) and Random Access High Efficiency 10 bit (RAHE-10) are tested.

Table 1 Summary of the results.

|  |  | RA-HE | LB-HE | RA-LC | LB-LC | RA-HE10 |
| --- | --- | --- | --- | --- | --- | --- |
| 4x8, 8x4 bi-pred and 4x4 PU restriction (8x8 SCU size) | proposal | 0.1 % | 0.1 % | 0.2 % | 0.3 % | X.X % |
|  | encoder only | 0.4 % | 0.5 % | 0.5 % | 0.8 % | 0.2 % |
| 8x8 bi-pred restriction and less than 8x8 PU restriction (16x16 SCU emulation) | proposal | 1.8 % | 2.0 % | 3.0 % | 3.7 % | X.X % |
|  | encoder only | 2.2 % | 2.7 % | 3.5 % | 4.6 % | 1.2 % |

Table 2 Summary of the results of Class A / B

|  |  | RA-HE | LB-HE | RA-LC | LB-LC | RA-HE10 |
| --- | --- | --- | --- | --- | --- | --- |
| 8x8 bi-pred restriction and less than 8x8 PU restriction (16x16 SCU emulation) | proposal | 1.1 % / 1.0 % | / 1.0 % | 2.0%/ 1.7 % | / 2.1 % | X.X% / X.X% |
|  | encoder only | 1.8 % / 1.4 % | / 1.7 % | 2.9%/ 2.1 % | / 2.9 % | 0.9 % / 1.4 % |

Table 3 shows the result of the 4x8 and 8x4 proposal. Average BD-rate loss of 0.1 % to 0.3 % are observed.

Table 4 shows the result of the 4x8 and 8x4 encoder only results. Average BD-rate loss of 0.2 % to 0.8 % are observed.

Table 5 shows the result of the 8x8 proposal. Average BD-rate loss of 0.X % to 3.7 % are observed.

Table 6 shows the result of the 8x8 encoder only results. Average BD-rate loss of 1.2 % to 4.6 % are observed.

Table 3: Performance of 4x8 and 8x4 bi-prediction restriction (proposed)

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | -0.1% | 0.1% | 0.2% | 0.1% |  |  |  |
| Class B | 0.1% | 0.0% | 0.0% | 0.2% | 0.2% | 0.1% |  |  |  |
| Class C | 0.1% | 0.2% | 0.3% | 0.3% | 0.3% | 0.3% |  |  |  |
| Class D | 0.3% | 0.2% | 0.2% | 0.3% | 0.1% | 0.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.1% | 0.1% | 0.1% | 0.2% | 0.2% | 0.2% |  |  |  |
|  | 0.1% | 0.1% | 0.1% | 0.2% | 0.2% | 0.2% |  |  |  |
| Class F | -0.3% | -0.1% | -0.2% | -0.2% | -0.1% | -0.2% |  |  |  |
| Enc Time[%] | 94% |  |  | 91% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.1% | 0.1% | 0.3% | 0.2% | 0.0% | 0.0% |  |  |  |
| Class C | 0.2% | 0.2% | 0.1% | 0.4% | 0.5% | 0.2% |  |  |  |
| Class D | 0.3% | -0.2% | 0.1% | 0.4% | 1.0% | 0.8% |  |  |  |
| Class E | 0.0% | -0.2% | -0.3% | 0.3% | -0.1% | 0.2% |  |  |  |
| Overall | 0.1% | 0.0% | 0.1% | 0.3% | 0.3% | 0.3% |  |  |  |
|  | 0.1% | 0.0% | 0.0% | 0.3% | 0.3% | 0.3% |  |  |  |
| Class F | -0.5% | -0.3% | -0.6% | -0.6% | -1.2% | -0.7% |  |  |  |
| Enc Time[%] | 92% |  |  | 90% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |  |  |  |

Table 4: Performance of 4x8 and 8x4 bi-prediction restriction (encoder only)

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.3% | 0.3% | 0.2% | 0.5% | 0.5% | 0.4% | 0.2% | 0.3% | 0.2% |
| Class B | 0.2% | 0.2% | 0.1% | 0.4% | 0.3% | 0.2% | 0.2% | 0.2% | 0.1% |
| Class C | 0.4% | 0.5% | 0.4% | 0.6% | 0.6% | 0.5% |  |  |  |
| Class D | 0.6% | 0.5% | 0.5% | 0.7% | 0.5% | 0.4% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.4% | 0.4% | 0.3% | 0.5% | 0.4% | 0.4% | 0.2% | 0.2% | 0.2% |
|  | 0.4% | 0.4% | 0.3% | 0.5% | 0.4% | 0.4% | 0.2% | 0.2% | 0.2% |
| Class F | -0.2% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 93% |  |  | 91% |  |  | 93% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.3% | 0.2% | 0.3% | 0.5% | 0.3% | 0.2% |  |  |  |
| Class C | 0.6% | 0.4% | 0.4% | 0.9% | 0.8% | 0.9% |  |  |  |
| Class D | 0.9% | 0.6% | 0.8% | 1.1% | 1.1% | 0.6% |  |  |  |
| Class E | 0.3% | 0.4% | 0.2% | 0.7% | 0.1% | 0.8% |  |  |  |
| Overall | 0.5% | 0.4% | 0.4% | 0.8% | 0.6% | 0.6% |  |  |  |
|  | 0.5% | 0.4% | 0.4% | 0.8% | 0.6% | 0.6% |  |  |  |
| Class F | -0.2% | -0.3% | -0.5% | 0.0% | 0.3% | -0.5% |  |  |  |
| Enc Time[%] | 92% |  |  | 89% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |  |  |  |

Table 5: Performance of 8x8 bi-prediction restriction (proposed)

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 1.1% | 1.9% | 1.6% | 2.0% | 3.1% | 2.8% |  |  |  |
| Class B | 1.0% | 0.7% | 0.7% | 1.7% | 1.4% | 1.5% |  |  |  |
| Class C | 1.9% | 2.2% | 2.4% | 3.3% | 3.6% | 3.9% |  |  |  |
| Class D | 3.1% | 3.1% | 3.3% | 4.8% | 5.1% | 5.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 1.8% | 1.9% | 2.0% | 3.0% | 3.2% | 3.3% |  |  |  |
|  | 1.8% | 1.9% | 2.0% | 3.0% | 3.2% | 3.3% |  |  |  |
| Class F | 1.7% | 2.0% | 2.0% | 2.7% | 3.2% | 3.1% |  |  |  |
| Enc Time[%] | 79% |  |  | 71% |  |  |  |  |  |
| Dec Time[%] | 97% |  |  | 95% |  |  |  |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 1.0% | 0.2% | 0.4% | 2.1% | 1.0% | 1.2% |  |  |  |
| Class C | 2.4% | 2.2% | 2.5% | 4.2% | 4.1% | 4.0% |  |  |  |
| Class D | 3.7% | 2.9% | 4.3% | 6.3% | 6.5% | 6.4% |  |  |  |
| Class E | 0.8% | 0.5% | 0.0% | 2.1% | 1.1% | 1.4% |  |  |  |
| Overall | 2.0% | 1.5% | 1.8% | 3.7% | 3.2% | 3.2% |  |  |  |
|  | 2.0% | 1.5% | 1.8% | 3.7% | 3.1% | 3.2% |  |  |  |
| Class F | 2.9% | 3.0% | 2.9% | 4.7% | 4.3% | 4.2% |  |  |  |
| Enc Time[%] | 78% |  |  | 69% |  |  |  |  |  |
| Dec Time[%] | 98% |  |  | 95% |  |  |  |  |  |

Table 6: Performance of 8x8 bi-prediction restriction (encoder only)

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 1.8% | 2.3% | 2.0% | 2.9% | 3.7% | 3.5% | 0.9% | 1.1% | 1.3% |
| Class B | 1.4% | 1.0% | 1.0% | 2.1% | 1.8% | 1.8% | 1.4% | 1.1% | 1.0% |
| Class C | 2.2% | 2.5% | 3.0% | 3.8% | 4.2% | 4.6% |  |  |  |
| Class D | 3.4% | 3.4% | 3.7% | 5.3% | 5.7% | 5.7% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 2.2% | 2.2% | 2.4% | 3.5% | 3.8% | 3.8% | 1.2% | 1.1% | 1.1% |
|  | 2.2% | 2.2% | 2.4% | 3.5% | 3.8% | 3.8% | 1.2% | 1.1% | 1.1% |
| Class F | 2.1% | 2.3% | 2.3% | 3.1% | 3.6% | 3.5% |  |  |  |
| Enc Time[%] | 77% |  |  | 69% |  |  | 77% |  |  |
| Dec Time[%] | 98% |  |  | 96% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 1.7% | 0.7% | 0.8% | 2.9% | 1.7% | 1.9% |  |  |  |
| Class C | 3.2% | 3.0% | 3.7% | 5.3% | 5.5% | 5.8% |  |  |  |
| Class D | 4.5% | 4.0% | 5.2% | 7.4% | 8.1% | 8.0% |  |  |  |
| Class E | 1.3% | 0.4% | 0.1% | 2.9% | 1.9% | 2.7% |  |  |  |
| Overall | 2.7% | 2.1% | 2.5% | 4.6% | 4.3% | 4.5% |  |  |  |
|  | 2.7% | 2.1% | 2.5% | 4.6% | 4.3% | 4.5% |  |  |  |
| Class F | 3.7% | 4.1% | 3.7% | 5.7% | 5.6% | 6.3% |  |  |  |
| Enc Time[%] | 75% |  |  | 67% |  |  |  |  |  |
| Dec Time[%] | 97% |  |  | 94% |  |  |  |  |  |

# Conclusion

This contribution presents mainly two parts. The first part is introducing inter_prediction_restriction_flag, which enables restriction for both PU size and bi-prediction PU size. This flag is more flexible and more effective for memory band-width reduction than the current inter_4x4_enable_flag. The second part is bi-pred to uni-pred conversion at merge mode candidate derivation process. This part solves the merge mode candidate problem that when bi-prediction is restricted, some or all merge mode candidates might be impossible to select. With the second part, the coding efficiency can be improved when both 8x4 and 8x8 bi-prediction restriction case. We recommend these proposals adopted in HM and CD.

# References

[1] J. Jung, B. Bross, P. Chen, and W.-J. Han, “CE9: Summary report of core experiment 9 on MV Coding and Skip/Merge operations” , JCT-VC 6th Meeting, Torino, 2011

[2] T. Ikai, “Bi-prediction restriction in small PU” ,JCTVC-G307, JCT-VC 7th Meeting, Geneva, November, 2011

# IPR Statement

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).