| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0098 |
| --- | --- |

| Title: | Modified definitions of intra mode and most probable mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomoyuki Yamamoto 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba JAPAN | Tel: Email: | +81 43 299 8526 yamamoto.tomoyuki@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution proposes two simplifications with regard to intra mode coding. The First simplification is on the numbering of the intra prediction modes. With the proposed numbering, the main direction of intra modes can be calcurated instead of referencing table. The second simplification is on the MPM derivation. The proposal omits conditional swapping of two MPM candidates. Experimental results show that those simplifications bring almost no impact (0.0% on average) in terms of coding efficiency.

# Introduction

In the last meeting, it was decided to remove contexts which are used to decode syntax relating intra mode such as mpm_idx and rem_intra_luma_pred_mode. From the change, the order of two MPMs and the order of remaining modes are no more important. In other words, changing the order of MPMs or remaining modes should not cause degradation of coding efficiency.

By utilizing the above characteristics, two simplifications of intra mode coding is proposed as detailed in the following sections.

# Renumbering of intra prediction modes

The numbering of intra modes in the proposed method and HM-5.0 are shown in table 1. The proposed numbering has following properties compared to the original one.

a) Even numbers are assigned to VER modes whereas odd numbers to HOR modes.

b) Four successive numbers are assigned to the directional modes having the same gradient index (the first two corresponds to negative index and the latter two to positive index).

These properties could be used to simplify prediction process as well as other intra mode dependent processes.

Property a) could simplify intra prediction process. In the current HM, intra mode is firstly reordered using table (intraPredOrder) to determine the main direction: VER or HOR. With the proposed numbering, the main direction can be determined by checking the least significant bit of intraPredMode (i.e. m&1 where m is the value of intraPredMode). The proposed numbering is also beneficial to mode dependent coefficient scan and mode dependent DCT/DST selection which also utilize the main direction. In addition, it is also good to the non-normative intra mode dependent algorithms such as simplified mode decision and post filtering.

Property b) can be used to reduce table size for deriving angle (intraPredAngle) used in angular prediction. Absolute value of angle can be indexed using (m+1) >> 2. The sign of the angle can be determined by checking the value of (m+1) & 2.

Table 1. Numbering of intra prediction mode

| intraPred Mode | Proposed | HM-5.0 |
| --- | --- | --- |
| 0 | Planar | Planar |
| 1 | VER | VER |
| 2 | HOR | HOR |
| 3 | VER-8 | DC |
| 4 | DC | VER-8 |
| 5 | VER+8 | VER-4 |
| 6 | HOR+8 | VER+4 |
| 7 | VER-2 | VER+8 |
| 8 | HOR-2 | HOR-4 |
| 9 | VER+2 | HOR+4 |
| 10 | HOR+2 | HOR+8 |
| 11 | VER-4 | VER-6 |
| 12 | HOR-4 | VER-2 |
| 13 | VER+4 | VER+2 |
| 14 | HOR+4 | VER+6 |
| 15 | VER-6 | HOR-6 |
| 16 | HOR-6 | HOR-2 |
| 17 | VER+6 | HOR+2 |
| 18 | HOR+6 | HOR+6 |

| intraPred Mode | Proposed | HM-5.0 |
| --- | --- | --- |
| 19 | VER-1 | VER-7 |
| 20 | HOR-1 | VER-5 |
| 21 | VER+1 | VER-3 |
| 22 | HOR+1 | VER-1 |
| 23 | VER-3 | VER+1 |
| 24 | HOR-3 | VER+3 |
| 25 | VER+3 | VER+5 |
| 26 | HOR+3 | VER+7 |
| 27 | VER-5 | HOR-7 |
| 28 | HOR-5 | HOR-5 |
| 29 | VER+5 | HOR-3 |
| 30 | HOR+5 | HOR-1 |
| 31 | VER-7 | HOR+1 |
| 32 | HOR-7 | HOR+3 |
| 33 | VER+7 | HOR+5 |
| 34 | HOR+7 | HOR+7 |
| 35 | LM | LM |

# Omitting conditional swap of MPM candidates

In the HM-5.0, two MPM candidates are conditionally swapped so that intra mode with the smaller intraPredMode value is assigned to the MPM corresponding to mpm_idx=0. In the proposed method, such conditional swap is omitted in MPM derivation process. Intra mode decoding process of the proposed method and HM-5.0 are shown in Figure 1. Although the conditional swap is still required in the remaining mode index derivation, the proposed method can reduce the decoding process when MPM is selected.

Figure 1. Decoding process of intra mode

a) Proposed

b) HM-5.0

# Experimental Result

The experiments are conducted to evaluate coding efficiency impacts of the above two modifications. Three configurations: i) modified intra mode numbering, ii) omitting conditional swap, iii) combination of i) and ii), are tested. Results are summarized in Table 2, Table 3, and Table 4. It can be said that no major impact is observed with the proposed modifications.

Table 2. Result of modified intra mode numbering

Table 3. Result of omitting conditional swap

Table 4. Result of combination of modified intra mode numbering and omitting conditional swap

# Conclusion

This contribution proposes two modifications with regard to intra mode coding: modified intra mode numbering and omitting conditional swap. The two modifications can simplify intra prediction process as well as other intra mode dependent process such as MDCS and MDDT. Therefore, it is recommended to adopt these modifications into HM and WD.

# Patent rights declaration(s)

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).