| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-Hxxx |
| --- | --- |

| Title: | On CABAC context for partition mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomoyuki Yamamoto 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba JAPAN | Tel: Email: | +81 43 299 8526 yamamoto.tomoyuki@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution report the difference between HM and WD on CABAC context derivation for partition mode information. In order to improve the situation, it is proposed to apply bypass mode for the 3rd and 4th bins of the syntax part_mode. The proposed modification is reported to cause no impact on coding efficiency.

# Binarization and context for partition mode information

In the last meeting, JCTVC-G1042 [1] was adopted and it was integrated into the latest HM [2] and WD [3]. Figure 1 illustrates binarization applied to partition mode information, pred_mode_flag and part_mode, when inter 4x4 partition is disabled. This binarization is used for both in the latest HM and WD. However, with regard to the context, what is described in WD is different from what is implemented in HM.

Table 1 is the definition of contexts in WD. In B_Slice or P_Slice, two contexts were used for part_mode: the one is for binIdx 0 and 2, the other is for binIdx 1 and 3.

In HM, for the binIdx 0 and 1 of part_mode, the context is applied as indicated in WD. But for binIdx 2 and 3, additional contexts are used depending on whther PU is divided horizontally or vertically. In other words, depending on the decode bin value of binIdx 1 of part_mode, the different contexts are used.

In order to improve the situation, two solutions are presented in the following sections.

# Solution 1: WD modification

First solution is to modify WD so that HM implementation is clearly described. Table 2 shows the example modification of the table used for ctxIdx derivation. As you could notice, binIdx 2 and 3 of part_mode depends on the value of binIdx 1. In our understanding, it is the undesirable characteristics that the context of part of bins depends on the preceeding bins of the same syntax.

# Solution 2: context removal

Second solution is to apply bypass mode for binIdx 2 and 3. Actually, this change is what is proposed in the last meeting [4]. Table 3 shows the modification of the table for ctxIdx derivation. Since solution 2 changes the bitstream, the performance is evaluated. As summarized in Table 4, no major impact on coding efficiency is observed. Note that because binIdx 2 and 3 are used when AMP is enabled, only HE inter configurations were tested.

Figure 1. Illustration of binarization of prediction partition information when inter 4x4 partition is disabled (Note: some bins may conditionally be omitted.)

Table 1. Definition of ctxIdx in WD5 (d4)

| Syntax element | ctxIdxTable, ctxIdxOffset |  | binIdx |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 | 3 | >=4 |
| pred_mode_flag | Table 9-25 | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |
| part_mode | Table 9-25 | 2 | 0 | na | na | na | na |
|  |  | 3 | 0 | 1 | 0 | 1 | na |
|  |  | 6 | 0 | 1 | 0 | 1 | na |
| part_mode (cLog2CUSize = = Log2MinCUSize && cLog2CUSize > 3 ¦ ¦ inter_4x4_enabled_flag) | Table 9-25 | 3 | 0 | 1 | 2 | na | na |
|  |  | 6 | 0 | 1 | 2 | na | na |

Table 2. Definition of ctxIdx in solution 1 (Implementation of HM-5.0. ‘bin1’ is bin value of binIdx=1)

| Syntax element | ctxIdxTable, ctxIdxOffset |  | binIdx |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 | 3 | >=4 |
| pred_mode_flag | Table 9-25 | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |
| part_mode | Table 9-25 | 2 | 0 | na | na | na | na |
|  |  | 3 | 0 | 1 | 3 (bin1 is 1) 4 (bin1 is 0) | 5 (bin1 is 1) 6 (bin1 is 0) | na |
|  |  | 10 | 0 | 1 | 3 (bin1 is 1) 4 (bin1 is 0) | 5 (bin1 is 1) 6 (bin1 is 0) | na |
| part_mode (cLog2CUSize = = Log2MinCUSize && cLog2CUSize > 3 ¦ ¦ inter_4x4_enabled_flag) | Table 9-25 | 3 | 0 | 1 | 2 | na | na |
|  |  | 10 | 0 | 1 | 2 | na | na |

Table 3. Definition of ctxIdx in solution 2

| Syntax element | ctxIdxTable, ctxIdxOffset |  | binIdx |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 | 3 | >=4 |
| pred_mode_flag | Table 9-25 | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |
| part_mode | Table 9-25 | 2 | 0 | na | na | na | na |
|  |  | 3 | 0 | 1 | na | na | na |
|  |  | 6 | 0 | 1 | na | na | na |
| part_mode (cLog2CUSize = = Log2MinCUSize && cLog2CUSize > 3 ¦ ¦ inter_4x4_enabled_flag) | Table 9-25 | 3 | 0 | 1 | 2 | na | na |
|  |  | 6 | 0 | 1 | 2 | na | na |

Table 4. Coding efficiency of solution 2 compared to HM-5.0.

# Conclusion

This contribution reports the discrepancy between HM and WD on CABAC context of part_mode and two solutions are presented. The solution 1 only needs WD changes, but the context of derivation of part of bins of part_mode depends on the other bin within the same syntax. The solution 2 requires changes for both WD and HM, but removes the dependency and uses fewer contexts. In addition, WD text is simpler in solution 2. Therefore, we recommend to adopt solution 2.

# References

[1] S.Oudin et al., “Harmonization of the prediction and partitioning mode binarization of P and B slices”, JCTVC-G1042_r2, Geneva, November, 2011.

[2] JCT-VC, “HEVC Test Model Software 5.0,” 
   https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-5.0

[3] JCT-VC, “WD5: Working Draft 5 of High-Efficiency Video Coding,” JCTVC-G1103_d6, Geneva, November, 2011.

[4] T.Yamamoto, K.Misra, and A.Segall “On partition size information coding in B-slices using CABAC,” JCTVC-G709, Geneva, November, 2011.

# Patent rights declaration(s)

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).