| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-H0102 |
| --- | --- |

| Title: | CE8.a.1: Crosscheck of MQT's One-stage SAO and ALF with LCU-based syntax (JCTVC-H0272) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Elena Alshina Samsung #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba, JAPAN | Tel: Email: | +82-10-3026-1305 elena_a.alshina@samsung.com +81 43 299 8526 ikai.tomohiro@sharp.co.jp |
| Source: | Samsung Corporation and SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution provides cross-check results of “CE8.a.1: Crosscheck of MQT's One-stage SAO and ALF with LCU-based syntax” (JCTVC-H0272). The coding results perfectly match with those provided by proponents. The current HM5.0’s ALF uses picture level area classification, so it is difficult to apply in low latency encoding. The evaluated method named LCU base syntax, enables both picture-base encoding and LCU-base encoding, which is applicable in low latency encoding.

On single-slice per picture case, the average BD-rate loss of AI-HE, LB-HE, RA-HE, RA10-HE of picture based encoding is 0.0 %, 0.0 %, 0.2 % and 0.5 % at picture based encoding and 0.9 %, 0.9 %, 1.5 % at LCU-based encoding, compared with HM50 (ALF on). On multi-slice per picture case, the average BD-rate gain is 4.2 % and 10.3 % at LCU-based encoding, compared to multi-slice anchor (ALF off).

# Validation

Each LCU of the slice is classified into regions.

- Each region uses

  - SAO (if MODE_SAO)

  - ALF + SAO (if MODE_FLT)

- The regions is represented by the following merge technique

  - row merge

  - up merge

  - left merge

  - direct merge (referred by region index)

- The maximum number of filters / offsets per color component is coded in SPS

  - number of filters is up to 16 (for test)

  - number of filters is up to 1024 (for test)

# Experimental result

## Single-slice per picture

Table 1: Simulation results of test1 (picture based encoding)

|  | All Intra HE |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U |  |
| Class A (8bit) | -0.2% | -0.8% | -0.3% |  |  |  |
| Class B | 0.0% | -0.1% | -0.2% |  |  |  |
| Class C | -0.2% | 0.4% | 0.4% |  |  |  |
| Class D | 0.0% | 0.3% | 0.6% |  |  |  |
| Class E | 0.0% | 1.4% | 1.0% |  |  |  |
| Overall | 0.0% | 0.3% | 0.3% |  |  |  |
|  | 0.0% | 0.3% | 0.3% |  |  |  |
| Class F | -0.2% | 0.0% | 0.3% |  |  |  |
| Enc Time[%] | 114% |  |  |  |  |  |
| Dec Time[%] | 104% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.1% | -0.8% | -0.3% | 0.4% | 6.0% | 6.6% |
| Class B | 0.1% | -0.6% | -0.4% | 0.5% | 1.5% | 1.8% |
| Class C | 0.0% | -0.2% | 0.3% |  |  |  |
| Class D | 0.0% | 0.4% | 0.4% |  |  |  |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.2% | 0.0% | 0.5% | 3.5% | 3.9% |
|  | 0.0% | -0.3% | 0.0% | 0.5% | 3.4% | 3.9% |
| Class F | 0.1% | 0.5% | 0.5% |  |  |  |
| Enc Time[%] | 102% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 87% |  |  |
|  | Low delay B HE |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.3% | -0.3% | 0.1% |  |  |  |
| Class C | 0.2% | -1.3% | -0.5% |  |  |  |
| Class D | 0.2% | 1.0% | 1.6% |  |  |  |
| Class E | 0.1% | 5.7% | 0.7% |  |  |  |
| Overall | 0.2% | 0.9% | 0.5% |  |  |  |
|  | 0.2% | 1.0% | 0.5% |  |  |  |
| Class F | 0.2% | 0.3% | 0.8% |  |  |  |
| Enc Time[%] | 102% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  |  |  |  |

Table 2: Simulation results of test1 (LCU based encoding)

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A (8bit) | 1.3% | 2.0% | 2.4% |
| Class B | 0.9% | 2.8% | 2.9% |
| Class C | 0.4% | 3.3% | 3.8% |
| Class D | 0.6% | 3.1% | 3.7% |
| Class E | 1.5% | 6.4% | 7.2% |
| Overall | 0.9% | 3.5% | 4.0% |
| Class F | 0.0% | 2.2% | 2.3% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 91% |  |  |
|  | Random Access HE |  |  |
| Class A (8bit) | 1.2% | 0.6% | 0.9% |
| Class B | 1.0% | 1.7% | 1.2% |
| Class C | 0.6% | 1.0% | 1.8% |
| Class D | 0.8% | 0.9% | 1.4% |
| Class E |  |  |  |
| Overall | 0.9% | 1.2% | 1.4% |
| Class F | 0.8% | 2.6% | 2.6% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 95% |  |  |
|  | Low delay B HE |  |  |
| Class A |  |  |  |
| Class B | 2.0% | 4.1% | 4.8% |
| Class C | 1.1% | 2.1% | 3.3% |
| Class D | 1.5% | 1.9% | 3.4% |
| Class E | 1.2% | 10.0% | 6.0% |
| Overall | 1.5% | 4.2% | 4.3% |
| Class F | 1.7% | 4.5% | 5.8% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 88% |  |  |

## Multi-slice per picture

Table 3: Simulation results of test1 (LCU based encoding)

|  | Low delay B HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -2.8% | -3.7% | -4.2% |
| Class C | -3.4% | -5.2% | -7.5% |
| Class D | -1.8% | -2.2% | -2.5% |
| Class E | -4.9% | -5.7% | -5.3% |
| Class F | -8.4% | -6.3% | -8.6% |
| Overall | -4.2% | -4.5% | -5.6% |
|  | -4.1% | -4.5% | -5.6% |
| Enc Time[%] | 109% |  |  |
| Dec Time[%] | 111% |  |  |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -13.7% | -6.4% | -6.7% |
| Class C | -8.3% | -7.0% | -8.9% |
| Class D | -4.3% | -3.5% | -4.0% |
| Class E | -16.4% | -9.3% | -9.1% |
| Class F | -9.7% | -6.9% | -9.1% |
| Overall | -10.3% | -6.5% | -7.5% |
|  | -10.4% | -6.5% | -7.5% |
| Enc Time[%] | 113% |  |  |
| Dec Time[%] | 113% |  |  |

# Conclusion

The coding results and software was confirmed.

# References

[2] C.-Y. Chen, C.-Y. Tsai, C.-M. Fu, Y.-W. Huang, S. Lei, I. S. Chong, M. Karczewicz, T. Yamakage, T. Itoh, T. Watanabe, T. Chujoh, “CE8.a.1: One-stage SAO and ALF with LCU-based syntax” , JCTVC-H0272, JCT-VC 8th Meeting, San Jose, February 2012