| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0103_r2 |
| --- | --- |

| Title: | CE8.a.3: Crosscheck of MQS's SAO with LCU-based syntax (JCTVC-H0273) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba, JAPAN | Tel: Email: | +81 43 299 8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution provides cross-check results of “CE8.a.3: SAO with LCU-based syntax” (JCTVC-H0273). The coding results perfectly match with those provided by proponents. The current HM5.0’s SAO uses quad-tree partitioning at picture-level, so it is difficult to apply in low latency encoding. The evaluated method named LCU base syntax, enables both picture-base encoding and LCU-base encoding, which is applicable in low latency encoding.

It is reported that the average BDrate loss of AI-HE, RA-HE, LB-HE, RA-HE10 is 0.0 %, 0.1 %, 0.3 % and 0.1 % at picture-base encoding and 0.0 %, 0.4 %, 0.7 % and 0.5 % at LCU-base encoding when single-slice per picture case. The combination of LCU-base syntax and H0114’s edge offset encoding method is also tested. On the combined results, it is reported that the average BDrate loss of AI-HE, RA-HE, LB-HE, RA-HE10 is 0.0 %, 0.1 %, 0.3 % and 0.1 % at picture-base encoding and 0.0 %, 0.3 %, 0.6 % and 0.5 % at LCU-base encoding when single-slice per picture case.

When single-slice per picture case, average BDrate gain of LB-HE, LP-HE10 is 1.0 %, 1.5 % respectively.

# Methods

- There are some merge syntaxs

  - row merge

  - left merge

  - up merge

- The merge process is summarized as the following

  - If row merge is used, some consecutive LCUs parameters are copied from upper LCU

    - the number of consecutive LCUs is derived from upper LCU parameters.

  - Otherwise, if left merge is used, the consecutive LCUs parameter are copied from left LCU

    - the number of consecutive LCUs is derived a syntax.

  - Otherwise, if up merge is used, the LCU parameter is copied from upper LCU

  - Otherwise, the LCU parameter is decoded from bitstream.

# Experimental result

## Single-slice per picture

Table 1: Summary of single-slice per picture case

|  | AI-HE | RA-HE | LB-HE | RA-HE10 | AI-LC | RA- LC | LB- LC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| test1 (picture-based encoding) | 0.0 % | 0.1 % | 0.3 % | 0.1 % | -0.6 % | -1.0 % | -1.8 % |
| test2 (LCU-based encoding) | 0.0 % | 0.4 % | 0.7 % | 0.5 % | -0.6 % | -0.8 % | -1.6 % |

Table 2: Summary of single-slice per picture case with JCTVC-H0114)

|  | AI-HE | RA-HE | LB-HE | RA-HE10 | AI-LC | RA- LC | LB- LC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| test3 (test1 + JCTVC-H0114) | 0.0 % | 0.1 % | 0.3 % | 0.1 % | -0.7 % | -1.0 % | -1.9 % |
| test4 (test2 + JCTVC-H0114) | 0.0 % | 0.3 % | 0.6 % | 0.5 % | -0.7 % | -0.8 % | -1.6 % |

Table 1: Simulation results of test1 (picture based encoding)

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U |  |
| Class A (8bit) | 0.0% | 0.1% | 0.1% | -1.1% | -2.1% | -2.3% |  |  |  |
| Class B | 0.0% | 0.2% | 0.4% | -0.5% | -2.3% | -3.3% |  |  |  |
| Class C | 0.1% | 0.1% | 0.1% | -0.6% | -2.4% | -3.4% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | -0.5% | -1.7% | -2.3% |  |  |  |
| Class E | 0.1% | 0.1% | 0.1% | -0.8% | -4.0% | -3.7% |  |  |  |
| Overall | 0.0% | 0.1% | 0.2% | -0.6% | -2.4% | -3.1% |  |  |  |
|  | 0.0% | 0.1% | 0.2% | -0.6% | -2.4% | -3.0% |  |  |  |
| Class F | 0.1% | 0.1% | 0.1% | -2.8% | -3.6% | -5.4% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 104% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.1% | 0.1% | 0.1% | -1.2% | -2.3% | -3.0% | 0.1% | 0.3% | 0.3% |
| Class B | 0.2% | 0.3% | 0.5% | -1.7% | -5.1% | -5.0% | 0.1% | 0.1% | 0.3% |
| Class C | 0.1% | 0.1% | 0.1% | -0.8% | -3.9% | -5.3% |  |  |  |
| Class D | 0.1% | 0.1% | 0.1% | -0.3% | -2.2% | -2.5% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.1% | 0.2% | 0.2% | -1.0% | -3.6% | -4.1% | 0.1% | 0.2% | 0.3% |
|  | 0.1% | 0.2% | 0.2% | -1.0% | -3.6% | -4.2% | 0.1% | 0.2% | 0.3% |
| Class F | 0.2% | 0.2% | 0.2% | -5.1% | -6.5% | -9.5% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 104% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.3% | 0.6% | 0.9% | -2.4% | -8.5% | -9.4% |  |  |  |
| Class C | 0.2% | 0.2% | 0.2% | -1.5% | -7.3% | -10.9% |  |  |  |
| Class D | 0.1% | 0.1% | 0.1% | -0.7% | -3.8% | -5.5% |  |  |  |
| Class E | 0.6% | 0.6% | 0.6% | -2.8% | -15.3% | -13.7% |  |  |  |
| Overall | 0.3% | 0.4% | 0.5% | -1.8% | -8.3% | -9.6% |  |  |  |
|  | 0.3% | 0.4% | 0.4% | -1.8% | -8.2% | -9.6% |  |  |  |
| Class F | 0.4% | 0.4% | 0.4% | -8.8% | -12.2% | -16.0% |  |  |  |
| Enc Time[%] | 101% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 104% |  |  |  |  |  |

Table 1: Simulation results of test2 (LCU based encoding)

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U |  |
| Class A (8bit) | 0.0% | 0.1% | 0.1% | -1.1% | -2.1% | -2.2% |  |  |  |
| Class B | 0.0% | 0.3% | 0.5% | -0.5% | -2.2% | -3.2% |  |  |  |
| Class C | 0.1% | 0.1% | 0.1% | -0.6% | -2.3% | -3.4% |  |  |  |
| Class D | 0.0% | 0.1% | 0.1% | -0.5% | -1.7% | -2.3% |  |  |  |
| Class E | 0.1% | 0.2% | 0.1% | -0.8% | -3.9% | -3.7% |  |  |  |
| Overall | 0.0% | 0.2% | 0.2% | -0.6% | -2.4% | -3.0% |  |  |  |
|  | 0.0% | 0.2% | 0.2% | -0.6% | -2.4% | -3.0% |  |  |  |
| Class F | 0.1% | 0.2% | 0.2% | -2.8% | -3.5% | -5.3% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 104% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.4% | 0.1% | 0.3% | -0.9% | -2.5% | -3.2% | 0.5% | 3.0% | 3.5% |
| Class B | 0.5% | 1.2% | 1.2% | -1.4% | -4.6% | -4.8% | 0.5% | 2.2% | 2.4% |
| Class C | 0.4% | 0.7% | 1.0% | -0.7% | -3.8% | -5.1% |  |  |  |
| Class D | 0.3% | 0.2% | 0.6% | -0.1% | -2.2% | -2.5% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.4% | 0.6% | 0.9% | -0.8% | -3.5% | -4.1% | 0.5% | 2.6% | 2.9% |
|  | 0.4% | 0.7% | 0.9% | -0.8% | -3.5% | -4.1% | 0.5% | 2.6% | 2.9% |
| Class F | 0.8% | 1.5% | 1.6% | -4.8% | -5.5% | -8.3% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 103% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.9% | 3.9% | 4.9% | -1.8% | -5.9% | -6.6% |  |  |  |
| Class C | 0.6% | 2.3% | 2.9% | -1.4% | -6.0% | -9.2% |  |  |  |
| Class D | 0.5% | 1.3% | 2.7% | -0.4% | -3.4% | -4.7% |  |  |  |
| Class E | 0.9% | 5.9% | 8.1% | -2.8% | -7.5% | -6.9% |  |  |  |
| Overall | 0.7% | 3.2% | 4.4% | -1.6% | -5.6% | -6.9% |  |  |  |
|  | 0.7% | 3.2% | 4.4% | -1.5% | -5.6% | -6.8% |  |  |  |
| Class F | 1.1% | 4.1% | 4.3% | -8.3% | -9.0% | -12.3% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 98% |  |  | 101% |  |  |  |  |  |

## Multi-slice per picture

Table 2: Summary of multi-slice per picture case (anchor is SAO off multi-slice)

|  | Low delay B HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.7% | -4.2% | -4.7% |
| Class C | -1.3% | -5.4% | -7.4% |
| Class D | -0.7% | -2.4% | -3.7% |
| Class E | -1.8% | -2.0% | -3.9% |
| Overall | -1.0% | -3.6% | -5.0% |
|  | -1.0% | -3.6% | -5.0% |
| Class F | -6.6% | -6.2% | -9.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -1.1% | -5.9% | -6.0% |
| Class C | -1.7% | -6.4% | -8.5% |
| Class D | -1.2% | -4.2% | -5.2% |
| Class E | -2.2% | -2.4% | -3.9% |
| Overall | -1.5% | -4.9% | -6.0% |
|  | -1.5% | -4.9% | -6.1% |
| Class F | -7.5% | -7.2% | -9.7% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

# Conclusion

The coding results and software was confirmed. The evaluated method named LCU base syntax, enables both picture-base encoding and LCU-base encoding, which is applicable in low latency encoding.

At picture level encoding the average loss is small (up to 0.3 %) in comparison with HM50 quad-tree base anchor (SAO on). At low latency encoding, the LCU base SAO gain is 0.6 %, 0.8 %, 1.6 % in AI-LC, RA- LC, LB- LC respectively in comparison with HM50 (SAO off) anchor. In multi-slice case the LCU base SAO gain is 1.0 % and 1.5 % in LB-HE and LP-HE case in comparison with HM (SAO off multi slice anchor).

# References

[2] C.-M. Fu, C.-Y. Chen, C.-Y. Tsai, Y.-W. Huang, S. Lei, I. S. Chong, M. Karczewicz, E. Alshina, A. Alshin, “CE8.a.3: SAO with LCU-based syntax” , JCTVC-H0273, JCT-VC 8th Meeting, San Jose, February 2011