| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0105r1 |
| --- | --- |

| Title: | AHG7: Crosscheck of Sony's memory bandwidth reduction proposal (JCTVC-H0221) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba, JAPAN | Tel: Email: | +81 43 299 8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution provides cross-check results of “AHG7: Modification of merge candidate derivation to reduce MC memory bandwidth” (JCTVC-H0221). The coding results perfectly match with those provided by proponents and the source code has been checked to verify its consistency with the proposal description. JCTVC-H0221 introduced a SPS syntax named disable_bip_small_mrg_idc to add bi-pred to uni-pred conversion process at the merge candidate derivation process. The additional process is controlled by the value of disable_bip_small_mrg_idc. When disable 4x4, 4x8, 8x4 and 8x8 bi-prediction cases, it is reported that average BD-rate loss is 0.6 % to 1.4 %, while those loss of encoder only restriction is 0.8 % to 2.3 %.

# Validation

JCTVC-H0221 addressed the merge mode candidate issue that when bi-prediction is restricted, some or all merge mode candidates might be impossible to select, which results in large coding loss. The issue can be solved with bi-pred to uni-pred conversion. Considering the PU size where bi-prediction is restricted depend on use case, the PU sizes is controlled by a newly introduced syntax disable_bip_small_mrg_idc.

- introduce disable_bip_small_mrg_idc in SPS

  - 0: no restriction

  - 1: bi-prediction 4x4 is restricted (uni-prediction 4x4 is possible).

  - 2: bi-prediction 4x4, 8x4, and 4x8 are restricted.

  - 3: bi-prediction 4x4, 8x4, 4x8, and 8x8 are restricted.

- bi-pred to uni-pred conversion, at merge candidate derivation process is processed depend on the value of disable_bip_small_mrg_idc.

- The conversion is processed after merge candidate list is constructed as

  - predFlagL1 = 0 // this means uni (L0) prediction

  - mvL1 = (0, 0)

  - refIdxL1 = -1

And inter_4x4_flag is enabled.

# Experimental result

Table1: Comparison the H0221 and encoder only restriction

|  | RA-HE | RA-LC | RA-HE10 | LB-HE | LB-LC |
| --- | --- | --- | --- | --- | --- |
| H0221 (disable_bip_small_mrg_idc = 3 bi-prediction 4x4, 8x4, 4x8, and 8x8 are restricted) | 0.8 % | 1.2 % | 0.6 % | 0.8 % | 1.4 % |
| encoder only (bi-prediction 4x4, 8x4, 4x8, and 8x8 are restricted)) | 1.1 % | 1.6 % | 0.8 % | 1.4 % | 2.3 % |

Table 2: Simulation result (with bi-pred to uni-pred conversion)

of Level3: bi-prediction 4x4, 8x4, 4x8, and 8x8 are restricted.

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 1.1% | 0.8% | 0.8% | 1.7% | 1.5% | 1.5% | 0.6% | 0.4% | 0.5% |
| Class B | 1.0% | 0.5% | 0.6% | 1.4% | 1.0% | 0.9% | 1.0% | 0.6% | 0.6% |
| Class C | 1.1% | 1.2% | 1.4% | 1.7% | 1.7% | 1.9% |  |  |  |
| Class D | 1.4% | 1.1% | 1.4% | 1.7% | 1.6% | 1.5% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 1.1% | 0.9% | 1.0% | 1.6% | 1.4% | 1.4% | 0.8% | 0.5% | 0.5% |
|  | 1.1% | 0.9% | 1.1% | 1.6% | 1.4% | 1.4% | 0.8% | 0.5% | 0.6% |
| Class F | -0.1% | 0.1% | 0.1% | 0.1% | 0.2% | 0.1% |  |  |  |
| Enc Time[%] | 98% |  |  | 98% |  |  | 97% |  |  |
| Dec Time[%] | 98% |  |  | 97% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 1.2% | 0.2% | 0.2% | 2.0% | 1.0% | 1.1% |  |  |  |
| Class C | 1.6% | 1.3% | 1.6% | 2.6% | 2.4% | 2.5% |  |  |  |
| Class D | 1.8% | 0.6% | 1.7% | 2.4% | 2.0% | 2.0% |  |  |  |
| Class E | 1.0% | 0.3% | 0.6% | 2.1% | 1.3% | 2.2% |  |  |  |
| Overall | 1.4% | 0.6% | 1.0% | 2.3% | 1.6% | 1.9% |  |  |  |
|  | 1.4% | 0.6% | 1.0% | 2.3% | 1.7% | 1.9% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.1% | -0.4% | -0.2% |  |  |  |
| Enc Time[%] | 95% |  |  | 94% |  |  |  |  |  |
| Dec Time[%] | 98% |  |  | 96% |  |  |  |  |  |

Table 3: Simulation result (encoder only)

of Level3: bi-prediction 4x4, 8x4, 4x8, and 8x8 are restricted.

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 1.1% | 0.8% | 0.8% | 1.7% | 1.5% | 1.5% | 0.6% | 0.4% | 0.5% |
| Class B | 1.0% | 0.5% | 0.6% | 1.4% | 1.0% | 0.9% | 1.0% | 0.6% | 0.6% |
| Class C | 1.1% | 1.2% | 1.4% | 1.7% | 1.7% | 1.9% |  |  |  |
| Class D | 1.4% | 1.1% | 1.4% | 1.7% | 1.6% | 1.5% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 1.1% | 0.9% | 1.0% | 1.6% | 1.4% | 1.4% | 0.8% | 0.5% | 0.5% |
|  | 1.1% | 0.9% | 1.1% | 1.6% | 1.4% | 1.4% | 0.8% | 0.5% | 0.6% |
| Class F | -0.1% | 0.1% | 0.1% | 0.1% | 0.2% | 0.1% |  |  |  |
| Enc Time[%] | 98% |  |  | 98% |  |  | 97% |  |  |
| Dec Time[%] | 98% |  |  | 97% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 1.2% | 0.2% | 0.2% | 2.0% | 1.0% | 1.1% |  |  |  |
| Class C | 1.6% | 1.3% | 1.6% | 2.6% | 2.4% | 2.5% |  |  |  |
| Class D | 1.8% | 0.6% | 1.7% | 2.4% | 2.0% | 2.0% |  |  |  |
| Class E | 1.0% | 0.3% | 0.6% | 2.1% | 1.3% | 2.2% |  |  |  |
| Overall | 1.4% | 0.6% | 1.0% | 2.3% | 1.6% | 1.9% |  |  |  |
|  | 1.4% | 0.6% | 1.0% | 2.3% | 1.7% | 1.9% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.1% | -0.4% | -0.2% |  |  |  |
| Enc Time[%] | 95% |  |  | 94% |  |  |  |  |  |
| Dec Time[%] | 98% |  |  | 96% |  |  |  |  |  |

# Conclusion

The coding results and software was confirmed. It can be said that the bi-pred to uni-pred conversion at merge mode candidate derivation process solves the issue on bi-prediction restriction. Because the conversion process must be controlled whether bi-prediction is restricted or not, a kind of syntax to show the restriction size is also needed.

# References

[2] K. Kondo, T. Suzuki, “AHG7: Modification of merge candidate derivation to reduce MC memory bandwidth” , JCTVC-H0221, JCT-VC 8th Meeting, San Jose, February 2011