| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0107 |
| --- | --- |

| Title: | Cross check on Logical intra mode numbering (JCTVC-H0407) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomoyuki Yamamoto 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba JAPAN | Tel: Email: | +81 43 299 8526 yamamoto.tomoyuki@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution provides cross-check results of Logical Intra Mode Numbering proposed in JCTVC-H0407 The algorithm and software implementation are stduied. It is confirmed that the bitrate and PNSR values completely match with those provided by the proponents.

# Introduction

In the proposal, JCTCV-H0407, two main algorithms are included:

a) Logical intra mode numbering

b) Logical intra mode numbering with 3MPM extension.

Logical intra mode numbering orders intra modes starting with Planar and DC, then from HOR+8 to HOR-7, and then from VER-8 to VER+8. With the new mode numbering, a table for reordering intra prediction is removed. In addition, several tables for intra mode dependent processes, such as MDIS, MDCS, and mode dependent DCT/DST are removed. In their proposal, 19 modes for 4x4 intra parition are used. MPM derivation is basically same as what is in HM-5.0. In 4x4 intra partition, MPM candidates (neighbor modes) are mapped to the nearest direction when it is not included in 19 modes.

In the 3MPM extension, when two neigbor modes are the same, the modes having nearest directions are used as additional MPMs. Thanks to the logical mode numbering, deriving the modes with nearest directions can be done withouot referring to any table. When two neighbor modes are different, basic mode such as Planar, DC, or Vertical mode is added as third MPM.

# Experimental Result

The experiments are carried out using the software provided by the proponents. Two configurations are tested which correspond to case a) and b) described above. Table 1 and 2 show the result of each configurations. It is confirmed that, in both cases, bitrates and PSNR values match with those provided by the proponents. Regarding run time, no significant difference is observed.

Table 1. Result of a) Logical intra mode numbering

Table 2. Result of b) Logical intra mode numbering and 3MPM extention

# Additional results

Additional cross check was conducted to check combination of changes discussed during the meeting. Changes incldue 3MPM extension, logical mode numbering, MPM sort postpone, and 35 modes for all sizes. Table 3 shows the experimental results and it is confirmed that results match with those proveded by the proponent.

Table 2. Result of b) Logical intra mode numbering and 3MPM extention

# Conclusion

This contribution provides cross-check results of Logical intra mode numbering proposed in JCTVC-H0407. The algorithm and software implementation are stduied. And it is confirmed that the bitrate and PSNR values completely match with those provided by the proponents. Since the proposed numbering is friendly to mode dependent processes, it seems to be a good candidate for replacing current one. It is suggested to discuss on this proposal in comparison with other related contribution such as JCTVC-H0175 and JCTVC-H0098.