| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, CA, USA, 1-10 February, 2012 | Document: JCTVC-H0108 |
| --- | --- |

| Title: | CE10 Subtest3: Simplified BS calculation process in deblocking filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seungwook Park, Byeongmoon Jeon 221, Yangjae-Dong, Seocho-Gu, Seoul, Korea | Tel: Email: | +81-10-3922-5896 seungwook.park@ lge.com bm.jeon@lge.com |
| Source: | LG |  |  |

_____________________________

# Abstract

This contribution reports that BS calculation process can be simplified without any degradation in a coding performance and visual quality aspects. In the current HEVC, BS calculation process consists of two step; 1) BS decision on 4-pels basis and 2) BS unification on 8-pels basis. The second process is not necessary anymore, because filtering on/off decision and strong/weak filtering decision are performed on every 4-pels basis. The proposed method removes the second step and reduces the number of BS calculation in order to reduce the complexity and line buffer. This contribution claims that there is no subjective and objective performance degradation (avg. 0% loss) with a reduced complexity compared with HM5.0.

# Introduction

HEVC deblock filtering process consists of three steps; 1) BS calculation, 2) filtering on/off decision and 3) strong/weak filtering. Basically BS value was decided for each 4-pels at PU/TU boundaries and two adjacent BS values were unified to one BS value for 8-pels in order to decide filtering on/off of its 8-pel boundary. However, it was decided that 2) and 3) processes were performed on 4-pels basis instead of using 8-pels in Geneva meeting. Accordingly, BS unification process for 8-pels is not needed anymore in BS calculation process.

- 1st step: BS decision process on             (b) 2nd step: BS unification process on
4 line basis except inside of 8x8               8 line basis

Figure 1. BS calculation process in HEVC

Meanwhile, a BS decision process still consumes un-negligible memory access and operation power as depicted in Figure 2. When 4xN/Nx4 of TU and PU is used, every 4-pels edge is checked in order to determine a proper BS value in both vertically and horizontally. However it is possible to have the same BS values between adjacent 4-pels boundaries when real 4-pels boundary of TU and PU does not exist within a CU. It means that every 4-pels boundaries are not necessary to be investigated for BS decision which is one of reasons to increase the BS-related complexity.

Figure 2. BS decision tree

Therefore, in addition to the proposal about removing BS unification process from HM5.0, we are proposing the simplified BS decision process in which every other 4-pels boundary is investigated.

# Proposed method

## Removing BS unification process

As described in Introduction, BS unification process is not needed anymore in HM5.0. So we propose to remove this process.

## Simplified BS decision process

As we mentioned, there is a high probability for adjacent 4-pels edges to have the same BS values. Accordingly, we just calculate BS value for above (or left) 4-pels and then its BS values is duplicated to the next 4-pels (below) as depicted in Figure 3. As a result, the complexity of BS calculation process can be reduced to 50% in maximum.

Figure 3. Simplified BS calculation process

And besides, line buffer for storing cbf information can be reduced as a half size of HM5.0.

# Experimental results

The proposed method is implemented on HM5.0 and tested with common condition [2]. There is no performance degradation in objective quality (with some marginal gain in classF).

Table 1. Experimental results (Y BD rate only)

|  | HE |  |  |  |  | LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | RA10 | LB | LP | AI | RA | LB | LP |
| All classes | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Only classF | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | -0.3% | 0.0% |
| Enc time(%) | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
| Dec time(%) | 98% | 99% | 98% | 98% | 97% | 100% | 98% | 98% | 99% |

For an informal visual quality test, we have used following sequences and coding conditions used in the previous BoG related to an in-loop filtering.

- BQMall, low-delay low-complexity, QP 32
- BQMall, random-access high-efficiency, QP 37 
- Kimono, low-delay low-complexity, QP 32
- Kimono, random-access high-efficiency, QP 37 
- Vidyo3, low-delay low-complexity, QP 32
- Vidyo3, low-delay high-efficiency, QP 37

As a result, there is no visual quality difference between the proposed method and the anchor.

# Conclusions

This contribution reports the removal of BS unification process and the simplified BS decision process without a degradation in both coding performance and visual quality, which reduces not only the overall BS calculation complexity but also the size of line buffer for cbf.

# References

- M. Narroschke, T. Wedi and S. Esenlik “CE12 Subtest 1: Results for modified decisions for deblocking”, JCTVC-G590, JCT-VC Meeting, Geneva, November 2011.

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-G1200, JCT-VC Meeting, Geneva, November. 2011.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Draft changes

#### 8.6.1.3Derivation process of boundary filtering strength Inputs of this process are: – a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture, – a variable log2CUSize specifying the size of the current coding unit, – a two-dimensional arrays of size (nS)x(nS), horEdgeFlags and verEdgeFlags. Output of this process is an array of size (2)x(nS)x(nS), bS specifying the boundary filtering strength. Let ( xEk, yEj ) with k = 0..nE-1 and j = 0..nE-1 specify a set of edge sample locations where nE is set equal to ( ( 1 << log2CUSize ) >> 2 ), xE0 = 0, yE0 = 0, xEk+1 = xEk + 8 and yEj+1 = yEj + 8. For ( xEk, yEj ) with k = 0..nE-1 and j = 0..nE-1, the following applies. - If horEdgeFlags[ xEk ][ yEj ] is equal to 1, for ( xEk+j, yEj ) with j=0, 1, the following applies. - Set sample p0 = recPicture[ xC + xEk ][ yC + yEj – 1 ] and q0 = recPicture[ xC + xEk ][ yC + yEj ]. - The variable filterDir is set equal to 1. - Otherwise, if verEdgeFlags[ xEk ][ yEj ] is equal to 1, for ( xEk, yEj +j) with j=0, 1, the following applies. - Set sample p0 = recPicture[ xC + xEk – 1 ][ yC + yEj ] and q0 = recPicture[ xC + xEk ][ yC + yEj ]. - The variable filterDir is set equal to 0. - Depending on the value of filterDir, the variable bS[ filterDir ][ xEk ][ yEj ] is derived as follows. - If the following condition is true, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 2. - The sample p0 or q0 is in a coding unit coded with intra prediction mode - Otherwise, if the block edge is also a transform unit edge and the following condition is true, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 1. - The sample p0 or q0 is in a transform unit which contains non-zero transform coefficient level. - Otherwise, if any of the following conditions are true, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 1. - The prediction unit containing sample p0 has different reference pictures or a different number of motion vectors with the prediction unit containing the sample q0. NOTE – The determination of whether the reference pictures used for the two prediction are the same or different is based on which pictures are referenced, without regard to whether a prediction is formed using an index into list 0 or an index into list 1, and also without regard to whether or not the index position within a reference picture list is different or not. - If horEdgeFlags[ xEk ][ yEj ] is equal to 1 and yC + yEj – 1 is smaller than YLCU, the following applies. - One motion vector is used to predict the prediction unit containing sample recPicture[ xL ][ yC + yEj – 1 ], one motion vector is used to predict the prediction unit containing sample q0, and the absolute difference between the horizontal or vertical component of the motion vector used is greater than or equal to 4 in units of quarter luma samples, where xL is set equal to ( ( ( xC + xEk ) >> Log2MinCUSize ) << Log2MinCUSize ) + ( ( ( xC + xEk ) >> Log2MinCUSize ) & 1 ) * ( MinPuSize * 2 – 1) and MinPuSize is set equal to 1<<(Log2MinCUSize – 1). - Otherwise, the following applies. - One motion vector is used to predict the prediction unit containing sample p0, one motion vector is used to predict the prediction unit containing sample q0, and the absolute difference between the horizontal or vertical component of the motion vector used is greater than or equal to 4 in units of quarter luma samples. - [Ed.: (WJ) needs to be checked again whether this condition covers all 2-motion cases] Two motion vectors are used to predict the prediction unit containing sample p0, two motion vectors are used to predict the prediction unit containing sample q0, and at least one of the motion vector pairs corresponding the same reference pictures and the different boundary samples p0 and q0 satisfies the following condition: - The absolute difference between the horizontal or vertical component of a motion vector used in the prediction of the two prediction units is greater than or equal to 4 in units of quarter luma samples. - Otherwise, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 0. - If filterDir is equal to 0, bS[ filterDir][ xEk ][ yEj +4 ], is set to equal to bS[filterDir ][ xEk ][ yEj ] - Otherwise, bS[ filterDir][ xEk + 4][ yEj ], is set to equal to bS[filterDir ][ xEk ][ yEj ] #### 8.6.1.4Filtering process for coding unit Inputs of this process are: – a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture, – a variable log2CUSize specifying the coding unit size, – an array bS specifying the boundary filtering strength. Output of this process is: – modified reconstruction of the picture. The filtering process for luma edges in the current coding unit consists of the following ordered steps: - The variable nD is set equal to 1 << ( log2CUSize – 3 ). - All elements of the three-dimensional array of size (2)x(nD)x(nD), dEdge are initialized to zero. - All elements of the three-dimensional array of size (2)x(nD)x(1<<log2CUSize), dSample are initialized to zero. - All elements of the three-dimensional array of size (2)x(nD)x(nD), bStrength ae initialized to zero. - For xDk set equal to xC+( k << 3 ), k=0..nD – 1, the following applies: - For yDm set equal to yC+( m << 3 ), m=0..nD – 1, the following ordered steps apply: - Boundary filtering strength bSVer is derived as follows: bSVer = Max( bS[ 0 ][ xDk ][ yDm + i ] ) for i = 0..7 (8-1) - bStrength[1][k][m] is set equal to bS[ 0 ][ xDk ][ yDm ]. - Decision process for luma block edge in subclause Error: Reference source not found is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 1, and the boundary filtering strength bStrength[1][k][m] as inputs and the decision dEdge[1][k][m] and an array dS of size (8) as outputs. - dSample[1][k][(m<<3)+i] is set equal to dS[i] for i=0..7. - For xDk set equal to xC+( k << 3 ), k=0..nD - 1, the following applies: - For yDm set equal to yB+( m << 3 ), m=0..nD – 1, the following ordered steps apply: - dS[i] is set equal to dSample[1][k][(m<<3)+i] for i=0..7. - Filtering process for luma block edge in subclause Error: Reference source not found is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 1, the boundary filtering strength bStrength[1][k][m], the decision dEdge[1][k][m], and the array of size (8), dS as inputs and the modified luma picture buffer as outputs. - For xDk set equal to xC + ( k << 3 ), k = 0..nD - 1, the following applies: - Boundary filtering strength bSHor is derived as follows: bSHor = Max( bS[ 1 ][ xDk + i ][ yDm ] ) for i = 0..7 (8-2) - bStrength[0][k][m] is set equal to bS[ 1 ][ xDk ][ yDm ]. - Decision process for luma block edge in subclause Error: Reference source not found is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 0, and the boundary filtering strength bStrength[0][k][m] as inputs and the decision dEdge[0][k][m] and an array dS of size (8) as outputs. - dSample[0][k][(m<<3)+i] is set equal to dS[i] for i=0..7. - For yDm set equal to yC+( m << 3 ), m=0..nD - 1, the following applies: - ...