| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0109 |
| --- | --- |

| Title: | Modification/simplification of intra angular prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shohei Matsuo Motohiro Takagi Seishi Takamura Atsushi Shimizu Hirohisa Jozawa | Tel: Email: | +81-46-859-2070 matsuo.shohei@lab.ntt.co.jp +81-46-859-3173 takagi.motohiro@lab.ntt.co.jp +81-46-859-2371 takamura.seishi@lab.ntt.co.jp +81-46-859-2448 shimizu.atsushi@lab.ntt.co.jp +81-46-859-3382 jozawa.hirohisa@lab.ntt.co.jp |
| Source: | NTT (Nippon Telegraph and Telephone) Corporation |  |  |

_____________________________

# Abstract

This document introduces an intra angular prediction method exploiting interpolation technique whose tap length is other than two. In the proposed method, reference samples for intra angular prediction are generated by the conventional 2-tap filter or 4-tap DCT-IF. The overall average coding gains against the HM5.0 anchor were about 0.3%, 0.8%, 0.8% (Y, Cb, Cr in HE), 0.4%, 0.2% and 0.3% (Y, Cb, Cr in LC). The maximum coding gains were about 1.8%, 2.5% and 3.4% (Y, Cb and Cr) for the sequence “BasketballDrill” in LC case. The average run-times of encoding and decoding were 99.8% and 99.5%, respectively.

# Introduction

In HM5.0, intra prediction modes are defined as follows: planar (mode 0), vertical (mode 1), horizontal (mode 2), DC (mode 3), angular (mode 4 to 34) and fromLuma (mode 35: this mode is used only for chroma). In the angular mode, 31 prediction directions are prepared as shown in Section 8.3.1 of Working Draft 5 [1]. The number of the prediction directions of HM is much larger than that of H.264/AVC standard. The angular prediction of HM can generate more flexible prediction signals and effectively reduce the prediction error energy. When the angular prediction mode is selected, in some prediction modes, the reference samples are generated by linear interpolation. Specifically, the following equation defined in the equation (8-44) of Working Draft 5 is used for generating the prediction samples.

predSamples[ x, y ] = ( ( 32 – iFact )*refMain[ x+iIdx+1 ] + iFact*refMain[ x+iIdx+2] + 16 ) >> 5

The array predSamples[ x, y ] and refMain[ x, y ] denote the predicted samples and the reference pixels, respectively. The parameter iIdx and iFact denote the index and the multiplication factor determined by the intra prediction mode. From the equation above, the predicted samples are generated by adding the weighted values of neighboring reference pixels. In other words, the reference pixels are interpolated by 2-tap simple interpolation filter. In this contribution, an interpolation filter whose tap length is other than 2 is applied to the generation of reference pixels for intra angular prediction.

# Modification and simplification of intra angular prediction

## Basic idea

This section explains the motivation of this proposal. Assume that Ai,j and Pi,j are the locally-decoded reference pixels and the pixels to be predicted, respectively, as shown in Figure 1. The index i and j denote spatial coordinates for horizontal and vertical position, respectively. When the vertical mode is selected, for example, the value of A0,0 is copied to P0,0, P0,1, …, P0,b-1. The parameter b denotes the prediction block size. When the angular mode is selected, the reference pixels such as A,0 (0<<1) are generated to make the prediction signals. In the current implementation of HM5.0, the reference pixels are generated by 2-tap linear interpolation filter. The filter tap length is fixed regardless of the characteristics of the input image and coding information.

When the prediction block size is larger, the texture tends to be flat and simple. Each reference pixel may have the similar characteristics. Therefore, the 2-tap or shorter interpolation would be sufficient. In addition, when the prediction block size is larger, the pixels at the bottom-right corner in the prediction block are far from the reference pixels. There is a possibility that the effect of modification of interpolation filter is small. However, when the prediction block size is smaller, the texture would be complicated. In addition, the pixels at the bottom-right corner in the prediction block are close to the reference pixels. Therefore, the longer-tap interpolation may be able to generate the prediction signals that reduce the prediction error energy.

DCT-based Interpolation Filter (DCT-IF) was proposed in the contribution [2, 3] and it is improved and currently employed in HM5.0 because of its effectiveness and unified design. In this contribution, when the intra angular prediction is selected, n(>2)-tap DCT-IF is applied to smaller block sizes. DCT-IF approach is currently applied to the interpolation of the proposal. However, of course, other interpolation approaches can be also easily applied to the proposal.

Figure 1: Intra angular prediction

## Encoding algorithm

The detailed encoding algorithm is summarized as follows. In the following explanation, k-tap and l-tap (k < l) filters are used.

S1: Set m that indicates the threshold value of the prediction block size.

S2: Input the block size of intra prediction. If the block size is larger than mxm, the tap length of the interpolation filter for angular prediction is set to k. Otherwise, the tap length of interpolation filter for angular prediction is set to l(>k).

S3: Input the value of the intra prediction mode. If the input value is equal to the angular prediction, go to S4. Otherwise, the corresponding conventional intra prediction process is performed as defined in Working Draft 5 [1]. After the process is done, go to S5.

S4: Angular prediction is done by using the interpolation filter defined in S2. Specifically, if the tap length is equal to 2, the conventional angular prediction is performed. After the angular prediction is done, go to S5.

S5: Move on to the next prediction block. Repeat the loop (S1 to S5) until all the intra prediction process is completed.

## Intra prediction parameter syntax

In the current implementation, the proposed encoder and decoder share the tap length (k and l) and the threshold value of the block size (m). Therefore, there is no additional syntax related to the proposed intra prediction.

## Decoder description

In Sec. 8.3.3.1.6 of Working Draft 5 [1], the specification of intra angular prediction is described. In the section, the equation (8-44) is defined as follows.

predSamples[ x, y ] = ( ( 32 – iFact )*refMain[ x+iIdx+1 ] + iFact*refMain[ x+iIdx+2] + 16 ) >> 5

Assume that the 4-tap interpolation filter is employed. The corresponding equation is replaced as follows.

predSamples[ x, y ] = ( a0*refMain[ x+iIdx] + a1*refMain[x+iIdx+1]

+ a2*refMain[x+iIdx+2] + a3*refMain[x+iIdx+3] + r ) >> s

In the equation above, ai denotes the interpolation filter coefficients. The coefficients ai, the offset parameter r and the shift parameter s are determined by the employed interpolation filter.

# Experimental results

The proposal was implemented in HM 5.0. It is applied to chrominance components as well as luminance component. The configurations of the proposal are listed in Table 1. In this experiment, k and l are set to two and four, respectively. Therefore, 2-tap and 4-tap combination was tested. The threshold value of the block size m is set to eight. To reduce the encoding and decoding complexity, when the tap length is equal to four, the intra smoothing is turned off. The other configurations are subject to the common test condition [4].

Table 1: Configurations of the proposed angular prediction.

The simulation results are shown in Table 2. The gains (BD-rate [5]) against the anchor of HM 5.0 were calculated. The overall average gains were about 0.3%, 0.8%, 0.8% (Y, Cb, Cr in HE), 0.4%, 0.2% and 0.3% (Y, Cb, Cr in LC). The maximum coding gains were about 1.8%, 2.5% and 3.4% (Y, Cb and Cr) for the sequence “BasketballDrill” in LC case. The average run-times of encoding and decoding were 99.8% and 99.5%, respectively.

Table 2: Simulation results (anchor: HM5.0).

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -0.5% | -0.3% | 0.0% | 0.2% | 0.2% |
| Class B | -0.2% | -0.8% | -0.9% | -0.2% | -0.2% | -0.3% |
| Class C | -0.5% | -1.1% | -1.1% | -0.6% | -0.7% | -0.9% |
| Class D | -0.3% | -0.8% | -0.8% | -0.4% | -0.4% | -0.4% |
| Class E | -0.4% | -0.7% | -0.7% | -0.5% | 0.7% | 0.6% |
| Overall | -0.3% | -0.8% | -0.8% | -0.4% | -0.2% | -0.3% |
|  | -0.3% | -0.8% | -0.8% | -0.4% | -0.2% | -0.3% |
| Class F | -0.4% | -0.6% | -0.6% | -0.4% | -0.4% | -0.7% |
| Enc Time[%] | 99.96% |  |  | 99.6% |  |  |
| Dec Time[%] | 99.4% |  |  | 99.7% |  |  |

# Conclusion

In this contribution, modification and simplification of intra angular prediction was proposed. As for the modification, by using 2-tap and 4-tap interpolation, average coding gains against the anchor HM5.0 were 0.3%, 0.8%, 0.8% (Y, Cb, Cr in HE), 0.4%, 0.2% and 0.3% (Y, Cb, Cr in LC). The maximum coding gains were about 1.8%, 2.5% and 3.4% (Y, Cb and Cr) for the sequence “BasketballDrill” in LC case. The average run-times of encoding and decoding were 99.8% and 99.5%, respectively. To turn off the intra smoothing process results in the complexity reduction. It is recommended to incorporate the proposal into intra prediction method of HEVC.

Thanks to KDDI for the cross-check of the coding results and the run-times [6].

# References

[1] B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan and T. Wiegand, “WD5: Working Draft 5 of High-Efficiency Video Coding,” JCTVC-G1103_d6, 7th. JCT-VC Meeting, Geneva, CH, 21-30 November, 2011.

[2] K. McCann, W.-J. Han, I.-K. Kim, J.-H Min, E. Alshina, A. Alshin, T. Lee, J. Chen, V. Seregin, S. Lee, Y.-M. Hong, M.-S. Cheon and N. Shlyakhov, “Samsung’s Response to the Call for Proposals on Video Compression Technology,” JCTVC-A124_r2, 1st. JCT-VC Meeting, Dresden, DE, 15-23 April, 2010.

[3] E. Alshina, J. Chen, A. Alshin, N. Shlyakhov, and W.-J. Han, “CE3: Experimental results of DCTIF by Samsung,” JCTVC-D344_r1, 4th. JCT-VC Meeting, Daegu, KR, 20-28 January, 2011.

[4] F. Bossen, “Common test conditions and software reference configurations,” JCTVC-F900, 6th. JCT-VC Meeting, Torino, IT, 14-22 July, 2011.

[5] G. Bjontegaard, “Calculation of average PSNR differences between RD-curves,” ITU-T SG16 Q.6 Document, VCEG-M33, 13th VCEG Meeting, Austin, Texas, USA, April 2001.

[6] T. Yoshino, K. Kawamura and S. Naito, “Non-CE6: Cross-check of modified/simplified intra angular prediction (JCTVC-H0109),” JCTVC-H0109, 8th. JCT-VC Meeting, San José, CA, USA, 1–10 February, 2012.

[7] S. Matsuo, S. Takamura and H. Jozawa, “Modification of angular intra prediction,” JCTVC-G350, 7th JCT-VC Meeting, Geneva, CH, 21-30 November, 2011.

# Patent rights declaration(s)

NTT Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

## Performance of inter-frame coding

To confirm the effect of the proposed intra angular prediction for the inter-frame coding, the additional results were obtained, as shown in Table 3. As for the overall average coding gains, 0.01 to 1.0% improvement was confirmed.

Table 3: Simulation results with inter-frame coding (anchor: HM5.0).

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -0.5% | -0.3% | 0.0% | 0.2% | 0.2% |  |  |  |
| Class B | -0.2% | -0.8% | -0.9% | -0.2% | -0.2% | -0.3% |  |  |  |
| Class C | -0.5% | -1.1% | -1.1% | -0.6% | -0.7% | -0.9% |  |  |  |
| Class D | -0.3% | -0.8% | -0.8% | -0.4% | -0.4% | -0.4% |  |  |  |
| Class E | -0.4% | -0.7% | -0.7% | -0.5% | 0.7% | 0.6% |  |  |  |
| Overall | -0.3% | -0.8% | -0.8% | -0.4% | -0.2% | -0.3% |  |  |  |
|  | -0.3% | -0.8% | -0.8% | -0.4% | -0.2% | -0.3% |  |  |  |
| Class F | -0.4% | -0.6% | -0.6% | -0.4% | -0.4% | -0.7% |  |  |  |
| Enc Time[%] | 99.96% |  |  | 99.6% |  |  |  |  |  |
| Dec Time[%] | 99.4% |  |  | 99.7% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -0.7% | -0.7% | 0.0% | -0.2% | -0.3% | 0.1% | -0.7% | 0.2% |
| Class B | -0.1% | -0.9% | -0.9% | -0.1% | -0.9% | -1.1% | -0.1% | -1.0% | -1.0% |
| Class C | -0.3% | -0.8% | -0.7% | -0.3% | -1.0% | -1.3% |  |  |  |
| Class D | -0.2% | -0.7% | -0.5% | -0.2% | -0.3% | -0.8% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.2% | -0.8% | -0.7% | -0.2% | -0.7% | -1.0% | 0.0% | -0.9% | -0.5% |
|  | -0.2% | -0.8% | -0.7% | -0.2% | -0.7% | -1.0% | 0.0% | -0.9% | -0.5% |
| Class F | -0.2% | -0.6% | -0.5% | -0.2% | -0.9% | -0.9% |  |  |  |
| Enc Time[%] | 100.0% |  |  | 99.99% |  |  | 100.0% |  |  |
| Dec Time[%] | 99.40% |  |  | 99.66% |  |  | 100.1% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | -0.4% | -0.1% | -0.1% | -0.9% | -0.6% |  |  |  |
| Class C | -0.1% | -0.5% | -0.2% | -0.1% | -0.3% | -0.9% |  |  |  |
| Class D | 0.0% | -0.8% | -0.5% | -0.2% | 0.3% | -0.7% |  |  |  |
| Class E | 0.0% | 0.2% | -1.2% | -0.3% | -0.5% | -1.4% |  |  |  |
| Overall | 0.0% | -0.4% | -0.4% | -0.1% | -0.4% | -0.9% |  |  |  |
|  | 0.0% | -0.4% | -0.4% | -0.1% | -0.5% | -0.8% |  |  |  |
| Class F | -0.3% | -0.2% | -0.7% | 0.1% | -1.2% | -0.6% |  |  |  |
| Enc Time[%] | 100.0% |  |  | 100.0% |  |  |  |  |  |
| Dec Time[%] | 100.1% |  |  | 99.9% |  |  |  |  |  |

## Performance of different tap-length

In the previous work [7], only the combination of 2-tap and 4-tap was investigated. To reduce the encoding and decoding complexity, this contribution newly introduces a direct copy (1-tap) method. When the 1-tap filter is used, the value of the nearest neighbor decoded pixel is copied to the reference pixel. As shown in Figure 2, for example, assume that the values of the decoded pixels are A0,0 and A1,0. When the reference pixel is in the position 0 to 15, A0,0 is copied to the value of the reference pixel. Otherwise, A1,0 is copied to the value of the reference pixel. The selected value is copied to the predicted pixels along the direction determined by the angular mode. Note that, in this section 7.2, the vertical horizontal filter defined in F172 is applied to chrominance signals as well as luminance signal.

Figure 2: Definition of 1-tap filter (nearest neighbor filter)

Assume that the 1-tap filter is employed. The corresponding equation (8-44) in Working Draft 5 [1] is replaced as follows.

predSamples[x,y]=refMain[x+iIdx+1](0 ≤iFact≤15)

In this appendix, three different filters are defined as follows.

- 1-tap filter: the nearest neighbor filter described in Figure 2.

- 2-tap filter: the conventional 2-tap linear filter employed in HM5.0.

- 4-tap filter: the 4-tap DCT-IF.

The configurations of the proposal are listed in Table 4. The other configurations are subject to the common test condition [4]. The simulation results are shown in Table 5 and 6. The gains (BD-rate [5]) against the anchor of HM 5.0 were calculated.

Table 4: Configurations of the proposed angular prediction with different combinations.

Table 5: Simulation results of Case 1 (anchor: HM5.0).

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -0.5% | -0.2% | 0.0% | 0.1% | 0.1% |
| Class B | -0.2% | -0.8% | -0.8% | -0.3% | -0.3% | -0.3% |
| Class C | -0.6% | -1.0% | -1.0% | -0.7% | -1.0% | -1.3% |
| Class D | -0.4% | -0.7% | -0.7% | -0.5% | -0.8% | -0.9% |
| Class E | -0.4% | -0.6% | -0.6% | -0.6% | 0.9% | 0.7% |
| Overall | -0.4% | -0.7% | -0.7% | -0.4% | -0.3% | -0.4% |
|  | -0.4% | -0.7% | -0.7% | -0.4% | -0.3% | -0.4% |
| Class F | -0.5% | -0.5% | -0.6% | -0.5% | -0.8% | -1.1% |
| Enc Time[%] | 97.2% |  |  | 99.6% |  |  |
| Dec Time[%] | 99.0% |  |  | 99.7% |  |  |

Table 6: Simulation results of Case 2 (anchor: HM5.0).

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.1% | 0.1% | 0.0% | -0.1% | -0.1% |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class C | -0.1% | 0.0% | 0.1% | -0.1% | -0.3% | -0.3% |
| Class D | -0.1% | 0.1% | 0.0% | -0.1% | -0.4% | -0.5% |
| Class E | 0.0% | 0.1% | 0.2% | 0.0% | 0.3% | 0.0% |
| Overall | 0.0% | 0.1% | 0.1% | -0.1% | -0.1% | -0.2% |
|  | 0.0% | 0.1% | 0.1% | -0.1% | -0.1% | -0.2% |
| Class F | 0.0% | 0.0% | 0.1% | -0.1% | -0.1% | -0.3% |
| Enc Time[%] | 99.9% |  |  | 99.6% |  |  |
| Dec Time[%] | 99.5% |  |  | 99.99% |  |  |

In Case 1 (2-tap and 4-tap combination), the overall average gains were about 0.4 %, 0.7%, 0.7% (Y, Cb, Cr in HE), 0.4%, 0.3% and 0.4% (Y, Cb, Cr in LC). The maximum coding gains were about 1.9%, 2.9% and 3.7% (Y, Cb and Cr) for the sequence “BasketballDrill” in LC case. It is confirmed that the proposal is effective for Class C. The average run-times of encoding and decoding were 96.4% and 99.3%, respectively. This reduction is achieved by turning off the intra smoothing and the angular mode for 64x64. In Case 1, the required buffer as well as the calculation number is reduced because the intra smoothing is turned off for all block sizes.

In Case 2 (1-tap and 2-tap combination), the overall average gains were about 0.04%, −0.08%, −0.11% (Y, Cb, Cr in HE), 0.07%, 0.13% and 0.20% (Y, Cb, Cr in LC). The maximum coding gains were about 0.2%, 0.5% and 0.5% (Y, Cb and Cr) for the sequence “RaceHorses” of Class D in LC case. The average run-times of encoding and decoding were 99.8% and 99.7%, respectively. By using 1-tap filtering, calculation of reference pixels is reduced. In addition, the required memory bandwidth is also reduced. As shown in Table 3, this simplification provided about 0.1% coding gains for LC. This is because the rounding error does not occur when the 1-tap filter is used.