| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0111r2 |
| --- | --- |

| Title: | CE9: Modification of bi-prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshinori Suzuki 3-6 Hikarinooka, Yokosuka-shi Kanagawa, 239-8536 Japan | Tel: Email: | +81 46 840 3517 suzukiyos@rd.nttdocomo.co.jp |
| Source: | NTT DOCOMO, Inc. |  |  |

_____________________________

# Abstract

This contribution reports the simulation results of section 2.1 in Core Experiment 9 (CE9). 8 experiments, BP01 to BP08, were established to evaluate the proposed method described in JCTVC-G305, in which the one of two motion vector differences (MVDs) for bi-prediction is not signaled and it is set to (0, 0) when a flag to switch this syntax modification on/off in slice header equals to 1.

In the proposed methods (BP01 and BP02), the on/off flag is set to 1 when reference picture list 1 is identical to reference picture list 0 (GBP picture). MVD for List 1 is not signaled in BP01 or for List 0 in BP02. They achieved Y BD-rates of -1.0% in LDB (average of HE and LC) and -0.2% in RA (average of HE, LC and HE10) with no encoding time increase. When the on/off flag is set to 1 for all pictures, 0.9% loss in BD-rate was caused in RA (BP03).

After the 7th JCT-VC meeting, the experiments on the HM ticket #175 which is related to bi-predictive motion estimation, was requested. The BP07 (HM5.0 with a bug fix of ticket #175) achieved Y BD-rates of -0.9% in LDB and -0.6% in RA for class F, though it did not achieve any gain for classes A to E. In the BP08 (BP01 with a bug fix of ticket #175), Y BD-rates of -1.1% in LDB and -0.3% in RA were achieved for classes A to E and, Y BD rates of -1.9% in LDB and -0.7% in RA were achieved for class F with no encoding time increase.

To clarify the effect of syntax changes in proposed method, the method with encoder only change, in which MVD for L1 is set to (0, 0) at encoder for GPB pictures, was tested as BP04. B04 achieved Y BD-rates of -0.2% in LDB and 0% in RA. And then, the additional motion estimation (ME) for bi-prediction was tested on the top of HM5.0 and BP01. The BP05 (HM5.0 with additional ME) achieved Y BD-rate of -0.7% in LDB with 31% encoding time increase. BP05 and BP06 (BP01 with additional ME in RA) achieved Y BD rates of -0.6% and -0.7% with 21% and 19% encoding time increase in RA, respectively.

For all the experiments, the decoding time was similar to that of HM5.0.

# Introduction

At the 7th JCT-VC meeting in Geneva, CH, the modification of syntax on bi-prediction was proposed by JCTVC-G305[1], in which the motion vector difference (MVD) for List 1 is not signaled and it is set to (0, 0) for GPB pictures (reference picture list 1 is identical to reference picture list 0) and section 2.1 of CE9 [2] was established to evaluate this proposal. Since the proposal does not change any decoding process, the comparison of the proposed method with syntax change and a method with encoder only change was requested. Additionally, after 7th JCTVC meeting, NTT DOCOMO suggested to test the HM ticket #175 reported at the HM SVN. It is related to the bi-predictive ME and it fixes the problem that a clipping range is smaller than a dynamic range of source signal.

In section 2.1 of CE9, five items were evaluated,

1) Proposal with syntax change (BP01) versus Encoder only change in JCTVC-G305 (BP04),

2) HM5.0 versus HM5.0 or BP01 with a bug fix of ticket #175,

3) HM5.0 with 2nd ME for bi-prediction (BP05) versus BP01 with 2nd ME for bi-prediction (BP06),

4) MVD for L1 sets to (0, 0) for all B-pictures (BP03) versus for GPB pictures (BP01) and

5) MVD for L0 sets to (0, 0) (BP02) versus MVD for L1 sets to (0, 0) (BP01).

Table 1 shows the brief description of experiments in section 2.1 of CE9.

Table 1 Description of experiments – BP series

| Test | Description |
| --- | --- |
| BP01 | L1 MVD is set to (0,0) and not signaled for GPB pictures |
| BP02 | L0 MVD is set to (0,0) and not signaled for GPB pictures |
| BP03 | L1 MVD is set to (0, 0) and not signaled for all inter pictures |
| BP04 | L1 MVD is set to (0, 0) at encoder for GPB pictures |
| BP05 | HM5.0 with the 2nd motion estimation for bi-prediction |
| BP06 | BP01 with the 2nd motion estimation for bi-prediction in non-GPB pictures |
| BP07 | HM5.0 with a bug fix of HM ticket #175 |
| BP08 | BP01 with a bug fix of HM ticket #175 |

- Changes to the WD

The modifications to WD text [3] on BP01 (BP03, BP06 and BP08) and BP02 are shown in this section.

1)	One flag to identify if the list 1/0 motion vector difference is parsed is added to the slice header (see 2.1.1 or 2.2.1),

2) Prediction unit is modified so that the vector components of list 1/0 motion vector difference are not parsed and they are set to (0, 0) (see 2.1.2 or 2.2.2)

## Changes for BP01/BP03/BP06/BP08

### Signaling of mvd_l1_zero_flag

The syntax element for mvd_l1_zero_flag is defined below. It is signaled in slice_header() when slice_type is B-slice.

### Slice header semantics

mvd_l1_zero_flag equals to 1 indicates that difference between a list 1 vector component and its prediction, mvd_l1[][][], is not parsed and it is set equal to 0. This flag equals to 1 indicates that mvd_l1[][][] is parsed.

### Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
|  |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| } |  |
| if( slice_type != I ) |  |
| cabac_init_idc | ue(v) |
|  |  |
| } |  |

### Signaling of mvd_l1

The syntax element for mvd_l1[][][] is not signaled and it is set to 0 when mvd_l1_zero_flag equals to 1. Prediction unit syntax is modified to support this option.

### Prediction unit syntax

| prediction_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
|  |  |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_BI ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| if( mvd_l1_zero_flag ) { |  |
| mvd_l1[ x0 ][ y0 ][ 0 ] = 0 |  |
| mvd_l1[ x0 ][ y0 ][ 1 ] = 0 |  |
| } else { |  |
| mvd_coding(mvd_l1[ x0 ][ y0 ][ 0 ], mvd_l1[ x0 ][ y0 ][ 1 ]) |  |
| } |  |
| mvp_idx_l1[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

## Changes for BP02

### Signaling of mvd_l0_zero_flag

The syntax element for mvd_l0_zero_flag is defined below. It is signaled in slice_header() when slice_type is B-slice.

### Slice header semantics

mvd_l0_zero_flag equals to 1 indicates that difference between a list 0 vector component and its prediction, mvd_l0[][][], is not parsed and it is set equal to 0. This flag equals to 1 indicates that mvd_l0[][][] is parsed.

### Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
|  |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| if( slice_type = = B ) |  |
| mvd_l0_zero_flag | u(1) |
| } |  |
| if( slice_type != I ) |  |
| cabac_init_idc | ue(v) |
|  |  |
| } |  |

### Signaling of mvd_l0

The syntax element for mvd_l0[][][] is not signaled and it is set to 0 when mvd_l0_zero_flag equals to 1. Prediction unit syntax is modified to support this option.

### Prediction unit syntax

| prediction_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| … |  |
| } |  |
| else { /* Pred_L0 or Pred_BI */ |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| if( slice_type = = B && mvd_l0_zero_flag ) { |  |
| mvd_l0[ x0 ][ y0 ][ 0 ] = 0 |  |
| mvd_l0[ x0 ][ y0 ][ 1 ] = 0 |  |
| } else { |  |
| mvd_coding(mvd_l0[ x0 ][ y0 ][ 0 ], mvd_l0[ x0 ][ y0 ][ 1 ]) |  |
| } |  |
| mvp_idx_l0[ x0 ][ y0 ] | ae(v) |
| } |  |
| … |  |
| } |  |

# Results

The experiments were conducted in accordance with the 5 common test conditions for inter coding described in [4]. The test methods described in 2.1 of [2] was implemented in HM5.0-tag (revision 1682). The excel sheet corresponding to the results of BP01 to BP08 are attached as part of the contribution. BP01 and BP04 were cross verified by HHI. BP02 and BP06 were cross verified by Orange. BP03 was cross verified by JVC Kenwood. BP05 was cross verified by Panasonic. And BP07 and BP08 were cross verified by Samsung.

Bitstreams of HM5.0, BP01, BP02, BP07 and BP08 were uploaded to the NTT DOCOMO’s ftp site to verify that proposed method does not show any unpleasant effect on reconstructed images.

The overall average of Y BD-rate and encoding time for classes A-E and classes A-F are shown in Table 2 and Table 3, respectively. The decoding time of BP01 to BP08 was similar to that of HM5.0. These results show that BP08 obtains the best tradeoff between coding performance and encoder complexity. The Y BD-rates of -1.1% in LDB (average of HE and LC) and -0.3% in RA (average of HE, LC and HE10) were achieved with no encoding time increase.

NTT DOCOMO checked that BP08 did not show any unpleasant side effect on decoded sequences in following conditions:

Sequences; class B and class C,

Configuration; LDB-HE and RA-HE,

QP; 37 and 32.

When an artifact found on decoded sequences of BP08, the similar artifact also found on that of HM5.0. The subjective quality of BP08 is very similar to that of HM5.0.

Table 2 Summary of overall results without class F

|  | Overall average Y BD rate [%] |  |  |  |  | Encoding time [%] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | LDBHE | LDBLC | RAHE | RALC | RAHE10 | LDBHE | LDBLC | RAHE | RALC | RAHE10 |
| BP01 | -0.8 | -1.1 | -0.2 | -0.2 | -0.3 | 100 | 99 | 99 | 99 | 99 |
| BP02 | -0.8 | -1.1 | -0.2 | -0.2 | -0.2 | 99 | 99 | 99 | 99 | 99 |
| BP03 | -0.8 | -1.1 | 1.0 | 0.9 | 0.8 | 99 | 99 | 99 | 99 | 99 |
| BP04 | -0.1 | -0.4 | 0.0 | 0.0 | -0.1 | 99 | 98 | 99 | 99 | 99 |
| BP05 | -0.5 | -0.8 | -0.5 | -0.7 | -0.7 | 129 | 132 | 120 | 124 | 120 |
| BP06 | -0.8 | -1.1 | -0.6 | -0.7 | -0.7 | 100 | 99 | 117 | 120 | 117 |
| BP07 | 0.0 | -0.1 | -0.1 | 0.0 | 0.0 | 100 | 99 | 100 | 100 | 99 |
| BP08 | -0.9 | -1.2 | -0.3 | -0.3 | -0.3 | 99 | 99 | 99 | 99 | 99 |

Table 3 Summary of overall results with class F

|  | Overall average Y BD rate [%] |  |  |  | Encoding time [%] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | LDBHE | LDBLC | RAHE | RALC | LDBHE | LDBLC | RAHE | RALC |
| BP01 | -0.7 | -0.9 | -0.2 | -0.2 | 100 | 99 | 99 | 99 |
| BP02 | -0.6 | -0.7 | -0.2 | -0.2 | 100 | 99 | 99 | 99 |
| BP03 | -0.7 | -0.9 | 1.0 | 0.9 | 100 | 99 | 100 | 99 |
| BP04 | -0.1 | -0.1 | 0.0 | 0.0 | 99 | 99 | 99 | 99 |
| BP05 | -0.5 | -0.8 | -0.5 | -0.7 | 129 | 133 | 121 | 124 |
| BP06 | -0.7 | -0.9 | -0.6 | -0.7 | 100 | 99 | 118 | 121 |
| BP07 | -0.2 | -0.2 | -0.1 | 0.0 | 100 | 100 | 100 | 100 |
| BP08 | -1.1 | -1.3 | -0.3 | -0.3 | 99 | 99 | 99 | 99 |

## Effect of syntax changes in JCTVC-G305

The proposal with syntax change (BP01) was compared the method with the encoder only change in JCTVC-G305 (BP04). The results of BP01 and BP04 are shown in Table 4 and Table 5, respectively. BP01 achieved the Y BD-rates of -1.0% and -0.2% in LDB and RA with no enc/dec time increase, respectively, whereas BP04 achieved the Y BD-rates of -0.2% and 0% in LDB and RA, respectively. These results show that the syntax change in proposal is effective and it achieves 0.8% gain in LDB and 0.2% gain in RA without any enc/dec time increase.

Table 4 Results of BP01

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.8% | -1.0% | -0.9% | -1.3% | -1.7% | -1.4% |
| Class C | -0.9% | -0.9% | -0.8% | -1.1% | -0.7% | -1.0% |
| Class D | -0.9% | -0.7% | -0.6% | -1.0% | -0.3% | -0.6% |
| Class E | -0.8% | -0.7% | -0.5% | -1.0% | -2.1% | -0.7% |
| Overall | -0.8% | -0.8% | -0.7% | -1.1% | -1.2% | -1.0% |
|  | -0.8% | -0.8% | -0.8% | -1.1% | -1.1% | -1.0% |
| Class F | -0.2% | -0.4% | -0.6% | 0.0% | -1.1% | -0.5% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.1% | -0.2% | -0.2% | -0.2% | -0.2% | -0.3% | -0.3% | -0.3% | -0.4% |
| Class B | -0.3% | -0.3% | -0.3% | -0.4% | -0.4% | -0.5% | -0.2% | -0.4% | -0.4% |
| Class C | -0.2% | -0.1% | 0.0% | -0.2% | -0.1% | 0.0% |  |  |  |
| Class D | -0.2% | -0.4% | -0.4% | -0.1% | -0.2% | -0.2% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.2% | -0.3% | -0.2% | -0.2% | -0.2% | -0.3% | -0.3% | -0.3% | -0.4% |
|  | -0.2% | -0.3% | -0.2% | -0.2% | -0.2% | -0.3% | -0.3% | -0.3% | -0.4% |
| Class F | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |

Table 5 Results of BP04

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | -0.2% | 0.1% | -0.6% | -0.7% | -0.5% |
| Class C | -0.2% | -0.3% | -0.2% | -0.3% | -0.1% | -0.2% |
| Class D | -0.1% | 0.4% | 0.3% | -0.2% | 0.0% | 0.5% |
| Class E | -0.1% | -0.6% | -0.2% | -0.3% | -0.8% | -0.1% |
| Overall | -0.1% | -0.2% | 0.0% | -0.4% | -0.4% | -0.1% |
|  | -0.1% | -0.1% | 0.0% | -0.4% | -0.4% | -0.1% |
| Class F | 0.3% | 0.4% | -0.1% | 0.8% | 0.2% | 0.3% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | -0.2% | 0.0% | -0.1% |
| Class B | -0.1% | -0.1% | -0.1% | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.2% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% |
| Class F | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |

## Influence on HM ticket #175

The influence of HM ticket #175 was tested on the top of HM5.0 and BP01. The results of BP07 (HM5.0 with a bug fix of ticket #175) and BP08 (BP01 with a bug fix of ticket #175) are shown in Table 6 and Table 7, respectively. BP07 did not have any impact on a coding performance for classes A to E, but Y BD-rates of -0.9% in LDB and -0.6% in RA were achieved for class F. In BP08, Y BD-rates of -1.1% in LDB and -0.3% in RA were achieved for classes A to E, and Y BD rates of -1.9% in LDB and -0.7% in RA were achieved for class F without any enc/dec time increase. For class F, the improvement obtained by the bug fix of HM ticket #175 is remarkable.

Table 6 Results of BP07

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.2% | -0.1% | -0.2% | 0.1% |
| Class C | -0.1% | -0.2% | 0.1% | 0.0% | 0.2% | 0.1% |
| Class D | -0.1% | 0.1% | 0.0% | -0.1% | 0.5% | -0.1% |
| Class E | 0.0% | -0.3% | 0.6% | 0.0% | -0.5% | -0.1% |
| Overall | 0.0% | -0.1% | 0.2% | -0.1% | 0.0% | 0.0% |
|  | -0.1% | -0.1% | 0.2% | -0.1% | 0.0% | 0.0% |
| Class F | -0.9% | -0.6% | -1.0% | -0.9% | -1.2% | -1.6% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.1% | 0.0% | -0.2% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.2% |
| Class B | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |  |  |  |
| Class D | -0.1% | -0.2% | -0.1% | -0.1% | 0.0% | -0.2% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
|  | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class F | -0.6% | -0.5% | -0.5% | -0.6% | -0.6% | -0.6% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 98% |  |  |

Table 7 Results of BP08

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.8% | -1.1% | -0.8% | -1.4% | -1.6% | -1.2% |
| Class C | -1.0% | -1.0% | -1.1% | -1.1% | -0.8% | -0.8% |
| Class D | -1.0% | -1.3% | -0.8% | -1.2% | -0.7% | -1.1% |
| Class E | -0.8% | -1.2% | -1.0% | -1.0% | -2.3% | -0.7% |
| Overall | -0.9% | -1.2% | -0.9% | -1.2% | -1.3% | -1.0% |
|  | -0.9% | -1.2% | -0.9% | -1.2% | -1.3% | -1.0% |
| Class F | -1.9% | -1.5% | -1.8% | -1.9% | -2.6% | -2.2% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.2% | -0.3% | -0.3% | -0.3% | -0.4% | -0.5% | -0.4% | -0.3% | -0.5% |
| Class B | -0.3% | -0.4% | -0.4% | -0.4% | -0.4% | -0.6% | -0.3% | -0.3% | -0.3% |
| Class C | -0.2% | -0.2% | -0.2% | -0.2% | -0.3% | -0.2% |  |  |  |
| Class D | -0.3% | -0.5% | -0.4% | -0.3% | -0.3% | -0.4% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.3% | -0.3% | -0.3% | -0.3% | -0.4% | -0.4% | -0.3% | -0.3% | -0.4% |
|  | -0.3% | -0.3% | -0.3% | -0.3% | -0.4% | -0.4% | -0.3% | -0.3% | -0.4% |
| Class F | -0.7% | -0.6% | -0.6% | -0.7% | -0.7% | -0.7% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |

## Second motion estimation for bi-prediction

The 2nd motion estimation (ME) for bi-prediction was tested on the top of both HM5.0 and BP01. In the HM5.0, the ME for bi-prediction is conducted on reference pictures in only one reference picture list after the ME for uni-prediction of both reference picture lists. This experiment evaluates the impact on ME for 2nd reference picture list in bi-prediction. On BP01, the 2nd ME was conducted only for non-GPB pictures in RA, since the proposed method does not need the ME of List 1.

The results of BP05 (HM5.0 with 2nd ME) and BP06 (BP01with 2nd ME) are shown in Table 8 and Table 9, respectively. BP05 achieved the Y BD-rates of -0.7% in LDB and -0.6% in RA with 31% and 21% encoding time increase, respectively, whereas BP06 achieved the Y BD-rates of -1.0% in LDB and -0.7% in RA with 0% and 17% encoding time increase, respectively. These results show that the 2nd ME for bi-prediction does not improve the tradeoff between performance and complexity. And it is confirmed that BP06 is better than BP05 by 0.3% and 0.1% in LDB and RA in Y BD-rate, and the encoder of BP05 is 31% and 3% slower than that of BP06 in LDB and RA, respectively. Note that a coding result of class F shown in Table 8 and Table 9 are inaccurate because of the HM ticket #175. The simulation results enabling the HM ticket #175 are shown in Table 10 and Table 11.

Table 8 Results of BP05

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.4% | -0.7% | -0.5% | -1.0% | -1.5% | -1.0% |
| Class C | -0.6% | -0.8% | -0.5% | -0.8% | -0.6% | -0.9% |
| Class D | -0.6% | -0.7% | -0.6% | -0.6% | 0.0% | -0.7% |
| Class E | -0.3% | -0.5% | -0.2% | -0.8% | -1.8% | -0.9% |
| Overall | -0.5% | -0.7% | -0.5% | -0.8% | -1.0% | -0.9% |
|  | -0.5% | -0.7% | -0.5% | -0.8% | -0.9% | -0.8% |
| Class F | -0.8% | -0.5% | -1.0% | -0.6% | -0.6% | -0.9% |
| Enc Time[%] | 129% |  |  | 132% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.1% | -0.3% | -0.3% | -0.4% | -0.5% | -0.4% | -0.7% | -1.1% | -1.1% |
| Class B | -0.6% | -0.6% | -0.6% | -1.0% | -0.9% | -1.0% | -0.6% | -0.6% | -0.7% |
| Class C | -0.5% | -0.5% | -0.5% | -0.7% | -0.7% | -0.5% |  |  |  |
| Class D | -0.5% | -0.6% | -0.6% | -0.4% | -0.4% | -0.4% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.5% | -0.5% | -0.6% | -0.7% | -0.7% | -0.6% | -0.7% | -0.9% | -0.9% |
|  | -0.5% | -0.5% | -0.6% | -0.7% | -0.7% | -0.7% | -0.7% | -0.8% | -0.8% |
| Class F | -0.3% | -0.3% | -0.3% | -0.2% | -0.1% | -0.1% |  |  |  |
| Enc Time[%] | 120% |  |  | 124% |  |  | 120% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |

Table 9 Results of BP06

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.8% | -1.0% | -0.9% | -1.3% | -1.7% | -1.4% |
| Class C | -0.9% | -0.9% | -0.8% | -1.1% | -0.7% | -1.0% |
| Class D | -0.9% | -0.7% | -0.6% | -1.0% | -0.3% | -0.6% |
| Class E | -0.8% | -0.7% | -0.5% | -1.0% | -2.1% | -0.7% |
| Overall | -0.8% | -0.8% | -0.7% | -1.1% | -1.2% | -1.0% |
|  | -0.8% | -0.8% | -0.8% | -1.1% | -1.1% | -1.0% |
| Class F | -0.2% | -0.4% | -0.6% | 0.0% | -1.1% | -0.5% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.3% | -0.4% | -0.4% | -0.5% | -0.5% | -0.5% | -0.7% | -0.9% | -1.0% |
| Class B | -0.7% | -0.7% | -0.7% | -0.9% | -0.8% | -0.9% | -0.7% | -0.7% | -0.7% |
| Class C | -0.6% | -0.6% | -0.5% | -0.7% | -0.5% | -0.4% |  |  |  |
| Class D | -0.5% | -0.5% | -0.5% | -0.5% | -0.3% | -0.5% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.6% | -0.6% | -0.6% | -0.7% | -0.6% | -0.6% | -0.7% | -0.8% | -0.9% |
|  | -0.6% | -0.6% | -0.6% | -0.7% | -0.6% | -0.6% | -0.7% | -0.8% | -0.8% |
| Class F | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 117% |  |  | 120% |  |  | 117% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 100% |  |  |

Table 10 Results of BP05 with Ticket #175

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.5% | -0.6% | -0.7% | -1.1% | -1.5% | -0.9% |
| Class C | -0.6% | -0.9% | -0.6% | -0.9% | -0.7% | -0.9% |
| Class D | -0.7% | -0.8% | -0.5% | -0.9% | -0.3% | -1.1% |
| Class E | -0.3% | -0.3% | -0.7% | -0.9% | -1.9% | -1.2% |
| Overall | -0.5% | -0.7% | -0.6% | -0.9% | -1.1% | -1.0% |
|  | -0.5% | -0.7% | -0.6% | -0.9% | -1.0% | -1.0% |
| Class F | -1.7% | -1.6% | -2.0% | -1.7% | -2.5% | -1.5% |
| Enc Time[%] | 128% |  |  | 132% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.2% | -0.5% | -0.6% | -0.5% | -0.6% | -0.6% | -0.7% | -1.3% | -1.2% |
| Class B | -0.7% | -0.7% | -0.7% | -1.1% | -0.9% | -1.0% | -0.7% | -0.7% | -0.7% |
| Class C | -0.7% | -0.6% | -0.6% | -0.8% | -0.7% | -0.7% |  |  |  |
| Class D | -0.6% | -0.7% | -0.6% | -0.6% | -0.6% | -0.7% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.6% | -0.7% | -0.6% | -0.8% | -0.8% | -0.8% | -0.7% | -1.0% | -0.9% |
|  | -0.6% | -0.7% | -0.6% | -0.8% | -0.8% | -0.8% | -0.7% | -1.0% | -0.9% |
| Class F | -1.1% | -0.9% | -0.9% | -1.1% | -1.0% | -1.0% |  |  |  |
| Enc Time[%] | 120% |  |  | 123% |  |  | 120% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |

Table 11 Results of BP06 with Ticket #175

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.8% | -1.1% | -0.8% | -1.4% | -1.6% | -1.2% |
| Class C | -1.0% | -1.0% | -1.1% | -1.1% | -0.8% | -0.8% |
| Class D | -1.0% | -1.3% | -0.8% | -1.2% | -0.7% | -1.1% |
| Class E | -0.8% | -1.2% | -1.0% | -1.0% | -2.3% | -0.7% |
| Overall | -0.9% | -1.2% | -0.9% | -1.2% | -1.3% | -1.0% |
|  | -0.9% | -1.2% | -0.9% | -1.2% | -1.3% | -1.0% |
| Class F | -1.9% | -1.5% | -1.8% | -1.9% | -2.6% | -2.2% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.3% | -0.6% | -0.6% | -0.6% | -0.7% | -0.7% | -0.7% | -0.9% | -1.2% |
| Class B | -0.8% | -0.7% | -0.8% | -1.0% | -0.8% | -0.9% | -0.8% | -0.8% | -0.8% |
| Class C | -0.7% | -0.7% | -0.6% | -0.8% | -0.8% | -0.6% |  |  |  |
| Class D | -0.7% | -0.8% | -0.6% | -0.7% | -0.7% | -0.7% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.7% | -0.7% | -0.7% | -0.8% | -0.8% | -0.7% | -0.7% | -0.8% | -1.0% |
|  | -0.7% | -0.7% | -0.7% | -0.8% | -0.8% | -0.8% | -0.7% | -0.8% | -1.0% |
| Class F | -1.1% | -0.9% | -0.9% | -1.1% | -1.0% | -1.0% |  |  |  |
| Enc Time[%] | 117% |  |  | 119% |  |  | 117% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  | 100% |  |  |

## Influence on non-GPB pictures in RA

The proposed modification on MVD for L1 for non-GPB pictures in RA was evaluated as BP03. In BP03, the MVD for L1 sets to (0, 0) and it is not signaled for all inter pictures. Since the proposed modification on MVD for L1 is applied to all inter pictures in LDB, the simulation results of BP03 are identical to that of BP01 in LDB. The results of BP03 are shown in Table 12. BP03 causes 0.9% loss in RA. The result shows that the proposed modification is not effective for non-GPB picture in HM.

Table 12 Results of BP03

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.8% | -1.0% | -0.9% | -1.3% | -1.7% | -1.4% |
| Class C | -0.9% | -0.9% | -0.8% | -1.1% | -0.7% | -1.0% |
| Class D | -0.9% | -0.7% | -0.6% | -1.0% | -0.3% | -0.6% |
| Class E | -0.8% | -0.7% | -0.5% | -1.0% | -2.1% | -0.7% |
| Overall | -0.8% | -0.8% | -0.7% | -1.1% | -1.2% | -1.0% |
|  | -0.8% | -0.8% | -0.8% | -1.1% | -1.1% | -1.0% |
| Class F | -0.2% | -0.4% | -0.6% | 0.0% | -1.1% | -0.5% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.4% | 0.3% | 0.3% | 0.4% | 0.3% | 0.3% | 0.2% | 1.7% | 1.4% |
| Class B | 1.2% | 1.2% | 1.2% | 1.0% | 0.9% | 0.8% | 1.2% | 1.3% | 1.3% |
| Class C | 1.0% | 1.1% | 1.3% | 0.9% | 1.0% | 1.2% |  |  |  |
| Class D | 1.0% | 1.0% | 1.2% | 1.0% | 1.0% | 1.0% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 1.0% | 1.0% | 1.1% | 0.9% | 0.9% | 0.9% | 0.8% | 1.5% | 1.3% |
|  | 1.0% | 1.0% | 1.1% | 0.9% | 0.8% | 0.9% | 0.8% | 1.5% | 1.4% |
| Class F | 1.0% | 1.4% | 1.5% | 1.2% | 1.3% | 1.3% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  | 100% |  |  |

## Comparison of List 1 and List 0 on modification of bi-prediction

The method in which MVD for List 0 of bi-prediction is not signaled and it sets to (0, 0) for GPB-pictures is compared with BP01. The results of BP01 and BP02 are shown in Table 4 and Table 13, respectively. These results show their coding performances are comparable.

Table 13 Results of BP02

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.8% | -0.9% | -0.7% | -1.3% | -1.6% | -1.2% |
| Class C | -0.8% | -1.0% | -0.7% | -1.0% | -0.9% | -0.9% |
| Class D | -0.8% | -0.7% | -0.7% | -0.9% | -0.3% | -0.8% |
| Class E | -0.8% | -1.6% | -0.7% | -1.0% | -2.2% | -1.4% |
| Overall | -0.8% | -1.0% | -0.7% | -1.1% | -1.2% | -1.1% |
|  | -0.8% | -0.9% | -0.7% | -1.1% | -1.2% | -1.1% |
| Class F | 0.3% | 0.2% | 0.2% | 0.7% | -0.2% | -0.1% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.1% | -0.1% | -0.1% | -0.2% | -0.2% | -0.2% | -0.3% | -0.1% | -0.4% |
| Class B | -0.2% | -0.2% | -0.3% | -0.3% | -0.3% | -0.4% | -0.2% | -0.2% | -0.2% |
| Class C | -0.1% | -0.2% | -0.2% | -0.1% | -0.2% | -0.1% |  |  |  |
| Class D | -0.2% | -0.2% | -0.2% | -0.1% | -0.1% | -0.2% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% | -0.3% | -0.2% | -0.2% | -0.3% |
|  | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% | -0.3% | -0.2% | -0.2% | -0.3% |
| Class F | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  | 100% |  |  |

# Conclusion

This contribution reports the simulation results of CE9 2.1. The results show the proposed method with syntax changes achieves 1% gain in LDB and 0.2% gain in RA with no encoding /decoding time increase.

The influence on HM ticket #175 was also tested. It is confirmed that it can improve the coding performance of class F by 0.9% and 0.6% in LDB and RA, respectively. The total gain by the proposal with a bug fix of ticket #175 is 1.1% gain in LDB and 0.3% in RA with no encoding /decoding time increase.

Two methods of encoder only change were tested.

1) The encoder only change described in JCTVC-G305 was not effective. The gain obtained by encoder only change of proposal was 0.2% in LDB and 0% in RA. The syntax change achieves 0.8% gain in LDB and 0.2% gain in RA.

2) The 2nd motion estimation for bi-prediction did not improve the tradeoff performance between coding performance and encoder complexity. It improved the coding performance of HM5.0 by 0.7% in LDB, but the encoding time was increased by 31%. To achieve 1% gain in LDB, the HM 5.0 encoder needs more works. It also improved the coding performance of proposal by 0.7% in RA, but the encoding time was increased by 17%.

It is recommended that

- the syntax change described in section 2.1 in this contribution is adopted into the CD and

- the problem of the HM ticket #175 is fixed by the adoption of BP08 to HM6.0.

# Patent rights declaration(s)

NTT DOCOMO, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Y. Suzuki, A. Fujibayashi., “Non-CE9: Bi-prediction for low delay coding”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G305, 7th Meeting: Geneva, CH, 21-30 November, 2011.

- I. Kim, J. Jung and S. Oudin, A. Fujibayashi., “Description of Core Experiment 9 (CE9): MV Coding and Skip/Merge Operations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G1209, 7th Meeting: Geneva, CH, 21-30 November, 2011.

- B. Bross et al., “WD5: Working Draft 5 of High-Efficiency Video Coding”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G1103_d9, 7th Meeting: Geneva, CH, 21-30 November, 2011.

- F. Bossen., “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G1200, 7th Meeting: Geneva, CH, 21-30 November, 2011.