| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0112 |
| --- | --- |

| Title: | CE1, subtest C4: Multi-parameter probability estimation for CABAC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Elena Alshina, Alexander Alshin, JeongHoon Park Samsung #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea | Tel: Email: | Elena Alshina +82-10-3026-1305 {elena_a.alshina, alexander_b.alshin, jeonghoon}@samsung.com |
| Source: | Samsung Electronics Co. Ltd.. |  |  |

_____________________________

# Abstract

This is performance test results for experiment C4 in CE1. High precision probability estimation in CABAC is studied. Average performance improvement provided by studied tool is 0.3% if one-parameter model is used and 0.7% in case of two parameters. No probability initialization adjustment was used during this test.

# Introduction

Multi-parameter probability estimation for CABAC was proposed in [1-2]. Main idea is to introduce high precision probability which is up-dated using direct calculation instead look-up table. Parameters for probability up-date was chosen as power of 2 in order to have shifts-add implementation only. The parameter in probability up-date model (M) can be interpreted as number of bins which have effect on current probability up-date.

One parameter probability estimation model is identical to HM 5.0 and AVC CABAC, just more distance symbol information is taken into account during probability up-date (M=32). We also tested longer distance prediction in one parameter probability up-date model M=64. In mandatory tests it provides slightly better results. But for LD(B) and LD(P) tests and tests in slice mode M=32 is preferable. So we don’t report M=64 performance here.

Two parameters model uses extremely short distance prediction (M0=16) during probability estimation during short transition period after entropy coder initialization. This allows fast stabilization near actual probability value (which for current content might be different from predefined initialization value). After stabilization multi-parameter probability up-date models use semi-sum of short (M0=16) and long distance (M1=128) predictions. This makes probability estimation more robust.

Interval sub-division in entropy coder is implemented as look-up table. Compare to [2] we reduced LUT size 8 times without elements recalculation (just removed redundant lines). This doesn’t affect on performance (0.00X% in terms of BD-rate).

For initialization of high precision probability we use not-changed HM5.0 8-bits initialization and convert it to 15-bits probability we use in calculations.

As well as in [1-2] we use 15 bits representation for probability. Bit-depth can be reduced (to 12 bits almost without performance change), but still 12 bits number should be represented as unsigned short in s/w implementation (2 bytes variable). So this bit-depth reduction doesn’t bring big benefits.

# Simulation results

For performance test Multi parameter probability up-date model was implemented in HM5.0 and tested according to [3]. Brief summary of test results for one parameter and two-parameter models are represented in Table 1 (average across mandatory tests Luma BD-rates is reported). These test data demonstrate that multi-parameter model provides twice higher Luma gain compare to one parameter. Chroma BD-rate for multi-parameter model is also 0.7% better compare to one parameter model.

Table 1. Test results summary for One and Two-parameter probability up-date models.

| Test | One parameter | Multi-parameters |
| --- | --- | --- |
| AI | -0.4% | -0.8% |
| RA | -0.3% | -0.7% |
| LB | -0.2% | -0.6% |
| All | -0.3% | -0.7% |
| Avg Enc Time | 100% | 103% |
| Avg Dec Time | 100% | 101% |

More details can be found in Tables 2,3 and excel spread-sheets attached to this contribution.

We would like to thank Canon for verification of our test in CE1.

# Conclusions

Based on reported performance improvement (0.7% in terms of Luma BD-rate) Samsung propose to adopt multi-parameter CABAC probability up-date model to WD and HM6.0.

# Patent rights declarations

Samsung Electronics Co. Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Alexander Alshin, Elena Alshina “Multi-parameter probability up-date for CABAC”, JCTVC-F254, Torino, ITA, July 2011

- A. Alshin, E. Alshina, J.H. Park “CE1 (subset B): Multi-parameter probability up-date for CABAC,” Document of Joint Collaborative Team on Video Coding, JCTVC-G764, November 2011.

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-G1200, November 2011.

Appendix

Table 2. Performance test results for one parameter probability estimation model (M=32).

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.5% | 0.6% | 0.5% | -0.4% | -0.1% | -0.1% | -0.5% | 0.7% | 0.5% |
| Class B | -0.4% | 0.0% | 0.0% | -0.3% | -0.3% | -0.2% | -0.4% | 0.0% | 0.0% |
| Class C | -0.4% | -0.3% | -0.2% | -0.3% | -0.3% | -0.3% | -0.4% | -0.2% | -0.1% |
| Class D | -0.4% | -0.1% | -0.2% | -0.4% | -0.4% | -0.4% | -0.4% | -0.2% | -0.1% |
| Class E | -0.3% | 0.4% | 0.5% | -0.3% | -0.1% | -0.1% | -0.3% | 0.5% | 0.5% |
| Class F | -0.2% | 0.0% | 0.2% | -0.3% | 0.1% | -0.2% | -0.3% | 0.1% | 0.2% |
| Overall | -0.4% | 0.1% | 0.1% | -0.4% | -0.2% | -0.2% | -0.4% | 0.1% | 0.1% |
|  | -0.4% | 0.1% | 0.1% | -0.4% | -0.2% | -0.2% | -0.4% | 0.1% | 0.1% |
| Enc Time[%] | 101% |  |  | 100% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 100% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.5% | 1.0% | 1.3% | -0.4% | 0.1% | 0.1% | -0.4% | 1.6% | 1.4% |
| Class B | -0.3% | 0.2% | 0.1% | -0.3% | 0.0% | -0.1% | -0.2% | 0.2% | 0.2% |
| Class C | -0.3% | -0.2% | 0.0% | -0.3% | -0.3% | -0.4% | -0.2% | 0.0% | -0.1% |
| Class D | -0.2% | -0.3% | 0.1% | -0.3% | -0.1% | -0.4% | -0.2% | -0.1% | 0.6% |
| Class E |  |  |  |  |  |  |  |  |  |
| Class F | -0.2% | 0.1% | 0.2% | -0.2% | -0.2% | -0.2% | -0.1% | 0.1% | 0.1% |
| Overall | -0.3% | 0.2% | 0.3% | -0.3% | -0.1% | -0.2% | -0.2% | 0.4% | 0.4% |
|  | -0.3% | 0.1% | 0.3% | -0.3% | -0.1% | -0.2% | -0.2% | 0.4% | 0.4% |
| Enc Time[%] | 100% |  |  | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.2% | 0.6% | 0.7% | -0.3% | 0.8% | 0.9% | -0.2% | 0.7% | 0.7% |
| Class C | -0.1% | 0.0% | 0.3% | -0.2% | 0.2% | 0.2% | -0.1% | 0.2% | 0.3% |
| Class D | -0.2% | 0.1% | 0.3% | -0.2% | 1.5% | 0.3% | -0.3% | 0.1% | 0.4% |
| Class E | -0.2% | 0.4% | 0.1% | -0.2% | -0.3% | -0.3% | -0.3% | -0.3% | 1.8% |
| Class F | -0.1% | 0.3% | 0.2% | -0.2% | -0.3% | -0.1% | 0.0% | 0.6% | 0.4% |
| Overall | -0.2% | 0.3% | 0.3% | -0.2% | 0.4% | 0.3% | -0.2% | 0.3% | 0.7% |
|  | -0.2% | 0.3% | 0.3% | -0.2% | 0.4% | 0.2% | -0.2% | 0.3% | 0.7% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  | 100% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  | Low delay P HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.3% | 0.9% | 0.7% | -0.2% | 1.2% | 0.5% | -0.3% | 0.9% | 0.8% |
| Class C | -0.2% | 0.4% | -0.1% | -0.2% | 0.1% | 0.4% | -0.2% | 0.1% | 0.3% |
| Class D | -0.3% | 0.5% | -0.1% | -0.3% | 1.0% | 0.6% | -0.2% | 0.3% | 0.7% |
| Class E | -0.2% | 0.5% | 0.0% | -0.2% | -0.3% | -0.5% | -0.3% | 0.2% | 1.1% |
| Class F | 0.0% | 0.4% | 0.8% | 0.0% | 0.5% | 0.8% | -0.4% | 0.2% | 0.4% |
| Overall | -0.2% | 0.5% | 0.3% | -0.2% | 0.6% | 0.4% | -0.3% | 0.4% | 0.7% |
|  | -0.2% | 0.4% | 0.2% | -0.2% | 0.5% | 0.4% | -0.3% | 0.4% | 0.7% |
| Enc Time[%] | 100% |  |  | 101% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  | 101% |  |  |

Table 3. Performance test results for multi-parameter probability estimation model (M0=16, M1=128).

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.9% | 0.1% | -0.1% | -0.8% | -0.7% | -0.6% | -0.9% | 0.2% | 0.0% |
| Class B | -0.8% | -0.7% | -0.7% | -0.8% | -0.9% | -0.9% | -0.8% | -0.7% | -0.6% |
| Class C | -0.8% | -0.7% | -0.7% | -0.7% | -0.8% | -0.8% | -0.7% | -0.7% | -0.6% |
| Class D | -0.8% | -0.4% | -0.5% | -0.7% | -0.8% | -0.8% | -0.8% | -0.5% | -0.4% |
| Class E | -0.7% | -0.8% | -0.6% | -0.8% | -0.9% | -0.8% | -0.7% | -0.7% | -0.5% |
| Class F | -0.5% | -0.6% | -0.3% | -0.6% | -0.4% | -0.6% | -0.6% | -0.5% | -0.3% |
| Overall | -0.7% | -0.5% | -0.5% | -0.8% | -0.8% | -0.8% | -0.7% | -0.5% | -0.4% |
|  | -0.7% | -0.5% | -0.4% | -0.8% | -0.8% | -0.8% | -0.7% | -0.5% | -0.4% |
| Enc Time[%] | 105% |  |  | 103% |  |  | 105% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  | 100% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.9% | 0.2% | 0.2% | -0.8% | -0.7% | -0.6% | -0.9% | 0.6% | 0.6% |
| Class B | -0.7% | -0.6% | -0.5% | -0.7% | -0.8% | -1.1% | -0.7% | -0.6% | -0.5% |
| Class C | -0.6% | -0.5% | -0.4% | -0.6% | -0.9% | -0.8% | -0.6% | -0.4% | -0.3% |
| Class D | -0.5% | -0.3% | -0.2% | -0.5% | -0.7% | -0.9% | -0.5% | -0.3% | 0.1% |
| Class E |  |  |  |  |  |  |  |  |  |
| Class F | -0.5% | -0.3% | -0.2% | -0.5% | -0.7% | -0.8% | -0.4% | -0.2% | 0.0% |
| Overall | -0.7% | -0.3% | -0.2% | -0.6% | -0.8% | -0.8% | -0.6% | -0.2% | -0.1% |
|  | -0.6% | -0.3% | -0.2% | -0.7% | -0.8% | -0.9% | -0.6% | -0.1% | -0.1% |
| Enc Time[%] | 103% |  |  | 102% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.7% | -0.3% | 0.1% | -0.8% | -0.5% | -0.5% | -0.6% | -0.2% | 0.0% |
| Class C | -0.5% | -0.3% | 0.1% | -0.5% | -0.4% | -0.8% | -0.5% | 0.0% | 0.0% |
| Class D | -0.4% | 0.0% | 0.0% | -0.4% | 0.1% | -0.5% | -0.4% | -0.4% | 0.4% |
| Class E | -0.5% | 0.4% | -0.1% | -0.6% | -1.0% | -1.2% | -0.5% | -0.7% | -1.0% |
| Class F | -0.5% | -0.1% | -0.1% | -0.5% | -1.6% | 0.0% | -0.4% | 0.1% | 0.1% |
| Overall | -0.5% | -0.1% | 0.0% | -0.6% | -0.6% | -0.5% | -0.5% | -0.2% | -0.1% |
|  | -0.5% | -0.1% | 0.0% | -0.6% | -0.6% | -0.6% | -0.5% | -0.2% | -0.1% |
| Enc Time[%] | 102% |  |  | 102% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  | 101% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  | Low delay P HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.7% | 0.1% | 0.0% | -0.7% | -0.1% | -0.4% | -0.7% | -0.1% | 0.0% |
| Class C | -0.5% | 0.0% | -0.1% | -0.5% | -0.5% | -0.3% | -0.5% | -0.1% | 0.0% |
| Class D | -0.5% | 0.7% | -0.2% | -0.5% | -0.5% | -0.1% | -0.4% | 0.0% | 0.9% |
| Class E | -0.5% | 1.0% | 0.4% | -0.5% | -1.1% | -1.3% | -0.6% | -0.6% | -0.7% |
| Class F | -0.5% | 0.3% | 0.2% | -0.2% | -0.8% | 0.2% | -0.6% | 0.1% | 0.1% |
| Overall | -0.5% | 0.4% | 0.0% | -0.5% | -0.5% | -0.3% | -0.6% | -0.1% | 0.1% |
|  | -0.5% | 0.3% | 0.0% | -0.5% | -0.5% | -0.3% | -0.6% | -0.1% | 0.2% |
| Enc Time[%] | 103% |  |  | 103% |  |  | 103% |  |  |
| Dec Time[%] | 101% |  |  | 103% |  |  | 101% |  |  |