| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0114 |
| --- | --- |

| Title: | Non-CE8: Sample adaptive offset with constrained ranges of edge offset |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Elena Alshina, Alexander Alshin, JeongHoon Park Samsung #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea In Suk Chong, Marta Karczewicz Qualcomm 5775 Morehouse Dr, San Diego, CA 92121, USA Chih-Ming Fu, Ching-Yeh Chen, Chia-Yang Tsai, Yu-Wen Huang, Shawmin Lei MediaTek No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Elena Alshina +82-10-3026-1305 {elena_a.alshina, alexander_b.alshin, jeonghoon}@samsung.com Marta Karczewicz +1-858-658-5673 {ichong, martak}@qualcomm.com Shawmin Lei +886-3-5670766 ext. 25555 {chihming.fu, chingyeh.chen, chiayang.tsai, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | Samsung Electronics Co. Ltd., Qualcomm Inc., MediaTek Inc.. |  |  |

_____________________________

# Abstract

In this contribution range of edge offset is restricted to allow fixed length coding. Prediction for edge offset value similar to JCTVC-G222 is also exploited. This contribution resolves pepper-salt noise which is theoretically possible with current Sample Adaptive Offset implementation in HM-5.0. Proposed edge offset coding modification provides in average -0.1% luma BD-rate and -0.4% chroma BD-rate.

# Introduction

It is known that Sample Adaptive Offset (SAO) [1-2] improves both objective and subjective quality of reconstructed signal. But in very rare cases as was pointed out in [3] it might cause salt and pepper noises because current HM5.0 encoder only optimizes PSNR without considering subjective quality. Pepper-salt noise appears when EO operates as sharpening filter making minimums and maximums extremely sharp. In order to avoid this we propose to constrain edge offsets by the predetermined limits. The range for EO was chosen based on analysis of statistics for edge offsets in HM5.0. The same statistical analysis for edge offsets in HM4.0 was included into [4].

In details proposed algorithm will be introduced in Section 2, simulation results will be shown in Section 3, and conclusions will be given in Section 4.

# Algorithm description

## EO statistics

Figure 1 and Figure 2 show the edge offset distribution of 10-bit pixels and that of 8-bit pixels, respectively, for different categories in EO. There is almost no difference between Figures 1-2 and corresponding figures in [4] where HM-4.0 EO statistics was analyzed. This means that statistical distribution of EO is stable.

It can be seen that

- zero offset is not always the most selected (especially for 10 bits samples);

- EO efficient range is non-symmetric around zero value

- EO efficient range is wider for higher bit-depth

- EO efficient range for categories 0,3 is wide compare to category 1,2

So current offset codeword design in EO of HM-5.0 is not proper.

Fig. 1. Offset distribution of 10-bit pixels for different categories in EO

Fig. 2. Offset distribution of 8-bit pixels for different categories in EO

The meaning of EO is illustrated by Figure 3a and 3b. If pixel is classified as minimum (category 0) then positive offset operates as smoothing filter (minimum becomes less deep). The same affect have negative offsets in category 3 (local maximum). Negative EO in local minimum (category 0) or positive EO in local maximum (category 3) makes reconstructed signal more sharp. The similar situation with edges classifies to categories 1 and 2 (refer Fig. 3b).

Fig. 3. Edge offset physical meaning

Statistics on Fig.1-2 shows that the most frequently SAO in HM5.0 operates as smoothing filter. But total restriction of EO sharpening effect leads to overall SAO performance degradation.

## EO range and most probable value

There is clear anti-symmetry between EO of category “i” and EO category “3-i” (i=0,1):

- the most probable values EO (i) and EO(3-i) have the same magnitude but opposite signs;

- ranges are mirrored:

  - Minimum (EO(3-i)) = -Maximum(EO(i));

  - Maximum(EO(3-i)) = -Minimum(EO(i)).

EO statistics analysis shows that the most probable value for EO is non-zero in many cases.

Most probable EO value and EO range depend on category and internal bit-depth. We suggest to limit EO range into diapason with length 2(uiEOBits-1)+1 in order to encode EO with predetermined and fixed number of bits.

Suggested ranges for EO and Most Probable EO values are summarized Table 1:

- iEOMin is predefined lower limit for EO,

- uiEOBits is predefined number of bits to encode EO,

- iEOMax is predefined upper limit for EO (iEOMax = iEOMin + 2(uiEOBits-1)+1),

- iEOMostProbable is expected EO value.

Table 1. Proposed EO range and most probable EO value.

| Bit-depth | 8 bits |  | 10 bits |  |
| --- | --- | --- | --- | --- |
| EO category i | 0 | 1 | 0 | 1 |
| Example |  |  |  |  |
| iEOMin(i) | -1 | 0 | -5 | -1 |
| iEOMax(i) | 3 | 1 | 11 | 3 |
| uiEOBits(i) | 3 | 1 | 5 | 2 |
| iEOMostProbable(i) | 1 | 0 | 1 | 1 |

## EO prediction

Neighbours blocks utilising EO likely have the same set EO values. So EO(i) in left block can be used as iEOMostProbable(i) in current block.

As it mentioned above EO(3-i) is likely to be –EO(i), i=0,1. So we suggest to use -EO(i), i=0,1 as iEOMostProbable(3-i).

## Proposed EO syntax

EO syntax in HM5.0 is described in Table 2; it uses signed integer Exp-Golomb-code.

We suggest to use 1 bit flag for signaling whether EO(i) matches with iEOMostProbable (i) and if not then signal remainder with (uiEOBits(i)-1) bits. Decoding process for EO is shown on Figure 4.

Figure 4. Proposed EO syntax.

# Simulation results

## Objective performance

This proposal was integrated on top of HM5.0 and tested according to [5]. Tables 1 ad 2 below show results for HE-AI, HE-RA, HE-LDB, HE-LDP and HE10-AI, HE10-RA, HE10-LDB, HE10-LDP test cases. Additionally performance test was done for LC-AI, LC -RA, LC -LDB, LC –LDP test cases (SAO was enabled for both anchor and modification).

This proposal consist of 2 parts:

- Limitation of EO;

- EO syntax modification.

EO limitation can be applied as encoder only modification. Table 1 shows results when we only restrict EO range on encoder side. There is no noticeable performance change in average for Luma (0.0%) and small average drop for Chroma (0.1%).

Table 2 shows test results if additionally to EO limitation we apply proposed syntax modification. This combination provides in average 0.1% (Luma)/ 0.4%(Chroma) performance improvement compare to HM5.0-HE. We would like to thank Toshiba for cross-check of these test results.

## Proposed EO syntax modification was also tested with LCU based SAO syntax in [6] and show very similar performance.Pepper and salt noise removal

It was reported in [3] that sometimes SAO produces pepper and salt noise. January, 13 authors of [3] sent the list of video sequences suggested for visual quality test and provided still image of HM5.0 with SAO on, off and [3] integrated in HM5.0.

Figures 5-13 illustrate visual quality according to this list. For HM5.0 with SAO on/off and JCTVC-H434 [7] we re-use images provided by TI.

| HM5.0 | HM5.0 SAO off |
| --- | --- |
| HM5.0 + G680 | HM5.0 + proposed modification |

Fig. 5. Part of BasketballDrive AI-HE QP37 Frame 286

| HM5.0 | HM5.0 SAO off |
| --- | --- |
| HM5.0 + G680 | HM5.0 + proposed modification |

Fig. 6. Part of BQTerrace AI-HE QP37 Frame 194

| HM5.0 | HM5.0 SAO off |
| --- | --- |
| HM5.0 + G680 | HM5.0 + proposed modification |

Fig. 7. Part of ChinaSpeed AI-HE QP37 Frame 499

| HM5.0 | HM5.0 SAO off |
| --- | --- |
| HM5.0 + G680 | HM5.0 + proposed modification |

Fig. 8. BasketballDrive LB-HE QP37 Frame 146

| HM5.0 | HM5.0 SAO off |
| --- | --- |
| HM5.0 + G680 | HM5.0 + proposed modification |

Fig.9. BasketballDrive LB-HE QP37 Frame 423

| HM5.0 | HM5.0 SAO off |
| --- | --- |
| HM5.0 + G680 | HM5.0 + proposed modification |

Fig.10. BasketballDrillText LB-HE QP37 Frame 351

| HM5.0 | HM5.0 SAO off |
| --- | --- |
| HM5.0 + G680 | HM5.0 + proposed modification |

Fig.11.BQMall LB-HE QP37 Frame 29

| HM5.0 | HM5.0 SAO off |
| --- | --- |
| HM5.0 + G680 | HM5.0 + proposed modification |

Fig.12. BQMall LB-HE QP37 Frame 102

| HM5.0 | HM5.0 SAO off |
| --- | --- |
| HM5.0 + G680 | HM5.0 + proposed modification |

Fig.13. Vidyo3 LB-HE QP37 Frame 226

Pepper and salt noise problem is resolved by proposed modification as well as by JCTVC-H434 [7]. But in both case (if EO limitation is applied as encoder only change or with syntax modification) proposed algorithm demonstrates better performance compare to JCTVC-H434. One more benefit of proposed EO syntax modification compare to JCTVC-H434 is fixed length coding.

Proposed EO constrains resolve pepper-salt noise but in opposite to JCTVC-H0434 allows sharpening effect. This allows preserving texture as it is demonstrated by still images in presentation attached to this contribution.

# Conclusions

Samsung, Qualcomm, and MediaTek propose to limit EO to the range described above and modify EO syntax to use fixed length coding. Proposed modifications resolve potential salt and pepper noise and provide 0.1% Luma and 0.4% Chroma BR-rate improvement (in average for HE test case).

# Patent rights declarations

Samsung Electronics Co. Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- C.-M. Fu, C.-Y. Chen, C.-Y. Tsai, Y.-W. Huang, S. Lei (MediaTek), “CE13: Sample Adaptive Offset with LCU-Independent Decoding,” Document of Joint Collaborative Team on Video Coding, JCTVC-E049, March 2011.

- C.-M. Fu, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek), S. Park, B. Jeon (LGE), A. Alshin, E. Alshina (Samsung), “Sample Adaptive Offset for Chroma,” Document of Joint Collaborative Team on Video Coding, JCTVC-F057, July, 2011.

- W.-S. Kim, D.-K. Kwon (TI), “Non-CE8: Method of visual coding artifact removal for SAO,” Document of Joint Collaborative Team on Video Coding, JCTVC-G680, November, 2011

- C.-M. Fu, Y.-W. Huang, S. Lei (MediaTek), I. S. Chong, M. Karczewicz (Qualcomm), “Non-CE8: Offset coding in SAO,” Document of Joint Collaborative Team on Video Coding, JCTVC-G222, November, 2011

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-G1200, November 2011.

- Chih-Ming Fu, Ching-Yeh Chen, Chia-Yang Tsai, Yu-Wen Huang, Shawmin Lei (MediaTek), I. S. Chong, M. Karczewicz(Qualcomm), E. Alshina, A. Alshin (Samsung) “CE8.a.3: SAO with LCU-based syntax,” Document of Joint Collaborative Team on Video Coding, JCTVC-H0273, February 2012.

- W.-S. Kim, D.-K. Kwon (TI) “CE8 Subset c: Necessity of Sign Bits for SAO Offsets,” Document of Joint Collaborative Team on Video Coding, JCTVC-H0273, February 2012

Table 1.  Results of the proposed EO limitation on encoder side.

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.1% | 0.2% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 99% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 101% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | -0.2% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | -0.2% |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |  |  |  |
| Class F | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.3% | 0.3% | 0.0% | 0.3% | 0.3% | 0.0% | 0.0% | 0.3% |
| Class C | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.2% |
| Class D | 0.0% | -0.3% | 0.5% | 0.0% | 0.0% | -0.4% | 0.0% | 0.1% | -0.1% |
| Class E | 0.0% | 0.1% | 0.2% | -0.1% | -0.5% | 0.7% | 0.1% | -0.2% | 0.2% |
| Class F | -0.2% | -0.1% | -0.2% | 0.0% | 0.2% | 0.3% | 0.0% | 0.1% | 0.1% |
| Overall | 0.0% | 0.0% | 0.2% | 0.0% | 0.1% | 0.2% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  | 101% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  | Low delay P HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.3% | -0.2% | 0.0% | 0.1% | 0.3% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.3% | 0.0% | 0.0% | 0.1% | 0.5% | 0.0% | 0.0% | 0.2% |
| Class D | 0.0% | 0.1% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | -0.3% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.1% | 0.9% | 0.1% | 0.0% | 0.1% | -0.2% |
| Class F | 0.0% | 0.5% | 0.3% | -0.1% | 0.2% | 0.0% | 0.0% | 0.2% | 0.2% |
| Overall | 0.0% | 0.2% | 0.0% | 0.0% | 0.2% | 0.2% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  | 102% |  |  |

Table 2. Results of the proposed EO syntax change.

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.2% | 0.0% | -0.2% | -0.2% | 0.0% | 0.1% | 0.1% |
| Class B | 0.0% | -0.2% | -0.3% | 0.0% | -0.2% | -0.3% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | -0.3% | -0.3% | 0.0% | -0.3% | -0.3% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | -0.2% | -0.2% | 0.0% | -0.2% | -0.2% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | -0.3% | -0.3% | 0.0% | -0.4% | -0.3% | 0.0% | 0.0% | 0.0% |
| Class F | -0.1% | -0.2% | -0.2% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | -0.2% | -0.2% | 0.0% | -0.2% | -0.3% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 99% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  | 99% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.3% | -0.5% | -0.1% | -0.1% | -0.4% | 0.1% | 0.1% | 0.1% |
| Class B | -0.1% | -0.4% | -0.4% | -0.1% | -0.2% | -0.2% | 0.0% | 0.1% | 0.1% |
| Class C | -0.1% | -0.5% | -0.4% | -0.1% | -0.3% | -0.4% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | -0.5% | -0.6% | 0.0% | -0.2% | -0.3% | 0.0% | 0.1% | 0.1% |
| Class E |  |  |  |  |  |  |  |  |  |
| Class F | -0.2% | -0.3% | -0.3% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.1% |
| Overall | -0.1% | -0.4% | -0.5% | -0.1% | -0.2% | -0.3% | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.1% | -0.3% | -0.3% | 0.0% | -0.1% | -0.1% | 0.0% | 0.2% | 0.2% |
| Class C | -0.1% | -0.5% | -0.6% | -0.1% | -0.2% | -0.2% | 0.0% | 0.0% | 0.0% |
| Class D | -0.1% | -0.6% | -0.6% | 0.0% | -0.5% | -0.2% | 0.0% | 0.1% | 0.1% |
| Class E | -0.1% | -0.4% | -1.1% | -0.2% | -0.4% | -0.5% | 0.0% | -0.5% | 0.3% |
| Class F | -0.4% | -0.5% | -0.6% | -0.1% | -0.2% | 0.2% | 0.0% | 0.2% | 0.0% |
| Overall | -0.2% | -0.5% | -0.6% | -0.1% | -0.2% | -0.2% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  | 100% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  | Low delay P HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.2% | -0.5% | -0.7% | -0.2% | -0.4% | -0.3% | 0.0% | 0.1% | 0.0% |
| Class C | -0.1% | -0.5% | -0.7% | -0.1% | -0.2% | 0.2% | 0.0% | 0.2% | 0.1% |
| Class D | -0.1% | -0.4% | -0.9% | -0.2% | -0.2% | -0.8% | 0.0% | -0.3% | 0.1% |
| Class E | -0.2% | -0.9% | -1.0% | -0.2% | 0.6% | -1.5% | -0.1% | -0.3% | 0.1% |
| Class F | -0.2% | -0.2% | -0.4% | -0.2% | -0.1% | -0.4% | -0.1% | 0.3% | 0.1% |
| Overall | -0.2% | -0.5% | -0.7% | -0.2% | -0.1% | -0.5% | -0.1% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  | 99% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  | 100% |  |  |

WD changes

SAO syntax in HM5.0

| if( sao_type_idx != 0 ) |  |
| --- | --- |
| for( i = 0; i < NumSaoClass[ sao_type_idx ]; i++ ) |  |
| sao_offset[ i ] | se(v) |

Proposed SAOsyntax.

| if( sao_type_idx!= 0 ){ |  |
| --- | --- |
| if( sao_type_idx<5 ){ |  |
| for( i = 0; i < NumSaoClass[ sao_type_idx ]; i++ ){ |  |
| EOEqualToMostProbable[ i ] | u(1) |
| if (EOEqualToMostProbable[ i ] && uiEOBits[i]>1 ){ |  |
| if (sao_offset[ i ]> iEOMostProbable[i]) |  |
| sao_offset[ i ]--; |  |
| sao_offset[ i ]-= iEOMin[i]); |  |
| sao_offset[ i ]; | u(uiEOBits[i]-1) |
| } |  |
| } |  |
| } else { |  |
| for( i = 0; i < NumSaoClass[ sao_type_idx ]; i++ ) |  |
| sao_offset[ i ] | se(v) |
| } |  |
| } |  |

#### Sample adaptive offset parameter semantics

sao_offset[ cIdx ][ saoDepth ][ rx ][ ry ][ i ] indicates the offset value of i-th category for the color component cIdx of the region specified by saoDepth, rx and ry.

The variable bitDepth is dervied as follows.

- If cIdx is equal to 0, bitDepth is set equal to BitDepthY..

- Otherwise (cIdx is equal to1 or 2), bitDepth is set equal to BitDepthC.

leftIdx is derived as follows.

leftIdx = ((cIdx -1 >= 0) && (cIdx % (1<< saoDepth ) != 0))? cIdx -1: -1

If leftIdx is not equal to -1 and bleft_partition_available = 1

sao_offset[ cIdx ][ saoDepth ][rx][ry][i] values are modified as follows.

If sao_type_idx [ cIdx ][ saoDepth ][ rx ][ ry ] is equal to 1, 2, 3, or 4, then iEOMostProbable[ cIdx ][ saoDepth ][rx][ry][i] , iEOMin[ cIdx ][ saoDepth ][rx][ry][i] , iEOMax[ cIdx ][ saoDepth ][rx][ry][i] , iEOBits[ cIdx ][ saoDepth ][rx][ry][i] (i=0,3) are set as described below.

If bitDepth is larger than 9, Depth_th =1, otherwise Depth_th =0,

iEOMostProbable [ cIdx ][ saoDepth ][rx][ry][i] = iEOParam [0][Depth_th ][i],

iEOMin [ cIdx ][ saoDepth ][rx][ry][i] = iEOParam [1][Depth_th ][i],

iEOBits [ cIdx ][ saoDepth ][rx][ry][i] = iEOParam [2][Depth_th ][i],

iEOMax [ cIdx ][ saoDepth ][rx][ry][i] = iEOMin [ cIdx ][ saoDepth ][rx][ry][i] + (1<<( iEOBits [ cIdx ][ saoDepth ][rx][ry][i]-1)).

where parameters iEOParam are show as follows:

iEOParam [3][2][4] =

{

{{1,0,0,-1}, {1,1,-1,-1},},

{{-1,0,-1,-3},{-5,-1,-3,-11},},

{{3,1,1,3},{5,3,3,5},}

};

If bleft_partition_available is equal to 1 and i=0,1 then

iEOMostProbable [ cIdx ][ saoDepth ][rx][ry][i] = sao_offset[ cIdx-1 ][ saoDepth ][rx][ry][i]

If i=2,3 then

iEOMostProbable [ cIdx ][ saoDepth ][rx][ry][i] = -sao_offset[ cIdx ][ saoDepth ][rx][ry][3-i].