| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0116 |
| --- | --- |

| Title: | [AHG 8] Objective and subjective evaluation of HM5.0. |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | TK Tan A. Fujibayashi Y. Suzuki J.Takiue 3-6 Hikarinooka, Yokosuka-shi Kanagawa, 239 8536 Japan | Tel: Email: | +65 97993861 tktan.p@rd.nttdocomo.co.jp +81-468-40-6833 fujibayashi@nttdocomo.co.jp suzukiyos@nttdocomo.co.jp takiue@nttdocomo.co.jp |
| Source: | NTT DOCOMO, Inc |  |  |

_____________________________

# Abstract

This contribution reports objective and subjective performance measure of the HM5.0 compared to JM18.  In addition to high efficiency (HE) and low complexity (LC) common conditions as specified in JCTVC-G1200, combinations of LC with one or more tools enabled in HE but not in LC were also investigated.  It describes a step by step assessment of the performance of HM5.0 and its tools combinations starting from objective measurements, followed by informal subjective assessments and finally a formal subjective assessment.

Graphs of the average BD-rate vs. encoding / decoding time reportedly show that the objective gains of HE compared to LC were mainly contributed by RDOQ, ALF and SAO.  It was also reported that RDOQ does not incur any additional decoding time for the decoder, but SAO and ALF contributes most if not all of the additional decoding time needed by HE compared to LC. Between SAO and ALF, ALF contributes the larger portion of the decoding time.

This contribution also asserted that visual comparison showed that all the tools except for SAO, did not give any significant subjective quality improvement. Based on these results, clustering of coding tools was made and LC with RDOQ and SAO enabled (LC+RDOQ+SAO) was selected as the subset of tools combination that gave the best trade off in terms of coding gains and decoding time.

A formal subjective test on the selected subset of tools combinations was conducted by NTT DOCOMO, Inc to verify the above findings for the random access case. The results show HM5.0 with HE tools is comparable in subjective quality to JM18.2 coded at twice or more the bitrate.  The same result is also obtained for HM5.0 with LC tools and RDOQ and SAO enabled. The results also show that HM5.0 with HE tools and HM5.0 with LC+RDOQ+SAO  is indistinguishable when coded at the same bitrate.

# Introduction

Several sets of test were performed and reported in this contribution. Objective performance was first collected to confirm the various objective gains contributed by the different tools. From this the BD-rate were plotted against the encoding time and decoding time to get a very rough measure of how much processing is needed in the encoder and the decoder by each tool. Then subjective performances were evaluated by performing visual tests on the decoded sequences. The tools are clustered based on the results obtained. It was identified that the LC with RDOQ and SAO enabled (LC+RDOQ+SAO) represents the subset of tools combination that gave the best trade off in terms of coding gains and decoding time.

A formal subjective test on the selected subset of tools combinations was conducted by NTT DOCOMO, Inc to compare the following codecs and the following random access coding conditions:

1) JM18.2* high profile (modified JM18.2 as described in [2])

2) HM5.0 high efficiency (HM5.0 HE)

3) HM5.0 low complexity with RDOQ and SAO (HM5.0 LC+RDOQ+SAO)

The following sections describe the steps that were taken leading to the formal subjective test and the findings.  It was found that the intermediate steps leading to the formal subjective test have a lot of useful information that may be helpful in defining profiles and levels.  Therefore the findings of these intermediate steps are reported in the next few sections.

# Objective comparison to JM

The following HM5.0 coding structures as defined in JCTVC-G1200[1] were tested and the corresponding JM18.0 references for the objective measure are as shown in the table below.

Table 1: Table of coding structure tested and corresponding JM reference.

| HM coding structure | JM configuration |
| --- | --- |
| Random Access (RA) | Alpha anchor (IbBbBbBbP) |
| Low Delay B (LB) | Beta* anchor (IPPP) |
| Low Delay P (LP) | Beta* anchor (IPPP) |
| All Intra (AI) | Alpha anchor with all I only. |

Alpha and Beta anchors are the JM references used in the HVC call for proposal [3] Beta* anchor, represent the Beta anchor with the exception that there is no longer temporal nesting and all frames are reference frames.

The HM5.0 was used to encode the test points of the objective measure using the JCTVC-G1200 common conditions for high efficiency (HE), low complexity (LC) and other combinations of LC + tools enabled in HE but not in LC.  A value of “1” in the table indicates that the tool is enabled; a value of “0” indicates that the tool is disabled.

Table 2: Table of tools combination tested.

| Tools | RDOQ | ALF | SAO | LMChroma | AMP | NSQT |
| --- | --- | --- | --- | --- | --- | --- |
| HE | 1 | 1 | 1 | 1 | 1 | 1 |
| LC | 0 | 0 | 0 | 0 | 0 | 0 |
| LC+RDOQ | 1 | 0 | 0 | 0 | 0 | 0 |
| LC+ALF | 0 | 1 | 0 | 0 | 0 | 0 |
| LC+SAO | 0 | 0 | 1 | 0 | 0 | 0 |
| LC+ALF+SAO | 0 | 1 | 1 | 0 | 0 | 0 |
| LC+RDOQ+ALF | 1 | 1 | 0 | 0 | 0 | 0 |
| LC+RDOQ+SAO | 1 | 0 | 1 | 0 | 0 | 0 |
| LC+LMChroma | 0 | 0 | 0 | 1 | 0 | 0 |
| LC+AMP | 0 | 0 | 0 | 0 | 1 | 0 |
| LC+NSQT | 0 | 0 | 0 | 0 | 0 | 1 |
| LC+AMP+NSQT | 0 | 0 | 0 | 0 | 1 | 1 |

  - Objective results

All the simulations were conducted on a common cluster running Ubuntu Release 10.04 (Kernel Linuz 2.6.32-33-generic) where the majority of the machines have Intel® Xeon® E5450 @ 3.00GHz cores.

The full results of the simulation can be found in the attached spreadsheets.  Table 3 to  summarize the coding performance (BD-rate for Y, U and V components and the encoding and decoding time) obtained from each of the coding structure for each of the combination of tools.  The results are also plotted in graphs 1 to 8 found in the Annex A of this document.

The following can be summarised from the tables and graphs.

The luma BD-rate gains of HM5.0 HE & LC compared to the JM18.0 for the following coding conditions are:

|  | Random Access | Low Delay B | Low Delay P | All Intra |
| --- | --- | --- | --- | --- |
| High efficiency (HE) | -35.9% | -41.2% | -37.4% | -22.9% |
| Low delay (LC) | -28.5% | -34.7% | -26.2% | -16.5% |

When various tools are added to the LC common conditions, the effects on BD-Rate (+ve is gains) are as follows.  Note that the BD-rate reported here are BD-rate LC+X – BD-rate LC with JM as the reference and X is one of {RDOQ, SAO, ALF, AMP or NSQT}.

- The gains are mostly contributed by RDOQ, ALF and to a lesser extent SAO.

- Random access (cf. alpha):		RDOQ (3.5%), ALF (2.6%) and SAO (0.8%)

- Low delay B (cf. beta): 		RDOQ (2.3%), ALF (2.0%) and SAO (1.3%)

- Low delay P (cf. beta): 		ALF (6.4%), SAO (5.4%) and RDOQ (1.4%)

- Intra only (cf AI alpha):		RDOQ (4.2%) and ALF (1.7%)

- Together ALF and SAO account for most of the difference in decoding time between HE and LC.

- AMP increases encoding time, but contribute marginally to the coding gains.

- Random access: 	Enc Time: +8%,	BD-Rate gains: 0.6%

- Low delay B: 	Enc Time: +14%,	BD-Rate gains: 0.7%

- Low delay P: 	Enc Time: +10%,	BD-Rate gains: 0.6%

- NSQT has negligible impact on encoding and decoding time, but contribute coding gains as follows

- Random access: 	BD-Rate gains: 0.2%

- Low delay B: 	BD-Rate gains: 0.6%

- Low delay P: 	BD-Rate gains: 0.5%

Table 3: Performance of different combination of tools for random access coding structure

| Random Access |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Tools | Y | U | V | Enc Time[%] | Dec Time[%] |
| HE | -35.9% | -38.0% | -37.4% | 112.2% | 160.3% |
| LC | -28.5% | -25.2% | -25.1% | 77.3% | 129.2% |
| LC+RDOQ | -32.0% | -30.1% | -30.6% | 90.5% | 128.2% |
| LC+ALF | -31.1% | -25.7% | -25.3% | 87.2% | 158.0% |
| LC+SAO | -29.2% | -27.8% | -28.2% | 77.1% | 133.1% |
| LC+ALF+SAO | -31.7% | -28.4% | -28.7% | 87.2% | 163.1% |
| LC+RDOQ+ALF | -34.2% | -30.4% | -30.5% | 100.2% | 157.7% |
| LC+RDOQ+SAO | -33.0% | -32.3% | -32.9% | 89.8% | 132.0% |
| LC+LMChroma | -28.7% | -30.8% | -29.6% | 76.9% | 128.3% |
| LC+AMP | -29.1% | -25.9% | -25.9% | 85.6% | 125.3% |
| LC+NSQT | -28.7% | -25.4% | -25.2% | 77.8% | 128.0% |
| LC+AMP+NSQT | -29.4% | -26.2% | -26.1% | 88.8% | 128.4% |

Table 4: Performance of different combination of tools for low delay B coding structure

| Low delay B |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Tools | Y | U | V | Enc Time[%] | Dec Time[%] |
| HE | -41.2% | -42.9% | -43.8% | 368.6% | 177.6% |
| LC | -34.7% | -30.2% | -31.5% | 265.3% | 143.2% |
| LC+RDOQ | -37.0% | -33.5% | -34.8% | 303.6% | 144.3% |
| LC+ALF | -36.7% | -31.3% | -31.2% | 290.0% | 174.1% |
| LC+SAO | -36.0% | -35.7% | -37.5% | 265.6% | 149.9% |
| LC+ALF+SAO | -37.8% | -36.0% | -37.4% | 289.9% | 180.1% |
| LC+RDOQ+ALF | -38.8% | -34.6% | -34.7% | 322.7% | 170.6% |
| LC+RDOQ+SAO | -38.4% | -38.8% | -40.2% | 302.3% | 150.4% |
| LC+LMChroma | -34.8% | -32.4% | -33.4% | 265.7% | 143.3% |
| LC+AMP | -35.4% | -31.1% | -32.4% | 299.2% | 144.2% |
| LC+NSQT | -35.3% | -31.1% | -32.4% | 270.0% | 143.9% |
| LC+AMP+NSQT | -36.2% | -32.2% | -33.4% | 302.2% | 141.7% |

Table 5: Performance of different combination of tools for low delay P coding structure

| Low delay P |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Tools | Y | U | V | Enc Time[%] | Dec Time[%] |
| HE | -37.4% | -40.1% | -40.8% | 254.9% | 155.6% |
| LC | -26.2% | -24.2% | -25.8% | 163.8% | 119.2% |
| LC+RDOQ | -27.6% | -27.1% | -28.7% | 203.3% | 120.8% |
| LC+ALF | -32.6% | -26.9% | -26.8% | 188.8% | 151.8% |
| LC+SAO | -31.6% | -32.4% | -34.2% | 166.5% | 129.6% |
| LC+ALF+SAO | -34.0% | -32.9% | -34.2% | 187.8% | 156.7% |
| LC+RDOQ+ALF | -34.7% | -29.9% | -30.3% | 227.4% | 151.9% |
| LC+RDOQ+SAO | -33.8% | -35.2% | -36.7% | 205.8% | 131.7% |
| LC+LMChroma | -26.3% | -27.1% | -28.4% | 164.3% | 120.0% |
| LC+AMP | -26.8% | -25.3% | -26.8% | 187.4% | 120.8% |
| LC+NSQT | -26.7% | -25.4% | -26.7% | 167.4% | 121.5% |
| LC+AMP+NSQT | -27.4% | -26.2% | -27.5% | 191.5% | 121.2% |

Table 6: Performance of different combination of tools for all intra coding structure

| All Intra |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Tools | Y | U | V | Enc Time[%] | Dec Time[%] |
| HE | -22.9% | -26.0% | -26.1% | 273.4% | 220.2% |
| LC | -16.5% | -18.9% | -18.5% | 118.0% | 177.7% |
| LC+RDOQ | -20.7% | -18.6% | -18.9% | 180.3% | 170.7% |
| LC+ALF | -18.2% | -21.1% | -20.9% | 207.7% | 221.1% |
| LC+SAO | -17.1% | -20.7% | -20.8% | 119.4% | 185.3% |
| LC+ALF+SAO | -18.6% | -22.2% | -22.4% | 205.7% | 226.4% |
| LC+RDOQ+ALF | -22.1% | -20.4% | -20.9% | 264.5% | 208.2% |
| LC+RDOQ+SAO | -21.3% | -20.3% | -21.0% | 180.5% | 176.9% |
| LC+LMChroma | -17.1% | -23.0% | -22.0% | 123.1% | 180.8% |

# Subjective comparison for HM tools

Subjective comparison of the coding performance was done for HE, LC and other combinations of LC with RDOQ, ALF and SAO to see if the trend found in the objective measure is the same visually.  A subset of the decoded sequences obtained from the common conditions simulation done in section 2 was viewed.  9 sequences from class B and C encoded with QP=32,37 for both random access and low delay B structure were used. The difference of bitrate between bitstream for HE and all the case of LC+ other tools were within 5%.  The monitors used in this test were: EIZO ColorEdge CE210W (21 inch LCD, resolution: 1920x1080) and Dell Ultra Sharp U3011 (30 inch LCD, resolution: 2560x1600).

The following tables summarize the subjective difference observed in the subjective comparison for the random access and low delay coding structure.

Table 7: Subjective performances of different combination of tools for random access coding structure

| Random Access |  |
| --- | --- |
| Tools | Observations |
| LC | Some colour bleeding, aliasing, wobbling were observed. |
| LC+RDOQ | Similar quality to LC. But artifact seems more pronounced. |
| LC+ALF | Similar quality to LC. Some colour bleedings were slightly reduced but impact seems small. |
| LC+SAO | Same quality as HE. |
| LC+ALF+SAO | Same quality as HE. |
| LC+RDOQ+ALF | Similar quality to LC + RDOQ |
| LC+RDOQ+SAO | Same quality as HE. |

Table 8: Subjective performances of different combination of tools for low delay B coding structure

| Low delay B |  |
| --- | --- |
| Tools | Observations |
| LC | Some colour bleeding, aliasing, blockiness were observed. |
| LC+RDOQ | Similar problems but seems to be worse than LC |
| LC+ALF | Similar quality to LC. |
| LC+SAO | Same quality as HE. |
| LC+ALF+SAO | Same quality as HE |
| LC+RDOQ+ALF | Similar quality to LC or LC + RDOQ |
| LC+RDOQ+SAO | Same quality as HE |

The main observations found in this comparison are as follows:

- Enabling RDOQ can sometimes cause some noisiness in the subjective quality of the video.

- Enabling SAO results in the visible improvement for some sequences especially BasketballDrill and Kimono1.

- Clustering of HM tools

Based on the results in section 3 and 4, the following clusters the tools that are enabled in the HE but not in the LC common conditions are made.

Table 9: Clustering of tools enabled in HE but not in LC common conditions

| Tool cluster | Cluster 0: RDOQ | Cluster 1: LMChroma | Cluster 2: AMP NSQT | Cluster 3 SAO | Cluster 4 ALF |
| --- | --- | --- | --- | --- | --- |
| Impact on decoding time | None | Negligible | Negligible | Low to medium | High |
| Objective Gains (BD-rate relative to BD-rate LC) | 1.4% to 4.2% | Mostly chroma only 1.9% to 5.6% | Small (less than 1.0% BD-rate) | 0.5% to 5.4% | 1.7% to 6.4% |
| Subjective improvements | Can sometime cause some noisiness in the subjective quality |  |  | Visible on some sequences: BasketballDrill and Kimono1 | Not visible |

- Subjective comparison to JM

Based on the clusters described in Table 9, LC+RDOQ+SAO was selected as a minimum tool set which has the potential to achieve the same subjective quality to HE.

The following codecs are tested to verify if the HM5.0 has achieved the 50% bitrate savings target compared to JM18.

1) JM 18.2* high profile (modified JM18.2 as described in [2])

2) HM5.0 high efficiency (HM5.0 HE)

3) HM5.0 low complexity with RDOQ and SAO (HM5.0 LC+RDOQ+SAO)

The random access coding structure was used for all codecs as this was found to be the most efficient coding structure.  The JM 18.2* bitstreams are coded at QP = 27, 30, 33, 36 and the HM5.0 HE and HM5.0 LC+RDOQ+SAO bitstreams are coded at not more than half the bitrate of the JM 18.2* bitstreams.

Two tests were conducted. The first is based on the informal subjective test method used in [4].  The only difference to this method is the subject is asked to vote either A is better, B is better or A and B are the same.  The second is a formal subjective test using the double stimulus integer scale (DSIS) method similar to the method used in the HVC call for proposal [3].  Please refer to Annex B for the detailed description of these test methods.

## Results of informal subjective test

An informal blind comparison between JM 18.2*, HM5.0 HE and HM5.0 LC with minimum tool set which was judged similar quality to HE was performed as an initial test to verify the test material selection before committing to a formal subjective test.

The results of blind comparison of HM5.0 HE and LC+RDOQ+SAO to JM18.2* are summarized in Table 10 and Table 11 below.. he combination of votes made by the 4 observers for a given test case is listed in the first 3 columns.  The number of test cases which received a given combination of votes are counted and shown in the 4th column.  The rows where the combination of votes indicate that the majority voted that HM5.0 at half the bitrate (or less) has a quality better than or equal to JM 18.2*.are shaded green.  The cumulative % where the majority of the observers voted that HM5.0 at half the bitrate (or less) has a quality better than or equal to JM 18.2*, is shown in the last column.

Table 10: Informal test comparing HM5.0 high efficiency to JM18.2*

| # of observer votes received by a test case (combination of votes) |  |  | # of test cases where the combination of votes occurred | Cumulative % where majority voted that HM5.0 HE at half the bitrate has a quality better than or equal to JM 18.2*. |
| --- | --- | --- | --- | --- |
| HM5.0 HE at half the bitrate is better than JM 18.2* | HM5.0 HE at half the bitrate and JM 18.2* are comparable | JM 18.2* is better than HM5.0 HE at half the bitrate |  |  |
| 4 | 0 | 0 | 20 | 56% |
| 3 | 1 | 0 | 2 | 61% |
| 3 | 0 | 1 | 5 | 75% |
| 2 | 2 | 0 | 1 | 78% |
| 2 | 1 | 1 | 1 | 81% |
| 1 | 3 | 0 | 1 | 83% |
| 1 | 0 | 3 | 2 |  |
| 0 | 2 | 2 | 2 |  |
| 0 | 0 | 4 | 2 |  |

Table 11: Informal test comparing HM5.0 low complexity+RDOQ+SAO to JM18.2*

| # of observer votes received by a test case (combination of votes) |  |  | # of test cases where the combination of votes occurred | Cumulative % where majority voted that HM5.0 LC+RDOQ+SAO at half the bitrate has a quality better than or equal to JM 18.2*. |
| --- | --- | --- | --- | --- |
| HM5.0 LC+RDOQ+SAO at half the bitrate is better than JM 18.2* | HM5.0 LC+RDOQ+SAO at half the bitrate and JM 18.2* are comparable | JM 18.2* is better than HM5.0 LC+RDOQ+SAO at half the bitrate |  |  |
| 4 | 0 | 0 | 21 | 58% |
| 3 | 1 | 0 | 3 | 67% |
| 2 | 2 | 0 | 1 | 69% |
| 3 | 0 | 1 | 2 | 75% |
| 2 | 0 | 2 | 3 |  |
| 1 | 1 | 2 | 2 |  |
| 1 | 0 | 3 | 1 |  |
| 0 | 2 | 2 | 1 |  |
| 0 | 1 | 3 | 1 |  |
| 0 | 0 | 4 | 1 |  |

These tables show that in a large percentage of the test cases (20 and 21 out of 36 in each case), observers voted unanimously that the HM5.0 HE and LC+RDOQ+SAO at half the bitrate was giving a subjective quality better than JM18.2*.

In total, 83% and 75% of the vote cases indicated that HM5.0 HE and LC+RDOQ+SAO at half the bitrate was giving a subjective quality equal to or better than JM18.2*, respectively.

This is an indication that the 50% bitrate savings over JM target has been reached and quite possibly have been exceeded.

Table 12 shows the results of the informal test comparing HM5.0 HE and LC+RDOQ+SAO.  Rows where the combinations of votes indicate that the HM5.0 HE and LC+RDOQ+SAO have comparable quality are shaded green. In 30 out of the 36 test cases, the observers voted unanimously that there is no subjective quality difference between HM5.0 HE and LC+RDOQ+SAO.

In 94% of the vote cases indicated that HM5.0 HE and HM5.0 LC+RDOQ+SAO at the same bitrate are comparable.

Table 12: Informal test comparing HM5.0 high efficiency to HM5.0 low complexity+RDOQ+SAO

| # of observer votes received by a test case (combination of votes) |  |  | # of test cases where the combination of votes occurred | Cumulative % where majority voted that HM5.0 HE and HM5.0 LC+RDOQ+SAO have the same quality. |
| --- | --- | --- | --- | --- |
| HM5.0 LC+RDOQ+SAO is better than HM5.0 HE at the same bitrate | HM5.0 HE and HM5.0 LC+RDOQ+SAO at the same bitrate are comparable | HM5.0 HE is better than HM5.0 LC+RDOQ+SAO at the same bitrate |  |  |
| 0 | 4 | 0 | 30 | 83% |
| 1 | 3 | 0 | 2 | 89% |
| 0 | 3 | 1 | 2 | 94% |
| 0 | 2 | 2 | 2 |  |

The main indicative findings of the informal subjective test comparing HM5.0 HE, HM5.0 LC+RDOQ+SAO and JM18.2* are:

- Both HM5.0 HE and HM5.0 LC+RDOQ+SAO has achieved the target of 50% bitrate savings over JM and it is quite possible than the bitrate saving is even higher than 50%.

- HM5.0 HE and HM5.0 LC+RDOQ+SAO at the same bitrate have the same subjective quality.

Having confirmed the preliminary screening test, the formal subjective test was conducted.

## Results of formal subjective test

A formal subjective assessment was conducted and the results are as shown in Annex C.  The graphs of the mean opinion score (MOS) are shown for the 9 test sequences in Figure 9 to Figure 17.

For all the test cases, the MOS scores assigned to HM HE and HM LC+RDOQ+SAO coded at comparable bitrates are not statistically distinguishable.

In comparison to the corresponding JM bitstreams, except for the lower 3 bitrate points of "PartyScene", HM at half the bitrate or less (52% to 36%) achieves similar subjective quality to JM. (33/36 test points).

The BD-rate calculations method was applied to the MOS vs. bitrate curves to compare the HM codecs to the JM18.2* reference.  The results are shown in Table 13.  The average BD-rate gains achieved by HM5.0 RA-HE and HM5.0 RA-LC+RDOQ+SAO compared to JM are 51.2% and 49.4%, respectively.

The difference between HM5.0 RA-HE and HM5.0 RA-LC+RDOQ+RA is only 1.8%.

Table 13: BD-Rate calculated from MOS vs. bitrate plots.

|  | A) | B) |  |
| --- | --- | --- | --- |
|  | RAHE vs JM | RA LC+RDOQ+SAO vs. JM | A) - B) |
| BQTerrace | -68.4% | -62.5% | -5.9% |
| BasketballDrive | -69.8% | -67.2% | -2.6% |
| cactus | -52.4% | -50.6% | -1.8% |
| Kimono1 | -50.7% | -55.2% | 4.6% |
| ParkScene | -51.9% | -50.1% | -1.8% |
| BQMall | -46.1% | -42.2% | -3.9% |
| BasketballDrill | -46.4% | -45.9% | -0.5% |
| PartyScene | -25.2% | -29.0% | 3.9% |
| RaceHorses | -49.8% | -41.8% | -8.0% |
| Average | -51.2% | -49.4% | -1.8% |

- Conclusions

The following are concluded in this contribution.

- BD-rate measures give a very rough indication of the actual performance gains achieved by HEVC.  Sometimes, subjective gains are not visible despite significant objective gains and vice-versa.

  - It is recommended that subjective assessment should be the norm rather than the exception when accessing the performance of the HEVC.

- For the random access coding structure, the formal subjective test confirmed that:

  - HM5.0 HE and HM5.0 LC+RDOQ+SAO at the same bitrate have the same subjective quality.

  - HM5.0 at half the bitrate or less (52% to 36%) has the same subjective quality compared to JM18.2*.

  - References

- F. Bossen., “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G1200, 7th Meeting: Geneva, CH, 21-30 November, 2011.

- Bin Li et. al., “Comparison of Compression Performance of HEVC Working Draft 5 with AVC High Profile”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G399, 8th Meeting: San José, CA, USA, 1–10 February, 2012.

- ITU-T Q6/16 Visual Coding and ISO/IEC JTC1/SC29/WG11 Coding of Moving Pictures and Audio, “Joint Call for Proposals on Video Compression Technology”, MPEG Document N11113, January 2010, Kyoto, JP.

- Vivienne Sze and Vittorio Baroncini “Report on subjective viewing test for configurations in JCTVC-F320”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F772, 6th Meeting: Torino, IT, 14-22 July, 2011.

# Acknowledgment

The authors wishes to thank University of Science and Technology of China and Microsoft Corp for providing the JM18.2*software

# Annex A: Graphs

Figure 1: BD-rate vs. encoding time (HM5.0 cf. JM18.0) for all intra

Figure 2: BD-rate vs. decoding time (HM5.0 cf. JM18.0) for all intra

Figure 3: BD-rate vs. encoding time (HM5.0 cf. JM18.0) for random access

Figure 4: BD-rate vs. decoding time (HM5.0 cf. JM18.0) for random access

Figure 5: BD-rate vs. encoding time (HM5.0 cf. JM18.0) for low delay B

Figure 6: BD-rate vs. decoding time (HM5.0 cf. JM18.0) for low delay B

Figure 7: BD-rate vs. encoding time (HM5.0 cf. JM18.0) for low delay P

Figure 8: BD-rate vs. decoding time (HM5.0 cf. JM18.0) for low delay P

# Annex B: Description of subjective test methods.

## Informal subjective test

This is an informal blind comparison between JM18.0*, HM5.0 HE and LC with minimum tool set which was judged similar quality to HE. It was performed as an initial test to verify the test material selection before committing to a formal subjective test. This informal subjective test is based on JCTVC-F772, Vivienne Sze and Vittorio Baroncini, “Report on subjective viewing test for configurations in JCTVC-F320”).

For this test, following condition was used.

- Monitors:

- EIZO ColorEdge CE210W (21 inch LCD, resolution: 1920x1080) and

- Dell Ultra Sharp U3011 (30 inch LCD, resolution: 2560x1600).

- Viewing condition: Viewing distance was set to about 60cm

- Test material / sequences:

- 9 sequences defined as class B and C in JCTVC-G1200[1]

- Codecs tested:

- JM 18.2* high profile (modified JM18.2 as described in [2])

- HM5.0 high efficiency (HM5.0 HE)

- HM5.0 low complexity with RDOQ and SAO (HM5.0 LC+RDOQ+SAO)

- Encoding parameters:

- Random access coding structure,

- QP=31,34,37,40 for HM5.0

- QP=27,30,33,36 for JM18.2*.

- It was confirmed that the bitrates of HM5.0 HE and LC+RDOQ+SAO range from 35% to 52% (47% on average) of the corresponding JM18.2* bitrate. The majority (30/36 bitstreams) of bitrate difference between HM5.0 HE and LC+RDOQ+SAO is within 2% of the bitrate and the maximum deviation was 3.6%.

- Test method:

- The unit consisting of a pair decoded sequences from 2 of the codecs tested was presented to the observer as sequence A followed by sequence B

- The observer are asked to vote one of the following

  - Sequence A is better than sequence B

  - Sequences A and B are comparable

  - Sequence A is worse than sequence B

## Formal subjective test

This is a formal visual test among JM18.2*, HM5.0 HE and LC with minimum tool set which was judged similar quality to HE. It was performed as a verification of informal subjective test described in 9.1 in Annex B. This formal subjective test is based on the subjective viewing test defined in HVC call for proposal [3].

For this test, following condition was used.

- Monitors:

- TH-50PF11KR(50 inch plasma, resolution: 1920x1080) and

- Viewing condition: Viewing distance was set from 2H to 4H (H is about 60cm).

- Test material / sequences:

- 9 sequences defined as class B and C in JCTVC-G1200[1]

- Codecs tested:

- JM 18.2* high profile (modified JM18.2 based on sequences generated by AHG22)

- HM5.0 high efficiency (HM5.0 HE)

- HM5.0 low complexity with RDOQ and SAO (HM5.0 LC+RDOQ+SAO)

- Encoding parameters:

- Random access coding structure,

- QP=31,34,37,40 for HM5.0

- QP for HM5.0 LC+RDOQ+SAO is selected to achieve bitrate difference between HM5.0 HE and LC+RDOQ+SAO is within 2%.

- QP=27,30,33,36 for JM18.2*.

- It was confirmed that the bitrates of HM5.0 HE and LC+RDOQ+SAO range from 35% to 52% (47% on average) of the corresponding JM18.2* bitrate.

- Test method:

- DSIS (Double Stimulus Impairment Scale) defined in the HVC call for proposal [3] is used

- This method will be used under the schema of evaluation of the quality (and not of the impairment); for this reason a quality rating scale made of 11 levels will be adopted, ranging from "0" (lowest quality) to "10" (highest quality).

- The structure of the Basic Test Cell (BTC) of DSIS method is made by two consecutive presentations of the sequence under test; at first the original version of the sequence is displayed, immediately afterwards the decoded sequence is presented; then a message displays for 5 seconds asking the viewers to vote (see Figure 9). The presentation of the video clips will be preceded by a mid-grey screen displaying for one second.

Figure 9:  DSIS  BTC

Other setting and environment not described here follows the manner described in the HVC call for proposal [3].

# Annex C: Formal subjective test results

This annex provides the plots of the mean opinion score (MOS) for the formal subjective test.

Figure 9: MOS vs. BD-Rate for BQTerrace

Figure 10: MOS vs. BD-Rate for BasketballDrive

Figure 11: MOS vs. BD-Rate for cactus

Figure 12: MOS vs. BD-Rate for Kimono1

Figure 13: MOS vs. BD-Rate for ParkScene

Figure 14: MOS vs. BD-Rate for BQMall

Figure 15: MOS vs. BD-Rate for BasketballDrill

Figure 16: MOS vs. BD-Rate for PartyScene

Figure 17: MOS vs. BD-Rate for RaceHorses