| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0119 |
| --- | --- |

| Title: | Non-CE4 Subtest2 : Extension of Adaptation Parameter Sets syntax for scaling list matrix |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junichi Tanaka Yoshitaka Morigami Teruhiko Suzuki Sony City Osaki, 2-10-1 Osaki, Shinagawa-ku, Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2745 tanaka.junichi@jp.sony.com Yoshitaka.Morigami@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

Scaling list parameter set are defined in APS[1]. APS has a parameter “aps_id” which is referred to in the slice header. In APS, ALF and SAO parameters are also defined. If one of the parameters (ALF / SAO / Scaling Lists) is changed, APS is transmitted. But the frequency of changing scaling list parameter set is different from ALF and SAO parameters. This contribution propose,

- New syntax “scaling_list_reference_aps_id” is defined.

- If aps_scaling_list_data_present_flag equal to 0, scaling_list_reference_aps_id is transmitted.

- “scaling_list_reference_aps_id” refers to the scaling list parameter set that is transmitted in the past.

# Problem

According to APS semantics, aps_scaling_list_data_present_flag equal to 0 specifies that scaling list parameters do not exist in this APS. This is unclear expression. The process for aps_scaling_list_data_present_flag equal to 0 should be clarified. The frequency of changing scaling list parameter set is different from ALF and SAO parameters. Scaling list parameter set should be able to transmit regardless change of SAO or ALF. In AVC, there is a use case of using different scaling list sets for each slice type (P-slice or B-slice). But in current design, it is necessary to send scaling list parameter set in any time when slice type is changed.

Figure 1 : Example of NAL unit structure (current APS)

# Proposal

New syntax “scaling_list_reference_aps_id” is defined. If aps_scaling_list_data_present_flag equal to 0, scaling_list_reference_aps_id is transmitted. “scaling_list_reference_aps_id” refers to the scaling list parameter set that is transmitted in the past. It is possible to transmit scaling list parameter set in less bit.

Figure 2 : Example of NAL unit structure (proposed APS)

Table 1 shows the memory usage of scaling lists. In HEVC, memory usage is 4064 bytes which is 18 times as compared with AVC. In AVC, scaling list parameter syntax is in SPS/PPS. Max size of SPS is 32, and max size of PPS is 256, so max memory usage of scaling lists are 224*(32+256) = 64512 bytes. It is reasonable value that max aps_id is 16, because 64512 / 4064 = 15.87.

Table 1 – Memory usage of scaling lists

| Scaling List Matrix | Memory usage in AVC (byte) | Memory usage in HEVC (byte) |
| --- | --- | --- |
| 4x4 | 96 | 96 |
| 8x8 | 128 | 384 |
| 16x16 | - | 1536 |
| 32x32 | - | 2048 |
| Total | 224 | 4064 |

Proposal syntax and semantics are as follows.

# Syntax

## Adaptation parameter set RBSP syntax

| aps_rbsp( ) { | Descriptor |
| --- | --- |
| aps_id | ue(v) |
| aps_scaling_list_data_present_flag | u(1) |
| aps_sample_adaptive_offset_flag | u(1) |
| aps_adaptive_loop_filter_flag | u(1) |
| if( aps_scaling_list_data_present_flag ) |  |
| scaling_list_param( ) |  |
| else |  |
| scaling_list_reference_aps_id | ue(v) |
| if( aps_sample_adaptive_offset_flag ) |  |
| sao_param( ) |  |
| if( aps_adaptive_loop_filter_flag ) |  |
| alf_param( ) |  |
| aps_extension_flag | u(1) |
| if( aps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| aps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

# Semantics

## Adaptation parameter set RBSP semantics

scaling_list_reference_aps_id refers to scaling list parameter set in the active adaption parameter set. The value of scaling_list_reference_aps_id shall be in the range of 0 to TBD, inclusive.

# Conclusion

In this contribution, the extension of Adaptation Parameter Sets syntax for scaling lists is proposed. It clarifies the process for aps_scaling_list_data_present_flag equal to 0, and it is possible to reduce the redundant data in APS. This contribution proposes,

- New syntax “scaling_list_reference_aps_id” is defined.

- If aps_scaling_list_data_present_flag equal to 0, scaling_list_reference_aps_id is transmitted.

- “scaling_list_reference_aps_id” refers to the scaling list parameter set that is transmitted in the past.

# Reference

- “JCT-VC break-out report: Adaptation Parameter Set issues”
	JCTVC-G1016, Geneva, November 2011

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).