| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC- H0120-v3 WG11 number: m22992 |
| --- | --- |

| Title: | Non-CE6: Improvement of Intra DC prediction for cases without neighbouring samples |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alberto Dueñas, Francisco Javier Roncero, Oscar Patino and Adam Malamy 2315 N. First Street, San Jose, CA 95131 USA | Tel: Email: | +1-408-943-7150 alberto@cavium.com kroncero@cavium.com oscar.patino@cavium.com adam.malamy@cavium.com |
| Source: | Cavium Inc. |  |  |

_____________________________

# Abstract

This contribution proposes an improved intra prediction process for the cases where no neighbouring samples are available. This contribution includes adding new elements to the syntax and a decoding process that will support this new Intra prediction method.

- Problem Statement

On the current WD5[1] in the cases where intra prediction is required for a CU where no neighbouring samples are available, value of 128 for all neighbors is used to perform the intra prediction.  We may define this as a 128-value prediction. The usage of the current 128-value prediction may cause a large error that in turn may cause both, a noticeable visually artifact, and a very noticeable coding efficiency lost in some occasions.

This kind of prediction without neighbouring samples will be used among others in the following scenarios:

- Slice/Tile boundary in Intra frames.

- Constrained intra prediction for isolated CU (no intra neighbors available).

- Top-Left CU Frame.

We must note that 1) and 2) are the most critical scenarios for this problem to happen. Examples of the visual artifact can be seen in the image below:

Original WD5:

Propose Contribution:

Marked zones correspond to an Intra prediction with no neighbors for both cases current and contribution. The sequence was encoded at QP 40 in Low Delay P Low complexity conditions forcing scenario 3) described above.

The severity of the artifact and the coding efficiency will depend on:

- Type of video processed:

  - The less texture on the video sequence the more severe this artifact will be.

  - The more different the 128-Value prediction than the real texture is, the less efficient will be 128-Value prediction.

- QP used for processing:

  - The higher the QP, the more likely this artifact will appear.

  - The lower the QP, the higher the cost of 128-Value prediction coding efficiency lost.

- CU size used for processing:

  - The larger the CU the more severe this artifact will be.

  - The larger the CU the higher the cost of 128-Value prediction coding efficiency lost.

The artifacts become more annoying when displaying changing video compared with static images.

Based on the experimental results, low texture content leads to a more obvious presence of this type of artifacts. In addition to this, it may lead to the artifact not being visible in some cases as the 128-value prediction may produce positive results in some occasions. However, the proposed algorithm will deliver BD-Rate gains compared with only using the 128-value prediction.

As well as the capability of removing the artifact described in some cases, in the end this proposal provides a more efficient Intra prediction for the no neighboring cases. Thus a better performance in terms of BD-Rate may be expected and has been verified with the results.

Due to the way it is implemented (only plays in mode decision in no neighboring cases), none or not significant impact will be expected for normal neighboring cases.

# Algorithm description

The current contribution proposed is to add an intra_DC_X_value parameter to the syntax (where X = luminance, Cb and Cr) that could be used on the cases where the current 128-value prediction is currently utilized to perform intra prediction. The decoding process of the DC value where no neighbouring samples are available will change in the following way: In the relevant cases it will modify the current assumption that the DC value is 128 by the values mentioned that will be transmitted as a new bit-stream element.

This proposal implies the following new main tasks in the bitstream generation process:

- Adding the new syntax elements – intra_DC_X_value – to the bitstream for the cases described. Fully described in Appendix A.

- The calculation of the value for intra_DC_X_value during the prediction process. Proposal in appendix B.

Both steps are assumed to have a low implementation cost.

## Derivation process for luma intra prediction mode

There are two possibilities to perform this process. In either case, both of them try to minimize the impact of the proposal in the bit-stream (bit rate performance):

- Using the “not used” Intra prediction mode 35 for luma to signal the special Intra_DC prediction proposed enables new syntax elements intra_DC_X_value that must be expected in the bit-stream.

Table 8-1 – Specification of intra prediction mode and associated names

| Intra prediction mode | Associated names |
| --- | --- |
| 0 | Intra_Planar |
| 1 | Intra_Vertical |
| 2 | Intra_Horizontal |
| 3 | Intra_DC |
| Otherwise (4..34) | Intra_Angular |
| 35 | Intra_FromLuma (used only for chroma) |

- Using a new flag to signal the intra DC special prediction. When this flag is enabled new syntax elements intra_DC_X_value must be expected in the bit-stream

Note that in both cases:

- The special Intra_DC prediction must be expected for both luma and chroma. The intra_chroma_pred_mode syntax element can be omitted in the bit-stream, since if special Intra_ DC prediction is signaled for luma, the same prediction mode uses the corresponding intra_DC_X_value could be assumed.

- With minimum changes and similar impact in the bit-stream, we could make independent the use of the special Intra_DC prediction for luma and chroma.

- For any purpose, sending the intra luma prediction (if mode 1) is used), this special Intra_DC prediction will be considered as Intra_DC (mode 3, for both luma and chroma also if 1)), then nothing changes about any other tool that could depend on the intra prediction mode.

This initial proposal considers case a) for the moment.

In section 7.1 (Appendix A) can be found a detailed description of this process.

## Derivation process for chroma intra prediction mode

There are no modifications in this process. When the special Intra_DC prediction proposed is used, we only have to signal in the bitstream the usual Intra_DC mode for chroma. Decoder will use the corresponding DC value depending on the Intra_DC mode signaled for luma:

- New intra_DC_Cx_value syntax elements if special Intra_DC prediction is signaled.

- DC value calculated as usual if special Intra_DC is not signaled.

## Intra sample prediction process

Current implementation calculates DCVal as:

DCVal = x'=0nS−1p[x', −1]+y'=0nS−1p[−1, y']+nS>>(k+1), with x, y = 0..nS-1

The proposal will use the new syntax elements intra_DC_X_value (X = luma, Cb and Cr).

# Experimental results

The experimental results has been obtained by modifying the common conditions and software reference configurations[2] and adding especial encoding configuration that will create more cases where no neighbouring samples are available.

Two new configurations are being tested:

- Low delay P LC + Force Intra Refresh + Constrained Intra. HM has been modified to enforce the usage of Intra LCUs (10% of total LCUs in a frame), a LCU is considered Intra at regular intervals every 10 LCUs, leading to 10% of the LCUs on the P frames being Intra predicted. By enabling “ConstrainedIntraPred” we simulate one of the conditions described above for this proposal to improve the performance. We also modify the current HM5 force Intra LCUs and to use “ConstrainedIntraPred”.

- All Intra LC + Slices. HM has been modified to force slices boundaries (slices of 10 LCUs in size are used in this case). This will be the other scenario where the proposal will improve performance.

Performance results are also provided for the unmodified common conditions and software reference configurations:

- Low delay P LC

- All Intra LC

The table below shows results:

Test A):

|  | Low Delay P LC +Force Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | - | - | - |
| Class B | -0.8 | -3.8 | -2.7 |
| Class C | -2.3 | -7.0 | -8.0 |
| Class D | 0.7 | -1.1 | -2.0 |
| Class E | 0.6 | 0.3 | -0.5 |
| Overall | -0.6 | -3.1 | -3.4 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | -- |  |  |

Test B):

|  | All Intra LC + Slices |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | - | - | - |
| Class B | -0.3 | -0.4 | -0.6 |
| Class C | -0.2 | -0.1 | -0.3 |
| Class D | 0.1 | -0.3 | 0.8 |
| Class E | -0.2 | -1.2 | -0.2 |
| Overall | -0.1 | -0.5 | -0.2 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | -- |  |  |

Test C):

|  | Low Delay P LC |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | - | - | - |
| Class B | 0.1 | 0.0 | 0.2 |
| Class C | 0.1 | 0.0 | -0.3 |
| Class D | -0.4 | 0.0 | -0.9 |
| Class E | -1.1 | -0.7 | -1.3 |
| Overall | -1.7 | -1.3 | -1.6 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | -- |  |  |

Test D):

|  | All Intra LC |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | - | - | - |
| Class B | 0.0 | -0.2 | -0.1 |
| Class C | 0.0 | 0.1 | 0.4 |
| Class D | 0.0 | 0.4 | 0.1 |
| Class E | 0.1 | -0.9 | 0.4 |
| Overall | 0.0 | -0.1 | 0.2 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | -- |  |  |

# Conclusions

The proposed algorithm deals with an issue that is producing both noticeable visual degradation and coding efficiency degradation in the cases where no neighbouring samples are available as described on this contribution. The key conclusions from this proposal are:

- Simple encoding/decoding process modifications.

- In case where no neighbouring samples are available:

  - Visual improvement are produced.

  - BD-Rate improvement is produced in spite of adding new syntax elements, since special Intra_DC prediction will be fairly better than common 128-value prediction in coding efficiency terms, so improvement in both texture bits and PSNR performance of the new prediction will be expected.

- In case where neighbouring samples are available, there no impact since the new special Intra_DC prediction will not be signaled on the bitstream.

This proposal proposes a new procedure that has improvement in both visual quality and BD-Rate terms on the cases where no neighbouring samples are available for some cases and it does not have any significant effect on the rest of the cases.

## Pending work

As commented, conditions of experimental results may be acceptable but are not complaint with reference tests. So some work is pending:

- Final HM Code implementation (for both encoder and decoder) selecting the best way to mark the special Intra_DC prediction mode. Taking into account the open issues:

  - Select the best way to mark the special Intra_DC prediction mode.

  - Decide if force same mode for chroma.

- Final Reference Tests. Including the rest of reference scenarios, only a subset has been used on this contribution.

# References

- “WD5: Working Draft 5 of High-Efficiency Video Coding”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC- G1103, 7th Meeting: Geneva, CH, November 2011.

- F. Bossen, “Common conditions and software reference configurations,” in JCTVC-G1200, 7th Meeting: Geneva, CH, November 2011.

# Patent rights declaration(s)

Cavium Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix A – Normative descriptions to be reflected to WD text

## intra_DC_X_value systax element addition

Changes to Section 7.3.7 on prediction unit syntax:

### Using “not used” Intra prediction mode 35 for luma

From the current syntax and derivation process for luma intra prediction mode (section 8.3.1), the way to mark the new Intra_DC prediction derivation process will be:

- prev_intra_luma_pred_flag[ x0 ][ y0 ] = 0.

- rem_intra_luma_pred_mode[ x0 ][ y0 ] = 33.

From the normal derivation process in 8.3.1 (steps “6 + Otherwise”) the IntraPredMode[xB][yB] will be 35. The algorithm requires at this point:

- Creates a IntraDCValueFromStream that will be used in the specification of Intra_DC prediction mode (See section 7.3).

- Override IntraPredMode[xB][yB] = 3. So this special mode will be treated as a normal Intra_DC prediction mode for the rest of decoding processing (See section 7.2).

- intra_chroma_pred_mode will not be read, since same special Intra_DC  prediction is assumed for chroma to use the new syntax elements intra_DC_Cx_value.

| prev_intra_luma_pred_flag[ x0 ][ y0 ] | ae(v) |
| --- | --- |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| mpm_idx[ x0 ][ y0 ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ae(v) |
| if(IntraPredMode[ xB ][ yB ] == 35) { |  |
| intra_DC_luma_value | ae(v) |
| intra_DC_Cb_value | ae(v) |
| intra_DC_Cr_value | ae(v) |
| } else { |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| SignaledAsChromaDC = ( chroma_pred_from_luma_enabled_flag ? intra_chroma_pred_mode[ x0 ][ y0 ] == 3 : intra_chroma_pred_mode[ x0 ][ y0 ] == 2 ) |  |

### New flag to signal special Intra_DC prediction

Current syntax and derivation process for luma intra prediction mode (section 8.3.1), the way to mark the new Intra_DC prediction derivation process will be adding the new flag special_intra_dc_pred_mod.

From the normal derivation process in 8.3.1 (steps “6 + Otherwise”) the IntraPredMode[xB][yB] will be 3, this is, as usual. Before reading rem_intra_luma_pred_mode. The new flag will be read. If this flag is enabled new syntax elements intra_DC_X_value will be read. As in previous mode, intra_chroma_pred_mode will not be read, since same special Intra_DC  prediction is assumed for chroma to use the new syntax elements intra_DC_Cx_value.

| prev_intra_luma_pred_flag[ x0 ][ y0 ] | ae(v) |
| --- | --- |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| mpm_idx[ x0 ][ y0 ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ae(v) |
| if(special_intra_dc_pred_mod) { |  |
| intra_DC_luma_value | ae(v) |
| intra_DC_Cb_value | ae(v) |
| intra_DC_Cr_value | ae(v) |
| } else { |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| SignaledAsChromaDC = ( chroma_pred_from_luma_enabled_flag ? intra_chroma_pred_mode[ x0 ][ y0 ] == 3 : intra_chroma_pred_mode[ x0 ][ y0 ] == 2 ) |  |

## Derivation process for luma intra prediction mode

Changes to Section 8.3.1

### Using “not used” Intra prediction mode 35 for luma

Table 8-2 – Specification of intra prediction mode and associated names

| Intra prediction mode | Associated names |
| --- | --- |
| 0 | Intra_Planar |
| 1 | Intra_Vertical |
| 2 | Intra_Horizontal |
| 3 | Intra_DC |
| Otherwise (4..34) | Intra_Angular |
| 35 | Intra_DC special prediction (for luma() Intra_FromLuma (for chroma) |

- IntraPredMode[ xB ][ yB ] is derived by applying the following procedure:

- If prev_intra_pred_flag[ xB ][ yB ] is true, the IntraPredMode[ xB ][ yB ] is set equal to  candModeList[ mpm_idx ][ xB ][ yB ]]

- Otherwise IntraPredMode[ xB ][ yB ] is derived by applying the following ordered steps:

  - IntraPredMode[ xB ][ yB ] = rem_intra_luma_pred_mode

  - When IntraPredMode[ xB ][ yB ] is equal or greater than candModeList[ 0 ], the value of IntraPredMode[ xB ][ yB ] is increased by one

  - When IntraPredMode[ xB ][ yB ] is equal or greater than candModeList[ 1 ], the value of IntraPredMode[ xB ][ yB ] is increased by one

  - If IntraPredMode[ xB ][ yB ] is equal to 35;

    - IntraPredMode[ xB ][ yB ] = Intra_DC.

    - IntraDCValueFromStream = 1.

### New flag to signal special Intra_DC prediction

No changes are needed.

## Specification of Intra_DC prediction mode

Changes to Section 8.3.3.1.5

Inputs to this process are:

–	neighbouring samples p[ x, y ], with x, y = -1..2*nS-1,

–	a variable nS specifying the prediction block size.

–	a variable cIdx specifying the chroma component of the current block.

–	a variable intraDCValueFromStream from derivation process for luma intra prediction.

Output of this process is:

–	predicted samples predSamples[ x, y ], with x, y =0..nS-1.

This intra prediction mode is invoked when intraPredMode is equal to 2.

The values of the prediction samples predSamples[ x, y ], with x, y = 0..nS-1, are derived as the following ordered steps:

- A variable DCVal is derived as:

  - If intraDCValueFromStream is equal to 1:

DCVal = intra_DC_luma_value				if cIdx = 0

DCVal = intra_DC_Cb_value and intra_DC_Cr_value	if cIdx = 1, for each chroma component

  - Otherwise

DCVal = x'=0nS−1p[x', −1]+y'=0nS−1p[−1, y']+nS>>(k+1), with x, y = 0..nS-1	(8-1)
where k=log2(nS)

- Depending on the prediction block size nS and the chroma component index cIdx, the following applies.

- If cIdx is equal to 0 and nS is less than 64, the following applies.

predSamples[ 0, 0 ] = ( 1*p[ -1, 0 ] + 2*DCVal + 1*p[ 0, -1 ] + 2 ) >> 2	(8-2)
predSamples[ x, 0 ] = ( 1*p[ x, -1 ] + 3*DCVal + 2 ) >> 2, with x = 1..nS-1	(8-3)
predSamples[ 0, y ] = ( 1*p[ -1, y ] + 3*DCVal + 2 ) >> 2, with y = 1..nS-1	(8-4)
predSamples[ x, y ] = DCVal, with x, y = 1..nS-1	(8-5)

- Otherwise, the prediction samples predSamples[ x, y ] are derived as

predSamples[ x, y ] = DCVal, with x, y = 0..nS-1

# Appendix B – Proposal for intra_DC_X_value calculation

Average value of the PU will be an optimal proposal.

For example if average value in a plane red PU is:

- For Luma Avg = 60: the value for intra_DC_luma_value = 60 (instead of 128).

- For Cb Avg = 102: the value for intra_DC_luma_value = 102 (instead of 128).

- For Cr Avg = 206: the value for intra_DC_luma_value = 206 (instead of 128).