| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0122 WG11 Number: m22994 |
| --- | --- |

| Title: | Cross-check report for CE2 Subtest 4.4 (JCTVC-H0308) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Youji Shibahara Takahiro Nishi | Tel: Email: | +81-50-3587-2044 shibahara.youji@jp.panasonic.com nishi.takahiro@jp.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution is a report of cross-check result for CE2 subtest 4.4 in JCTVC-H0308.

This cross-check was done for the second version of the software released on January 18th since it was reported by the proponent that the first version released on January 10th was including some bugs. It was reportedly confirmed that an experimental result perfectly matches with the one provided from the proponent in the rate-distortion performance.

Concern on the implementation impact was expressed by the cross-checker since the memory requirement for residual signals will be increased if chroma 32x32 TU were introduced.

- Analysis

In the JCT-VC Geneva meeting, it was agreed not to introduce chroma 32x32 TU in order not to increase the residual buffer size as suggested in JCTVC-G112 [1]. Residual buffer holds three color components of inverse transform output. In the HM5.0, the maximum TU size for luminance is 32x32 and the maximum CU size is 64x64. For the efficient memory usage, typical decoder may be designed in 32x32 unit pipeline. In this design, the residual buffer may hold 32x32 pixel (having 3 components in 4:2:0 color format). On the other hand, if chroma 32x32 TU is introduced, the residual buffer has to hold 64x64 pixels and its size becomes 4 times large. The proposal defined CE2 Subtest 4.4 [2] was planned to evaluate for the cases with/without chroma 32x32TU. This contribution suggests that the group should be aware of the increase of the residual buffer size on Subtest 4.4 (test_id=1).

- 2	Experimental Condition

CE2 subtest 4.4 plans to test on following cases.

- not introduce chroma TU 32x32 (test_id=0)

- introduce chroma TU 32x32 (test_id=1)

This contribution reports the cross check result for 2).

The software to be tested was provided by the proponent and its performance relative to HM5.0 software was checked under the common test condition described in JCTVC-G1200 [3]. The computing platform is Linux 64bits and the executables were compiled on 64-bit Linux with gcc 4.4.3.

# Results

Table 1 show RD performance and execution time for test_id=1.

Table 1: SubTest 4.4 (test_id=1, with Chroma TU32x32)

# Conclusion

Experimental result provided by the proponent for CE2 Subtest 4.4 (test_id=1) was confirmed. Provided source code was reviewed, and none of software implementation issue was found.

As the memory requirement for residual signals will be increased if chroma 32x32 TU were introduced, we suggest to give priority to CE2 Subtest 4.4 (test_id=0) which does not include chroma 32x32 TU rather than CE2 Subtest 4.4 (test_id=1).

# References

- T. Hellman, Y. Yu, “Changing Luma/Chroma Coefficient Interleaving from CU to TU level”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G112, 7th JCT-VC Meeting, Geneva, CH, November 2011.

- X. Zhao, L. Guo, A. Minezawa, A. Ichigaya, S. Ma et al, “CE2: Subset 4.4: Test Results on Harmonization of JCTVC-G283, JCTVC-G442 and JCTVC-G980”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-H0308, San José, CA, February 2012.

- F. Bossen, “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G1200, Geneva, CH, November 2011.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).