| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0123 WG11 Number: m22995 |
| --- | --- |

| Title: | Merge transform_tree and transform_coeff |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Youji Shibahara Takahiro Nishi | Tel: Email: | +81-50-3587-2044 shibahara.youji@jp.panasonic.com nishi.takahiro@jp.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

This contribution proposes to merge transform_tree syntax and transform_coeff syntax for simplification. In the WD5.0, split_transform_flag and coded_block_flag are coded in transform_tree, and transformed coefficients are separately coded in transform_coeff even though the structure of these two trees is identical. An experimental result reportedly shows negligible differences in coding efficiency (in range of 0.001%) and no increase in software runtime relative to HM5.0 for all configurations in the common condition.

# Problem Statement

In the WD5.0 [1], split_transform_flag and coded_block_flag are coded in transform_tree. And then transform coefficient is coded in the transform_coeff syntax which forms a tree structure as same as transform_tree syntax. In the viewpoint of the computational complexity, the twice traversing is just a cost factor. In the viewpoint of the temporary memory, the decoder is necessary to hold the transform_split_flag and the coded_block_flag from transform_tree syntax to transform_coeff syntax. And for the encoder, it is necessary to hold the all transform coefficients in one CU until all coded_block_flag in the CU are decided.

# Solutions

In order to reduce the redundancy of twice traversing of transform tree structure, this contribution proposes a unified tree for the transform syntax. In the proposal, the transform coefficient and deltaQP are moved into the leaf node of the transform_tree syntax and the transform_coeff tree is removed. The summary of the coded flags are shown in table 1 and one example of tree structure is briefly shown in figure 1.

Table 1: summary of the coded syntax elements in transform related tree structures

|  | WD5.0 / HM5.0 |  | Proposed |
| --- | --- | --- | --- |
|  | transform_tree | transform_coeff | transform_tree |
| split_transform_flag |  |  |  |
| coded_block_flag |  |  |  |
| transform coefficient |  |  |  |
| delta QP |  |  |  |

Figure 1: one example of tree structure

# Experimental results

Table 2 shows RD performance and execution time of the software relative to HM version 5.0 software. The simulation is conducted under common condition as specified in JCTVC-G1200 [2]. There is no change of BD-rate (and PSNR-rate) for all configurations and there is no difference of encoder and decoder runtime.

Table 2: The RD performance (anchor: HM version 5.0)

# Conclusion

In this contribution, an unified tree syntax is proposed for coded_block_flag and transformed coefficients. Proposed tree structure can make the parsing process of the syntax simpler. An experimental result shows negligible difference in coding efficiency and software runtime relative to HM5.0. We recommend that this simplification be adopted in HM/CD of HEVC.

# References

- B. Bross, et al., “WD5: Working Draft 5 of High-Efficiency Video Coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G1103_d4, Geneva, CH, November 2011.

- F. Bossen., “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G1200, 7th JCT-VC Meeting, Geneva, CH, November 2011.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Working draft modification

The following shows the modified working draft based on JCTVC-G1103_d4 [1] and the proposed fix on WD ticked #295.

## coding_unit

| coding_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| else if( slice_type != I ¦ ¦ log2CUSize = = Log2MinCUSize ) { |  |
| pred_type | ae(v) |
| x1 = x0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| x2 = x1 − ( ( 1 << log2CUSize ) >> 2 ) |  |
| y2 = y1 − ( ( 1 << log2CUSize ) >> 2 ) |  |
| x3 = x1 + ( ( 1 << log2CUSize ) >> 2 ) |  |
| y3 = y1 + ( ( 1 << log2CUSize ) >> 2 ) |  |
| if( PartMode == PART_2Nx2N ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| } else if( PartMode == PART_2NxN ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x0, y1 , log2CUSize ) |  |
| } else if( PartMode == PART_Nx2N ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x1, y0 , log2CUSize ) |  |
| } else if( PartMode == PART_2NxnU ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x0, y2 , log2CUSize ) |  |
| } else if( PartMode == PART_2NxnD ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x0, y3 , log2CUSize ) |  |
| } else if( PartMode == PART_nLx2N ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x2, y0 , log2CUSize ) |  |
| } else if( PartMode == PART_nRx2N ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x3, y0 , log2CUSize ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x1, y0 , log2CUSize ) |  |
| prediction_unit( x0, y1 , log2CUSize ) |  |
| prediction_unit( x1, y1 , log2CUSize ) |  |
| } |  |
| if( !pcm_flag ) { |  |
| transform_tree( x0, y0, log2CUSize, log2CUSize, 0, 0 ) |  |
| transform_coeff( x0, y0, x0, y0, log2CUSize, log2CUSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |

## transform_tree

| transform_tree( x0, y0, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| if( trafoDepth = = 0 && IntraSplitFlag = = 0 && PredMode != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0]) ) |  |
| no_residual_data_flag | ae(v) |
| if( !no_residual_data_flag ) { |  |
| log2TrafoSize = ( log2TrafoWidth + log2TrafoHeight ) >> 1 |  |
| intraSplitFlag = ( IntraSplitFlag && trafoDepth = = 0 ? 1 : 0 ) |  |
| maxDepth = ( PredMode = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| xBase = x0 − ( x0 & ( 1 << log2TrafoWidth ) ) |  |
| yBase = y0 − ( y0 & ( 1 << log2TrafoHeight ) ) |  |
| if( log2TrafoSize <= Log2MaxTrafoSize && log2TrafoSize > Log2MinTrafoSize && trafoDepth < maxDepth && !intraSplitFlag ) |  |
| split_transform_flag[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| if( log2TrafoSize <= Log2MaxTrafoSize ) { |  |
| firstChromaCbf = ( log2TrafoSize = = Log2MaxTrafoSize ¦ ¦ trafoDepth = = 0 ? 1 : 0 ) |  |
| if( firstChromaCbf ¦ ¦ log2TrafoSize > Log2MinTrafoSize ) { |  |
| if( firstChromaCbf ¦ ¦ cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ] ) { |  |
| readCbf = true |  |
| if( blkIdx = = 3 && log2TrafoSize < Log2MaxTrafoSize ) |  |
| readCbf = cbf_cb[ xBase ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cb[ xBase + ( 1 << log2TrafoWidth ) ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cb[ xBase ][ yBase + ( 1 << log2TrafoHeight ) ][ trafoDepth ] |  |
| if ( !readCbf ) |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth ] = 1 |  |
| else |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } |  |
| if( firstChromaCbf ¦ ¦ cbf_cr[ xBase ][ yBase ][ trafoDepth − 1 ] ) { |  |
| readCbf = true |  |
| if( blkIdx = = 3 && log2TrafoSize < Log2MaxTrafoSize ) |  |
| readCbf = cbf_cr[ xBase ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cr[ xBase + ( 1 << log2TrafoWidth ) ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cr[ xBase ][ yBase + ( 1 << log2TrafoHeight ) ][ trafoDepth ] |  |
| if ( !readCbf ) |  |
| cbf_cr[ x0 ][ y0 ][ trafoDepth ] = 1 |  |
| else |  |
| cbf_cr[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } |  |
| } |  |
| } |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| if( InterTUSplitDirection = = 2 ) { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 1 ) |  |
| y1 = y0 |  |
| x2 = x0 |  |
| y2 = y0 + ( ( 1 << log2TrafoHeight ) >> 1 ) |  |
| x3 = x1 |  |
| y3 = y2 |  |
| } else { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y1 = y0 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| x2 = x1 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y2 = y1 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| x3 = x2 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y3 = y2 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| log2TrafoHeight = log2TrafoHeight + 2 * InterTUSplitDirection − 1 |  |
| log2TrafoWidth = log2TrafoWidth − 2 * InterTUSplitDirection + 1 |  |
| } |  |
| transform_tree( x0, y0, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 0 ) |  |
| transform_tree( x1, y1, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 1 ) |  |
| transform_tree( x2, y2, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 2 ) |  |
| transform_tree( x3, y3, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 3 ) |  |
| } else { |  |
| if( PredMode = = MODE_INTRA ¦ ¦ trafoDepth != 0 ¦ ¦ cbf_cb[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| readCbf = true |  |
| if( blkIdx = = 3 && log2TrafoSize < Log2MaxTrafoSize && PredMode != MODE_INTRA ) |  |
| readCbf = cbf_luma[ xBase ][ yBase ][ trafoDepth ] ¦ ¦ cbf_luma[ xBase + ( 1 << log2TrafoWidth ) ][ yBase ][ trafoDepth ] ¦ ¦ cbf_luma[ xBase ][ yBase + ( 1 << log2TrafoHeight ) ][ trafoDepth ] ¦ ¦ cbf_cb[ xBase ][ yBase ][ trafoDepth – 1] ¦ ¦ cbf_cr[ xBase ][ yBase ][ trafoDepth – 1] |  |
| if ( !readCbf ) |  |
| cbf_luma[ x0 ][ y0 ][ trafoDepth ] = 1 |  |
| else |  |
| cbf_luma[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } |  |
| if( cbf_luma[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cb[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta |  |
| IsCuQpDeltaCoded = 1 |  |
| } |  |
| log2TrafoSize = ( ( log2TrafoWidth + log2TrafoHeight ) >> 1 ) |  |
| log2TrafoSizeC = ( ( log2TrafoSize == Log2MinChromaTrafoSize) ? log2TrafoSize : (log2TrafoSize – 1) |  |
| if ( PredMode = = MODE_INTRA ) { |  |
| scanIdxL = ScanType[ log2TrafoSize – 2 ][ IntraPredMode ] |  |
| scanIdxC = ScanType[ log2TrafoSize – 2 ][ IntraPredModeC ] |  |
| } else { |  |
| scanIdxL = 0 |  |
| scanIdxC = 0 |  |
| } |  |
| if ( cbf_luma[ x0 ][ y0 ][ trafoDepth ] ) |  |
| residual_coding_cabac( x0, y0, log2TrafoSize, trafoDepth, scanIdxL, 0 ) |  |
| if ( log2TrafoSize > 2) { |  |
| if (cbf_cb[ x0 ][ y0 ][ trafoDepth ] ) |  |
| residual_coding_cabac( x0, y0, log2TrafoSizeC, trafoDepth, scanIdxC, 1 ) |  |
| if (cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) |  |
| residual_coding_cabac( x0, y0, log2TrafoSizeC, trafoDepth, scanIdxC, 2 ) |  |
| } else if ( blkIdx = = 3) { |  |
| if (cbf_cb[ xC ][ yC ][ trafoDepth ] ) |  |
| residual_coding_cabac( xC, yC, log2TrafoSizeC, trafoDepth, scanIdxC, 1 ) |  |
| if (cbf_cr[ xC ][ yC ][ trafoDepth ] ) |  |
| residual_coding_cabac( xC, yC, log2TrafoSizeC, trafoDepth, scanIdxC, 2 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

## transform_coeff

| transform_coeff( x0, y0, xC, yC, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| if(cbf_luma[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cb[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta | se(v) ¦ ae(v) |
| IsCuQpDeltaCoded = 1 |  |
| } |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| if( InterTUSplitDirection = = 2 ) { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 1 ) |  |
| y1 = y0 |  |
| x2 = x0 |  |
| y2 = y0 + ( ( 1 << log2TrafoHeight ) >> 1 ) |  |
| x3 = x1 |  |
| y3 = y2 |  |
| } else { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y1 = y0 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| x2 = x1 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y2 = y1 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| x3 = x2 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y3 = y2 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| log2TrafoHeight = log2TrafoHeight + 2 * InterTUSplitDirection − 1 |  |
| log2TrafoWidth = log2TrafoWidth − 2 * InterTUSplitDirection + 1 |  |
| } |  |
| transform_coeff( x0, y0, x0, y0, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 0 ) |  |
| transform_coeff( x1, y1, x0, y0, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 1) |  |
| transform_coeff( x2, y2, x0, y0, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 2) |  |
| transform_coeff( x3, y3, x0, y0, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 3) |  |
| } else { |  |
| log2TrafoSize = ( ( log2TrafoWidth + log2TrafoHeight ) >> 1 ) |  |
| log2TrafoSizeC = ( ( log2TrafoSize == Log2MinChromaTrafoSize) ? log2TrafoSize : (log2TrafoSize – 1) |  |
| if ( PredMode = = MODE_INTRA ) { |  |
| scanIdxL = ScanType[ log2TrafoSize – 2 ][ IntraPredMode ] |  |
| scanIdxC = ScanType[ log2TrafoSize – 2 ][ IntraPredModeC ] |  |
| } else { |  |
| scanIdxL = 0 |  |
| scanIdxC = 0 |  |
| } |  |
| if ( cbf_luma[ x0 ][ y0 ][ trafoDepth ] ) |  |
| residual_coding_cabac( x0, y0, log2TrafoSize, trafoDepth, scanIdxL, 0 ) |  |
| if ( log2TrafoSize > 2) { |  |
| if (cbf_cb[ x0 ][ y0 ][ trafoDepth ] ) |  |
| residual_coding_cabac( x0, y0, log2TrafoSizeC, trafoDepth, scanIdxC, 1 ) |  |
| if (cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) |  |
| residual_coding_cabac( x0, y0, log2TrafoSizeC, trafoDepth, scanIdxC, 2 ) |  |
| } else if ( blkIdx == 3) { |  |
| if (cbf_cb[ xC ][ yC ][ trafoDepth ] ) |  |
| residual_coding_cabac( xC, yC, log2TrafoSizeC, trafoDepth, scanIdxC, 1 ) |  |
| if (cbf_cr[ xC ][ yC ][ trafoDepth ] ) |  |
| residual_coding_cabac( xC, yC, log2TrafoSizeC, trafoDepth, scanIdxC, 2 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |