| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0124 WG11 Number: m22996 |
| --- | --- |

| Title: | CE7: Cross check of Tool 1, Mode dependent secondary transform |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Atsuro Ichigaya Yasuko Sugito NHK Science & Technology Research Laboratories 1-10-11, Kinuta, Setagaya-ku, Tokyo, Japan | Tel: Email: | +81-3-5494-3354 ichigaya.a-go@nhk.or.jp |
| Source: | NHK (Japan Broadcasting Corporation) |  |  |

_____________________________

# Abstract

The contribution reports NHK’s cross check results on mode-dependent secondary transforms (MD-ST) for intra prediction residual, which is reported in JCTVC-H0126. The tool was proposed at the Geneva meeting in Nov. 2011. The proposal applies secondary transform to low-frequency NxN DCT coefficients of 8x8 DCT or larger. Two test cases were conducted. One was to apply 4x4 secondary transform to DCT coefficients. The second was to apply 8x8 secondary transform to DCT coefficients. The experimental results were identical to the results provided by the proponents.

# Mode dependent secondary transform

The secondary transforms are applied to low-frequency NxN DCT coefficients according to intra prediction modes [1]. The MD-ST is applied from 8x8 to 32x32 core transform outputs, DCT. We tested the coding performance of 4x4 and 8x8 secondary transforms. The secondary transforms are implemented by using NxN matrices. The matrices are defined as below.

#if SEC_TR_SIZE ==4

{   123,   -35,    -8,    -3,},

{   -32,  -120,    30,    10,},

{    14,    25,   123,   -22,},

{     8,    13,    19,   126,},

#elif SEC_TR_SIZE ==8

{   123,   -35,    -8,    -3,    -2,    -1,    -1,    -1},

{    32,   120,   -29,   -10,    -5,    -3,    -2,    -1},

{   -14,   -24,  -123,    21,     8,     4,     3,     2},

{     7,    11,    17,   125,   -16,    -7,    -4,    -2},

{    -4,    -7,    -8,   -13,  -126,    13,     5,     3},

{     3,     4,     5,     6,    11,   127,   -10,    -5},

{    -2,    -3,    -3,    -4,    -5,    -9,  -127,     9},

{     2,     2,     3,     3,     3,     5,     8,   128},

#endif

# Cross-checking

## Test conditions

The source code, of which MD-ST to be integrated into HM5.0, was provided from the proponents. Given that the proposal applies only to intra TUs, the cross-checking conditions are AI (All Intra) and RA (Random Access) cases. All experiments were performed on Linux 64 bit OS with gcc-4.1.2 with the same computer systems. Default conditions were according to JCEVC-G1200 [2].The conducted condition were according to JCTVC-G1207, which was CE7 description [3].

## Test results

Table 1 shows the test results of 4x4 MD-ST. Table 2 shows the test results of 8x8 MD-ST.

Table 1-a. Test results of 4x4 MD-ST on AI conditions

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.8% | -0.5% | -0.4% | -1.1% | -0.5% | -0.4% |  |  |  |
| Class B | -0.5% | -0.2% | -0.3% | -0.7% | -0.2% | -0.2% |  |  |  |
| Class C | -0.3% | -0.3% | -0.3% | -0.4% | -0.3% | -0.3% |  |  |  |
| Class D | -0.2% | -0.2% | -0.2% | -0.3% | -0.2% | -0.2% |  |  |  |
| Class E | -0.8% | -0.6% | -0.5% | -1.0% | -0.6% | -0.5% |  |  |  |
| Overall | -0.5% | -0.3% | -0.3% | -0.6% | -0.3% | -0.3% |  |  |  |
|  | -0.5% | -0.3% | -0.3% | -0.6% | -0.3% | -0.3% |  |  |  |
| Class F | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% | -0.2% |  |  |  |
| Enc Time[%] | 101% |  |  | 102% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |  |  |  |

Table 1-b. Test results of 8x8 MD-ST on RA conditions

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.3% | 0.2% | 0.0% | -0.6% | -0.1% | -0.2% | -0.3% | 0.5% | 0.7% |
| Class B | -0.3% | -0.3% | -0.2% | -0.4% | -0.1% | -0.2% | -0.3% | -0.2% | -0.2% |
| Class C | -0.2% | -0.2% | -0.2% | -0.2% | -0.1% | -0.3% |  |  |  |
| Class D | -0.1% | -0.3% | 0.0% | -0.2% | 0.1% | -0.2% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.2% | -0.2% | -0.1% | -0.3% | 0.0% | -0.2% | -0.3% | 0.1% | 0.2% |
|  | -0.2% | -0.2% | -0.1% | -0.3% | 0.0% | -0.2% | -0.3% | 0.2% | 0.3% |
| Class F | -0.1% | -0.2% | -0.3% | -0.1% | -0.2% | -0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 99% |  |  |

Table 2-a. Test results of 8x8 MD-ST on AI conditions

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -1.2% | -0.9% | -0.9% | -1.5% | -1.0% | -0.9% |  |  |  |
| Class B | -0.7% | -0.7% | -0.8% | -1.0% | -0.6% | -0.7% |  |  |  |
| Class C | -0.4% | -0.5% | -0.5% | -0.5% | -0.5% | -0.5% |  |  |  |
| Class D | -0.3% | -0.3% | -0.3% | -0.4% | -0.3% | -0.3% |  |  |  |
| Class E | -1.1% | -1.1% | -1.1% | -1.2% | -1.1% | -1.0% |  |  |  |
| Overall | -0.7% | -0.6% | -0.7% | -0.8% | -0.6% | -0.6% |  |  |  |
|  | -0.7% | -0.6% | -0.7% | -0.8% | -0.6% | -0.6% |  |  |  |
| Class F | -0.1% | -0.1% | -0.2% | -0.1% | -0.3% | -0.4% |  |  |  |
| Enc Time[%] | 103% |  |  | 106% |  |  |  |  |  |
| Dec Time[%] | 104% |  |  | 104% |  |  |  |  |  |

Table 2-b. Test results of 8x8 MD-ST on RA conditions

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.5% | -0.2% | -0.3% | -0.7% | -0.4% | -0.2% | -0.4% | 0.0% | 0.1% |
| Class B | -0.4% | -0.3% | -0.3% | -0.6% | -0.4% | -0.4% | -0.4% | -0.4% | -0.5% |
| Class C | -0.3% | -0.4% | -0.2% | -0.3% | -0.4% | -0.4% |  |  |  |
| Class D | -0.2% | -0.3% | -0.1% | -0.2% | 0.1% | -0.5% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.3% | -0.3% | -0.2% | -0.4% | -0.2% | -0.4% | -0.4% | -0.2% | -0.2% |
|  | -0.3% | -0.3% | -0.2% | -0.4% | -0.3% | -0.4% | -0.4% | -0.2% | -0.2% |
| Class F | -0.1% | -0.2% | -0.2% | -0.1% | -0.3% | -0.3% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  | 100% |  |  |

# Summary

We confirmed the implementation of MD-ST and cross-checked the coding results. In the case of 4x4 MD-ST, the results show 0.5% gain with AIHE, 0.6 % gain with AILC, 0.2 % gain with RAHE, 0.3% gain RALC and 0.3% gain with RAHE-10. The encoding process time was increased by a small percentage and few negligible errors were observed with decoding time. In the case of 8x8 MD-ST, the results show 0.7% gain with AIHE, 0.8 % gain with AILC, 0.3 % gain with RAHE, 0.4% gain RALC and 0.4% gain with RAHE-10. The encoding process time was increased by 3 - 6 % and few negligible errors were observed with decoding time. These results are identical to the result of the proponents. MDST is implemented clear and the results were identical to the results of the proponents.

# Reference

[1] A. Saxena, Y. Shibahara, F. Fernandes and T. Nishi, “CE7: On secondary transforms for inter prediction residual,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-H0126, 8th JCT-VC Meeting, San Jose, USA, Feb. 2012.

[2] F. Bossen, “Common HM test conditions and software reference configurations,” Joint Collaborative Team on Video Coding(JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G1200, 7th JCT-VC Meeting, Geneva, CH, Nov. 2011.

[3] R. Cohen, C. Yeo, R. Joshi and F. Felnandes “Description of Core Experiment 7: Additional Transforms,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G1207, 7th JCT-VC Meeting, Geneva, CH, Nov. 2011.