| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0125 WG 11Number :_m22997 |
| --- | --- |

| Title: | CE7: On secondary transforms for intra prediction residual |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ankur Saxena, Youji Shibahara Felix C. Fernandes Takahiro Nishi | Tel: Email: | 1-9727617761 asaxena@sta.samsung.com, +81-50-3587-2044 shibahara.youji@jp.panasonic.com, felix.f@sta.samsung.com, nishi.takahiro@jp.panasonic.com |
| Source: | Samsung Electronics Co., Ltd & Panasonic |  |  |

_____________________________

# Abstract

It was previously shown by Han, Saxena & Rose in ICASSP 2010, that following intra prediction, the optimal transform is not DCT, but DST Type-7 with performance close to KLT, along the direction of prediction, for the horizontal and vertical modes. The 4x4 DST by Saxena & Fernandes: JCTVC-E125 was adopted in the HEVC Geneva meeting in March 2011. Saxena, Shibahara, Fernandes and Nishi previously had proposed various mode-dependent secondary transforms in JCTVC-G108. This contribution presents the experimental results for these mode-dependent secondary transforms (MD-ST). The proposed MD-ST transform scheme is applied following the DCT output for block sizes 8x8 and higher. No additional signaling information or R-D search is required during the encoding, and the algorithm works in a single-pass. The conventional quantization tables for HM 5.0 are retained and no changes have been made to the scanning order. Various schemes to reduce the latency of secondary transform in hardware are also proposed. Experimental results are provided with HM 5.0 as anchor for the test conditions as stipulated in Core Experiment 7. Average BD Rate gains of upto 0.7 %, 0.8 %, 0.3 % and 0.4% are obtained for Intra High Efficiency, Intra Low Complexity, Random Access High Efficiency, and Random Access Low complexity settings respectively by the proposed MD-ST scheme.

# Introduction

In HM 5.0, mode-dependent DCT/DST scheme from JCTVC-E125 [1] is used at block size 4x4. The mode-dependent DCT/DST scheme was published in literature in [2]. Currently, in Core Experiment 7 [3], mode-dependent secondary transforms (MD-ST) from JCTVC-G108 [4] are being investigated. This contribution presents the results of proposed MD-ST’s, which are used after DCT based on the intra prediction direction in HM 5.0 for Luma and Intra TU only. The proposed MD-ST scheme does not require R-D search, or any additional signaling information.

Simulation results are provided for the proposed MD-ST scheme with HM 5.0 as anchor.

# Mode-Dependent Secondary Transforms

## Application of Secondary Transform Based On Intra-Prediction Modes

Fig. 1 shows the decoder operations when a K-point mode-dependent secondary transform is applied as a row or column transform depending on the intra prediction mode for a block of size NxN. Note that N is typically greater than or equal to K. Extension to rectangular blocks of size N1 x N (for example in Short Distance Intra Prediction) is straightforward [5].

The trigger conditions when the secondary transform is used are shown in the right column in Fig. 1 and depend on the categorization of the intra prediction modes.

Depending on whether the prediction for a block is performed from only from one of the top row or left column, a prediction mode is termed as Category 1 intra prediction mode. In case, the prediction for the block is performed from both the top row and left columns, it is termed as Category 2 prediction mode. More details are in JCTVC-C108 [6], and JCTVC-E125 [1], where the optimality of mode-dependent DCT/DST for 4x4 blocks is shown.

In Fig. 1, when the intra prediction mode is DC mode (a non-directional mode), no secondary transform is applied in horizontal and vertical directions. For category 1 intra prediction modes, if the prediction was performed from pixels only from the top row, i.e., in vertical direction, and the intra prediction modes are one of the VER, VER+1, VER+2, … VER+8, then the secondary transform is used only in the vertical direction (along columns). In case, the prediction was performed from pixels only from the left column, i.e., in horizontal direction, and intra prediction modes are HOR, HOR+1, HOR+2, … HOR+8, then the secondary transform is used only in the horizontal direction (along rows).

Finally for Category 2 intra prediction modes, if the prediction is performed using both the left column and the top row, i.e., intra prediction modes are VER-1, VER-2, … VER-8 or HOR-1, HOR-2, … HOR-7, or if the intra prediction mode is Planar (a non-directional mode), then the secondary transform is applied in both the horizontal and vertical directions. The encoder instantiation is a straightforward inverse of the decoder implementation.

Note that the trigger conditions re-use the existing mapping table in HM5.0 for DCT/DST at 4x4 blocks. For example, for Vertical mode, at size 4x4 DST is used in Vertical direction, and DCT in Horizontal. In the proposed scheme at size 8x8 (or higher), for Vertical mode, before 2-d DCT, a secondary transform is applied only in Vertical Direction (maintaining a correspondence with DCT/DST scheme at 4x4 size).

Trigger Conditions

Decoder Operations

NxN

Inverse DCT

Inverse Quantizer

K-Point Inverse Row

(Horizontal)

Secondary Transform

K-Point

Inverse Column (Vertical)

Secondary

Transform

NxN Transform Unit

when IntraPredMode

= Planar or VER -1,

VER -2, …, VER-8 , or HOR-1,  HOR-2, ….,

HOR-7.

NxN

Inverse DCT

Inverse Quantizer

NxN Transform Unit

when IntraPredMode

= DC Mode

NxN

Inverse DCT

Inverse Quantizer

K-Point Inverse Row

(Horizontal)

Secondary Transform

NxN Transform Unit

when IntraPredMode =

= HOR, HOR+1,

HOR+2,…, HOR+8

NxN

Inverse DCT

Inverse Quantizer

K-Point

Inverse Column (Vertical)

Secondary Transform

NxN Transform Unit

when IntraPredMode

VER, VER+1,

VER +2 , …, VER+8

Fig 1: Example decoder operations for mode-dependent secondary transform

## Mode-Dependent Secondary Transform

We derived secondary transform matrices for sizes 4x4 and 8x8 using an analytical model in JCTVC-G108 [4]. They are given in Appendix 1. The secondary transform matrices operate only on the KxK (K=4 or 8) low-frequency coefficients of the 8x8 (or larger) DCT output. Further details about the model used for deriving secondary transform matrices are given in Appendix 2.

Note that in order to improve the compression efficiency, primary alternate transforms other than conventional DCT can be applied at block sizes 8x8, 16x16 and 32x32. However, these primary transforms will have the same size as the block size. In general, these alternate transforms at higher block sizes such as 32x32 may have marginal gains that may not justify the enormous cost of supporting additional 32x32 transform in the hardware.

Also, since most of the energy in the DCT coefficients is concentrated in the low-frequency coefficients, it should be adequate to perform operations only on the first few coefficients (typically 4 to 8) of DCT output. These operations can be performed by simply using a MD-ST of size KxK (K<=8) as presented above. Furthermore, the same secondary transform derived for a block size (such as 8x8) can be applied at higher block sizes (such as 16x16 or 32x32), while this is not true for alternate primary transform. This re-utilization of MD-ST at higher block sizes is one of the most important advantages of secondary transform.

# Simulation Results

The test conditions as stipulated in Core Experiment 7 [3] were followed for conducting the Experiments. Here the anchor was HM 5.0 with the conventional DCT at block sizes 8x8 and larger. The performance of the proposed 4x4 and 8x8 MD-ST transforms was evaluated for the following settings: Intra High Efficiency, Intra Low Complexity, Random Access High Efficiency, Random Access-High Efficiency 10 bits, and Random Access Low Complexity. The simulations were performed in a Linux 64 bit environment. Additional results for the Low Delay-B configurations are also provided.

Tables 1 and 2 show the summary of simulation results (average BD-Rates over Class A to E, and encoding/decoding times relative to HM 5.0 anchor) for All Intra (HE and LC), Random Access (HE, HE-10 bit and LC), and Low Delay-B (HE and LC) settings when the proposed 8x8 and 4x4 MD-ST scheme was used respectively. Detailed results are available in the attached Excel sheets.

Table 1. Performance Summary of 8x8 secondary transform

|  |  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
|  | Class A (8bit) | -1.2% | -0.9% | -0.9% | -1.5% | -1.0% | -0.9% |
|  | Class B | -0.7% | -0.7% | -0.8% | -1.0% | -0.6% | -0.7% |
|  | Class C | -0.4% | -0.5% | -0.5% | -0.5% | -0.5% | -0.5% |
|  | Class D | -0.3% | -0.3% | -0.3% | -0.4% | -0.3% | -0.3% |
|  | Class E | -1.1% | -1.1% | -1.1% | -1.2% | -1.1% | -1.0% |
|  | Overall | -0.7% | -0.6% | -0.7% | -0.8% | -0.6% | -0.6% |
|  |  | -0.7% | -0.6% | -0.7% | -0.8% | -0.6% | -0.6% |
|  | Class F | -0.1% | -0.1% | -0.2% | -0.1% | -0.3% | -0.4% |
|  | Enc Time[%] | 103% |  |  | 105% |  |  |
|  | Dec Time[%] | 102% |  |  | 102% |  |  |
|  |  | Random Access HE |  |  | Random Access LC |  |  |
|  |  | Y | U | V | Y | U | V |
|  | Class A (8bit) | -0.5% | -0.2% | -0.3% | -0.7% | -0.4% | -0.2% |
|  | Class B | -0.4% | -0.3% | -0.3% | -0.6% | -0.4% | -0.4% |
|  | Class C | -0.3% | -0.4% | -0.2% | -0.3% | -0.4% | -0.4% |
|  | Class D | -0.2% | -0.3% | -0.1% | -0.2% | 0.1% | -0.5% |
|  | Class E |  |  |  |  |  |  |
|  | Overall | -0.3% | -0.3% | -0.2% | -0.4% | -0.2% | -0.4% |
|  |  | -0.3% | -0.3% | -0.2% | -0.4% | -0.3% | -0.4% |
|  | Class F | -0.1% | -0.2% | -0.2% | -0.1% | -0.3% | -0.3% |
|  | Enc Time[%] | 100% |  |  | 100% |  |  |
|  | Dec Time[%] | 100% |  |  | 101% |  |  |
|  |  | Low delay B HE |  |  | Low delay B LC |  |  |
|  |  | Y | U | V | Y | U | V |
|  | Class A |  |  |  |  |  |  |
|  | Class B | -0.2% | -0.2% | -0.1% | -0.2% | -0.7% | -0.6% |
|  | Class C | -0.2% | -0.4% | -0.2% | -0.2% | -0.2% | -0.3% |
|  | Class D | -0.1% | -0.6% | -0.3% | -0.2% | 0.1% | -0.3% |
|  | Class E | 0.2% | -0.2% | -1.0% | 0.2% | -1.7% | -0.5% |
|  | Overall | -0.1% | -0.4% | -0.3% | -0.1% | -0.5% | -0.4% |
|  |  | -0.1% | -0.3% | -0.3% | -0.1% | -0.6% | -0.4% |
|  | Class F | -0.2% | -0.4% | -0.8% | 0.0% | -0.5% | -0.6% |
|  | Enc Time[%] | 100% |  |  | 101% |  |  |
|  | Dec Time[%] | 101% |  |  | 102% |  |  |

|  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- |
|  |  | Y | U | V |
|  | Class A (8bit) | -0.4% | 0.0% | 0.1% |
|  | Class B | -0.4% | -0.4% | -0.5% |
|  | Class C |  |  |  |
|  | Class D |  |  |  |
|  | Class E |  |  |  |
|  | Overall | -0.4% | -0.2% | -0.2% |
|  |  | -0.4% | -0.2% | -0.2% |
|  | Class F |  |  |  |
|  | Enc Time[%] | 100% |  |  |
|  | Dec Time[%] | 101% |  |  |

Table 2. Performance Summary of 4x4 secondary transform

|  |  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
|  | Class A (8bit) | -0.8% | -0.5% | -0.4% | -1.1% | -0.5% | -0.4% |
|  | Class B | -0.5% | -0.2% | -0.3% | -0.7% | -0.2% | -0.2% |
|  | Class C | -0.3% | -0.3% | -0.3% | -0.4% | -0.3% | -0.3% |
|  | Class D | -0.2% | -0.2% | -0.2% | -0.3% | -0.2% | -0.2% |
|  | Class E | -0.8% | -0.6% | -0.5% | -1.0% | -0.6% | -0.5% |
|  | Overall | -0.5% | -0.3% | -0.3% | -0.6% | -0.3% | -0.3% |
|  |  | -0.5% | -0.3% | -0.3% | -0.6% | -0.3% | -0.3% |
|  | Class F | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% | -0.2% |
|  | Enc Time[%] | 101% |  |  | 102% |  |  |
|  | Dec Time[%] | 99% |  |  | 102% |  |  |
|  |  | Random Access HE |  |  | Random Access LC |  |  |
|  |  | Y | U | V | Y | U | V |
|  | Class A (8bit) | -0.3% | 0.2% | 0.0% | -0.6% | -0.1% | -0.2% |
|  | Class B | -0.3% | -0.3% | -0.2% | -0.4% | -0.1% | -0.2% |
|  | Class C | -0.2% | -0.2% | -0.2% | -0.2% | -0.1% | -0.3% |
|  | Class D | -0.1% | -0.3% | 0.0% | -0.2% | 0.1% | -0.2% |
|  | Class E |  |  |  |  |  |  |
|  | Overall | -0.2% | -0.2% | -0.1% | -0.3% | 0.0% | -0.2% |
|  |  | -0.2% | -0.2% | -0.1% | -0.3% | 0.0% | -0.2% |
|  | Class F | -0.1% | -0.2% | -0.3% | -0.1% | -0.2% | -0.1% |
|  | Enc Time[%] | 101% |  |  | 100% |  |  |
|  | Dec Time[%] | 101% |  |  | 102% |  |  |
|  |  | Low delay B HE |  |  | Low delay B LC |  |  |
|  |  | Y | U | V | Y | U | V |
|  | Class A |  |  |  |  |  |  |
|  | Class B | -0.2% | -0.2% | -0.3% | -0.2% | -0.7% | 0.1% |
|  | Class C | -0.1% | -0.3% | -0.1% | -0.1% | -0.1% | -0.3% |
|  | Class D | -0.1% | 0.1% | -0.2% | -0.1% | 0.4% | -0.5% |
|  | Class E | 0.1% | 1.0% | -0.2% | 0.1% | -0.4% | -0.8% |
|  | Overall | -0.1% | 0.1% | -0.2% | -0.1% | -0.2% | -0.3% |
|  |  | -0.1% | 0.0% | -0.2% | -0.1% | -0.2% | -0.4% |
|  | Class F | -0.3% | -0.6% | -0.6% | -0.1% | 0.0% | -0.6% |
|  | Enc Time[%] | 100% |  |  | 100% |  |  |
|  | Dec Time[%] | 100% |  |  | 100% |  |  |

|  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- |
|  |  | Y | U | V |
|  | Class A (8bit) | -0.3% | 0.5% | 0.7% |
|  | Class B | -0.3% | -0.2% | -0.2% |
|  | Class C |  |  |  |
|  | Class D |  |  |  |
|  | Class E |  |  |  |
|  | Overall | -0.3% | 0.1% | 0.2% |
|  |  | -0.3% | 0.2% | 0.3% |
|  | Class F |  |  |  |
|  | Enc Time[%] | 100% |  |  |
|  | Dec Time[%] | 100% |  |  |

From the above experimental results, average BD rate gains are 0.7%, 0.8 %, 0.3 % and 0.4 % are obtained for Intra High Efficiency, Intra Low complexity, Random Access High Efficiency and Random Access Low Complexity settings for the 8x8 secondary transforms. For the 4x4 secondary transform, the gains in Intra HE, LC, Random Access HE and Random Access LC configurations are 0.5 %, 0.6%, 0.2 % and 0.3 %. The additional encoding/decoding time due to the introduction of secondary transforms is negligible.

## Qualitative Results

We did not notice any subjective differences in the visual quality of the coded sequences.

## Training of Matrices

The proposed secondary transforms MD-ST’s were derived analytically. No training was performed on any video sequence.

## Complexity

Quantization & Scaling Matrices: We retain the same quantization and inverse quantization matrices as used in HM 5.0.

Transform Matrices: For a KxK MD-ST (K=4 or 8), we only require the storage of one additional KxK matrix.

Bit Shift at Intermediate Levels: In our implementation, we shift the output of MD-ST by 7 bits after vertical or horizontal transform.

The bit-precision for all the transforms is 16 bits, similar to in HM 5.0.

Scanning: In this proposal we have retained the conventional HM 5.0 scans

Table 3. Worst-Case Inverse transform operations count summary for the mode-dependent secondary transforms

| Transform Unit size |  | HM 5.0 DCT | HM 5.0 DST | 4x4 Secondary Transform | 8x8 Secondary Transform | Percentage Increase for HM DST over HM DCT | Percentage increase for 4x4 Secondary Transform Over HM DCT | Percentage increase for 8x8 Secondary Transform Over HM DCT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4x4 | Mults | 48 | 64 | 0 | 0 | 33.3 | N/A | N/A |
|  | Adds | 96 | 120 | 0 | 0 | 25 | N/A | N/A |
|  | Shifts | 32 | 32 | 0 | 0 | 0 | N/A | N/A |
|  | Clips | 32 | 32 | 0 | 0 | 0 | N/A | N/A |
| 8x8 | Mults | 352 | N/A | +128 | +1024 | N/A | 36.4 | 290.9 |
|  | Adds | 576 | N/A | +128 | +1024 | N/A | 22.2 | 177.8 |
|  | Shifts | 128 | N/A | +32 | +128 | N/A | 25.0 | 100.0 |
|  | Clips | 128 | N/A | +32 | +128 | N/A | 25.0 | 100.0 |
| 16x16 | Mults | 2752 | N/A | +128 | +1024 | N/A | 4.7 | 37.2 |
|  | Adds | 3712 | N/A | +128 | +1024 | N/A | 3.4 | 27.6 |
|  | Shifts | 512 | N/A | +32 | +128 | N/A | 6.3 | 25.0 |
|  | Clips | 512 | N/A | +32 | +128 | N/A | 6.3 | 25.0 |
| 32x32 | Mults | 21888 | N/A | +128 | +1024 | N/A | 0.6 | 4.7 |
|  | Adds | 25856 | N/A | +128 | +1024 | N/A | 0.5 | 4.0 |
|  | Shifts | 2048 | N/A | +32 | +128 | N/A | 1.6 | 6.3 |
|  | Clips | 2048 | N/A | +32 | +128 | N/A | 1.6 | 6.3 |

Table 4. Average-Case Inverse transform operations count summary for the mode-dependent secondary transforms

| Transform Unit size |  | HM 5.0 DCT | HM 5.0 Average for 4x4 blocks which use DCT and DST | 4x4 Secondary Transform | 8x8 Secondary Transform | Percentage Increase for HM DST over HM DCT | Percentage increase for 4x4 Secondary Transform Over HM DCT | Percentage increase for 8x8 Secondary Transform Over HM DCT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4x4 | Mults | 48 | 60 | 0 | 0 | 24.8 | N/A | N/A |
|  | Adds | 96 | 114 | 0 | 0 | 18.6 | N/A | N/A |
|  | Shifts | 32 | 32 | 0 | 0 | 0.0 | N/A | N/A |
|  | Clips | 32 | 32 | 0 | 0 | 0.0 | N/A | N/A |
| 8x8 | Mults | 352 | N/A | +91 | +731 | N/A | 26.0 | 207.8 |
|  | Adds | 576 | N/A | +91 | +731 | N/A | 15.9 | 127.0 |
|  | Shifts | 128 | N/A | +23 | +91 | N/A | 17.9 | 71.4 |
|  | Clips | 128 | N/A | +23 | +91 | N/A | 17.9 | 71.4 |
| 16x16 | Mults | 2752 | N/A | +91 | +731 | N/A | 3.3 | 26.6 |
|  | Adds | 3712 | N/A | +91 | +731 | N/A | 2.5 | 19.7 |
|  | Shifts | 512 | N/A | +23 | +91 | N/A | 4.5 | 17.9 |
|  | Clips | 512 | N/A | +23 | +91 | N/A | 4.5 | 17.9 |
| 32x32 | Mults | 21888 | N/A | +91 | +731 | N/A | 0.4 | 3.3 |
|  | Adds | 25856 | N/A | +91 | +731 | N/A | 0.4 | 2.8 |
|  | Shifts | 2048 | N/A | +23 | +91 | N/A | 1.1 | 4.5 |
|  | Clips | 2048 | N/A | +23 | +91 | N/A | 1.1 | 4.5 |

Remark: From the above table, we see that the worst-case increase in the complexity of the 4x4 and 8x8 secondary transform is 36.4 % and 290.9 %. Note that the gains for Random Access case, the 4x4 and 8x8 secondary transform schemes in JCTVC-H0126 are providing upto 0.7 % and 0.8 % gain respectively on average for the High Efficiency and Low Complexity settings in HM 5.0, while the 4x4 DST adopted in March 2011 provided 0.3 % on average gain in Random Access High Efficiency case at an almost additional worst-case complexity of 33.3 % (RA-LC setting results for 4x4 DST from E-125 are not reported because CAVLC was used).

Note that the worst-case complexity metrics should not be directly compares with average case gains, which are actually for the whole video codec. In fact, the transforms take a very nominal part of the whole video codec, especially for the inter case, and the additional complexity for the whole video codec by the introduction of new transform techniques could be gauged by the increase in encoding/decoding times.

## Techniques for reducing latency for secondary transform in hardware

In general, a secondary transform after a primary transform (e.g., DCT) would require some additional cycles for execution. This can pose latency overhead in the encoder/decoder, and throughput may decrease. To minimize this latency, we propose the following scheme in the context of 2-d transforms, and first consider the worst case scenario above in Fig. 1, when the mode-dependent secondary transform is applied along both the horizontal and vertical directions.

Note that what we present next is just an example of reducing latency for secondary transforms. Better implementations in hardware to reduce latency would exist, and can be employed in conjunction with the technique presented next.

For brevity, we show the analysis for forward transform at encoder. The analysis for the inverse transform is similar to being presented for the forward transform operations.

We assume that the NxN horizontal DCT is performed followed by the NxN vertical DCT at the encoder. Let C be the NxN DCT, X be the NxN input block, and S be the KxK secondary transform. The mathematical operations that need to be carried out are:

Y = CT * X * C   ;

and

Z(1:K,1:K)  =  ST * Y (1:K,1:K)  * S

Z(K+1:N,K+1:N)   =   Y(K+1:N,K+1:N)  ;

Z(1:K,K+1:N)   =   Y(1:K,K+1:N)  ;

Z(K+1:N,1:K)   =   Y(K+1:N,1:K)  ;

where Y is an intermediate NxN matrix, and Z is the output NxN transformed matrix (after DCT and secondary transform). Note that the above equations simply means that the KxK low frequency coefficients of Y are multiplied by the secondary transform which is pictorially shown in Fig. 4.

Fig. 4: Low-frequency components of a DCT transformed block

For vertical DCT to begin, the operations for all the N rows of horizontal DCT should finish. For example for the 8x8 DCT, we have:

i.e, the rows x1T to x8T of X are multiplied sequentially by the basis vectors of DCT i.e., c1 to c8. In the first clock cycle, the processing of x1T begins, i.e., x1T is multiplied by c1 to c8, to obtain the first row of X*C. This is finished by L = (1-1) +L cycles, where L denotes the latency for DCT.

At the beginning of 2nd clock cycle, processing of x2T starts to obtain the 2nd row of X*C. This finishes by 1+L = (2-1) + L cycles. Finally at the end of (8-1)+ L clock cycles, the 8 rows of horizontal DCT are obtained. To generalize, for a N-point DCT transform, it takes N+L-1 cycles to complete.

Once the horizontal DCT finishes, we need to perform the vertical DCT, i.e.,

This will take another 8 + L -1 cycles (the vertical DCT can be carried out step by step for the 1st column of the above matrix, followed by 2nd column and so on). Hence, in general a 2-d N-point DCT will require 2* (N+L-1) cycles for the horizontal and vertical transform.

Similarly for the 2-d K-point secondary transform, we assume the number of cycles required would be 2 * (K+M-1) where M is the latency for a row of secondary transform. Note that since the KxK secondary transforms have much lower operation counts, they can also be implemented via a parallel implementation, and the total number of cycles would be even lesser.

In general, the total worst case overhead if the 2-d secondary transform is applied after the 2-d DCT finishes would be:  2 * (K+M-1) /  [2*  (N+L-1)   ]    =   (K+M-1) / (N+L-1).

For example, for N =8, L =1, K =8 and M =1, we have Overhead = 100 %, which might not be acceptable in hardware. For N = 32, L =2, K = 8, M =1, Overhead = 8 / 33 =  24.24 %.

However, to reduce this overhead, a secondary transform can be applied for the rows/columns immediately after the DCT is over. Specifically, we propose to use the following transform order logic 1:

- Horizontal (Hor) DCT

- Vertical (Vert) DCT

- Vertical Secondary Transform

- Horizontal Secondary Transform

In fact, by using the above logic, as soon as the 1st column of the Vertical DCT is processed, the processing of 1st column of Vertical Secondary transform can begin via a “pipelined” architecture. Such flexibility would not have been available if the horizontal secondary transform (ST) was required to be taken after the Vertical DCT. In that case, we would need to wait until all the N rows of vertical DCT have been processed.

Assuming N >> K (e.g., N =32, and K =8), if we were to draw a timing diagram for the operations of Vertical DCT (which begins after N+L-1 cycles), Vertical ST, and Horizontal ST, it would look like:

Fig. 5: Timing diagram for the operations of DCT and secondary transform

In the above figure, the row corresponding to Hor DCT shows the time when a particular row of DCT finishes. For example, the 1st row of Hor DCT finishes after L clock cycles, and row N finishes after (N+L-1) cycles. Similarly, for Vertical DCT, the 1st column finishes at (N+L-1) + L cycles. Exactly at this point, the processing of the secondary transform for the first column of the secondary transform can begin. This gets over within additional M cycles at time (N+L-1)+ L + M.

The vertical ST gets over at (N+L-1) + L + M + K-1 as shown above and now the horizontal ST can begin. This finishes in another M+K-1 cycles. In this way, the total time for the Vertical and Horizontal ST to finish is

TST     =   (N+L-1) + L + 2*(M+K-1) cycles.

For 2-d DCT alone, the time was:

TDCT  = 2* (N+L-1) cycles

Therefore additional cycles due to the introduction of ST are:

TST  - TDCT  =  L + 2*(M+K-1)  -  (N+L-1)

= 2K + 2M -1 - N

Also, note that the secondary transform can sometimes finish before the DCT itself (e.g., when N = 32, K =8). In that case, TST  <  TDCT , and hence there is no overhead. So, we can write the above formula for additional cycles as :

Additional Cycles  =    max (0, 2K+2M-1-N)

Therefore overhead    = max (0,2K+2M-1-N) / TDCT

= max (0,2K+2M-1-N) / [2*(N+L-1)]

Examples: For the 32x32 DCT, when N=32, the overhead latency for secondary transform at size K=8 is (assuming M = L =1): max (0,-15)/[2*32]   =  0. Note that the values M=L=1 are just representative. Similarly for other sizes K, overhead = 0.

For N = 16, if K<=7, and M=1, then max (0,2K + 2-1-16)  <= max (0,-1). Therefore for secondary transforms of size upto 7x7, there is no overhead. For the 8x8 secondary transform, the overhead is 1/32 = 3.125 %.

For N =8, assuming L=M=1, total number of cycles for DCT is 2* (8+1-1) = 16. For the secondary transform, we have:

Table 5: Additional cycle table for various secondary transform sizes when DCT size is 8

| K (Secondary Transform Size) | Additional Cycles = max (0,2K+2M-1-N) = max (0,2K-7) | Overhead in Percentage = Extra secondary transform cycles/Cycles for 2-d DCT |
| --- | --- | --- |
| 2 | 0 | 0 |
| 3 | 0 | 0 |
| 4 | 1 | 6.25 |
| 5 | 3 | 18.75 |
| 6 | 5 | 31.25 |
| 7 | 7 | 43.75 |
| 8 | 9 | 56.25 |

From the above Table, it appears that for N=8, a secondary transform of size 8x8 has a large overhead of 56.25 %. However, in this case, we can actually reduce this latency by using a different logic for implementing 8x8 2-d DCT and 8x8 2-d Secondary Transform. Specifically, we can use the following logic 2:

- Hor DCT

- Hor Secondary Transform

- Vertical DCT

- Vertical Secondary Transform

In the above implementation, the 1st row of secondary transform can be started as soon as the 1st row of DCT finishes. The last of the 8th row of secondary transform takes additional M cycles after Horizontal DCT. Immediately after the horizontal secondary transform finishes, the Vertical DCT can begin, and the Vertical secondary transform can finish M cycles after vertical DCT.

So, the overall additional cycles = 2*M, and hence overhead = 2*M/ 2(8+L-1). Assuming M=L=1, overhead = 2/16 = 12.5 %, which is much less than 56.25% in the implementation above.

Further, if a parallel implementation for secondary transform is used, then the latency for the secondary transform can be further reduced. For example, if only 2 rows of Horizontal Secondary transform (for the Vertical ST, we still assume that there is no parallel implementation) are processed together, then the entries for K=2 to 6 in Table 5 become:

Table 6: Additional cycle table for various secondary transform sizes when DCT size is 8, and 2 rows can be processed in parallel for the horizontal secondary transform

| K (Secondary Transform Size) | Additional Cycles | Overhead in Percentage = Extra secondary transform cycles/Cycles for 2-d DCT |
| --- | --- | --- |
| 2 | 0 | 0 |
| 3 | 0 | 0 |
| 4 | 0 | 0 |
| 5 | 1 | 6.25 |
| 6 | 2 | 12.5 |

Hence, to summarize, at block sizes 8x8, 16x16 and 32x32 blocks, the additional latency in terms of cycles would be 0 for the 32x32 case, and at-most 2 for the other two cases by the above implementation.

## Further remarks

- Note that if the DCT is applied in Vertical direction first, and then in horizontal direction, then the secondary transform in Logic 1 needs to be applied as the horizontal secondary transform, followed by vertical secondary transform.

- Till now we only considered worst-case scenario, where secondary transform needs to be applied in both the directions. However, in many cases as in Fig. 1, the secondary transform would be applied only in one direction or none. In such cases, the additional latency would decrease, and in most cases be 0.

- At the decoder, the latencies and overhead for a particular size of NxN DCT, and KxK secondary transform via an inverse realization as compared to the encoder, would be exactly the same as for encoder. The following implementation at decoder can be used to minimize latency:

  - Horizontal Inverse Secondary Transform

  - Vertical Inverse Secondary Transform

  - Vertical Inverse DCT

  - Horizontal Inverse DCT

- In case at the decoder, horizontal inverse DCT is taken first and then vertical inverse DCT, the order for secondary transforms should be:

  - Vertical inverse secondary transform,

  - Horizontal inverse secondary transform

  - Horizontal inverse DCT

  - Vertical inverse DCT

- For the 8x8 secondary transform and 8x8 DCT, we can reduce the latency by the following logic (similar to what we did at encoder):

  - Vertical inverse secondary transform

  - Vertical inverse DCT

  - Horizontal inverse secondary transform

  - Horizontal inverse DCT

# Conclusions

In this contribution, we have presented various mode-dependent secondary transforms for intra-prediction residual coding instead of the conventional DCT at block sizes 8x8 and higher. The proposed MD-ST scheme requires the storage of only 1 additional MD-ST matrix, is based on the intra prediction mode, and does not require any explicit signaling. There is no requirement of introducing additional quantization matrices in our proposal. Simulation results show significant gains in compression performance as compared to HM 5.0 anchors. We also present various techniques to minimize the latency of secondary transforms in hardware. We recommend adopting this proposal into HM.

# References

1. A. Saxena and F. Fernandes, “CE7: Mode-dependent DCT/DST without 4*4 full matrix multiplication for intra prediction,” JCTVC-E125, Geneva, Switzerland, March 2011.

2. J. Han, A. Saxena and K. Rose, “Towards jointly optimal spatial prediction and adaptive transform in video/image coding,” IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP), March 2010, pp. 726-729.

3. R. Cohen et al., “Description of Core Experiment 7: Additional Transforms”, JCTVC-G1207, Geneva, Switzerland, Nov 2011.

4. A. Saxena, Y. Shibahara, F. Fernandes and T.Nishi, “On secondary transforms for intra prediction residual,” JCTVC-G108, Geneva, Switzerland, Nov 2011.

5. A. Saxena, Y. Shibahara, F. Fernandes and T.Nishi, “Harmonization of SDIP and mode-dependent secondary transforms,” JCTVC-G629, Geneva, Switzerland, Nov 2011.

6. A. Saxena and F. Fernandes, “Jointly optimal intra prediction and adaptive primary transform,” JCTVC-C108, Guangzhou, China, October 2010.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd., may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix 1

The 4x4 and 8x8 secondary transform matrices are given as follows, where the basis vectors are along rows, and the variable SEC_TR_SIZE denotes the size of secondary transform.

SEC_TR_SIZE ==4

{   123,   -35,    -8,    -3,},

{   -32,  -120,    30,    10,},

{    14,    25,   123,   -22,},

{     8,    13,    19,   126,},

SEC_TR_SIZE ==8

{   123,   -35,    -8,    -3,    -2,    -1,    -1,    -1},

{    32,   120,   -29,   -10,    -5,    -3,    -2,    -1},

{   -14,   -24,  -123,    21,     8,     4,     3,     2},

{     7,    11,    17,   125,   -16,    -7,    -4,    -2},

{    -4,    -7,    -8,   -13,  -126,    13,     5,     3},

{     3,     4,     5,     6,    11,   127,   -10,    -5},

{    -2,    -3,    -3,    -4,    -5,    -9,  -127,     9},

{     2,     2,     3,     3,     3,     5,     8,   128},

# Appendix 2

Secondary Transforms by smoothing the correlation matrix for dimension 32 intra prediction blocks

For deriving a 8-point transform (e.g., vertical transform after vertical prediction on intra blocks with vertical dimension 8), we used the following covariance matrix after smoothing:

R8  =  [1       1       1        1       1       1       1       1

1     1.5     1.5      1.5     1.5     1.5     1.5     1.5

1     1.5       2        2       2       2       2       2

1     1.5       2      2.5     2.5     2.5     2.5     2.5

1     1.5       2      2.5       3       3       3       3

1     1.5       2      2.5       3     3.5     3.5     3.5

1     1.5       2      2.5       3     3.5       4       4

1     1.5       2      2.5       3     3.5       4     4.5]

In the above correlation matrix as R8, 8 denotes the corresponding input vector length. In [1,2], the correlation matrix for intra 4x4 blocks for deriving the optimal transform was:

Noting the similarity between R8 and R4, we propose the following smoothing for term (i,j) of the NxN matrix of intra prediction residue block RN:

Note that in the above expression, the slope factor (p-1)/(N/4) can be further generalized to : β (p-1) / (N/4) where β > 0 (a positive real number) can control the slope by which we smooth the elements of the correlation matrix RN. Possible values of β, for example, are 0.6, 0.8, 1.2 etc. However, to avoid any tweaking of the parameter, we simply retain it as 1.Note that all the correlation matrices R4, R8, R16, R32 are simply special cases of the above NxN matrix RN. The matrices for R16 and R32 can be derived similarly.

Steps for Deriving Secondary Transform:

We next outline the steps for deriving a KxK secondary transform from the NxN correlation matrix RN.

Steps:

(a) Obtain the correlation matrix after applying DCT on the intra-prediction residuals, i.e., the resulting correlation matrix denoted as UN   = CNT * RN * CN, where CN denotes the conventional 2-d DCT matrix of size NxN and ‘*’ is the standard multiplication operator.

(b) Obtain the matrix for the top K rows and left-most K columns VK,N  = UN (1:K,1:K) where the sub-scripts K and N in VK,N denote that VK,N is obtained from the K top rows and K left columns of NxN correlation matrix UN.

(c) Find the KLT of VK,N of dimension KxK denoted as WK,N. The resulting matrix WK,N  is the secondary matrix of dimension K that can be used following the DCT as a K-point transform for the first K elements of the N-point DCT output.

(d) In case an integer based approximation of WK,N with m-bit precision (defined as YK,N) is required, multiply WK,N  by 2m and then round the matrix elements to the nearest integer, i.e., YK,N  = round (2m * WK,N).

As an example, we show how to derive an 8x8 secondary transform YK,N with 7-bit precision from R32. Following the aforementioned steps, we have DCT of size 32 as C32.

Then   U32  = C32T  * R32  * C32

Next obtain V8,32  as the top 8x8 portion of the matrix U32, i.e., V8,32  = U32 (1:8,1:8)

The KLT of V8,32 is:

W8,32   = KLT (V8,32). For the integer approximation, we multiply by 27 = 128, and round the resulting elements in the matrix as follows:

Y8,32  = round (128*W8,32)

[123   -35    -8    -3    -2    -1    -1    -1

32   120   -29   -10    -5    -3    -2    -1

-14   -24  -123    21     8     4     3     2

7    11    17   125   -16    -7    -4    -2

-4    -7    -8   -13  -126    13     5     3

3     4     5     6    11   127   -10    -5

-2    -3    -3    -4    -5    -9  -127     9

2     2     3     3     3     5     8   128]

where the basis vectors are along the rows in Y8,32. In our proposal, we use N=32 to derive the secondary transforms. This gives better performance than choosing N=8, and N=16, while deriving the secondary transform, which is applied to 8x8, 16x16 and 32x32 blocks, and is best “matched” to the 32x32 block.