Change of Woking Darft for proposal

JCTVC-H0125: On secondary transforms for intra prediction residual

The document is based on JCTVC-G1103_d8.doc and change is colored yellow, and are less than half page.

## Scaling, transformation and array construction process prior to deblocking filter process

### Scaling and transformation process

Inputs to this process are:

–	a luma location ( xT, yT ) specifying the top-left luma sample of the current transform unit relative to the top-left luma sample of the current picture,

–	a variable trafoDepth specifying the hierarchy depth of the current block relative to the coding unit,

–	a variable cIdx specifying the chroma component of the current block,

–	a variable nW specifying the width of the current transform block.

–	a variable nH specifying the height of the current transform block.

Output of this process is:

–	a modified version of the (nW)x(nH) array of residual samples r with elements rij.

The quantization parameter qP is derived as follows.

–	If cIdx is equal to 0,

qP = QP’Y			(8-1)

–	Otherwise,

qP = QP’C			(8-2)

The (nW)x(nH) array of residual samples r are derived as specified in the following ordered steps.

- The scaling process for transform coefficients as specified in subclause 8.1.2 is invoked with the width of the transform unit nW, the height of the transform unit nH, the (nW)x(nH) array of transform coefficients transCoeffLevel[ xT ][ yT ][ cIdx ], the chroma component variable cIdx and the quantization parameter qP as the inputs and the output is a scaled transform coefficient (nW)x(nH) array d.

- The transformation process for scaled transform coefficients as specified in subclause 8.1.3 is invoked with the width of the transform unit nW, the height of the transform unit nH, the scaled transform coefficient (nW)x(nH) array d and the chroma component variable cIdx as the inputs and the output is a residual samples (nW)x(nH) array r.

### Scaling process for transform coefficients

Inputs of this process are:

–	a variable nW specifying the width of the current transform unit,

–	a variable nH specifying the height of the current transform unit,

–	a (nW)x(nH) array c of transform coefficients with elements cij,

–	a variable cIdx specifying the chroma component of the current block,

–	a variable qP specifying the quantization parameter.

Output of this process is scaled transform coefficients as a (nW)x(nH) array of d with elements dij.

The variable log2TrSize is derived as follows:

log2TrSize = ( log2( NW ) + log2( NH ) ) >> 1		(8-3)

The variable shift is derived as follows:

–	If cIdx is equal to 0,

shift = BitDepthY + log2TrSize – 9		(8-4)

–	Otherwise,

shift = BitDepthC + log2TrSize – 9		(8-5)

The scaling array levelScale[·] is specified as levelScale[k] = { 40, 45, 51, 57, 64, 72 } with k=0..5.

The variable shiftScale is set equal to shift + 4 – QP/6.

The elements of array M[ i ][ j ] with i = 0..nW – 1, j = 0..nH – 1 are set equal to ScalingList[ SizeID ][ RefMatrixID ][ trafoType ][ i*nW+j ], where SizeID and RefMatrixID are specified in Error: Reference source not found and Error: Reference source not found, respectively, and trafoType is derived by

trafoType = ( ( nW = = nH ) ? 0 : ( ( nW > nH ) ? 1 : 2 ) )	(8-6)

The scaled transform coefficient dij with i = 0..nW − 1, j = 0..nH − 1 is derived as follows.

–	If seq_scaling_list_present_flag is equal to 1 or pic_scaling_list_present_flag is equal to 1, [Ed. (WJ): should be replaced by better conditioning]

dij = Clip3( -32768, 32767,( ( cij * levelScale[ qP%6 ] << ( qP/6 ) ) + ( 1 << ( shift – 1 ) ) ) >> shift )	(8-7)

–	Otherwise, if shiftScale is greater than 0,

dij = Clip3( -32768, 32767, ( cij * M[ i ][ j ]*levelScale[ qP%6 ] + 					( 1 << ( shiftScale – 1 ) ) ) >> shiftScale )	(8-8)

–	Otherwise,

dij = Clip3( -32768, 32767, ( cij * M[ i ][ j ]*levelScale[ qP%6 ] ) << ( -shiftScale ) )	(8-9)

[Ed. (WJ): do we need to clip cij to 16b before computing dij? – related to G719 – maybe not]

### Transformation process for scaled transform coefficients

Inputs of this process are:

–	a variable nW specifying the width of the current transform unit,

–	a variable nH specifying the height of the current transform unit,

–	a (nW)x(nH) array d of scaled transform coefficients with elements dij.

–	a variable cIdx specifying the chroma component of the current block,

Output of this process is residual samples as a (nW)x(nH) array r with elements rij.

Depending on PredMode and IntraPredMode, the following applies:

–	If PredMode is equal to MODE_INTRA, log2(nW*nH) is equal to 4 or 8 or 16 or 32, and cIdx is equal to 0, the variables horizTrType and vertTrType are specified as Table 8-1 with IntraPredMode as input. [Ed. (WJ): DST is applied only for luma 4x4 block]

–	Otherwise, the variables horizTrType and vertTrType are set equal to 0.

Table 8-1 – Specification of horizTrType and vertTrType

| IntraPredMode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| horizTrType | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 1 |
| vertTrType | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 |

| IntraPredMode | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| horizTrType | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| vertTrType | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 1 |

The constructed residual samples are derived as specified in the following ordered steps.

- Each (horizontal) row of scaled transform coefficients dij (i=0..nW−1, j=0..nH−1) is transformed to eij (i=0..nW−1, j=0..nH−1) by invoking the one-dimensional transformation process as specified in subclause Error: Reference source not found according to the width of the transform unit nW, with the (nW)x(nH) array d and the transform type variable horizTrType as the inputs and the output is the (nW)x(nH) array e.

- The intermediate sample values gij (i=0..nW-1, j=0..nH-1) are derived by

gij = Clip3( -32768, 32767, ( eij + 64 ) >> 7 )		(8-10)

- Each (vertical) column of the resulting matrix gij (i=0..nW−1, j=0..nH−1) is transformed to fij (i=0..nW−1, j=0..nH−1) by invoking the one-dimensional transformation process as specified in subclause Error: Reference source not found according to the height of the transform unit nH, with the (nW)x(nH) array e and the transform type variable vertTrType as the inputs and the output is the (nW)x(nH) array f.

- The variable shift is derived as follows:

–	If cIdx is equal to 0,

shift = 20 – BitDepthY		(8-11)

–	Otherwise,

shift = 20 – BitDepthC		(8-12)

- The residual sample value rij with i=0..(nW)−1, j=0..(nH)−1 is derived as follows.

rij = ( fij + (1 << ( shift – 1) ) ) >> shift		(8-13)

      - Transformation process

Inputs of this process are:

–	a variable nS specifying the sample size of scaled transform coefficients,

–	an array of scaled transform coefficients x with elements xi, with i=0..nS-1.

–	a transform type variable trType

Output of this process is an array of samples of the residual samples y with elements yi, with i = 0..nS-1.

Depending on trType, the following applies:

–	If nS is equal to 4 and trType is equal to 1, the following ordered steps apply:

- A set of intermediate values c0, c1, c2 and c3 is calculated as follows:

c0 = x0 + x2		(8-14)
c1 = x2 + x3		(8-15)
c2 = x0 – x3		(8-16)
c3 = 74 * x1		(8-17)

- The output values yi with i = 0..3 are then specified as follows:

y0 = ( 29*c0 + 55*c1 + c3 + 64 ) >> 7		(8-18)
y1 = ( 55*c2 – 29*c1 + c3 + 64 ) >> 7		(8-19)
y2 = ( 74*( x0 - x2 + x3 ) + 64 ) >> 7		(8-20)
y3 = ( 55*c0 + 29 * c2 – c3 + 64 ) >> 7		(8-21)

–	If (nS is equal to 8 or 16 or 32, and trType is equal to 1), the following applies:

- A set of intermediate values t0, to t7  from inut x0, to x7 is calculated as follows:

ti = xi for i=0..7		(8-22)

- Next values xi, i = 0..7 are modified as follows:

x0 = ( 123*t0 + 32*t1 - 14*t2 + 7*t3 -4* t4  + 3 t5 - 2*t6 + 2* t7 + 64 ) >> 7	(8-23)
x1 = ( – 35*t0 + 120*t1 -24 *t2 + 11 *t3 -7*t4 + 4*t5 -3*t6 +2* t7 + 64 ) >> 7	(8-24)
x2 = ( – 8*t0 – 29*t1 - 123*t2 + 17*t3 -8*t4 + 5*t5 - 3*t6 + 3*t7 + 64 ) >> 7	(8-25)
x3 = ( – 3*t0 – 10*t1 + 21*t2 + 125*t3 -13*t4 + 6*t5 -4*t6 +3* t7 + 64 ) >> 7	(8-26)

x4 = ( -2*t0 – 5*t1 +  8*t2 -16*t3 - 126* t4  + 11 *t5 -    5*t6 + 3*t7 + 64 ) >> 7	(8-27)
x5 = ( – t0     -3*t1 +  4*t2 -   7*t3 +  13*t4 + 127*t5 -     9*t6 + 5*t7 + 64 ) >> 7	(8-28)
x6 = ( – t0 –      2 t1 +  3*t2  -  4*t3 +    5*t4  -     10*t5 - 127*t6 + 8*t7 + 64 ) >> 7	(8-29)
x7 = (  -t0     – t1 +     2* t2   -   2* t3  +    3*t4 -      5*t5  +     9*t6 + 128*t7 + 64 ) >> 7	(8-30)

(Note: In case, a different size of Secondary Transforms is used such as 4x4 , then the Eqn 8.159 can be appropriately changed using the matrix in JCTVC-H0125: Appendix 1.)

–	[Delete this: Otherwise (nS is not equal to 4 or trType is not equal to 1), the following applies:]

–	If ( (nS is equal to 4, and trType is equal to 0) or (nS is equal to 8 or 16 or 32); ), the following applies:

yi = ∑j( ckj * xj ) with i=0..nS-1, j=0..nS-1		(8-31)

where k = 1 << ( 5 – log2( nS ) ) * i and the transform coefficient matrix c is specified as:

c =

{

{64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64}

{90 90 88 85 82 78 73 67 61 54 46 38 31 22 13  4 -4-13-22-31-38-46-54-61-67-73-78-82-85-88-90-90}

{90 87 80 70 57 43 25  9 -9-25-43-57-70-80-87-90-90-87-80-70-57-43-25 -9  9 25 43 57 70 80 87 90}

{90 82 67 46 22 -4-31-54-73-85-90-88-78-61-38-13 13 38 61 78 88 90 85 73 54 31  4-22-46-67-82-90}

{89 75 50 18-18-50-75-89-89-75-50-18 18 50 75 89 89 75 50 18-18-50-75-89-89-75-50-18 18 50 75 89}

{88 67 31-13-54-82-90-78-46 -4 38 73 90 85 61 22-22-61-85-90-73-38  4 46 78 90 82 54 13-31-67-88}

{87 57  9-43-80-90-70-25 25 70 90 80 43 -9-57-87-87-57 -9 43 80 90 70 25-25-70-90-80-43  9 57 87}

{85 46-13-67-90-73-22 38 82 88 54 -4-61-90-78-31 31 78 90 61  4-54-88-82-38 22 73 90 67 13-46-85}

{83 36-36-83-83-36 36 83 83 36-36-83-83-36 36 83 83 36-36-83-83-36 36 83 83 36-36-83-83-36 36 83}

{82 22-54-90-61 13 78 85 31-46-90-67  4 73 88 38-38-88-73 -4 67 90 46-31-85-78-13 61 90 54-22-82}

{80  9-70-87-25 57 90 43-43-90-57 25 87 70 -9-80-80 -9 70 87 25-57-90-43 43 90 57-25-87-70  9 80}

{78 -4-82-73 13 85 67-22-88-61 31 90 54-38-90-46 46 90 38-54-90-31 61 88 22-67-85-13 73 82  4-78}

{75-18-89-50 50 89 18-75-75 18 89 50-50-89-18 75 75-18-89-50 50 89 18-75-75 18 89 50-50-89-18 75}

{73-31-90-22 78 67-38-90-13 82 61-46-88 -4 85 54-54-85  4 88 46-61-82 13 90 38-67-78 22 90 31-73}

{70-43-87  9 90 25-80-57 57 80-25-90 -9 87 43-70-70 43 87 -9-90-25 80 57-57-80 25 90  9-87-43 70}

{67-54-78 38 85-22-90  4 90 13-88-31 82 46-73-61 61 73-46-82 31 88-13-90 -4 90 22-85-38 78 54-67}

{64-64-64 64 64-64-64 64 64-64-64 64 64-64-64 64 64-64-64 64 64-64-64 64 64-64-64 64 64-64-64 64}

{61-73-46 82 31-88-13 90 -4-90 22 85-38-78 54 67-67-54 78 38-85-22 90  4-90 13 88-31-82 46 73-61}

{57-80-25 90 -9-87 43 70-70-43 87  9-90 25 80-57-57 80 25-90  9 87-43-70 70 43-87 -9 90-25-80 57}

{54-85 -4 88-46-61 82 13-90 38 67-78-22 90-31-73 73 31-90 22 78-67-38 90-13-82 61 46-88  4 85-54}

{50-89 18 75-75-18 89-50-50 89-18-75 75 18-89 50 50-89 18 75-75-18 89-50-50 89-18-75 75 18-89 50}

{46-90 38 54-90 31 61-88 22 67-85 13 73-82  4 78-78 -4 82-73-13 85-67-22 88-61-31 90-54-38 90-46}

{43-90 57 25-87 70  9-80 80 -9-70 87-25-57 90-43-43 90-57-25 87-70 -9 80-80  9 70-87 25 57-90 43}

{38-88 73 -4-67 90-46-31 85-78 13 61-90 54 22-82 82-22-54 90-61-13 78-85 31 46-90 67  4-73 88-38}

{36-83 83-36-36 83-83 36 36-83 83-36-36 83-83 36 36-83 83-36-36 83-83 36 36-83 83-36-36 83-83 36}

{31-78 90-61  4 54-88 82-38-22 73-90 67-13-46 85-85 46 13-67 90-73 22 38-82 88-54 -4 61-90 78-31}

{25-70 90-80 43  9-57 87-87 57 -9-43 80-90 70-25-25 70-90 80-43 -9 57-87 87-57  9 43-80 90-70 25}

{22-61 85-90 73-38 -4 46-78 90-82 54-13-31 67-88 88-67 31 13-54 82-90 78-46  4 38-73 90-85 61-22}

{18-50 75-89 89-75 50-18-18 50-75 89-89 75-50 18 18-50 75-89 89-75 50-18-18 50-75 89-89 75-50 18}

{13-38 61-78 88-90 85-73 54-31  4 22-46 67-82 90-90 82-67 46-22 -4 31-54 73-85 90-88 78-61 38-13}

{ 9-25 43-57 70-80 87-90 90-87 80-70 57-43 25 -9 -9 25-43 57-70 80-87 90-90 87-80 70-57 43-25  9}

{ 4-13 22-31 38-46 54-61 67-73 78-82 85-88 90-90 90-90 88-85 82-78 73-67 61-54 46-38 31-22 13 -4}

}

(8-32)

[Ed. (WJ): better way?]