| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0126 WG 11Number : m22998 |
| --- | --- |

| Title: | CE 7: On secondary transforms for inter prediction residual |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ankur Saxena, Youji Shibahara Felix C. Fernandes Takahiro Nishi | Tel: Email: | 1-9727617761 asaxena@sta.samsung.com, +81-50-3587-2044 shibahara.youji@jp.panasonic.com, felix.f@sta.samsung.com, nishi.takahiro@jp.panasonic.com |
| Source: | Samsung Electronics Co., Ltd & Panasonic |  |  |

_____________________________

# Abstract

Mode-dependent secondary transforms have been previously presented by Saxena et al. in JCTVC-G108 for the intra prediction residual. In JCTVC-G281, a boundary based transform DCT/DST scheme was presented for the inter-predicted residues, while in JCTVC-G632 a boundary-based secondary transform scheme was proposed for the inter residue. This contribution proposes a boundary-dependent transform scheme: at size 4, the DCT/DST is used for inter prediction residue, and at sizes 8 and larger, a secondary transform scheme is employed. The secondary transform for the inter residue is same as being used in JCTVC-G108/JCTVC-H0125 for the intra prediction residuals. Average BD Rate gains of upto 0.5 % are obtained for Low Delay low Complexity setting by applying the proposed transform scheme only on inter residue. Results are also presented when the transform scheme is applied to both intra and inter residue, and gains upto 0.8 % are obtained for Random Access Low Complexity setting. In particular, the gains for higher resolution sequences in Class A are 1.3 % for Random Access Low complexity setting.

# Introduction

In HM 5.0, DCT is applied as the transform for inter predicted residues. In this proposal, at size 4, we use the boundary-dependent DCT/DST Type-7 scheme from JCTVC-G281[1]/JCTVC-H0309. At sizes 8 and larger, we propose the use a secondary transform (of size 4 or 8) following the DCT as earlier presented in JCTVC-G632 [4]. The 4x4 and 8x8 secondary matrices are from JCTVC-G108 [3]/JCTVC-H0125.

# Secondary Transforms on Inter Residues

In [1], alternate primary transforms at sizes 4x4, 8x8 and 16x16 are proposed for the inter prediction residuals. In this proposal, we retain the transform scheme in [1] at size 4x4. However, instead of applying DST Type-4 via full matrix multiplication at sizes 8x8 and 16x16 as in [1], which may have high computational cost, we propose to use secondary transforms, which can be applied following the DCT at block sizes 8x8 and higher, including 32x32.

## Applying secondary transform via multiple “flips”

We next present the concepts for using a secondary transform, instead of using a “flipped” DST. We present the steps for applying a secondary transform (for TU0 in [1]) at encoder and decoder, in which flipping of data is allowed:

Encoder

1. Flip the input data. i.e., for N-point input vector x, with entries xi, i=1..N, define y with elements yi = xN+1-i

2. Take the DCT of y. Denote the output as z.

3. Apply a K-point secondary transform on the first K-elements of z. Let the output be w, where we copy the remaining “N-K” high-frequency elements from z, on which the secondary transform was not applied.

Decoder

Similarly at the decoder, let the input for transform module be v, which is a quantized version of w. Following steps can be performed for taking the inverse transform:

4. Apply inverse secondary transform on the first K-elements of v. Let the output be b (where the “N-K” high frequency coefficients are identical to that of v).

5. Take the inverse DCT of b. Denote the output as d.

6. Flip the data in d. i.e., define f with elements fi = dN+1-i

Then f is the reconstructed values for the pixels x.

For TU1 in [1], the flipping operations need not be required, and DCT followed by secondary transform only needs to be taken at the encoder. At the decoder, one just needs to take inverse secondary transform followed by inverse DCT.

Figure 1 shows how to apply the secondary transform at size 8x8 and larger. For size 4x4, the scheme is identical to [1], and more details can be found therein. Appendix 1 shows an example why a DST (or similarly a DCT+ secondary transform) would be a better transform than DCT via some figures.

Conditions for

Hor Trans. and Vert Trans.

Figure 1. Decoder operations for boundary-dependent secondary transforms at size 8 and larger. When the Horizontal secondary transform is applied such as in Case 3 or 4 above, the data is also flipped afterwards if the left TU boundary is that of PU. By symmetry, in Case 2 or 4, when the vertical transform is applied, the data is flipped afterwards if the top TU boundary is that of PU.

Decoder Operations

HM 5.0 NxN Inverse DCT

HM 5.0 Inverse Quant

K-Point Inv. Row Transforms

K-Point Inv. Col. Transforms

HM 5.0 NxN Inverse Transform

HM 5.0 Inverse Quant

TU Hor boundary

(Left and Right) are

both non-PU boundaries; or both are PU boundaries; or TU is same as PU.

HM 5.0 NxN Inverse DCT

HM 5.0 Inverse Quant

K-Point Inv. Row Transforms

HM 5.0 NxN Inverse DCT

HM 5.0 Inverse Quant

K-Point Inv. Col. Transforms

TU Ver boundary

(Top and Down) are

both non-PU boundaries; or both are PU boundaries; or TU is same as PU

TU Hor boundary:

(Left and Right): one is non-PU boundary; and

other is PU boundary

TU Hor boundary:

(Left and Right): one is non-PU boundary; and

other is PU boundary

TU Ver boundary

(Top and Down): one is non-PU boundary; and other is PU boundary

TU Ver boundary

(Top and Down) are both non-PU boundaries; or both are PU boundaries

TU Ver boundary (Top and Down): one is non-PU boundary; and

other is PU boundary

TU Hor boundary

(Left and Right) are

both non-PU boundaries;

or both are PU  boundaries

# Simulation Results

The test conditions as stipulated in Core Experiment 7 [2] were followed for conducting the experiments. Here the anchor was HM 5.0 with the conventional DCT. Simulation results are provided for Random Access HE, LC and HE-10 bit, and Low Delay-B: HE and LC settings. The simulations were performed in a Linux 64 bit environment. In all the following tests, at size 4x4, the DCT/DST Type-7 scheme is applied for the inter residue.

Test 1: At size 8 and larger for the inter residue, a secondary transform of size 8x8 is used after the DCT. Table 1 shows the summary of simulation results (average BD-Rates over Class A to E, and encoding/decoding times relative to HM 5.0 anchor) for Random Access and Low Delay settings when the proposed transform scheme with was applied. Detailed results are available in the attached Excel sheets.

Table 1. BD Rate gains when proposed transform scheme is applied on Inter prediction Residue with 8x8 secondary transform

|  |  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
|  | Class A (8bit) | -0.4% | -0.2% | -0.2% | -0.6% | 0.1% | 0.0% |
|  | Class B | -0.3% | 0.0% | 0.0% | -0.4% | 0.3% | 0.2% |
|  | Class C | -0.3% | -0.1% | 0.0% | -0.3% | 0.1% | 0.2% |
|  | Class D | -0.2% | -0.2% | -0.2% | -0.3% | 0.3% | 0.2% |
|  | Class E |  |  |  |  |  |  |
|  | Overall | -0.3% | -0.1% | -0.1% | -0.4% | 0.2% | 0.2% |
|  |  | -0.3% | -0.1% | -0.1% | -0.4% | 0.2% | 0.1% |
|  | Class F | -0.1% | 0.1% | 0.0% | -0.1% | 0.1% | 0.0% |
|  | Enc Time[%] | 102% |  |  | 102% |  |  |
|  | Dec Time[%] | 100% |  |  | 101% |  |  |
|  |  | Low delay B HE |  |  | Low delay B LC |  |  |
|  |  | Y | U | V | Y | U | V |
|  | Class A |  |  |  |  |  |  |
|  | Class B | -0.4% | -0.1% | 0.1% | -0.6% | 0.4% | 0.8% |
|  | Class C | -0.4% | -0.2% | -0.1% | -0.5% | 0.4% | 0.3% |
|  | Class D | -0.3% | 0.2% | 0.0% | -0.5% | 0.8% | 0.8% |
|  | Class E | -0.2% | 0.4% | -0.3% | -0.4% | 0.1% | 0.8% |
|  | Overall | -0.4% | 0.1% | -0.1% | -0.5% | 0.4% | 0.7% |
|  |  | -0.4% | 0.1% | -0.1% | -0.5% | 0.5% | 0.5% |
|  | Class F | -0.3% | 0.0% | -0.2% | -0.7% | -1.0% | -0.3% |
|  | Enc Time[%] | 101% |  |  | 101% |  |  |
|  | Dec Time[%] | 99% |  |  | 100% |  |  |

|  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- |
|  |  | Y | U | V |
|  | Class A (8bit) | -0.2% | -0.2% | 0.1% |
|  | Class B | -0.2% | 0.1% | 0.1% |
|  | Class C |  |  |  |
|  | Class D |  |  |  |
|  | Class E |  |  |  |
|  | Overall | -0.2% | 0.0% | 0.1% |
|  |  | -0.2% | 0.0% | 0.1% |
|  | Class F |  |  |  |
|  | Enc Time[%] | 102% |  |  |
|  | Dec Time[%] | 101% |  |  |

Test 2: At size 8 and larger for the inter residue, a secondary transform of size 4x4 is used after the DCT. Table 2 shows the summary of simulation results (average BD-Rates over Class A to E, and encoding/decoding times relative to HM 5.0 anchor) for Random Access and Low Delay settings when the proposed transform scheme with was applied. Detailed results are available in the attached Excel sheets.

Table 2. BD Rate gains when proposed transform scheme is applied on Inter prediction Residue with 4x4 secondary transform

|  |  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
|  | Class A (8bit) | -0.4% | -0.2% | -0.2% | -0.5% | 0.1% | 0.0% |
|  | Class B | -0.2% | 0.0% | 0.0% | -0.3% | 0.2% | 0.2% |
|  | Class C | -0.2% | -0.1% | 0.0% | -0.3% | 0.1% | 0.2% |
|  | Class D | -0.3% | -0.2% | -0.1% | -0.3% | 0.1% | 0.1% |
|  | Class E |  |  |  |  |  |  |
|  | Overall | -0.3% | -0.1% | -0.1% | -0.3% | 0.1% | 0.1% |
|  |  | -0.3% | -0.1% | -0.1% | -0.3% | 0.1% | 0.1% |
|  | Class F | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
|  | Enc Time[%] | 102% |  |  | 101% |  |  |
|  | Dec Time[%] | 100% |  |  | 102% |  |  |
|  |  | Low delay B HE |  |  | Low delay B LC |  |  |
|  |  | Y | U | V | Y | U | V |
|  | Class A |  |  |  |  |  |  |
|  | Class B | -0.4% | -0.1% | 0.2% | -0.5% | 0.2% | 0.4% |
|  | Class C | -0.4% | -0.1% | 0.1% | -0.5% | 0.3% | 0.2% |
|  | Class D | -0.3% | 0.3% | 0.3% | -0.5% | 1.1% | 0.3% |
|  | Class E | -0.2% | -0.2% | 0.0% | -0.3% | 1.0% | 0.7% |
|  | Overall | -0.3% | 0.0% | 0.1% | -0.4% | 0.6% | 0.4% |
|  |  | -0.3% | 0.0% | 0.1% | -0.4% | 0.6% | 0.4% |
|  | Class F | -0.2% | 0.2% | -0.1% | -0.6% | -0.5% | -0.4% |
|  | Enc Time[%] | 101% |  |  | 101% |  |  |
|  | Dec Time[%] | 99% |  |  | 100% |  |  |

|  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- |
|  |  | Y | U | V |
|  | Class A (8bit) | -0.2% | 0.0% | 0.0% |
|  | Class B | -0.2% | 0.1% | 0.0% |
|  | Class C |  |  |  |
|  | Class D |  |  |  |
|  | Class E |  |  |  |
|  | Overall | -0.2% | 0.0% | 0.0% |
|  |  | -0.2% | 0.0% | 0.0% |
|  | Class F |  |  |  |
|  | Enc Time[%] | 101% |  |  |
|  | Dec Time[%] | 101% |  |  |

From the above experimental results, average BD rate gains of upto 0.3%, 0.4 %, 0.4 % and 0.5 % are obtained for Random Access HE and LC, and Low delay HE and LC settings with almost negligible increase in encoding/decoding run-times.

Next, we provide results when the transform scheme is applied to intra residue in addition to the inter residue in Tests 1 and 2.

Test 3: For the intra residue, at size 8 and larger, we apply the 8x8 secondary transform scheme from JCTVC-H0125. For the inter residue, we retain the setting in Test 1. Table 3 shows the summary of simulation results (average BD-Rates over Class A to E, and encoding/decoding times relative to HM 5.0 anchor) for Random Access and Low Delay settings when the proposed transform scheme with was applied. Detailed results are available in the attached Excel sheets.

Table 3. BD Rate gains when proposed transform scheme is applied on both intra and inter prediction residue with 8x8 secondary transform

|  |  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
|  | Class A (8bit) | -0.9% | -0.4% | -0.6% | -1.3% | -0.3% | -0.2% |
|  | Class B | -0.7% | -0.2% | -0.2% | -0.9% | -0.1% | -0.2% |
|  | Class C | -0.5% | -0.4% | -0.2% | -0.6% | -0.3% | -0.3% |
|  | Class D | -0.4% | -0.2% | -0.2% | -0.5% | 0.3% | -0.3% |
|  | Class E |  |  |  |  |  |  |
|  | Overall | -0.6% | -0.3% | -0.3% | -0.8% | -0.1% | -0.2% |
|  |  | -0.6% | -0.3% | -0.2% | -0.8% | -0.1% | -0.3% |
|  | Class F | -0.2% | -0.2% | -0.1% | -0.2% | -0.3% | -0.2% |
|  | Enc Time[%] | 102% |  |  | 102% |  |  |
|  | Dec Time[%] | 100% |  |  | 102% |  |  |
|  |  | Low delay B HE |  |  | Low delay B LC |  |  |
|  |  | Y | U | V | Y | U | V |
|  | Class A |  |  |  |  |  |  |
|  | Class B | -0.6% | -0.3% | -0.2% | -0.8% | -0.3% | -0.1% |
|  | Class C | -0.6% | -0.5% | -0.4% | -0.7% | 0.2% | 0.0% |
|  | Class D | -0.5% | -0.2% | -0.4% | -0.6% | 1.0% | 0.1% |
|  | Class E | -0.1% | -0.3% | -0.9% | -0.2% | -0.5% | 0.1% |
|  | Overall | -0.5% | -0.3% | -0.4% | -0.6% | 0.1% | 0.0% |
|  |  | -0.5% | -0.3% | -0.4% | -0.6% | 0.2% | 0.0% |
|  | Class F | -0.4% | -0.3% | -0.4% | -0.7% | -1.1% | -0.4% |
|  | Enc Time[%] | 102% |  |  | 102% |  |  |
|  | Dec Time[%] | 101% |  |  | 100% |  |  |

|  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- |
|  |  | Y | U | V |
|  | Class A (8bit) | -0.6% | 0.1% | 0.1% |
|  | Class B | -0.6% | -0.4% | -0.5% |
|  | Class C |  |  |  |
|  | Class D |  |  |  |
|  | Class E |  |  |  |
|  | Overall | -0.6% | -0.1% | -0.2% |
|  |  | -0.6% | -0.1% | -0.2% |
|  | Class F |  |  |  |
|  | Enc Time[%] | 102% |  |  |
|  | Dec Time[%] | 100% |  |  |

Test 4: For the intra residue, at size 8 and larger, we apply the 4x4 secondary transform scheme from JCTVC-H0125. For the inter residue, we retain the setting in Test 2. Table 4 shows the summary of simulation results (average BD-Rates over Class A to E, and encoding/decoding times relative to HM 5.0 anchor) for Random Access and Low Delay settings when the proposed transform scheme with was applied. Detailed results are available in the attached Excel sheets.

Table 4. BD Rate gains when proposed transform scheme is applied on both intra and inter prediction residue with 4x4 secondary transform

|  |  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
|  | Class A (8bit) | -0.7% | 0.0% | -0.2% | -1.1% | 0.0% | -0.2% |
|  | Class B | -0.5% | -0.2% | -0.2% | -0.7% | 0.1% | 0.0% |
|  | Class C | -0.4% | -0.3% | -0.2% | -0.5% | 0.0% | -0.1% |
|  | Class D | -0.4% | -0.1% | 0.1% | -0.5% | 0.2% | 0.1% |
|  | Class E |  |  |  |  |  |  |
|  | Overall | -0.5% | -0.2% | -0.1% | -0.7% | 0.1% | 0.0% |
|  |  | -0.5% | -0.2% | -0.1% | -0.7% | 0.1% | -0.1% |
|  | Class F | -0.2% | -0.2% | -0.4% | -0.2% | -0.1% | -0.1% |
|  | Enc Time[%] | 102% |  |  | 101% |  |  |
|  | Dec Time[%] | 102% |  |  | 101% |  |  |
|  |  | Low delay B HE |  |  | Low delay B LC |  |  |
|  |  | Y | U | V | Y | U | V |
|  | Class A |  |  |  |  |  |  |
|  | Class B | -0.5% | -0.2% | 0.1% | -0.7% | -0.2% | 0.1% |
|  | Class C | -0.5% | -0.4% | -0.3% | -0.6% | 0.1% | 0.1% |
|  | Class D | -0.4% | -0.2% | 0.1% | -0.5% | 0.6% | 0.3% |
|  | Class E | -0.1% | 1.6% | -0.3% | -0.2% | 0.1% | -0.4% |
|  | Overall | -0.4% | 0.1% | -0.1% | -0.5% | 0.2% | 0.0% |
|  |  | -0.4% | 0.1% | -0.1% | -0.5% | 0.1% | 0.0% |
|  | Class F | -0.4% | -0.5% | -0.4% | -0.8% | -0.9% | -1.1% |
|  | Enc Time[%] | 101% |  |  | 101% |  |  |
|  | Dec Time[%] | 101% |  |  | 101% |  |  |

|  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- |
|  |  | Y | U | V |
|  | Class A (8bit) | -0.5% | 0.5% | 0.7% |
|  | Class B | -0.5% | 0.0% | -0.1% |
|  | Class C |  |  |  |
|  | Class D |  |  |  |
|  | Class E |  |  |  |
|  | Overall | -0.5% | 0.2% | 0.2% |
|  |  | -0.5% | 0.2% | 0.3% |
|  | Class F |  |  |  |
|  | Enc Time[%] | 101% |  |  |
|  | Dec Time[%] | 100% |  |  |

From the above experimental results, average BD rate gains of upto 0.6%, 0.8 %, 0.5 % and 0.6 % are obtained for Random Access HE and LC, and Low delay HE and LC settings with almost negligible increase in encoding/decoding run-times when the 8x8 secondary transform is used for both intra and inter residue.

## Qualitative Results

We did not notice any subjective differences in the visual quality of the coded sequences.

## Training of Matrices

The proposed secondary transforms were derived analytically. No training was performed on any video sequence. For more details about the derivation, please refer to [3].

## Complexity

Quantization & Scaling Matrices: We retain the same quantization and inverse quantization matrices as used in HM 5.0.

Transform Matrices: For a KxK MD-ST (K=4 or 8), we only require the storage of one additional KxK matrix.

Bit Shift at Intermediate Levels: In our implementation, we shift the output of secondary transform by 7 bits after vertical or horizontal transform.

The bit-precision for all the transforms is 16 bits, similar to in HM 5.0.

Scanning: In this proposal we have retained the conventional HM 5.0 scans

Operation Counts: Tables 5 and 6 present the additional worst-case and average case operations for the proposed transform scheme for inter TU’s inverse secondary transform schemes. For the average case, the following assumptions are made:

For 32x32 TU, the DCT would be used as both the horizontal and vertical transform if PU size is 32x32. If the PU size is 64x64, then there are 4 TU’s, each of size 32x32 will use the secondary transform. We assume that 1/3 of the 32x32 TU come from 32x32 PU, and the rest 2/3 come from 64x64 PU, where they share either the left or right boundary with the PU. In this case, the secondary transform would be used. So, the secondary transform would be used on average in 2/3 cases.

For the 4x4 to 16x16 TU's, we assume the following 4 conditions for horizontal transform:

(a) The TU size is exactly the same as PU size. This corresponds to be a case from Row 1 of Fig. 1, where secondary transform will not be used.

(b) The TU and PU do not share any horizontal boundary. This corresponds to Row 1 or 2 of Fig.1, where secondary transform will not be used.

(c) The TU and PU share the left boundary. In this case, the secondary transform is invoked. This corresponds to the Row 3 or 4, where secondary transform will be used.

(d) The TU and PU share the right boundary. This again corresponds to the Row 3 or 4, where secondary transform will be used.

Assuming the above 4 cases occur with same probability; the secondary transform would be applied, on average, in 50 % cases. A similar analysis can be performed for the vertical transform.

So, for all TU sizes, on average, the additional operations are half of the worst-case scenario. Details about these operations are also available in the attached excel sheet Complexity_InterST.xls.

Table 5. Worst-Case Inverse transform operations count summary for the proposed transform scheme for Inter TU’s

| Transform Unit size |  | HM 5.0 DCT | HM 5.0 DST(currently Used for Intra TU’s only in HM) | Proposed scheme with 4x4 Secondary Transform | Proposed scheme with 8x8 Secondary Transform | Percentage Increase for HM DST over HM DCT in Intra TU’s | Percentage increase for proposed scheme with 4x4 secondary transform Over HM DCT | Percentage increase for proposed scheme with 8x8 secondary transform over HM DCT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4x4 | Mults | 48 | 64 | 64 | 0 | 33.3 | 33.3 | 33.3 |
|  | Adds | 96 | 120 | 120 | 0 | 25 | 25 | 25 |
|  | Shifts | 32 | 32 | 32 | 0 | 0 | 0 | 0 |
|  | Clips | 32 | 32 | 32 | 0 | 0 | 0 | 0 |
| 8x8 | Mults | 352 | N/A | +128 | +1024 | N/A | 36.4 | 290.9 |
|  | Adds | 576 | N/A | +128 | +1024 | N/A | 22.2 | 177.8 |
|  | Shifts | 128 | N/A | +32 | +128 | N/A | 25.0 | 100.0 |
|  | Clips | 128 | N/A | +32 | +128 | N/A | 25.0 | 100.0 |
| 16x16 | Mults | 2752 | N/A | +128 | +1024 | N/A | 4.7 | 37.2 |
|  | Adds | 3712 | N/A | +128 | +1024 | N/A | 3.4 | 27.6 |
|  | Shifts | 512 | N/A | +32 | +128 | N/A | 6.3 | 25.0 |
|  | Clips | 512 | N/A | +32 | +128 | N/A | 6.3 | 25.0 |
| 32x32 | Mults | 21888 | N/A | +128 | +1024 | N/A | 0.6 | 4.7 |
|  | Adds | 25856 | N/A | +128 | +1024 | N/A | 0.5 | 4.0 |
|  | Shifts | 2048 | N/A | +32 | +128 | N/A | 1.6 | 6.3 |
|  | Clips | 2048 | N/A | +32 | +128 | N/A | 1.6 | 6.3 |

Table 4. Average-Case Inverse transform operations count summary for the mode-dependent secondary transforms

| Transform Unit size |  | HM 5.0 DCT | HM 5.0 Average for 4x4 blocks which use DCT and DST (for Intra TU’s only) | Proposed scheme with 4x4 Secondary Transform | Proposed scheme with 8x8 Secondary Transform | Percentage Increase for HM DST over HM DCT in Intra TU’s | Percentage increase for proposed scheme with 4x4 secondary transform Over HM DCT | Percentage increase for proposed scheme with 8x8 secondary transform over HM DCT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4x4 | Mults | 48 | 60 | 56 | 56 | 24.8 | 16.7 | 16.7 |
|  | Adds | 96 | 114 | 108 | 108 | 18.6 | 12.5 | 12.5 |
|  | Shifts | 32 | 32 | 32 | 32 | 0.0 | 0.0 | 0.0 |
|  | Clips | 32 | 32 | 32 | 32 | 0.0 | 0.0 | 0.0 |
| 8x8 | Mults | 352 | N/A | +64 | +512 | N/A | 18.2 | 145.5 |
|  | Adds | 576 | N/A | +64 | +512 | N/A | 11.1 | 88.9 |
|  | Shifts | 128 | N/A | +16 | +64 | N/A | 12.5 | 50.0 |
|  | Clips | 128 | N/A | +16 | +64 | N/A | 12.5 | 50.0 |
| 16x16 | Mults | 2752 | N/A | +64 | +512 | N/A | 2.3 | 18.6 |
|  | Adds | 3712 | N/A | +64 | +512 | N/A | 1.7 | 13.8 |
|  | Shifts | 512 | N/A | +16 | +64 | N/A | 3.1 | 12.5 |
|  | Clips | 512 | N/A | +16 | +64 | N/A | 3.1 | 12.5 |
| 32x32 | Mults | 21888 | N/A | +85 | +683 | N/A | 0.4 | 3.1 |
|  | Adds | 25856 | N/A | +85 | +683 | N/A | 0.3 | 2.6 |
|  | Shifts | 2048 | N/A | +21 | +85 | N/A | 1.0 | 4.2 |
|  | Clips | 2048 | N/A | +21 | +85 | N/A | 1.0 | 4.2 |

# Conclusions

In this contribution, we have presented a transform scheme for the inter-prediction residual coding instead of the conventional DCT. The proposed scheme does not require any explicit signaling. Simulation results show gains in compression performance as compared to HM 5.0 anchors. We recommend adopting this proposal into HM and Working/Committee Draft.

# References

1. J. An, X. Zhao, X. Guo, S. Lei, “Non-CE7: Boundary dependent transform for inter predicted residue”, JCTVC-G281, Geneva, Switzerland, Nov 2011.

2. R. Cohen et al., “Description of Core Experiment 7: Additional Transforms”, JCTVC-G1207, Geneva, Switzerland, Nov 2011.

3. A. Saxena, Y. Shibahara, F. Fernandes and T. Nishi, “On secondary transforms for intra prediction residual”, JCTVC-G108, Geneva, Switzerland, November 2011.

4. A. Saxena, and F. Fernandes, “On secondary transforms for inter prediction residual”, JCTVC-G632, Geneva, Switzerland, November 2011.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd., may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix 1:

Fig. 2 Transform Units in a Prediction Unit. The numbers in 4x4 TU4 denote the energy of the prediction error.

(a)                                     (b)

Fig. 3

(a) DCT basis vectors: First basis vector is flat.

(b)DST basis vectors: First basis vector is zero at one boundary, and maximum at the other boundary. This mimics the prediction error residue behavior in TU4 more efficiently as compared to the DCT basis vector.