| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, USA, 1–10 February, 2012 | Document: JCTVC-H0131 |
| --- | --- |

| Title: | Non-CE11: Simplified multi-level significance map coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junghye Min Yinji Piao JeongHoon Park | Email: | jh643.min@samsung.com yj1003.piao@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution presents a simplified design for multi-level significance maps. In HM-5.0, a multi-level structure is employed to encode significance coefficients for large transform units. This multilevel structure reduces the number of bins for significant map by up to 11%. The proposed method provides simpler design for significant map coding with more reductions in the number of bins (up to 1.0%).

# Introduction

In HM-5.0, a multi-level structure is employed to encode significance coefficients for large transform units. In this two-level structure of significance maps, large TUs (of sizes 16x16, 16x4, 4x16, 32x32, 32x8, 8x32) are partitioned into 4x4 coefficient groups. The significant_coeffgroup_flag represents whether corresponding coefficient group has any non-zero coefficients or not. If significant_coeffgroup_flag is coded as 0, significant_coeff_flags for the corresponding group (16 significant coefficient flags) are not coded. For some cases, significant_coeffgroup_flag is inferred as 1 and not coded in the bitstream. When the current group is the first or last group in group scanning order, significant_coeffgroup_flag is inferred as 1. If lower and right significant_coeffgroup_flag in the neighboring groups are both 1, significant_coeffgroup_flag is also inferred as 1. In this proposal, condition checking for the neighboring groups is removed which results in simpler coding process. Following section describes significant_coeffgroup_flag coding process in HM5.0 and the new design.

## HM5.0

For each coefficient group, significant_coeffgroup_flags in the right and the lower neighboring coefficient groups are checked. If both are 1, significant_coeffgroup_flag is set to 1 and significant_coeffgroup_flag is not coded in the bitstream. The coding process is described in following flow chart.

## Simplified coding process

Checking process for neighbouring significant_coeffgroup_flags is removed. The coding process is described in following flow chart.

# Results

Table 1 compares the number of decoded bins for significance map coding in our proposal against HM5.0 anchor. (a negative value indicates reduction in the number of bins). For most of test conditions, number of decoded bins is reduced while maximum reduction is 1.0%.

Table 2 shows that the BD-rates of the proposal against HM-5.0 anchor. (a negative value indicates gain; and a positive value indicates loss).  For most of cases, no performance loss is observed while 0.1% loss is observed only for RA-10 test conditions. Table 3 shows that the BD-rates of the proposal against HM-5.0 anchor in low QP condition. And Table 4 shows the BD-rates of the proposal against HM-5.0 anchor in RDOQ off case. Performance loss is negligible for most of cases.

Table 1:  Numbers of bins decoded for significant map (significant_coeffgroup_flag + significant_coeff_flags ) coding in the proposal against those in HM 5.0 anchor

|  | All I HE |  |  | All I LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | sig flag | sig group flag | sum | sig flag | sig group flag | sum |
| Class A | -0.2% | 33% | 0.4% | -0.5% | 30% | 0.2% |
| Class B | -0.5% | 24% | 0.2% | -1.3% | 20% | -0.6% |
| Class C | -0.4% | 28% | 0.0% | -0.6% | 23% | -0.2% |
| Class D | -0.5% | 24% | -0.2% | -0.7% | 24% | -0.4% |
| Class E | -0.3% | 21% | 0.4% | -0.5% | 21% | 0.3% |
| overall | -0.4% | 26% | 0.2% | -0.7% | 24% | -0.1% |

|  | RA HE |  |  | RA LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | sig flag | sig group flag | sum | sig flag | sig group flag | sum |
| Class A | -0.4% | 20% | 0.0% | -0.4% | 14% | -0.1% |
| Class B | -0.8% | 16% | -0.4% | -0.9% | 9% | -0.6% |
| Class C | -0.7% | 11% | -0.4% | -0.6% | 7% | -0.5% |
| Class D | -0.7% | 6% | -0.6% | -0.7% | 5% | -0.7% |
| Class E |  |  |  |  |  |  |
| overall | -0.6% | 13% | -0.4% | -0.7% | 9% | -0.5% |

|  | LD HE |  |  | LD LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | sig flag | sig group flag | sum | sig flag | sig group flag | sum |
| Class A |  |  |  |  |  |  |
| Class B | -1.0% | 12% | -0.6% | -1.2% | 5% | -1.0% |
| Class C | -0.6% | 1% | -0.5% | -0.6% | 1% | -0.6% |
| Class D | -0.6% | 3% | -0.5% | -0.9% | 4% | -0.8% |
| Class E | -1.2% | 8% | -0.8% | -1.8% | 5% | -1.5% |
| overall | -0.8% | 6% | -0.6% | -1.1% | 4% | -1.0% |

|  | RA HE-10 |  |  |
| --- | --- | --- | --- |
|  | sig flag | sig group flag | sum |
| Class A | -2.7% | 47% | -1.4% |
| Class B | -0.7% | 16% | -0.3% |
| Class C |  |  |  |
| Class D |  |  |  |
| Class E |  |  |  |
| overall | -1.7% | 31% | -0.8% |

Table 2: BD-rate vs. HM-5.0 (common condition)

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class B | 0.0% | 0.1% | 0.1% | 0.1% | 0.2% | 0.3% |
| Class C | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class D | 0.0% | 0.1% | 0.0% | 0.0% | 0.2% | 0.1% |
| Class E | 0.0% | 0.2% | 0.1% | 0.0% | 0.1% | 0.1% |
| Overall | 0.0% | 0.1% | 0.1% | 0.0% | 0.2% | 0.2% |
| Class F | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.1% | -0.1% | 0.0% | 0.1% | 0.1% |
| Class B | 0.0% | 0.1% | 0.0% | 0.1% | 0.3% | 0.2% |
| Class C | 0.0% | 0.0% | 0.2% | 0.0% | 0.2% | 0.1% |
| Class D | 0.0% | 0.1% | 0.2% | 0.0% | 0.1% | -0.2% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.1% | 0.1% | 0.0% | 0.2% | 0.1% |
| Class F | 0.0% | 0.0% | 0.2% | 0.1% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.3% | 0.3% | 0.0% | -0.1% | 0.3% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% |
| Class D | 0.0% | -0.1% | 0.4% | 0.0% | 0.5% | 0.4% |
| Class E | 0.0% | 0.4% | -0.3% | 0.0% | -1.1% | -0.6% |
| Overall | 0.0% | 0.1% | 0.2% | 0.0% | -0.1% | 0.1% |
| Class F | 0.0% | -0.3% | -0.1% | -0.1% | -0.7% | -0.5% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

|  | Random Access HE-10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A (8bit) | 0.1% | 0.9% | 1.3% |
| Class B | 0.1% | 0.1% | 0.0% |
| Class C |  |  |  |
| Class D |  |  |  |
| Class E |  |  |  |
| Overall | 0.1% | 0.5% | 0.6% |
| Class F |  |  |  |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

Table 3: BD-rate vs. HM-5.0 (low QP conditions: 12, 17, 22, 27)

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class B | 0.1% | 0.0% | 0.1% | 0.1% | 0.2% | 0.2% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class D | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class E | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | 0.2% | 0.2% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class D | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class F | -0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | -0.1% | 0.1% | 0.1% | 0.2% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | -0.1% | 0.1% | 0.0% | 0.2% | 0.1% |
| Class E | 0.0% | 0.1% | -0.1% | 0.0% | -0.3% | 0.2% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.2% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% |
| Class F | -0.1% | -0.1% | -0.1% | 0.4% | 0.4% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

|  | Random Access HE-10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A (8bit) | 0.1% | 0.3% | 0.4% |
| Class B | 0.1% | -0.1% | -0.1% |
| Class C |  |  |  |
| Class D |  |  |  |
| Class E |  |  |  |
| Overall | 0.1% | 0.1% | 0.1% |
|  | 0.1% | 0.1% | 0.2% |
| Class F |  |  |  |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

Table 4: BD-rate vs. HM-5.0 (RDOQ off)

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class B | 0.1% | 0.2% | 0.2% | 0.1% | 0.2% | 0.3% |
| Class C | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class D | 0.1% | 0.2% | 0.2% | 0.0% | 0.2% | 0.1% |
| Class E | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Overall | 0.0% | 0.1% | 0.1% | 0.0% | 0.2% | 0.2% |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.2% | 0.2% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class B | 0.1% | 0.1% | 0.2% | 0.1% | 0.3% | 0.2% |
| Class C | 0.0% | 0.0% | -0.1% | 0.0% | 0.2% | 0.1% |
| Class D | 0.0% | 0.1% | 0.2% | 0.0% | 0.1% | -0.2% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.1% | 0.1% | 0.0% | 0.2% | 0.1% |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.2% | 0.0% |
| Class F | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.2% | 0.5% | 0.0% | -0.1% | 0.3% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class D | 0.0% | -0.1% | 0.2% | 0.0% | 0.5% | 0.4% |
| Class E | 0.1% | 1.3% | 0.0% | 0.0% | -1.1% | -0.6% |
| Overall | 0.0% | 0.3% | 0.2% | 0.0% | -0.1% | 0.1% |
|  | 0.0% | 0.3% | 0.2% | 0.0% | 0.0% | 0.1% |
| Class F | 0.0% | -0.1% | 0.1% | -0.1% | -0.7% | -0.5% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |

|  | Random Access HE-10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A (8bit) | 0.1% | 0.6% | 0.5% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C |  |  |  |
| Class D |  |  |  |
| Class E |  |  |  |
| Overall | 0.1% | 0.3% | 0.2% |
|  | 0.1% | 0.3% | 0.3% |
| Class F |  |  |  |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 101% |  |  |

# Conclusions

The simplified design for Multi-Level significance maps is proposed. The proposed design provides simpler coding process for two-level significance map coding with more reduction in number of decoded bins in significant maps. For most of cases, no performance loss is observed while negligible loss in RA-10 test condition.

# Patent rights declarations

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- JCT-VC, “Multi-level significance maps for large transform units”, JCTVC-G644, 7th JCT-VC Meeting, Geneva, CH, 21-30 Nov, 2011

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-G1200, 7th JCT-VC Meeting, Geneva, CH, 21-30 Nov, 2011

# Text for Working Draft

###### 9.2.3.1.1.4 Derivation process of ctxIdxInc for the syntax element significant_coeff_group_flag

Inputs to this process are the color component index cIdx, the current coefficient group scan position ( xCG, yCG ), the previously decoded bins of the syntax element significant_coeff_group_flag, the width of the current transform block, log2TrafoWidth and the height of the current transform block, log2TrafoHeight.

Output of this process is ctxIdxInc.

The variable ctxCG is derived using the current position ( xCG, yCG ), two previously decoded bins of the syntax element significant_coeff_group_flag in scan order, and the width of the current transform block, log2TrafoWidth and the height of the current transform block, log2TrafoHeight as follows.

- ctxCG is initialized with 0 as follows.

ctxCG  =  0		(9-1)

- When xCG is less than ( 1 << ( log2TrafoWidth − 2 ) ) − 1, ctxCG is modified as follows.

ctxCG  +=  significant_coeff_group_flag[ xCG + 1 ][ yCG ]	(9-2)

- When yCG is less than ( 1 << ( log2TrafoHeight − 2 ) ) − 1, ctxCG is modified as follows.

ctxCG  +=  significant_coeff_group_flag[ xCG ][ yCG + 1 ]	(9-3)

The context index increment ctxIdxInc is derived using the color component index cIdx and ctxCG as follows.

- If cIdx is equal to 0, ctxIdxInc is derived as follows.

ctxIdxInc  =  ctxCG		(9-4)

ctxIdxInc  =  min(ctxCG,1)		(9-5)

- Otherwise (cIdx is greater than 0), ctxIdxInc is derived as follows.

ctxIdxInc  =  2  +  ctxCG		(9-6)

ctxIdxInc  =  2  +  min(ctxCG,1)		(9-7)

NOTE – The values of ctxCG are in the range of 0 to 2, inclusive. A ctxCG value of 3 does not occur because significant_coeff_group_flag[ xCG ][ yCG ] is inferred when significant_coeff_group_flag[ xCG + 1 ][ yCG ] and significant_coeff_group_flag[ xCG ][ yCG + 1 ] are both equal to 1.