| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0132 |
| --- | --- |

| Title: | On Adaptation Parameter Signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yan Ye Eun-Seok Ryu 9710 Scranton Rd, Suite 250 San Diego, CA 92121 | Tel: Email: | +1-858-210-4803 yan.ye@interdigital.com eun.ryu@interdigital.com |
| Source: | InterDigital Communications |  |  |

_____________________________

# Abstract

In this proposal the error propagation effect of losing SAO and ALF parameters in APS is studied by analyzing the simple case of losing only the first APS. Quality degradation in terms of both PSNR and visual quality was analyzed. It is reported that error propagation due to the loss of SAO and ALF parameters is content dependent. In terms of PSNR degradation, the maximum loss is 2dB for Y, 4dB for U, and 5dB for V. For all three components, the minimum PSNR loss is close to 0dB. In terms of subjective quality, visual degradation ranges from unnoticeable for some test sequences to significant visual artifacts for others. Further, it is reported that for those sequences where visual artifacts were observed, at which frame and for how long the artifacts appear is also content dependent; in other words, some artifacts appear early in the sequence while others appear later in the sequence, and some artifacts appear for just a few frames while others propagate throughout many frames. Based on these observations, a method to improve error resilience of adaptation parameters carried in APS is proposed.

# Problem Statement

In the 6th and the 7th JCT-VC meetings, the concept of Adaptation Parameter Set (APS) was adopted. Currently 3 types of parameters can be carried in APS: quantization scaling matrices, SAO parameters, and ALF parameters [4]. The APS provides a convenient way to carry these parameters, and makes it easy to share the same APS parameters among different slices from the same picture. However, because the APS is sent as an individual NAL unit, it can suffer packet loss independently from the NAL units of the coded slices which will refer to it. When an APS is lost, even if the NAL units carrying the coded slices are received correctly, there will be reconstruction error at the decoder side, and the error will propagate due to temporal prediction. In this proposal, the effect of error propagation caused by APS loss is studied. Then a method is proposed to improve the error resilience property of these SAO and ALF parameters.

To study the effect of APS loss, a simple experiment is carried in this proposal: when decoding the first frame, the decoder “pretends” that the first APS is lost, and sets the ALF and SAO flags to 0. This is only done for the first frame (I frame), and all the other APS sets for all other frames are considered to be correctly received. Hence, all the other frames are decoded incorrectly only as a result of temporal error propagation. The low delay B high efficiency (LDB-HE) setting is selected to evaluate the effect of such loss and propagation. The HM5.0 software and the HM5.0 common testing conditions are used to generate the anchor bitstreams. Random Access is not tested as the extent of temporal error propagation is limited. And Low Complexity is also not tested as SAO and ALF are disabled.

# Simulation results and discussions

The effect of error propagation due to first APS loss is studied both in terms of objective (PSNR) and subjective quality. Figure 1 through Figure 3 show the average PSNR degradation (averaged over the entire sequence) due to the simulated first APS loss, for Y, U and V components, respectively. This is calculated by subtracting the average PSNR of simulated first APS loss from the average PSNR of error free decoding. In Figures 1 to 3, PSNR degradation due to APS loss is plotted for each sequence along the X axis. The sequences are ordered alphabetically, and the mapping between the sequence numbers and sequence names is given in Table 1. As shown in Figure 1, for the Y component, PSNR degradation is generally the largest for the highest quality (smallest QP), and can be as high as 2dB. For U and V components, amount of PSNR degradation is less related to the QP value, but can reach as high as 4dB to 5dB. Figure 4 shows the frame by frame PSNR degradation for class D sequence BlowingBubbles, at QP 22.

Table 1. List of sequences

| Seq. no | Seq. name | Resolution | Seq. no | Seq. name | Resolution |
| --- | --- | --- | --- | --- | --- |
| 1 | BasketballDrill | 832x480 | 11 | Kimono | 1920x1080 |
| 2 | BasketballDrillText | 832x480 | 12 | ParkScene | 1920x1080 |
| 3 | BasketballDrive | 1920x1080 | 13 | PartyScene | 832x480 |
| 4 | BasketballPass | 416x240 | 14 | RaceHorses | 416x240 |
| 5 | BlowingBubbles | 416x240 | 15 | RaceHorses | 832x480 |
| 6 | BQMall | 832x480 | 16 | SlideEditing | 1280x720 |
| 7 | BQSquare | 416x240 | 17 | SlideShow | 1280x720 |
| 8 | BQTerrace | 1920x1080 | 18 | Vidyo1 | 1280x720 |
| 9 | Cactus | 1920x1080 | 19 | Vidyo3 | 1280x720 |
| 10 | ChinaSpeed | 1024x768 | 20 | Vidyo4 | 1280x720 |

Figure 1. Average PSNR-Y degradation due to first APS loss

Figure 2. Average PSNR-U degradation due to first APS loss

Figure 3. Average PSNR-V degradation due to first APS loss

Figure 4. Frame-by-frame PSNR for BlowingBubbles at QP = 22

## Visual artifact examples

Viewing of the reconstructed sequences with APS error was performed separately from, as well as in comparison with the corresponding error free reconstruction. Some sequences suffer significant visual degradation due to loss of the first APS. A few examples were given in Figure 5 to Figure 9, using QP=22 sequences as example. In Figures 5 to 9, the one on the left is the reconstructed sequence with first APS loss, and the one on the right is the error-free reconstruction. Most of these artifacts shown are in the form of significant color shift. In general, the following observations were made:

- Visual degradation is content dependent, ranging from unnoticeable for some test sequences to significant visual artifacts for other sequences.

- For those sequences where visual artifacts were observed, some artifacts appear earlier while other appear later in the sequence. For example, for BasketballDrive, artifacts appeared as early as at ~0.6 sec (frames 32 and 33).

- Additionally, some artifacts appear for very short duration (e.g., in 2 frames), while other appear for a long time. For example, for PartyScene, color artifacts started appearing in frame 180 and gradually became more pronounced through the end of the sequence (frame 499).

- The most visible artifacts were in the form of chroma shifts, where unnatural and/or strange colors appear in isolated locations.

- Except class E (which has less color-rich content), there is at least one sequence in each sequence class where noticeable to significant visual artifacts were observed.

Figure 5. BasketballDrive, frame 129

Figure 6. BlowingBubbles, frame 499 (error started appearing from frame 290, and lasted till end)

Figure 7. BQTerrace, frame 499 (error started appearing in frame 467)

Figure 8. Cactus, frame 170

Figure 9. ChinaSpeed, frame 89

# Proposed method for improved error resilience

As shown in the previous section, it is very important to ensure robust error resilience when signalling the adaptation parameters such as SAO and ALF, as one single APS loss can cause significant visual quality problems. Further, though loss analysis was not done in this document, it is logical to assume that the same principle should apply to quantization scaling matrices.

To improve error resilience, we propose to add a flag to slice header to indicate whether the current slice derives its adaptation parameters from an APS. If the flag is set to 1, then the adaptation parameters from the APS with aps_id are used to reconstruct the current slice. If the flag is set to 0, then the adaptation parameters are signalled in the current slice header. Section 3.1 shows the proposed WD change.

There are various ways that the encoder and/or the decoder can use this flag to improve error resilience:

- Embed adaptation parameters in slice header when the slice is likely to be used frequently for temporal prediction. For example, if a slice is from lower hierarchy of the prediction structure and is coded using lower QP, then it is more likely to be used as temporal reference.

- Embed adaptation parameters in slice header when the slice is more likely to cause longer temporal propagation, for example, if the slice is close to the last intra refresh point (or if it is an I-slice used for intra refresh).

- Embed adaptation parameters in slice header when it is deemed that error propagation is more likely to be visible. For example, from our viewing experiments, it has been observed that sequences with “richer” colors tend to suffer more visible artifacts.

- As in the example in Figure 10, when a picture is coded using multiple slices, one slice can repeat the parameters in APS inside one of the slice headers again. Though this introduces additional redundancy, if the APS is lost, then all other slices can still use the embedded adaptation parameters. This mechanism can be applied to the scenario in 1, 2, or 3 to improve error resilience.

- If the encoder wants to derive SAO and ALF parameters on slice basis, instead of on picture basis, it can choose to signal these parameters inside the slice header, rather than inside APS.

Figure 10. Redundant signaling of adaptation parameters

## Proposed WD text change

| slice_header( ) { | Descriptor |
| --- | --- |
| entropy_slice_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| if( sample_adaptive_offset_enabled_flag ¦¦ adaptive_loop_filter_enabled_flag ) |  |
| { |  |
| use_adapt_param_from_aps_flag | u(1) |
| if( use_sao_alf_param_in_aps_flag ) |  |
| aps_id | ue(v) |
| else |  |
| { |  |
| scaling_list_data_present_flag | u(1) |
| sample_adaptive_offset_flag | u(1) |
| adaptive_loop_filter_flag | u(1) |
| if( scaling_list_data_present_flag ) |  |
| scaling_list_param( ) |  |
| if( sample_adaptive_offset_flag ) |  |
| sao_param( ) |  |
| if( adaptive_loop_filter_flag ) { |  |
| alf_param( ) |  |
| } |  |
| … |  |
| } |  |
| … |  |
| } |  |

use_adapt_param_from_aps_flag identifies if the syntax element aps_id is present. If use_adapt_param_from_aps_flag is equal to 1, then the syntax element aps_id is present, and the ALF and SAO parameters are retrieved from the APS associated with the aps_id. If use_adapt_param_from_aps_flag is equal to 0, then the syntax element aps_id is not present, and the ALF and SAO parameters are decoded directly from the slice header.

scaling_list_data_present_flag equal to 1 specifies that the scaling list parameters exist in this slice header, equal to 0 specifies that scaling list parameters do not exist in this slice header.

sample_adaptive_offset_flag equal to 1 specifies that the SAO is on for this slice; equal to 0 specifies that the SAO is off for this slice.

adaptive_loop_filter_flag equal to 1 specifies that the ALF is on for this slice; equal to 0 specifies that the ALF is off for this slice.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand. WD5: Working Draft 5 of High-Efficiency Video Coding. Document no JCTVC-G1103, November 2011.

- F. Bossen, Common HM test conditions and software reference configurations. Document no JCTVC-G1200, November 2011.

- S. Wenger, J. Boyce, Y. W. Huang, C. Y. Tsai, P. Wu, M. Li, Adaptation Parameter Set. Document no JCTVC-F747, July 2011.

- S. Wenger, BoG report on APS matters. Document no JCTVC-G1016, November 2011.

# Patent rights declaration(s)

InterDigital Communications may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).