| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, USA, 1-10 February, 2012 | Document: JCTVC-H0134 |
| --- | --- |

| Title: | Cross-check of CE11 subtest2: On context simplification for coefficients coding(JCTVC-H060) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yinji Piao JeongHoon Park | Email: | yj1003.piao@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results of level context simplification described in JCTVC-H060[1]. The number of context is reduced from 56 to 24 by mapping contexts with similar probabilities into 12 representative contexts both for coeff_abs_level_greater1_flag and coeff_abs_level_greater2_flag. The software implementation is not exactly same with the algorithm description of JCTVC-G301[2], but consistent with JCTVC-H060. The crosscheck results match the results from proponent except All Intra configuration.

# Code-level check

The proposal was implemented on top of HM5.0. BJUT_COEFF_G301 is used for the proposed modification. Look-up tables with size of 2x32 and 2x24 are introduced to map the derived context index to one of 12 contexts in case of intra and inter slices both for coeff_abs_level_greater1_flag and coeff_abs_level_greater2_flag. The specific mapping is summarized in Table I and Table II. As can be observed from the tables, luma and chroma shares some contexts and which is inconsistent with the initial idea of G301. In G301, contexts were grouped inside luma and chroma, but in H060 contexts were shared in luma and chroma.

Table I Context mapping of coeff_abs_level_greater1_flag

| GreaterT1 flag (Luma) |  |  |  | c1 | ctxIdx_Intra | ctxIdx_Inter |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | Sub-set 0 |  | 0 LargerT1 in previous subset | 1 or more LargerT1 | 0 | 0 |
| 1 |  |  |  | Initial - no trailing ones | 1 | 1 |
| 2 |  |  |  | 1 trailing one | 2 | 1 |
| 3 |  |  |  | 2 or more trailing ones | 3 | 2 |
| 4 |  |  | 1-3 LargerT1 in previous subset | 1 or more LargerT1 | 4 | 3 |
| 5 |  |  |  | Initial - no trailing ones | 5 | 2 |
| 6 |  |  |  | 1 trailing one | 3 | 4 |
| 7 |  |  |  | 2 or more trailing ones | 6 | 5 |
| 8 |  |  | >3 LargerT1 in previous subset | 1 or more LargerT1 | 7 | 6 |
| 9 |  |  |  | Initial - no trailing ones | 8 | 7 |
| 10 |  |  |  | 1 trailing one | 8 | 7 |
| 11 |  |  |  | 2 or more trailing ones | 8 | 8 |
| 12 | Other sub-sets |  | 0 LargerT1 in previous subset | 1 or more LargerT1 | 6 | 5 |
| 13 |  |  |  | Initial - no trailing ones | 9 | 9 |
| 14 |  |  |  | 1 trailing one | 9 | 9 |
| 15 |  |  |  | 2 or more trailing ones | 1 | 10 |
| 16 |  |  | 1-3 LargerT1 in previous subset | 1 or more LargerT1 | 10 | 11 |
| 17 |  |  |  | Initial - no trailing ones | 2 | 10 |
| 18 |  |  |  | 1 trailing one | 2 | 10 |
| 19 |  |  |  | 2 or more trailing ones | 5 | 1 |
| 20 |  |  | >3 LargerT1 in previous subset | 1 or more LargerT1 | 0 | 0 |
| 21 |  |  |  | Initial - no trailing ones | 6 | 4 |
| 22 |  |  |  | 1 trailing one | 3 | 4 |
| 23 |  |  |  | 2 or more trailing ones | 11 | 4 |
| GreaterT1 flag (Chroma) |  |  |  |  |  |  |
| 24 |  | 0 LargerT1 in previous subset |  | 1 or more LargerT1 | 0 | 0 |
| 25 |  |  |  | Initial - no trailing ones | 9 | 9 |
| 26 |  |  |  | 1 trailing one | 1 | 10 |
| 27 |  |  |  | 2 or more trailing ones | 11 | 2 |
| 28 |  | >0 LargerT1 in previous subset |  | 1 or more LargerT1 | 0 | 0 |
| 29 |  |  |  | Initial - no trailing ones | 2 | 2 |
| 30 |  |  |  | 1 trailing one | 5 | 2 |
| 31 |  |  |  | 2 or more trailing ones | 11 | 4 |

Table II Context mapping of coeff_abs_level_greater2_flag

| GreaterT2 flag (Luma) |  |  | c2 | ctxIdx_Intra | ctxIdx_Inter |
| --- | --- | --- | --- | --- | --- |
| 0 | Sub-set 0 | 0 LargerT1 in previous subset | Initial - no LargerT1 | 0 | 0 |
| 1 |  |  | 1 LargerT1 | 1 | 1 |
| 2 |  |  | 1 or more LargerT1 | 2 | 2 |
| 3 |  | 1-3 LargerT1 in previous subset | Initial - no LargerT1 | 3 | 3 |
| 4 |  |  | 1 LargerT1 | 1 | 1 |
| 5 |  |  | 1 or more LargerT1 | 2 | 2 |
| 6 |  | >3 LargerT1 in previous subset | Initial - no LargerT1 | 4 | 4 |
| 7 |  |  | 1 LargerT1 | 5 | 5 |
| 8 |  |  | 1 or more LargerT1 | 6 | 6 |
| 9 | Other sub-sets | 0 LargerT1 in previous subset | Initial - no LargerT1 | 7 | 7 |
| 10 |  |  | 1 LargerT1 | 0 | 0 |
| 11 |  |  | 1 or more LargerT1 | 1 | 1 |
| 12 |  | 1-3 LargerT1 in previous subset | Initial - no LargerT1 | 8 | 0 |
| 13 |  |  | 1 LargerT1 | 3 | 8 |
| 14 |  |  | 1 or more LargerT1 | 4 | 4 |
| 15 |  | >3 LargerT1 in previous subset | Initial - no LargerT1 | 9 | 3 |
| 16 |  |  | 1 LargerT1 | 10 | 9 |
| 17 |  |  | 1 or more LargerT1 | 5 | 5 |
| GreaterT2 flag (Chroma) |  |  |  |  |  |
| 18 |  | 0 LargerT1 in previous subset | Initial - no LargerT1 | 8 | 10 |
| 19 |  |  | 1 LargerT1 | 10 | 1 |
| 20 |  |  | 1 or more LargerT1 | 11 | 11 |
| 21 |  | >0 LargerT1 in previous subset | Initial - no LargerT1 | 0 | 8 |
| 22 |  |  | 1 LargerT1 | 9 | 9 |
| 23 |  |  | 1 or more LargerT1 | 5 | 2 |

# Test Results

The tests were carried out under the test conditions specified in the JCTVC-G1200[3] as well as LowQP and RDOQoff conditions. S/w provided by proponent was compiled on 64-bit Linux and executed on Linux cluster. The summary results are presented in following tables, and more details can be found in accompanying excel sheet.

Table III. Test results in HM5.0 under common test condition

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | -0.1 |  |  |  | 0.0 | 0.2 | 0.3 |
| Class B | 0.0 | 0.1 | 0.1 | 0.0 | 0.0 | 0.0 | -0.1 | 0.1 | -0.1 | 0.0 | 0.1 | -0.1 |
| Class C | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 | 0.1 | -0.1 | -0.4 | -0.1 |  |  |  |
| Class D | 0.0 | 0.1 | 0.1 | 0.0 | -0.2 | -0.1 | -0.1 | -0.5 | -0.3 |  |  |  |
| Class E | 0.0 | 0.0 | 0.0 |  |  |  | 0.0 | 0.1 | -0.6 |  |  |  |
| Overall | 0.0 | 0.1 | 0.1 | 0.0 | -0.1 | 0.0 | -0.1 | -0.2 | -0.3 | 0.0 | 0.1 | -0.1 |
| Enc Time | 100% |  |  | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time | 100% |  |  | 100% |  |  | 98% |  |  | 102% |  |  |
|  | All Intra LC |  |  | Random Access LC |  |  | Low delay B LC |  |  |  |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |  |  |  |
| Class A | 0.1 | 0.1 | 0.1 | 0.0 | 0.1 | -0.2 |  |  |  |  |  |  |
| Class B | 0.1 | 0.1 | 0.1 | 0.1 | 0.2 | 0.1 | 0.0 | -0.1 | 0.3 |  |  |  |
| Class C | 0.0 | 0.1 | 0.0 | 0.1 | -0.1 | 0.1 | 0.1 | 0.2 | -0.1 |  |  |  |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.4 | 0.0 | 0.0 | 0.8 | 0.0 |  |  |  |
| Class E | 0.1 | 0.1 | 0.1 |  |  |  | -0.1 | -0.9 | 0.3 |  |  |  |
| Overall | 0.0 | 0.1 | 0.1 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.1 |  |  |  |
| Enc Time | 100% |  |  | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time | 99% |  |  | 99% |  |  | 99% |  |  |  |  |  |

Table IV. Low QP test results in HM5.0

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.1 | 0.1 | 0.0 | 0.1 | 0.1 |  |  |  | 0.0 | 0.1 | 0.2 |
| Class B | 0.1 | 0.4 | 0.4 | 0.0 | 0.4 | 0.4 | 0.0 | 0.5 | 0.4 | 0.1 | 0.4 | 0.3 |
| Class C | 0.1 | 0.1 | 0.1 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 |  |  |  |
| Class D | 0.1 | 0.2 | 0.2 | 0.0 | 0.1 | 0.1 | 0.0 | 0.1 | 0.1 |  |  |  |
| Class E | 0.0 | 0.0 | 0.0 |  |  |  | 0.0 | -0.1 | -0.1 |  |  |  |
| Overall | 0.1 | 0.2 | 0.2 | 0.0 | 0.2 | 0.2 | 0.0 | 0.0 | 0.1 | 0.0 | 0.3 | 0.3 |
| Enc Time | 102% |  |  | 101 |  |  | 101 |  |  | 100% |  |  |
| Dec Time | 100% |  |  | 101 |  |  | 100 |  |  | 100% |  |  |
|  | All Intra LC |  |  | Random Access LC |  |  | Low delay B LC |  |  |  |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |  |  |  |
| Class A | 0.1 | 0.1 | 0.1 | 0.0 | 0.1 | 0.0 |  |  |  |  |  |  |
| Class B | 0.2 | 0.3 | 0.3 | 0.1 | 0.2 | 0.3 | 0.0 | 0.3 | 0.3 |  |  |  |
| Class C | 0.1 | 0.1 | 0.1 | 0.0 | 0.1 | 0.1 | 0.0 | 0.1 | 0.1 |  |  |  |
| Class D | 0.1 | 0.1 | 0.1 | 0.0 | 0.1 | 0.1 | 0.0 | 0.3 | 0.1 |  |  |  |
| Class E | 0.0 | 0.1 | 0.1 |  |  |  | 0.0 | -0.2 | -0.2 |  |  |  |
| Overall | 0.1 | 0.1 | 0.1 | 0.0 | 0.1 | 0.1 | 0.0 | 0.2 | 0.1 |  |  |  |
| Enc Time | 101% |  |  | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time | 100% |  |  | 101 |  |  | 100% |  |  |  |  |  |

Table V. RDOQoff test results in HM5.0

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.1 | 0.1 | 0.0 | 0.2 | 0.1 |  |  |  | 0.0 | 0.3 | 0.2 |
| Class B | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.2 | 0.0 | 0.1 | 0.3 | 0.0 | 0.1 | -0.1 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | -0.1 | 0.0 | -0.1 | 0.0 |  |  |  |
| Class D | 0.0 | 0.1 | 0.1 | 0.0 | 0.2 | 0.1 | 0.0 | 0.0 | -0.2 |  |  |  |
| Class E | 0.1 | 0.1 | 0.1 |  |  |  | 0.0 | -0.3 | -0.6 |  |  |  |
| Overall | 0.0 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.0 | 0.0 | -0.1 | 0.0 | 0.2 | -0.1 |
| Enc Time | 100% |  |  | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time | 99% |  |  | 100% |  |  | 99% |  |  | 101% |  |  |
|  | All Intra LC |  |  | Random Access LC |  |  | Low delay B LC |  |  |  |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |  |  |  |
| Class A | 0.1 | 0.1 | 0.1 | 0.0 | 0.1 | -0.2 |  |  |  |  |  |  |
| Class B | 0.1 | 0.1 | 0.1 | 0.1 | 0.2 | 0.1 | 0.0 | -0.1 | 0.3 |  |  |  |
| Class C | 0.0 | 0.1 | 0.0 | 0.1 | -0.1 | 0.1 | 0.1 | 0.2 | -0.1 |  |  |  |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.4 | 0.0 | 0.0 | 0.8 | 0.0 |  |  |  |
| Class E | 0.1 | 0.1 | 0.1 |  |  |  | -0.1 | -0.9 | 0.3 |  |  |  |
| Overall | 0.0 | 0.1 | 0.1 | 0.1 | 0.1 | 0.0 | 0.0 | 0.0 | 0.1 |  |  |  |
| Enc Time | 100% |  |  | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time | 99% |  |  | 98% |  |  | 99% |  |  |  |  |  |

# Conclusions

Test results from JCTVC-H060 are verified in this report. Except AI configuration in common test condition, the simulation results matched the data provided by the proponents. The reason we include the mapping tables in this report is to show the disorderly mapped conditions, such as highlighted in yellow, green and blue. Even if they may have similar probability, there is no consistency among those conditions which share same context.

# References

- JCT-VC, “CE11: SubTest2 Context simplification of coefficient entropy coding”, JCTVC-H060, 8th JCT-VC Meeting, San José, USA, 1-10 Feb., 2012

- JCT-VC, “Test results on context simplification for coefficients coding”, JCTVC-G301, 7th JCT-VC Meeting, Geneva, CH, 21-30 Nov, 2011

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-G1200, 7th JCT-VC Meeting, Geneva, CH, 21-30 Nov, 2011