| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, California, 1- 10 February, 2012 | Document: JCTVC-H0140 |
| --- | --- |

| Title: | CE1: Modified probability update and table removal for multi-parameter CABAC update |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Rosewarne 1 Thomas Holt Drive, North Ryde, NSW 2113, Australia. | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au |
|  | M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: | +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia (CiSRA), Canon Inc. |  |  |

_____________________________

# Abstract

This contribution presents a modification of the multi-parameter CABAC update contribution from Samsung (JCTVC-G764).  The presented method substitutes a 288Kb look-up table with a multiplication and restricts the range of the probability update function. The intention is to reduce area for hardware implementations. The intention of the modification to the probability estimate updating method is to remove one operation from the substituted function and it is asserted this provides a timing benefit for hardware implementation. The proposed technique gives gains in the luma channel of -0.8%, -0.7% and 0.5% for IA_HE, RA_HE and LB-HE configurations, respectively.

# Introduction

The JCTVC-G764 contribution by Samsung introduced a scheme for the context modeling and probability update mechanism. This scheme improved the accuracy of the probability estimation but introduced a 288Kb look-up table containing a function to map the probability estimate to the CABAC offset for each particular CABAC range value.

This contribution presents a method for replacing the look-up table with a multiplication operation and inserting a clipping operation into the probability update mechanism. This replicates the function embodied in the look-up table contents.

From a hardware implementation perspective, look-up tables are generally only preferred if they model a function that would be difficult to implement otherwise, as large look-up tables consume much area/memory, which is a key cost driver.

# Background

Changes introduced by JCTVC-G764 are as follows:

- Either one or two probability estimates may exist for each context (iP0, iP1).

  - iP0 and iP1 are average to create a combined probability estimate.

- Each probability estimate is 15 bits.

- There is no ‘valMPS’ for each context, instead each probability estimate covers the [0, 1] probability space.

  - Also, probability estimates are now evenly distributed over the [0, 1] probability space.

- The CABAC offset (“uiLPS”) is determined by performing a table look-up on a two dimensional table. The first dimension index is created by truncating the combined probability estimate from 15-bits to 9-bits. The second dimension index is created by truncating the 9-bit range value to 6-bits (the MSB is required to always be ‘1’ and is ignored). The table contains 9-bit entries, resulting in an overall table size of 288Kb.

# Proposed method

In CABAC, it is necessary to determine a comparison threshold for the arithmetic encode or decode operation. This comparison threshold is determined by subtracting an offset from a range value. The offset is dependent on the range value and the combined probability estimate.

This contribution removes the 288Kb look-up table by taking the following steps:

- A 15-bit combined probability estimate (either iP0 or average of iP0 and iP1) is right shifted by 6 bits, producing a 9-bit truncated probability estimate.

- The truncated probability estimate is inverted

- The 9-bit range value is masked by 0x1fc to produce a 7-bit value

- Multiply the truncated probability estimate (9-bits) with the 7-bit value to produce a 16 bit value

  - Note that MSB of 7-bit value is always 1, so multiplication logic is a bit simplified.

- Right shift the product by 9-bits to produce a 7-bit offset

- Clip the offset to a minimum value

We assert that a multiplier of this size is much simpler to implement than an equivalent table.

Replacing the look-up table with a multiplier introduces a second aspect to this contribution. This aspect is the need to clip the probability estimate range.

A minimum clipping operation is introduced into the updateLPS function for updating the probability estimates in the context model:

The previous updateLPS function is defined as:

iP0 -= iP0 >> ALPHA0

iP1 -= iP1 >> ALPHA1

The proposed updateLPS function is defined as:

iP0 = max(iP0 – iP0 >> ALPHA0, 2<<6)

iP0 = max(iP1 – iP1 >> ALPHA1, 2<<6)

(ALPHA1 and ALPHA2 are constants which set the rate of adaptation 0 and 1 bin values.)

The purpose of the probability estimate clipping operation is to restrict the lower bound of the truncated probability estimate. After inversion, this restricts the maximum offset value to maintain a required minimum difference between range and offset (δ in G764).

The clipping operation results in a small reduction in the dynamic range of each probability estimate, however the performance impact is asserted to be negligible and the hardware implementation is asserted to be improved firstly, by removal of the look-up table and secondly, by removing the clipping operation from the critical logic path in the range register.

# Experiment Results

Results are presented in Table 1 below. Refer to the accompanying spreadsheet for results with greater precision. Execution took place on a Linux cluster using GCC 3.4.6.

Table 1. Results for Modified probability update for multi-parameter CABAC update (relative to HM-5.0).

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.9% | -0.9% | -0.8% | -0.9% | -0.9% | -0.9% |
| Class B | -0.8% | -0.8% | -0.7% | -0.8% | -0.9% | -0.9% |
| Class C | -0.8% | -0.8% | -0.7% | -0.7% | -0.8% | -0.8% |
| Class D | -0.8% | -0.5% | -0.5% | -0.8% | -0.8% | -0.8% |
| Class E | -0.7% | -0.8% | -0.6% | -0.8% | -0.9% | -0.9% |
| Overall | -0.8% | -0.7% | -0.6% | -0.8% | -0.9% | -0.9% |
|  | -0.8% | -0.7% | -0.6% | -0.8% | -0.8% | -0.9% |
| Class F | -0.6% | -0.6% | -0.3% | -0.6% | -0.4% | -0.6% |
| Enc Time[%] | 105% |  |  | 106% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

|  |  | Random Access HE |  |  |  |  | Random Access LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U |  | V |  | Y |  | U | V |
| Class A (8bit) |  | -0.9% | -0.6% |  | -0.9% |  | -0.9% |  | -1.0% | -1.2% |
| Class B |  | -0.7% | -0.6% |  | -0.5% |  | -0.7% |  | -0.9% | -1.0% |
| Class C |  | -0.6% | -0.6% |  | -0.4% |  | -0.6% |  | -1.0% | -1.0% |
| Class D |  | -0.5% | -0.4% |  | -0.3% |  | -0.6% |  | -0.7% | -0.9% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  | -0.7% | -0.6% |  | -0.5% |  | -0.7% |  | -0.9% | -1.0% |
|  |  | -0.7% | -0.6% |  | -0.4% |  | -0.7% |  | -0.9% | -1.0% |
| Class F |  | -0.5% | -0.4% |  | -0.3% |  | -0.6% |  | -0.5% | -0.7% |
| Enc Time[%] |  | 102% |  |  |  |  | 100% |  |  |  |
| Dec Time[%] |  | 101% |  |  |  |  | 100% |  |  |  |
|  |  | Low delay B HE |  |  |  |  | Low delay B LC |  |  |  |
|  |  | Y | U |  | V |  | Y |  | U | V |
| Class A |  |  |  |  |  |  |  |  |  |  |
| Class B |  | -0.7% | -0.1% |  | -0.2% |  | -0.8% |  | -0.9% | -0.5% |
| Class C |  | -0.5% | -0.2% |  | -0.2% |  | -0.6% |  | -0.5% | -0.7% |
| Class D |  | -0.4% | -0.4% |  | 0.3% |  | -0.4% |  | 0.2% | -0.2% |
| Class E |  | -0.3% | 0.3% |  | -0.3% |  | -0.6% |  | -0.4% | -0.3% |
| Overall |  | -0.5% | -0.1% |  | -0.1% |  | -0.6% |  | -0.4% | -0.4% |
|  |  | -0.5% | -0.2% |  | -0.1% |  | -0.6% |  | -0.4% | -0.4% |
| Class F |  | -0.5% | -0.5% |  | -0.6% |  | -0.5% |  | -0.8% | -0.5% |
| Enc Time[%] |  | 103% |  |  |  |  | 102% |  |  |  |
| Dec Time[%] |  | 102% |  |  |  |  | 102% |  |  |  |
|  | Random Access HE-10 |  |  |  |  |  |  |  |  |  |
|  | Y |  |  | U |  | V |  |  |  |  |
| Class A (8bit) | -0.9% |  |  | 0.6% |  | 0.4% |  |  |  |  |
| Class B | -0.7% |  |  | -0.6% |  | -0.5% |  |  |  |  |
| Class C |  |  |  |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.8% |  |  | -0.1% |  | -0.1% |  |  |  |  |
|  | -0.8% |  |  | 0.0% |  | -0.1% |  |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |  |
| Enc Time[%] | 103% |  |  |  |  |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  |  |  |  |  |

Note: class F results are not included in the averages

# Summary

The presented scheme enables removal of the uiLPS look-up table, for a saving of 288Kb. The modified updateLPS function removes one clamp operation from the uiLPS function and inserts this operation into the probability estimate update operation. Relative to JCTVC-G764, it is asserted that the result is minimal impact on coding efficiency.

# References

- A.Alshin, E.Alshina, “Multi-parameter probability up-date for CABAC”, JCTVC-F254, 6th JCT-VC Meeting, Torino, IT, July 2011.

- A.Alshin, E.Alshina, “CE1 (subset B): Multi-parameter probability up-date for CABAC”, JCTVC-G764, 7th JCT-VC meeting, Geneva, CH, November, 2011.

- C. Rosewarne, M. Maeda, “Modified probability update and table removal for multi-parameter CABAC update (F254)”, JCTVC-G413, 7th JCT-VC meeting, Geneva, CH, November, 2011.

# Patent rights declaration(s)

CiSRA may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).