| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0141-r2 |
| --- | --- |

| Title: | Non-CE5: Complexity Reduction for Transform Skipping Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Dong Yan Ye 9710 Scranton Road, Suite 250, San Diego, CA 92121, USA | Tel: Email: | +1 858 210 4809 jie.dong@interdigital.com yan.ye@interdigital.com |
| Source: | InterDigital Communication, LLC |  |  |

_____________________________

# Abstract

This contribution presents a complexity-reduced design for the transform skipping mode (TSM) that was proposed in the 7th JCT-VC meeting (JCTVC-G575) and studied in core experiment (CE) 5 (JCTVC-G1205). First, the operation logics for TSM_1 and TSM_2 are unified with 16-bit implementation. Second, the number of arithmetic operations used in quantization and dequantization is reduced. Third, look-up tables used for TSM to deal with half-bit right shift are saved. This design was integrated into HM-5.0_tsm, which is the software base for CE5, and its R-D performance was tested under the common test conditions specified in JCTVC-G1200. The BD-rate performance is 0.0% to 0.1% increase on average for Class A-E and 0.0% to 0.2% increase for Class F, compared with the baseline TSM.

# Introduction

In the current TSM design [1], four modes are used: both horizontal and vertical transforms enabled (TSM_0), only horizontal transform enabled (TSM_1), only vertical transform enabled (TSM_2), and neither horizontal nor vertical transform enabled (TSM_3). TSM_0 uses the same operations as the core transform design in HEVC [2]. This contribution is focused on the improvement for the other three modes.

The forward transforms for TSM_1, TSM_2, and TSM_3 are expressed as in (1), (2), and (3), respectively.

TSM_1:		 Y = ( X × TNT + f ) >> B					(1)

TSM_2:		 Y = ( TN × X + f ) >> B					(2)

TSM_3:		 Y = X							(3)

X: input residual block, TN : N×N transform matrix, B: bits for right shift, f: rounding offset

The quantization is expressed by (4).

C = ( Y × S + f ) >> Q                                                               (4)

S: quant. scaling factor, Q: bits for right shift

The dequantization is expressed by (5).

Y = ( C × IS + f ) >> IQ                                                              (5)

IS: dequant. scaling factor, IQ: bits for right shift

The inverse transforms for TSM_1, TSM_2, and TSM_3 are expressed as in (6), (7), and (8), respectively.

TSM_1:		 X = ( Y × TN + f ) >> IB					(6)

TSM_2:		 X = ( TNT × Y + f ) >> IB				(7)

TSM_3:		 X = Y							(8)

TN : N×N transform matrix, IB: bits for right shift

B, Q, IQ, and IB for different TSM modes reuse those in core transform desing, as shown in TABLE 1, and by using the B, Q, IQ, and IB defined TABLE 1, the dynamic ranges after each stage are shown in TABLE 2. Note that ΔBD is equal to bit-depth minus 8 (e.g., input 10-bit video has ΔBD equal to 2) and the pattern A/B/C/D represents the four cases for 4×4, 8×8, 16×16, and 32×32 transforms, respectively.

TSM_1 and TSM_2 can be considered equivalent in terms of input dynamic range, output dynamic range, and intermediate dynamic range change. Using different right shift bits for TSM_1 and TSM_2 complicates the operation logic and results in inconsistent dynamic ranges for TSM_1 and TSM_2  (see the blue shaded entries in TABLE 2).

TABLE 1 B, Q, IQ, and IB used in the current TSM design

|  | TSM_1 | TSM_2 | TSM_3 |
| --- | --- | --- | --- |
| B | (1/2/3/4) + ΔBD | (8/9/10/11) |  |
| Q | (20/19.5/19/18.5) - ΔBD | (13/12.5/12/11.5) | 14 |
| IQ | (1/1.5/2/2.5) + ΔBD | (6/6.5/7/7.5) | 6 |
| IB | 12 - ΔBD | 7 |  |

TABLE 2 Dynamic ranges after stages in the current TSM design

|  | TSM_1 | TSM_2 | TSM_3 |
| --- | --- | --- | --- |
| Forward Trans. | 16 | 9 + ΔBD |  |
| Quant. | (10/10.5/11/11.5) + ΔBD | (10/10.5/11/11.5) + ΔBD | 9 + ΔBD |
| Dequant. | 15 | 10 + ΔBD | 9 + ΔBD |
| Inv. Trans. | 9 + ΔBD | 9 + ΔBD |  |

TABLE  Definition of bscale in the current TSM design

|  | TSM_1 & TSM_2 | TSM_3 |  |  |
| --- | --- | --- | --- | --- |
|  |  | N×N | 4×16 or 16×4 | 8×32 or 32×8 |
| bscale | 128/181/256/362 | 16384/32761/65536/131044 | 32768 | 65522 |

TABLE  Definition of bshift for quantization in the current TSM design

|  | TSM_1 | TSM_2 | TSM_3 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | N×N | 4×16 or 16×4 | 8×32 or 32×8 |
| bshift | 8/9/10/11 | 1/2/3/4 + ΔBD | 9/11/13/15 + ΔBD | 11 + ΔBD | 13 + ΔBD |

TABLE  Definition of bshift for dequantization in the current TSM design

|  | TSM_1 | TSM_2 | TSM_3 |
| --- | --- | --- | --- |
| bshift | 7 | 12 - ΔBD | 19 - ΔBD |

Furthermore, the quantization and dequantization in the current TSM implementation are not as simple as in (4) and (5). Instead, they are implemented as in (9) and (10), where QTSM0 and IQTSM0 are the Q and IQ for TMS_0, respectively, and bscale and bshift are defined in TABLE , TABLE  and TABLE , respectively. TABLE  to TABLE  can be replaced by logics (see Section 3 of [1]).

Quantization:		C = ( Y × ( ( S × bscale ) >> bshift ) + f ) >> QTSM0		(9)

Dequantization:  	Y = ( C × ( ( IS × bscale ) >> bshift ) + f ) >> IQTSM0		(10)

As (9) and (10) are used to approximate (4) and (5), the following conditions are satisfied. Note that using bscale and bshift to scale S and IS in (9) and (10) is performed once per block.

bscale / pow ( 2, bshift + QTSM0 )  ≈  1 / pow (2, Q)                                        (11)

bscale / pow ( 2, bshift + IQTSM0 )  ≈  1 / pow (2, IQ)                                       (12)

Although this method solves the problem of half-bit right shift in TABLE 1, it introduces a) two 2-D and one 1-D look-up tables, and b) one additional multiplication and one additional shift. Therefore, this method increases the computational complexity and causes additional precision loss.

# Proposed TSM Design

We propose the complexity-reduced TSM design to overcome the drawbacks introduced in Section 1. First, TSM_1 and TSM_2 share the same bits for right shift (see TABLE 3), and the intermediate dynamic ranges for TSM_1 and TSM_2 are 16 bits (see TABLE 4).

TABLE 3 Proposed B, Q, IQ, and IB

|  | TSM_1 & TSM_2 | TSM_3 |
| --- | --- | --- |
| B | (1/2/3/4) + ΔBD |  |
| Q | (20/19.5/19/18.5) - ΔBD | 14 |
| IQ | (0/0.5/1/1.5) + ΔBD | 6 |
| IB | 13 - ΔBD |  |

TABLE 4 Proposed Dynamic ranges

|  | TSM_1 & TSM_2 | TSM_3 |
| --- | --- | --- |
| Forward Trans. | 16 |  |
| Quant. | (10/10.5/11/11.5) + ΔBD | 9 + ΔBD |
| Dequant. | 16 | 9 + ΔBD |
| Inv. Trans. | 9 + ΔBD |  |

TABLE 5 S and IS defined in HEVC

| QP%6 | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| S | 26214 | 23302 | 20560 | 18396 | 16384 | 14564 |
| IS | 40 | 45 | 51 | 57 | 64 | 72 |

Second, unlike using (9) and (10) for approximation, the quantization and dequantization formulas in (4) and (5) are used without any scaling for S and IS. When Q or IQ is a half-bit value, the scaling factor S or IS, which is originally obtained by looking up TABLE 5 with the index QP%6, is obtained by using the index  (QP+3)%6. As the quantization stepsize increases about pow(2, 1/6) times with each increment of QP and exactly doubles for every 6 increments, replacing the original scaling factor with the one three entries away in TABLE 5 makes the new scaling factor pow(2, 1/2) times larger or smaller than the original one, and therefore accomplishes the 0.5-bit shift in the scaling operation (“×S” and “×IS”). The remaining integer-bit right shift can be accomplished by adjust Q or IQ accordingly, shown as below.

- For quantization

  - If ( QP%6 + 3 ) > 5, Q = Q + 0.5;

  - Otherwise, Q = Q - 0.5.

- For dequantization

  - If ( QP%6 + 3 ) > 5, IQ = IQ - 0.5;

  - Otherwise, IQ = IQ + 0.5.

By doing this, no additional arithmetic operations or look-up tables are introduced for TSM quantization and dequantization, and precision is better preserved.

# Experimental Results

The proposed TSM design is integrated into HM-5.0_tsm, and its R-D performance was tested under the common test conditions specified in [5]. All-intra settings were not tested in the test, because TSM is proposed only for inter coding and does not influence the coding efficiency for intra frames. The experimental results measured by BD-rate are reported in the attached spreadsheet “JCTVC-H0141.xls” and summarized in TABLE 6, TABLE 7, and TABLE 8.

TABLE 6 Performance measured by BD-rate on Random Access

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class C | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class F | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |

TABLE 7 Performance measured by BD-rate on Low Delay B

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class C | 0.1% | -0.2% | 0.1% | 0.0% | -0.1% | -0.1% |
| Class D | 0.1% | 0.1% | 0.2% | 0.0% | 0.1% | -0.2% |
| Class E | 0.0% | -0.6% | -0.5% | -0.1% | -0.7% | -0.1% |
| Overall | 0.1% | -0.1% | 0.0% | 0.0% | -0.2% | 0.0% |
|  | 0.1% | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% |
| Class F | 0.0% | 0.2% | 0.2% | 0.2% | 0.0% | 0.3% |

TABLE 8 Performance measured by BD-rate on Low Delay P

|  | Low delay P HE |  |  | Low delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | -0.1% | 0.0% | 0.1% | -0.1% |
| Class C | 0.1% | 0.0% | -0.1% | 0.0% | 0.1% | -0.1% |
| Class D | 0.2% | 0.2% | -0.4% | 0.0% | 0.5% | -0.5% |
| Class E | 0.0% | -0.3% | 0.1% | -0.1% | -0.9% | 0.2% |
| Overall | 0.1% | -0.1% | -0.2% | 0.0% | 0.0% | -0.1% |
|  | 0.1% | 0.0% | -0.2% | 0.0% | 0.0% | -0.1% |
| Class F | 0.2% | 0.3% | 0.5% | 0.1% | -0.8% | -0.5% |

# Conclusion

This contribution presents a complexity-reduced design for TSM. First, the operation logics for TSM_1 and TSM_2 are unified with 16-bit implementation. Second, the number of arithmetic operations used in quantization and dequantization is reduced. Third, three look-up tables used for TSM to deal with half-bit right shift are saved. This design was integrated into HM-5.0_tsm, and compared with the baseline TSM. The BD-rate performance is 0.0% to 0.1% increase on average for Class A-E and 0.0% to 0.2% increase for Class F.

# Reference

- M. Mrak, A. Gabriellini, D. Flynn (BBC), “Transform skip mode,” JCTVC-G575, Nov. 2011.

- A. Fuldseth, G. Bjøntegaard, M. Budagavi, “CE10: Core transform design for HEVC,” JCTVC-G495, Nov. 2011.

- M. Mrak, J. Sole, J. Xu, A. Saxena, “Description of core experiment 5 (CE5): Transform Skipping,” JCTVC-G1205, Nov. 2011.

- http://hevc.kw.bbc.co.uk/git/w/jctvc-hm/jctvc-hm-tsm.git/shortlog/refs/heads/HM-5.0_tsm

- F. Bossen, “Common HM test conditions and software reference configurations,” JCTVC-G1200, Nov. 2011.

# Patent rights declaration(s)

InterDigital Communications LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).