| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, California, 1- 10 February, 2012 | Document: JCTVC-H0145 |
| --- | --- |

| Title: | Non-CE11: Harmonisation of 8x8 TU residual scan |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Rosewarne 1 Thomas Holt Drive, North Ryde, NSW 2113, Australia. | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au |
|  | M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: | +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia (CiSRA), Canon Inc. |  |  |

_____________________________

# Abstract

This contribution proposes harmonizing the 8x8 TU residual scan with other TU sizes. A 4x4 sub-block scan is proposed for the 8x8 TU and significant coefficient groups are applied to the 8x8 TU.  Two variants are proposed, one with the upper level scan using the scan pattern specified by MDCS and the other uses the diagonal scan for the upper level.  When applying MDCS to the upper level, the results in luma for HE configurations for AI, RA and LB are 0.2%, 0.1% and 0.0% respectively.

# Introduction

The adopted straw man solution of JCTVC-G1038 harmonises processing 16x4, 4x16, 32x8 and 8x32 with 16x16 and 32x32 TUs. A non-overlapping 4x4 sub-block was extended to NSQT.  Furthermore, the 2-level significance map of JCTVC-G644 was adopted and extended to NSQT cases. This contribution extends the harmonization measure to 8x8 TUs, in order to reduce complexity for implementation of TU processing.

# Proposal

To harmonise 8x8 TU processing with other TU, this contribution proposes to extend the 4x4 sub-block scan to the 8x8 TU. No changes to context derivation are made.

## 4x4 sub-blocks with MDCS applied to both levels

The 8x8 TU is subdivided into four square 4x4 sub-blocks. The two-level significance map applied the TU.  Both the lower level and the upper level are scanned in the direction selected by MDCS.

The backward diagonal scan case is depicted in Figure 1 below.

Figure 1. 8x8 TU scan with 4x4 sub-blocks.

## 4x4 sub-blocks with upper level fixed to diagonal

This variant retains the selection of the lower level scan in accordance with MDCS, however the upper level scan is fixed at diagonal.

# Experiment Results

Results for 4x4 sub-blocks with MDCS applied to both levels presented in Table 1 below.  Results for 4x4 sub-blocks with upper level fixed to diagonal are presented in Table 2.

Refer to the accompanying spreadsheet for results with greater precision. Execution took place on a Linux cluster using GCC 3.4.6.

Table 1. Results for harmonizing 8x8 TU

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.2% | -0.1% | -0.1% | 0.2% | 0.1% | 0.0% |
| Class B | 0.3% | 0.0% | 0.1% | 0.2% | 0.0% | 0.0% |
| Class C | 0.3% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% |
| Class D | 0.2% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% |
| Class E | 0.6% | -0.1% | 0.0% | 0.4% | 0.1% | 0.1% |
| Overall | 0.3% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% |
|  | 0.3% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% |
| Class F | 0.2% | 0.1% | 0.0% | 0.1% | 0.2% | 0.1% |
| Enc Time[%] | 100% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.1% | 0.0% | -0.2% | 0.1% | 0.0% | 0.0% |
| Class B | 0.2% | -0.1% | 0.1% | 0.1% | 0.1% | 0.0% |
| Class C | 0.1% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.1% | -0.2% | 0.3% | 0.0% | 0.2% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.1% | -0.1% | 0.1% | 0.0% | 0.1% | 0.0% |
|  | 0.1% | -0.1% | 0.1% | 0.0% | 0.1% | 0.0% |
| Class F | 0.1% | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.2% | 0.1% | 0.0% | -0.4% | 0.3% |
| Class C | 0.1% | -0.2% | 0.4% | 0.0% | 0.2% | -0.2% |
| Class D | 0.1% | 0.1% | -0.1% | -0.1% | 0.4% | -0.6% |
| Class E | -0.1% | -0.3% | -1.0% | 0.1% | -1.6% | 0.2% |
| Overall | 0.1% | 0.0% | -0.1% | 0.0% | -0.3% | -0.1% |
|  | 0.1% | 0.0% | -0.2% | 0.0% | -0.2% | 0.0% |
| Class F | 0.0% | 0.0% | -0.4% | 0.1% | -0.8% | -0.2% |
| Enc Time[%] | 100% |  |  | 102% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  |

|  | Random Access HE-10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A (8bit) | 0.2% | -0.1% | 0.0% |
| Class B | 0.2% | -0.1% | 0.1% |
| Class C |  |  |  |
| Class D |  |  |  |
| Class E |  |  |  |
| Overall | 0.2% | -0.1% | 0.0% |
|  | 0.2% | 0.0% | 0.0% |
| Class F |  |  |  |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |

Table 2. Results for harmonizing 8x8 TU with upper level using diagonal scan only.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.3% | -0.1% | -0.1% | 0.2% | 0.1% | 0.0% |
| Class B | 0.4% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% |
| Class C | 0.4% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% |
| Class D | 0.3% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% |
| Class E | 0.6% | -0.2% | -0.1% | 0.5% | 0.1% | 0.1% |
| Overall | 0.4% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% |
|  | 0.4% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% |
| Class F | 0.3% | 0.0% | 0.0% | 0.1% | 0.2% | 0.0% |
| Enc Time[%] | 99% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.1% | 0.0% | -0.3% | 0.1% | 0.1% | -0.2% |
| Class B | 0.2% | 0.0% | 0.3% | 0.1% | 0.0% | 0.0% |
| Class C | 0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.1% | -0.2% | 0.1% | 0.0% | 0.3% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.1% | -0.1% | 0.1% | 0.0% | 0.1% | 0.0% |
|  | 0.1% | -0.1% | 0.1% | 0.0% | 0.1% | -0.1% |
| Class F | 0.1% | 0.2% | -0.1% | 0.1% | 0.0% | -0.1% |
| Enc Time[%] | 101% |  |  | 98% |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.2% | -0.1% | 0.0% | -0.3% | 0.3% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class D | 0.0% | -0.3% | -0.1% | -0.1% | 1.0% | 0.2% |
| Class E | 0.1% | -0.7% | -1.3% | 0.0% | -2.2% | -0.9% |
| Overall | 0.1% | -0.1% | -0.3% | 0.0% | -0.2% | 0.0% |
|  | 0.1% | -0.1% | -0.2% | 0.0% | -0.3% | 0.0% |
| Class F | 0.1% | 0.1% | -0.2% | -0.1% | -0.3% | -0.5% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

|  | Random Access HE-10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A (8bit) | 0.2% | -0.1% | 0.4% |
| Class B | 0.2% | -0.1% | 0.0% |
| Class C |  |  |  |
| Class D |  |  |  |
| Class E |  |  |  |
| Overall | 0.2% | -0.1% | 0.2% |
|  | 0.2% | -0.1% | 0.2% |
| Class F |  |  |  |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

# Working Draft text

## Changes for Sections 2.1 and 2.2.

### WD: 6.1.1	Up-right diagonal scanning array initialisation process

Input to this process is a block width blkWidth and a block height blkHeight.

Output of this process is the array DiagScan[ pos ][ comp ]. The array index pos specify the scan position ranging from 0 to ( blkWidthSize * blkHeightSize ) − 1. The array index comp equal to 0 specifies the horizontal component and the array index comp equal to 1 specifies the vertical component. Depending on the values of blkWidth and blkHeight, the array DiagScan is derived as follows.

–	If blkWidth is less than 16 and blkHeight is less than 16, the following applies.

i  =  0
x  =  0
y  =  0
stopLoop  =  false
while( !stopLoop ) {
	while( y  >=  0 ) {
		if ( x  <  blkWidth  &&  y  <  blkHeight ) {
			DiagScan[ i ][ 0 ]  =  x
			DiagScan[ i ][ 1 ]  =  y
			i++
		}
		y− −
		x++
	}
	y  =  x
	x  =  0
	if ( i  >=  blkWidth  *  blkHeight ) 
		stopLoop  =  true
}

–	Otherwise (blkWidth is greater than 8 or blkHeight is greater than 8), the following applies.

x_off  =  0
y_off  =  0
i_off  =  0
stopLoopSubblocks = false
while( !stopLoopSubblocks ) { 
	i  =  0
	x  =  0
	y  =  0
	stopLoop  =  false
	while( !stopLoop ) {
		while( y  >=  0 ) {
			if ( x  <  4  &&  y  <  4 ) {
				DiagScan [ i + i_off ][ 0 ]  =  x + x_off
				DiagScan [ i + i_off ][ 1 ]  =  y + y_off
				i++
			}
			y− −
			x++
		}
		y  =  x
		x  =  0
		if ( i  >=  16 ) 
			stopLoop  =  true
	}
	i_off  =  i_off + 16
	if ( i_off  >=  blkWidth * blkHeight ) {
		stopLoopSubblocks  =  true
	} else{
		do {
			y_off  =  y_off − 4
			x_off  =  x_off + 4
			if ( y_off  <  0 ) {
				y_off  =  x_off
 				x_off  =  0
			}
		} while( !( x_off  <  blkWidth  &&  y_off  <  blkHeight ) ) 
	}
}

### 7.1.1	Residual coding syntax

| residual_coding ( x0, y0, log2TrafoWidth, log2TrafoHeight, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| last_significant_coeff_x_prefix | ae(v) |
| last_significant_coeff_y_prefix | ae(v) |
| if ( last_significant_coeff_x_prefix > 3 ) |  |
| last_significant_coeff_x_suffix | ae(v) |
| if ( last_significant_coeff_y_prefix > 3 ) |  |
| last_significant_coeff_y_suffix | ae(v) |
| numCoeff = 0 |  |
| do { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 1 ] |  |
| numCoeff++ |  |
| } while( ( xC != LastSignificantCoeffX ) ¦¦ ( yC != LastSignificantCoeffY ) ) |  |
| numLastSubset = (numCoeff − 1) >> 4 |  |
| for( i = numLastSubset; i >= 0; i− − ) { |  |
| offset = i << 4 |  |
| if( max( log2TrafoWidth, log2TrafoHeight ) > 3) { |  |
| xCG = ScanOrder[ log2TrafoWidth – 2 ][ log2TrafoHeight – 2 ][ scanIdx ][ i ][ 0 ] |  |
| yCG = ScanOrder[ log2TrafoWidth – 2 ][ log2TrafoHeight – 2 ][ scanIdx ][ i ][ 1 ] |  |
| rightCGFlag = (xCG = = (1<< (log2TrafoWidth – 2)) − 1) ? 0 : significant_coeff_group_flag[ xCG + 1 ][ yCG ] |  |
| bottomCGFlag = (yCG = = (1 << (log2TrafoHeight – 2)) − 1) ? 0 : significant_coeff_group_flag[ xCG ][ yCG + 1 ] |  |
| if( (i < numLastSubset) && (rightCGFlag + bottomCGFlag < 2) && (i > 0) ) |  |
| significant_coeff_group_flag[ xCG ][ yCG ] | ae(v) |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if ( (n + offset) < (numCoeff − 1) && significant_coeff_group_flag[ xCG ][ yCG ] ) { |  |
| numNZInCG = (i = = numLastSubset) ? 1 : 0 |  |
| if ( (n > 0) ¦ ¦ (rightCGFlag + bottomCGFlag = = 2) ¦ ¦ (i = = 0) ¦ ¦ (numNZInCG > 0) ) { |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| numNZInCG += significant_coeff_flag[ xC ][ yC ] |  |
| Else |  |
| significant_coeff_flag[ xC ][ yC ] = 1 |  |
| } |  |
| } |  |
| } else { |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( (n + offset) < (numCoeff − 1) ) |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| } |  |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| if( coeff_abs_level_greater1_flag[ n ] ) |  |
| coeff_abs_level_greater2_flag[ n ] | ae(v) |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| if( coeff_abs_level_greater2_flag[ n ] ) |  |
| coeff_abs_level_minus3[ n ] | ae(v) |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( coeff_abs_level_minus3[ n ] + 3 ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| } else |  |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = 0 |  |
| } |  |
| } |  |
| } |  |

## Changes for Section 2.1 only

## WD: 6.2 Scanning order array initialisation process

Input to this process is a block widthsize blkWidth and a block height blkHeight.

Output of this process is the array ScanOrder[ scanIdx ][ pos ][ comp ]. The array index scanIdx equal to 0 specifies the up-right diagonal scan as specified in subclause  with blkWidth and blkHeight as inputs, scanIdx equal to 1 specifies a horizontal scan, and scanIdx equal to 2 specifies a vertical scan. The array index pos specifies the scan position ranging from 0 to ( blkWidth * blkHeight ) − 1. The array index comp equal to 0 specifies the horizontal component and the array index comp equal to 1 specifies the vertical component. The array ScanOrder is derived as follows.

ScanOrder[0] = DiagScan

i  =  0

y_off = 0

while( y_off < blkHeight ) {

x_off = 0

while( x_off < blkWidth ) {

y  =  0
		while( y  < blkHeight4 ) {
			x  =  0
			while( x  <  blkWidth4 ) {
					ScanOrder[ 1 ][ i ][ 0 ]  = x + x_off
					ScanOrder[ 1 ][ i ][ 1 ]  = y + y_off
					x++
					i++
			}
			y++
		}

x_off  += 4

}

y_off += 4

}

i  =  0

x_off = 0

while( x_off<blkWidth ) {

y_off = 0

while( y_off < blkHeight ) {
		x  =  0
		while( x  < blkWidth4 ) {
			y  =  0
			while( y  <  blkHeight4 ) {
				ScanOrder[ 2 ][ i ][ 0 ]  = x
				ScanOrder[ 2 ][ i ][ 1 ]  = y
				y++
				i++
			}
			x++
		}

y_off += 4

}

x_off  += 4

}

## Changes for Section 2.2 only

## WD: 6.2	Scanning order array initialisation process

x_off  =  0
y_off  =  0
i_off  =  0

stopLoopSubblocks = false
while( !stopLoopSubblocks ) { 
	i  =  0

y  =  0

while( y  < blkHeight4 ) {
		x  =  0
		while( x  <  blkWidth4 ) {
				ScanOrder[ 1 ][ i + i_off ][ 0 ]  = x + x_off
				ScanOrder[ 1 ][ i + i_off ][ 1 ]  = y + y_off
				x++
				i++
		}
		y++
	}

i = 0

x  =  0
	while( x  < blkWidth4 ) {
		y  =  0
		while( y  <  blkHeight4 ) {
			ScanOrder[ 2 ][ i + i_off ][ 0 ]  = x
			ScanOrder[ 2 ][ i + i_off ][ 1 ]  = y
			y++
			j++
		}
		x++
	}

i_off  =  i_off + 16

if ( i_off  >=  blkWidth * blkHeight ) {
		stopLoopSubblocks  =  true
	} else{
		do {
			y_off  =  y_off − 4
			x_off  =  x_off + 4
			if ( y_off  <  0 ) {
				y_off  =  x_off
 				x_off  =  0
			}
		} while( !( x_off  <  blkWidth  &&  y_off  <  blkHeight ) ) 
	}
}

# Summary

Two approaches to harmonization of 8x8 TU processing with other TU sizes has been presented.  We assert that these approaches are beneficial for implementation as they enable a 4x4 sub-block scan to be applied to all TU sizes.

# References

- J. Sole, “JCT-VC break-out report: Harmonization of NSQT with residual coding”, JCTVC-G1038_r1, 7th JCT-VC Meeting, Geneva, CH, 21-30 November, 2011.

- N. Nguyen et al, “Multi-level significance maps for Large Transform Units”, JCTVC-G644, 7th JCT-VC Meeting, Geneva, CH, 21-30 November, 2011.

# Patent rights declaration(s)

CiSRA may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).