| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0147 |
| --- | --- |

| Title: | CE10.1: Symmetric rounding in the deblocking filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Matthias Narroschke Anand Kotra Thomas Wedi Panasonic R&D Center Germany Monzastr. 4c 63225 Langen Germany | Tel: Email: | +49 6103 766-204 Matthias.Narroschke@eu.panasonic.com +49 6103 766-154 Anand.Kotra@eu.panasonic.com +49 6103 766-1194 Thomas.Wedi@eu.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

The current HM-5.0 deblocking filter generates an offset due to a non-symmetric rounding operation. Proposed is a symmetric rounding operation to eliminate this undesired offset. Compared to HM5.0, the following BD-bit rate reductions are achieved at similar complexity: I-HE: -0.1%, I-LC: -0.1%, RA-HE: -0.1%, RA-LC: 0.0%, RA-HE-10: 0.0%, LD-HE(B): -0.1%, LD-LC(B): -0.1%, LD-HE(P): 0.0%, LD-LC(P): 0.1%. Measured run-time ratios are all around 100%. It is proposed to adopt the symmetric rounding into the next version of the HM.

# Introduction

In deblocking filter of current HM-5.0, a delta value used for weak deblocking of the luminance signal is calculated as follows

Δ=9⋅q0i−p0i−3⋅q1i−p1i+8>>4

The add-and-shift operation can be interpreted as a quantization of Δk=9⋅q0i−p0i−3⋅q1i−p1i, see Figure 1. It can be seen from Figure 1, that the quantizer is not symmetric. Since the distribution of Δkis typically symmetric, the use of this non-symmetric quantizer results in EΔk≠EΔ.

Since EΔ=0for a symmetrically distributed Δk, this quantizer generates an offset between the signal before and after deblocking. This offset is undesired.

Figure 1: Illustration of the unsymmetric quantizer characteristic used in HM5.0 to quantize Δk.

# Symmetric rounding

In order to eliminate the undesired offset, a symmetric rounding is introduced. This is achieved by adding a value a=7 in the case of a positive Δk and a different value b=8 in the case of a negative Δk:

Δ=9⋅q0i−p0i−3⋅q1i−p1i+a>>4;Δk≥09⋅q0i−p0i−3⋅q1i−p1i+b>>4;Δk<0

It can be seen from Figure 2 that the proposed quantizer is symmetric.

Figure 2: Illustration of the proposed symmetric quantizer characteristic to quantize Δk.

The symmetric rounding requires an additional check whether the delta value is positive or negative.

However, such a check is already performed in the clipping operation, which is being executed directly subsequent to the rounding operation. Therefore, it is possible to keep the number of condition checks approximately constant by jointly implementing the rounding and the subsequent clipping-operation. The associated pseudo-code is shown in Figure 3.

Figure 3: Pseudo-code of HM5.0 and of the proposal.

For weak deblocking of the luminance signal, and for deblocking of the chrominance signal, there are different delta values being calculated, which use right shifts of 1 bit and 3 bits. The following values a and bare used in order to perform the symmetric rounding without generating offsets.

|  | Number of right shift bits | a | b |
| --- | --- | --- | --- |
| Luma weak filter for p0i and q0i | 4 | 7 | 8 |
| Luma weak filter for p1i and q1i | 1 | 0 | 1 |
| Chroma filter for p0i and q0i | 3 | 3 | 4 |

# Experimental Results

The proposed changes are introduced into HM5.0 [1]. Experiments were conducted according to the common conditions [2]. Figure 4 shows the measured BD-bit rate reductions as well as the measured run-time ratios.

Figure 4: Measured BD-bit rates and run time ratios.

# Required Changes in Working Draft

The working draft text modifications for symmetric rounding are as marked in yellow:

Clip4(int shift_value, int delta, int tc, int pv, int nv)

{

if (delta>0)

{

delta=((delta+pv)>>shift_value);

if(delta > tc)

delta =tc;

}

else

{

delta=((delta+nv)>>shift_value);

if(delta < -tc)

delta = -tc;

}

return delta;

}

##### Filtering process for a luma sample

Otherwise, the following weak filtering applies while nDp and nDq are set equal to 0:

 = ( 9 * ( q0 –  p0 ) – 3 * ( q1 – p1 ) + 8 ) >> 4		(8-1)

 = (9 * ( q0 –  p0 ) – 3 * ( q1 – p1 )                                                                                         (8-2)

  - When abs() is less than tc*10 tc*160, the following ordered steps apply:

  -  = Clip4(4,delta, tc, 7, 8)

    - The filtered sample values p0’ and q0’ are specified as follows:

p0’ = Clip1Y( p0 +  )		(8-3)

q0’ = Clip1Y( q0 -  )		(8-4)

    - If dEp1 is equal to 1, the filtered sample value pi’ is specified as follows:

p = Clip3( -(tc >> 1), tc >> 1, ( ( ( p2 + p0 + 1 ) >> 1 ) – p1 +  ) >>1 )	(8-5)

p = Clip4(1,  ( ( ( p2 + p0 + 1 ) >> 1 ) – p1 +  ) ,  tc >> 1, 0,1 )                                                   (8-6)

pi’ = Clip1Y( p1 + p )		(8-7)

    - If dEq1 is equal to 1, the filtered sample value qi’ is specified as follows:

q = Clip3( -(tc >> 1), tc >> 1, ( ( ( q2 + q0 + 1 ) >> 1 ) – q1 –  ) >>1 )	(8-8)

q = Clip4(1, ( ( ( q2 + q0 + 1 ) >> 1 ) – q1 –  )   tc >> 1,  0, 1 )

qi’ = Clip1Y( q1 + q )		(8-9)

    - nDp is set equal to dEp1+1 and nDq is set equal to dEq1+1.

Each of the filtered sample values, pi’ with i = 0..nDp-1, is substituted by the corresponding input sample value pi if all of the following conditions are true.

##### Filtering process for a chroma sample

[Ed: (WJ) no filtering when bS is equal or less than 2]

Inputs of this process are:

–	sample values, pi and qi with i = 0..1,

–	a variable bS specifying the boundary filtering strength.

–	a variable tC.

Output of this process is:

–	The filtered sample values, p0’ and q0’.

When the variable bS is greater than 2, the filtered sample values p0’ and q0’ are derived by

 = Clip3( -tC, tC, ( ( ( ( q0 – p0 ) << 2 ) + p1 – q1 + 4 ) >> 3 ) )	(8-10)

 = Clip4( 3,  ( ( ( ( q0 – p0 ) << 2 ) + p1 – q1 + 4 ) , tC, 3 , 4 )	(8-11)

p0’ = Clip1C( p0 +  )		(8-12)

q0’ = Clip1C( q0 -  )		(8-13)

The filtered sample value, p0’ is substituted by the corresponding input sample value p0 if all of the following conditions are true.

–	p0 is a sample of an I_PCM block.

–	pcm_loop_filter_disable_flag value is equal to 1.

Similary, the filtered sample value, q0’ is substituted by the corresponding input sample value q0 if all of the following conditions are true.

–	q0 is a sample of an I_PCM block.

–	pcm_loop_filter_disable_flag value is equal to 1.

[Ed. (WJ): for PCM case, deblocking filter applies first and the filtered pixels are restored. Rather than this, it’s better to skip the filtering itself for PCM samples since first filtering is actually not needed.]

# Conclusion

The current HM-5.0 deblocking filter generates an offset due to a non-symmetric rounding operation. Proposed is a symmetric rounding operation to eliminate this undesired offset. Compared to HM5.0, the following BD-bit rate reductions are achieved at similar complexity: I-HE: -0.1%, I-LC: -0.1%, RA-HE: -0.1%, RA-LC: 0.0%, RA-HE-10: 0.0%, LD-HE(B): -0.1%, LD-LC(B): -0.1%, LD-HE(P): 0.0%, LD-LC(P): 0.1%. Measured run-time ratios are all around 100%. It is proposed to adopt the symmetric rounding into the next version of the HM.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1]  HM-5.0 software, http://hevc.kw.bbc.co.uk/trac/browser/tags/HM-5.0

[2] F. Bossen, “Common test conditions and software reference configurations”, JCTVC-G1200, Geneva,       November, 2011.